3GPP TSG-RAN WG4 Meeting #86	R4-1802895

Athens, GR, 26th Feb. – 2nd March, 2018

Source:	NTT DOCOMO, INC.

Title:	Indoor anechoic chamber test method procedure for occupied bandwidth

Agenda item:	6.27.3.1.2

Document for:	Approval

- Introduction

This contribution presents occupied bandwidth test procedure for an indoor anechoic chamber test method for conformance testing

- Frequency error conformance requirement

For occupied bandwidth, the beam characteristics are not important. The requirement should however cover the fact that all transmitters are active and the system is operating at the maximum declared rated total radiated power. For the OTA requirement, the same occupied bandwidth value (i.e. /2= 0.5%) as for the conducted occupied bandwidth requirement for Rel-13 AAS BS is to be used. As the occupied bandwidth is a measure of the wanted signal its performance will be flat in space and can be completely characterized as a directional requirement or a TRP requirement. As performance of the wanted signal is generally characterized on the main beam as a directional requirement the same is done for occupied bandwidth. Occupied bandwidth is therefore specified as a directional requirement valid over the OTA coverage range.

- Indoor anechoic chamber test method for occupied bandwidth

The method measures the occupied bandwidth in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna. The measurement system setup is as depicted in figure 10.2.2.1-1.

The test consists of two stages: the calibration and the measurement. The test procedure is as follows.

# Figure 10.2.2.1-1: Indoor anechoic chamber measurement system setup for occupied bandwidth

# Figure 10.2.2.3-1: Indoor Anechoic Chamber calibration system setup for occupied bandwidth

Stage 1 - Calibration: [Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

1)	Connect the reference antenna and the receiving antenna to the measurement RF out port and RF in port of the network analyzer, respectively, as shown in figure 10.2.2.3-1.

2)	Install the reference antenna with its beam peak direction and the height of its phase centre aligned with the receiving antenna.

3)	Set the centre frequency of the network analyzer to the carrier centre frequency of the tested signal for occupied bandwidth measurement of AAS BS and measure LFEIRP, E→D , which is equivalent to 20log|S21| (dB) obtained by the network analyzer:

-	LFEIRP, E→D: Pathloss between E and D in figure 10.2.2.3-1.

4)	Measure the cable loss, LFEIRP, E→F between the reference antenna connector and the network analyzer connector:

-	LFEIRP, E→F: Cable loss between E and F in figure 10.2.2.3-1.

5)	Calculate the calibration value between A and D with the following formula:

-	LEIRP_cal, A→D = LFEIRP, E→D  + GREF_ANT_EIRP, A→F -LFEIRP, E→F.

-	LEIRP_cal, A→D:  Calibration value between  A and D in figure 10.2.2.3-1.

-	GREF_ANT_EIRP, A→F: Antenna gain of the reference antenna.

# Stage 2 - Measurement:

6)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

7)	Set the AAS BS to generate the tested beam with the beam peak direction intended to be the same as the testing direction.

8)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

9)	Set the AAS BS to transmit the test signal at the manufacturer's declared rated output power according to E-TM3.1 at 5MHz bandwidth configuration.

10)	Measure frequency error of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument) denoted in figure 2.

- Conclusion

This contribution presents:

- An option to combine occupied bandwidth and output power measurement

- Further investigation of the dynamic range of occupied bandwidth measurement equipment may be necessary.