3GPP TSG-RAN WG4 Meeting #86	R4-1803086
Athens, Greece, 26 Feb - 02 Mar 2018

Agenda item:	7.9.9.1.1

Source:	Intel Corporation

Title:	Updated Simulation results for PDCCH Performance for RLM

Document for:	Discussion

# Introduction

In RAN4-NRAH-1801, the simulation assumptions for PDCCH performance for RLM were agreed up on [1]. In this contribution we provide simulation results for PDCCH performance for In and Out of Sync hypothetical PDCCH BLER.

# Simulation Results

The simulation assumptions for PDCCH performance are listed in Appendix. The payload size for DCI Format 1-0 for 24 PRB is set as 56. Based on the DCI format 1-0 in TS 38.212, the time domain resource allocation bits are not included in the payload size. By considering the 4 bits for time domain resource allocation the payload size is 60 bits for 24PRB.

## Simulation Results for FR1

For FR1, the simulation results for Out of sync evaluation with AL=8 and BLER= 10% are captured in Table 1.

Table 1: PDCCH Performance results for Out of Sync for FR1

|  |  | AWGN | EPA; 3km/h | ETU; 3km/h | EPA; 42km/h | ETU; 42km/h |
| --- | --- | --- | --- | --- | --- | --- |
| SCS: 15KHz | 24PRB | -9.7 | -5.7 | -6.7 | -5.8 | -6.7 |
|  | 48PRB | -9.6 | -6.4 | -6.7 | -6.4 | -6.7 |
|  | 96PRB | -9.4 | -6.9 | -6.4 | -6.9 | -6.4 |
| SCS:30KHz | 24PRB | -9.7 | -6.6 | - | -6.7 | - |
|  | 48PRB | -9.6 | -7.3 | - | -7.3 | - |
|  | 96PRB | -9.4 | -7.3 | - | -7.4 | - |

For FR1 the PDCCH evaluation results for In Sync evaluation with AL=4 and BLER=2% are captured in Table 2.

Table 2: PDCCH Performance results for In Sync for FR1

|  |  | AWGN | EPA; 3km/h | ETU; 3km/h | EPA; 42km/h | ETU; 42km/h |
| --- | --- | --- | --- | --- | --- | --- |
| SCS:15KHz | 24PRB | -6.7 | -0.3 | -2.3 | -0.1 | -2.4 |
|  | 48PRB | -6.5 | -1.4 | -2.2 | -1.5 | -2.2 |
|  | 96PRB | -6.4 | -2.0 | -2.1 | -1.9 | -2.1 |
| SCS:30KHz | 24PRB | -6.8 | -1.7 | - | -1.6 | - |
|  | 48PRB | -6.5 | -2.5 | - | -2.5 | - |
|  | 96PRB | -6.4 | -2.3 | - | -2.3 | - |

Based on the results, we observe that SNR hysteresis is 3dB in AWGN out of sync and  in sync evaluation for 15KHz SCS and 30KHz SCS. In fading channels the SNR hysteresis is 4.3-5.5 dB between in-sync and out-of-sync.

Observation#1: In AWGN the SNR hysteresis is ~3dB between in-sync and out-of-sync across all BWs and SCS in FR1. In fading channels the SNR hysteresis is 4.3-5.5dB.

## Simulation Results for FR2

For FR2, the simulation results for Out of sync evaluation with AL=8 and BLER= 10% are captured in Table 4.

Table 3: PDCCH Performance for Out of Sync in FR2

|  |  | AWGN | TDL-C; 3km/h | TDL-C; 30km/h |
| --- | --- | --- | --- | --- |
| SCS:60KHz | 24PRB | -9.7 | -7.2 | -7.1 |
|  | 48PRB | -9.6 | -6.8 | -6.9 |
|  | 96PRB | -9.4 | -6.7 | -6.7 |
| SCS:120KHz | 24PRB | -9.7 | -5.6 | -7.1 |
|  | 48PRB | -9.6 | -6.5 | -6.4 |
|  | 96PRB | -9.4 | -6.4 | -6.3 |

For FR2, the simulation results for In sync evaluation with AL=4 and BLER= 2% are captured in Table 5.

Table 4: PDCCH Performance for In Sync in FR2

|  |  | AWGN | TDL-C; 3km/h | TDL-C; 30km/h |
| --- | --- | --- | --- | --- |
| SCS:60KHz | 24PRB | -6.7 | -2.3 | -2.5 |
|  | 48PRB | -6.5 | -1.9 | -1.8 |
|  | 96PRB | -6.4 | -2.1 | -2.1 |
| SCS:120KHz | 24PRB | -6.7 | -2.1 | -2.3 |
|  | 48PRB | -6.6 | -2.4 | -2.3 |
|  | 96PRB | -6.4 | -2.1 | -2.1 |

Based on the results for FR2, we observe that in AWGN the SNR hysteresis between out of sync and in sync is ~3dB across all BW and SCS. In fading channels the SNR hysteresis is 3.5-5.1 dB between in-sync and out-of-sync.

Observation#2: In FR2 the SNR hysteresis in AWGN is ~3dB between in-sync and out-of-sync evaluation across all BW and SCS. In fading channels the SNR hysteresis is 3.5-5.1dB.

# Conclusion

In this paper we have presented PDCCH simulation results for In and Out of sync evaluation. We recommend that RAN4 considers them in RLM requirements specification.

# References

- R4-1801068, “Simulation assumption for NR RLM”, Nokia, Nokia Shanghai Bell

# Appendix

Simulation assumptions for PDCCH simulations are listed in table below:

Table A-1: PDCCH Simulation Assumptions

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | FR1, 4 GHz | FR2, 30 GHz |
| DCI format / payload size | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs), 60 (96 PRBs); | Format 1-0 Number of bits: 56 (24 PRBs), 58 (48 PRBs), 60 (96 PRBs); |
| Number of control OFDM symbols | {2 symbols, 15 kHz, 24 PRB} {2 symbols, 15 kHz, 48 PRB} {1 symbol, 15 kHz, 96 PRB} {2 symbols, 30 kHz, 24 PRB} {1 symbol, 30 kHz, 48 PRB} {1 symbol, 30 kHz, 96 PRB} | {2 symbols, 60 kHz, 24 PRB} {2 symbols, 60 kHz, 48 PRB} {1 symbol, 60 kHz, 96 PRB} {2 symbols, 120 kHz, 24 PRB} {1 symbol, 120 kHz, 48 PRB} {1 symbol, 120 kHz, 96 PRB} |
| Bandwidth (MHz) |  |  |
| Sub-carrier spacing (kHz) |  |  |
| Aggregation level (CCE) | 4 and 8 | 4 and 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB | 0 dB |
| Precoder granularity in frequency domain | REG bundle size | REG bundle size |
| REG bundle size | 6 | 6 |
| CP length | Normal | Normal |
| Mapping from REG to CCE | Distributed | Distributed |
| SNR | [-12, -11, … 4, 5, 6] | [-12, -11, … 4, 5, 6] |
| Frequency error | 0.05 ppm | 0.05 ppm |
| Propagation condition | AWGN, EPA, ETU (only for 15kHz SCS) | AWGN, TDL-C (100ns RMS) |
| Antenna configuration | 2x2 Note | 2x2 Note |
| UE speed | 3km/h, 42km/h | 3km/h, 30km/h |
| Note: Companies are encouraged to clarify whether the 2Tx simulation results are based Tx diversity or not. |  |  |