3GPP TSG-RAN WG4 Meeting #86	R4-1803131

# Athens, Greece, February 26 – March 2, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5583 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of inter-RAT SFTD measurement requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-03-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 has decided that EN-DC capable UE shall be able to report SFTD between LTE PCell and NR inter-RAT neighbour cell for the case that no NR PSCell is configured. RAN4 has decided that one mechanism for inter-RAT SFTD measurement is based on that the UE conducts the measurement without associated measurement gaps. Corresponding core requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing a subclause 8.1.2.4.25 with core requirements for inter-RAT SFTD measurement, when such measurements are carried out without measurement gaps, and LTE duplex mode is FDD. Values are TBD until RAN4#86bis. Introducing a placeholder subclause 8.1.2.4.26 for corresponding requirements when LTE duplex mode is TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement on support of inter-RAT SFTD measurements without measurement gaps will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.4.25, 8.1.2.4.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ...36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<UNCHANGED SECTIONS OMITTED>

<FIRST CHANGE>

##### 8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements without measurement gaps

###### 8.1.2.4.25.1	Introduction

This clause contains requirements for a UE supporting E-UTRAN FDD-NR dual connectivity and is applicable in RRC_CONNECTED state and conditioned on that no NR PSCell is configured.

###### 8.1.2.4.25.2	SFTD Measurement requirements

The requirements on SFTD measurement defined in this section are applicable under the side condition SCH Ês/Iot ≥ -3 dB for the NR neighbour cell.

The UE shall be able to detect, identify and measure SFTD of the up to 3 strongest NR cell on the carrier frequency provided in the SFTD measurement configuration.

When configured to conduct the SFTD measurement without support of configured measurement gaps, the UE shall be capable of finding the NR cell regardless of its SSB position in the SMTC period. The SFTD measurement shall be conducted with sustained communication in the E-UTRA FDD PCell, however, the UE may be allowed to cause interruptions at the beginning and at the end of the SFTD measurement, as specified below, for reconfiguration of the radio receiver.

When no MCG DRX is used, the UE shall be capable of determining SFTD within a physical layer measurement period of Tmeasure_SFTD1 as follows;

- For NR cell in FR1, Tmeasure_SFTD1 = [TBD]

- For NR cell in FR2, Tmeasure_SFTD1 = [TBD]

- When other inter-RAT NR measurements

are configured, a longer physical layer measurement period can be expected.

When MCG DRX is used, the same Tmeasure_SFTD1 as for non-DRX applies, but the reporting delay depends on the DRX cycle length in use.

If PCell is changed without changing PCC while the UE is performing SFTD measurements, the UE shall still meet SFTD measurement and accuracy requirements for the new PCell. In this case the UE shall restart the SFTD measurement, and the total physical layer measurement period shall not exceed Tmeasure_SFTD2 as defined by the following expression:

Tmeasure_SFTD2 = (N+1)*(Tmeasure_SFTD1) + N*TPCell_change

where:

N is the number of times the E-UTRA PCell is changed over the measurement period (Tmeasure_SFTD2),

TPCell_change is the time necessary to change the PCell; it can be up to [25] ms.

The measurement accuracy for the SFTD measurement shall be as specified in the sub-clause 9.x.x.

<Editor’s note: requirements on interruptions with respect to UE RF capabilities are FFS>

8.1.2.4.25.3	SFTD Measurement reporting delay

This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

##### 8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements without measurement gaps

<Editor’s note: Placeholder for E-UTRAN TDD requirements>

<END OF FIRST CHANGE>

<UNCHANGED SECTIONS OMITTED>