| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (See R4-17121 19) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | 1) 60kHz subcarrier spacing for data channel in FR1 |  | Yes | UE does not support 60kHz of subcarrier spacing for data channel | Type 4 if needed | No Need | Applicable only to FR1 |  |  | RAN4 | TBD |  |
|  | 1-2 | Extended CP | 1) Extended CP | Only for UE that supports 60kNHz SCS | Yes | UE does not support extended CP | Type 4 if needed | No need | TBD |  |  | RAN4 | TBD |  |
|  | 1-3 | 64QAM modulation for FR2 PDSCH | 1) 64QAM modulation for FR2 PDSCH |  | Yes | 64QAM for PUSCH is not possible |  | No need | Applicable only to FR2 |  | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | RAN4 | Mandatory without capability |  |
|  | 1-4 | 64QAM for PUSCH | 1) 64QAM for PUSCH |  | Yes | 64QAM for PUSCH is not possible |  | No need | No need |  | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | RAN4 | Mandatory without capability |  |
|  | 1-5 | 256QAM for PDSCH | 1) 256QAM for PDSCH |  | Yes | 256QAM for PDSCH is not possible | Type4 for FR1 [Type1] if needed for FR2 | No need | Yes |  | For FR1, it can be revisited in the future whether the 256QAM is mandated in all UE types or categories For FR2, FFS on whether type 1 is sufficient considering the number of supporting MIMO layers. | RAN4 | Mandatory with capability for FR1 TBD for FR2 |  |
|  | 1-6 | 256QAM for PUSCH | 1) 256QAM for PUSCH |  | Yes | 256QAM for PUSCH is not possible | Type1 for FR1 [Type1] if needed for FR2 | No need | Yes |  | For FR1, RAN4 can further discuss to mandate 256QAM for PUSCH for FR1 in future release. For FR2, FFS on whether type 1 is sufficient considering the number of supporting MIMO layers. | RAN4 | Optional for FR1 TBD for FR2 |  |
|  | 1-7 | pi/2-BPSK for PUSCH | 1) pi/2-BPSK for PUSCH |  | Yes | pi/2-BPSK for PUSCH is not possible | Type 4 | No need | Yes |  | RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non pulse-shaped pi/2 BPSK for FR2. | RAN4 | Optional |  |
|  | 1-8 | pi/2-BPSK for PUCCH format 3/4 | 1) pi/2-BPSK for PUCCH format 3/4 |  | Yes | pi/2-BPSK for PUCCH format 3/4 is not possible | Type 4 | No need | Yes |  |  | RAN4 | Optional |  |
|  | 1-9 | BWP switching delay | 1) Support BWP switching delay specified in TS38.xxx, candidate values set: {type1, type2} |  | Yes | UE does not support BWP switching delay | Type 4 if needed | No need | TBD |  | Network cannot configure the shorter delay for certain UE type RAN4 may remove the capability signaling if RAN4 can agree single minimum delay for each scenario. | RAN4 | TBD |  |
|  | 1-10 | 1-symbol GP for 120kHz SCS in unpaired spectrum | 1) Slot formats with 1-symbol GP(s) for 120KHz SCS in unpaired spectrum in FR2 2) Slot formats with 1-symbol GP(s) for 60KHz SCS in unpaired spectrum in FR1 |  | Yes | UE does not support 1 symbol GP in unpaired spectrum | Type 4 if needed | Applicable only to TDD | Yes |  | RAN4 will further discuss whether it is feasible to support this feature or not in Rel-15. | RAN4 | TBD |  |
|  | 1-11 | Support of UL sharing from UE perspective | 1) Support of UL sharing from UE perspective |  | Yes | UE does not support UL sharing from UE perspective | TBD | No Need | Applicable only to FR1 |  |  | RAN4 | Optional |  |
|  | 1-12 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) ~0us 2) <20us | 1-11 | Yes | UE does not support UL subcarrier alignment between LTE and NR for EN-DC with LTE-NR coexistence in UL | Per band combination | No Need | Applicable only to FR1 |  |  | RAN4 | TBD |  |
|  | 1-13 | 7.5kHz UL raster shift | 1) 7.5kHz UL raster shift | [1-11] | Yes | UE does not support 7.5kHz UL raster shift |  |  |  |  |  | RAN4 | Mandatory in the SUL bands with uplink sharing either from UE perspective or from network perspective FFS: band n2, n5, n66 |  |
|  | 1-14 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL |  | Yes | UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL | TBD |  |  |  | Continue discuss in this meeting. | RAN4 | TBD |  |
|  | 1-15 | Supplemental uplink with different numerology | 1) Numerology other than that of associated DL |  | Yes | The UE will not be able to access or operate on a SUL carrier | TBD | Need Support not required for an FDD-only UE |  |  | Continue discuss in this meeting. | RAN4 | TBD |  |
|  | 1-16 | Supplemental uplink with dynamic switch | 1) DCI based selection of PUSCH carrier |  | Yes |  | TBD | Need Support not required for an FDD-only UE |  |  | Continue discuss in this meeting. | RAN4 | TBD |  |
| UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 |  | Yes | For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC For FR2, UE does not support some UE channel bandwidths | Type 1 | No Need | No Need |  | Overall EN-DC/NR CA capability framework will be discussed in RAN4 #86 | RAN2/4 | Optional |  |
|  | 2-2 | Mixed numerologies for CA | 1) SCS within the single carrier in the CA configuration |  | Yes | UE does not support mixed numerologies for CA | TBD | TBD | TBD |  | If a UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s), the UE shall support two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL with or without capability signaling | RAN4 | TBD |  |
|  | 2-3 | Non-contiguous intra-band CA frequency span | TBD |  | Yes | UE does not support non-contiguous intra-band CA frequency span | Option 1: Combine with CA BW class Option 2: Type 1 | No need | Applicable only to FR2 |  | FFS for whether applicable for downlink only or both downlink and uplink | RAN4 | TBD |  |
|  | 2-4 | Simultaneous reception and transmission for inter band CA or EN-DC (TDD-TDD or TDD-FDD) | 1) Simultaneous reception and transmission for inter CA or EN-DC (TDD-TDD or TDD-FDD) |  | Yes | UE does not support simultaneous reception and transmission for inter CA or EN-DC (TDD-TDD or TDD-FDD) | Per band combination | No Need | No Need |  | Whether to mandating this feature in certain band combinations will be FFS. | RAN4 | TBD |  |
|  | 2-5 | Asynchronous FDD-FDD intra-band LTE-NR DC | Asynchronous FDD-FDD intra-band LTE-NR DC |  | Yes | UE does not support asynchronous FDD-FDD intra-band LTE-NR DC | Per band combination | Applicable only to FDD | Applicable only to FR1 |  |  | RAN4 | Optional |  |
|  | 2-6 | Non-contiguous UL CP-OFDM |  |  |  |  |  |  |  |  | Postpone the non-continuous CP-OFDM for uplink feature to June 2018. | RAN4 |  |  |
|  | 2-7 | PA calibration gap | 1) Support of PA calibration gap |  | Yes | UE does not support PA calibration gap | Type4 if needed | No Need | Applicable only to FR2 |  | RAN4 will further discuss the UE capability in the future. We can revisit the need of calibration gap once RAN4 reach consensus. | RAN4 | TBD |  |
|  | 2-8 | UE power class | 1) Support of [non-default] UE power class |  | Yes | UE does not | Type 1 | No Need | No Need |  | FFS: Default power class(es) for FR2. ( different UE device types may have different default power class) FFS: Definition under inter-band NR CA and EN-DC RAN4 will further discuss the mandantory of PC2 for FR1 in certain region. | RAN4 | TBD |  |
| Baseband | 3-1 | Independent measurement gap configurations for FR1 and FR2 | 1) measurement gaps for FR1 and FR2 are configured independently |  | Yes | UE does not support independent gap configuration between FR1 and FR2 | Type 4 | No Need | No Need |  |  | RAN4 | Optional |  |
|  | 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | 1) Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement |  | Yes | UE does not support simultaneous reception of data and SS block with different numerologies | Type 4 | No Need | Applicable only to FR1 |  | FFS: inter-band CA case | RAN4 | Optional |  |
|  | 3-3 | [Number of UE TX ports] |  |  |  |  |  |  |  |  | No capability signalling for number of Rx/Tx ports. We can revisit the need of capability signalling of number of Tx ports when we discuss the number of MIMO layers in this week. | RAN4 |  |  |
| Removed features from RAN4 feature list | r-1 | Single switched UL transmission in EN-DC | 1) single switched UL transmission in EN-DC |  | Yes | UE does not support single switched UL | Type 4 | No Need | Applicable only to FR1 |  | For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5 of RP-1722833) is mandatory with capability signaling. (Decision was made in RAN#78, RP-172833) | RAN4 | Optional |  |
|  | r-2 | Dynamic transmission power sharing between NR and LTE in EN-DC | 1) dynamic transmission power sharing between NR and LTE in EN-DC |  | Yes | Support of single UL operation (operation A with Case 1) is mandatory | [Type 4] | No Need | Applicable only to FR1 |  | This feature is not mandatory in Rel.15. For UEs without this capability, the support of single UL operation (Operation A with Case 1 in Slide 5 of RP-1722833) is mandatory with capability signaling. (Decision was made in RAN#78, RP-172833). | RAN 1/4 | Optional |  |
|  | r-3 | DL MIMO layers (PDSCH MIMO layers) |  |  |  |  |  |  |  |  | Maximum number of MIMO layers shall be supported |  |  |  |
|  | r-4 | UL MIMO layers (PUSCH MIMO layers) |  |  |  |  |  |  |  |  | Maximum number of MIMO layers shall be supported |  |  |  |
|  | 3-3 | Number of UE RX ports |  |  |  |  | TBD |  |  |  | FR1: UE supports 2 or 4 RX chains [RP-172788]. FR2: Number of transceiver chains for FR2 is FFS FFS if separate capability signalling is needed or can be coupled with number of MIMO layers Necessity of this feature will be discussed in RAN4 #86 | RAN4 | TBD |  |