3GPP TSG-RAN WG4 Meeting #86	R4-1803641

Melbourne, Australia 16 April – 20 April 2018

Source:	Charter Communications

Title:	TP for Rel-15 4DL 36.715-00-00: Bandwidth combination set, REFSENS and insertion loss parameters for CA_4DL_48A-48A-48A-48A _1UL_BCS0

Agenda item:	6.1.2

Document for:	Approval

# 1. Introduction

This contribution provides a TP for TR 36.715-00-00 to include the requirements for the 4DL/1UL CA combination CA_4DL_48A-48A-48A-48A _1UL_BCS0 as defined in RP-172410 [1].

# 2. Text Proposal

##### ---Start of changes---

## 5.x	CA_4DL_48A-48A-48A-48A_1UL_BCS0

### 5.x.1	Channel bandwidths per operating band for CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48A-48A-48A-48A | - | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 80 | 0 |

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

### 5.x.2	Co-existence studies

For single UL the 2nd and 3rd harmonic analysis for B48, B48, B48 and B48  is already done in the 4DL/1UL TR 36.715-00-00.  As shown in the table 5.x.2-1. None of the products falls into the B48, B48, B48 or B48  BS/UE receiving bands. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.x.2-1 summarizes frequency ranges where harmonics occur due to Band 48 CA with 1 UL.

Table 5.x.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

. Table 5.x.2-1 shows there are no harmonic issues for this combination.

### 5.x.3	ΔTIB,c and ΔRIB,c values

For the UE which supports CA_4DL_48A-48A-48A-48A  the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 5.x.3-1 and 5.x.3-2 respectively assuming separate antenna architecture without HTF:

Table 5.x.3-1: ΔTIB,c

| Intra-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_4DL_48A-48A-48A-48A | 48 | 0 |

Table 5.x.3-2: ΔRIB,c

| Inra-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_4DL_48A-48A-48A-48A | 48 | 0 |

### The delta values are chosen by similarities to existing combination

### 5.x.4	MSD

##### No MSD needed

##### ---End of changes---

# Reference

[1]	RP-172410, “Revised WID: LTE Advanced intra-band CA Rel-15 for xDL/yUL including contiguous and non-contiguous spectrum”, RAN#78, Ericsson