3GPP TSG-RAN WG4 Meeting #86-bis	R4-1803667

Melbourne, Australia, 16 – 20 Apr, 2018

Source:	Mediatek Inc

Title:	TP for TR36.715-04-01: the support of CA_2A-66C-71A_1UL_BCS0

Agenda item:	6.4.2

Document for:	Approval

1.  Introduction

CA_2A-66C-71A_1UL_BCS0 was approved in 4DL/1UL basket WID in the RAN-P [1]. This paper is to provide TP for the completion of this combination.

2.  Reference

[1]      RP-180572 LTE Advanced inter-band CA Rel-15 for 4DL/1UL, RAN-P #79.

Annex: TP for TR36.715-04-01 (va.b.c)

[Unchanged portions skipped]

5.x	CA_2A-66C-71A _BCS0

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA operating Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE transmit / BS receive |  |  | UE receive / BS transmit |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-66-71 | 2 | 1850 MHz | – | 1980 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz |  |

Table 5.x.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-66C-71A | 2 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2	UE Co-existence studies

Table 5.x.2-1: Band 2 and Band 71 UL harmonics

|  |  |  |  |  | 3rd Harmonic |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 5790 | 5970 | 5550 | 5730 |
| 71 | 663 | 698 | 617 | 652 | 1851 | 1956 | 1989 | 2094 |

The uplink of Band 71 is 663 - 698 MHz for which third harmonic products fall within 1989 – 2094 MHz, while the downlink of Band 2 is 1930 - 1990 MHz.  Thus, it can be seen that there is potential for third harmonic interference from uplink in Band 71 to downlink in Band 2.  However, the potential is limited to edge channels since the overlap is only 1 MHz from 1989 – 1990 MHz.  Only an uplink allocation in the lowest edge of Band 71 paired with simultaneous downlink in the uppermost edge of Band 2 could be impacted.  Moreover, due the guard bands inherent in the definition of the transmission bandwidth configuration for LTE, there may not be any direct overlap at all.  However, there may still be some effect due to spectral spreading of the third harmonic even if direct overlap is averted.

Another potential mechanism for harmonic interference is mixing of the uplink in Band 2 with the third harmonic of the Rx LO.  In this case, the frequency range impacted is much larger.  The third harmonic of Rx extends from 1851 to 1956 MHz while the uplink in Band 2 ranges from 1850 to 1910 MHz.  Therefore, there is nearly complete overlap between these two ranges.  Depending upon the particular frequency assignments and allocation of PRB’s for uplink within Band 2 and downlink within Band 71, there may be interference from this mechanism.

5.x.3	∆TIB and ∆RIB values

the overlap between the third harmonic of the uplink in Band 71 and the downlink in Band 2 is small.  Therefore, the number of deployment scenarios that may be impacted are also small and may possibly be overcome by network scheduling or some other means.  Consequently, a harmonic trap filter is not assumed to be present and therefore not accounted for in the TIB and RIB values.  The combination of Band 2 and Band 66 are a high/high combination for which the TIB is 0.5 dB and RIB is 0.3 dB.  Band 71 is a low band combined by duplexing and therefore is specified with TIB of 0.3dB.

Table 5.x.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-66C-71A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |

# Table 5.x.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_2A-66C-71A | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0 |

5.x.4	REFSENS requirements

It is expected that MSD will be specified for the two types of harmonic interference described above.  MSD is captured by means of the refsens exception table for harmonics.

5.x.4.1	Band 2 MSD

For the case of 3rd harmonic of uplink in Band 71 interfering with downlink in Band 2, the harmonic interference level at Tx antenna port over the frequency range of the Band 2 receive channel was analyzed in its 3DL fallback modes.  The result can be reused.

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-66C-71A36 | 2 |  |  | -93.1 | -93.7 | -92.2 | -91.1 | FDD |
|  | 66 |  |  | -99.2 | -96.2 | -94.4 | -93.2 |  |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |
| CA_2A-66C-71A37 | 2 |  |  | -96.5 | -93.7 | -92.2 | -91.1 | FDD |
|  | 66 |  |  | -99.2 | -96.2 | -94.4 | -93.2 |  |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |
| NOTE 36: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 37: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-66C-71A | 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

5.x.4.2	Band 71 MSD

For the case of B71 Rx harmonic mixing with uplink in B2, the analysis also has been done in its 3DL fallback modes. The result can also be reused.

Table 7.3.1A-0a Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-66C-71A15,16 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 66 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | FDD |
|  | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-66C-71A | 2 |  |  | 25 | 50 | 50 | 50 | FDD |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"