3GPP TSG-RAN WG4 Meeting #86bis 	R4-1803767

Melbourne, Australia, 16th April  – 20th April, 2018

Source: 	ZTE

Title:	on EMC requirements for BS type 2-O

Agenda Item:	7.8.2

Document for:	Approval

Introduction

Core part for TS 38.113 has already been approved in December 2017. However, the EMC requirements for BS type 2-O has not been fully discussed, especially the radiated immunity test. In this contribution, we give some discussion on the applicability of all the EMC requirements for BS type 2-O.

Discussion

The applicability of BS type 2-O should be the same as that of BS type 1-O as the only difference is the frequency range which doesn’t apply any change in EMC requirements. So the applicability for BS type 2-O should be the same as chapter 7 of TS 38.113[1] as shown in table 1 and table 2 below.

Table 1 Emission requirements applicability

| Phenomenon | Application | Equipment test requirement |  | Reference subclause in the present document | Reference Standard |
| --- | --- | --- | --- | --- | --- |
|  |  | BS equipment | Ancillary equipment |  |  |
| Radiated emission | Enclosure | applicable for BS type 1-C and BS type 1-H (Note 1) | not applicable | 8.2.1 | ITU-R SM.329 [24] |
| Radiated emission | Enclosure of ancillary equipment | not applicable] | applicable | 8.2.2 | CISPR 32 [11] |
| Conducted emission | DC power input/output port | applicable | applicable | 8.3 | CISPR 32 [11], CISPR 16-1-1 [12] |
| Conducted emission | AC mains input/output port | applicable | applicable | 8.4 | CISPR 32 [11] |
| Conducted emission | Telecommunication port | applicable | applicable | 8.5 | CISPR 32 [11] |
| Harmonic current emissions | AC mains input port | applicable | applicable | 8.6 | IEC 61000-3-2 [13] or IEC 61000-3-12 [14] |
| Voltage fluctuations and flicker | AC mains input port | applicable | applicable | 8.7 | IEC 61000-3-3 [15] ] or IEC 61000-3-11 [16] |
| NOTE 1: Radiated emission requirements for BS type 1-O and BS type 2-O are described in subclause 8.2.1. |  |  |  |  |  |

Table 2 Immunity requirements applicability

| Phenomenon | Application | Equipment test requirement |  | Reference subclause in the present document | Reference standard |
| --- | --- | --- | --- | --- | --- |
|  |  | BS equipment | Ancillary equipment |  |  |
| RF electromagnetic field (80 – 6000 MHz) | Enclosure | applicable | applicable | 9.2 | IEC 61000-4-3 [18] |
| Electrostatic discharge | Enclosure | applicable | applicable | 9.3 | IEC 61000-4-2 [17] |
| Fast transients common mode | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.4 | IEC 61000-4-4 [19] |
| RF common mode 0.15 - 80 MHz | Signal, telecommunications and control ports, DC and AC power input ports | applicable | applicable | 9.5 | IEC 61000-4-6 [21] |
| Voltage dips and interruptions | AC mains power input ports | applicable | applicable | 9.6 | IEC 61000-4-11 [22] |
| Surges, common and differential mode | AC power input ports and telecommunications port | applicable | applicable | 9.7 | IEC 61000-4-5 [20] |

For emission tests in table 1, BS type 2-O share the same test configuration and test method for type 1-O BS, so there is no additional requirements for BS type 2-O compared to BS type 1-O.

Proposal 1: For BS type 2-O, there is no additional requirement for emission test compared to BS type 1-O.

For immunity tests in table 2, the only test need to be treated differently is the RF electromagnetic filed test (80--6000MHz) which is called RI test in following discussion. As the operating band of FR1 is below 6GHz which will fall within the test frequency range of the RI test (80MHz--6GHz), to avoid the overlapping, the transmitter and receiver exclusion band for BS type 1-O are set in subclause 4.4 in TS 38.113[1] respectively:

1, Transmitter exclusion band: There is no transmitter exclusion band.

2, Receiver exclusion band:

FUL_low  – ΔfOOB<f < FUL_high + ΔfOOB

ΔfOOB is defined in TS 38.104[2].

For the BS type 2-O whose operating band is far beyond 6GHz, as the exclusion band is considering the overlapping of the Uplink frequency range and the test frequency range, there should be no receiver exclusion band for BS type 2-O.

Proposal 2: For BS type 2-O, there is no receiver exclusion band.

Conclusion

In this contribution, we compared the requirements of BS type 2-O to requirements of BS type 1-O and the proposal are provided as:

Proposal 1: For BS type 2-O, there is no additional requirement for emission test compared to BS type 1-O.

Proposal 2: For BS type 2-O, there is no receiver exclusion band.

- References

- TS 38.113 v15.1.0

- TS 38.104 v15.1.0