3GPP TSG-RAN WG4 Meeting #86-Bis                                                            R4-1803811

Melbourne, Austrailia, 16-20 April, 2018

Source:	Samsung

Title:		TP for TR38.815 update

Agenda Item:	   7.1.1.3

Document for:	Approval

- Intoriduction

This paper is to update TR38.815 for new frequency range between 24.25-29.5 GHz from the draft TR v0.3.0 [1].

- Reference

[1] R4-1804001,	“Draft TR38.815 v0.3.0,” Samsung, 3GPP RAN4#86-Bis, Apr 2018

Text proposal for TR 38.815

<Start of TP>

6	Channel numbering and channel bandwidth

The NR-ARFCN that are applicable in the frequency range between 24.25-29.5 GHz are given in table 6-1.

Table 6-1: Applicable NR-ARFCN in the frequency range between 24.25-29.5 GHz

| NR Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 60 | 2054166 – <1> – 2104165 |
| n258 | 60 | 2016666 – <1> – 2070832 |

The synchronization raster in the frequency range between 24.25-29.5 GHz is given in Table 6-2. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 6-2 with the step size interval of 17.28MHz.

Table 6-2: Applicable SS raster entries in the frequency range between 24.25-29.5 GHz

| NR Operating Band | SS Block SCS | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- |
| n257 | 120 kHz | [24306 – <1> – 24476] |
|  | 240 kHz | [24308 – <2> – 24474] |
| n258 | 120 kHz | [24175 – <1> – 24361] |
|  | 240 kHz | [24176 – <2> – 24360] |

BS and UE channel bandwidth and Bandwidth combination sets have been defined as found in 3GPP TR38.817. They are captured to support at least 50 MHz as a minimum channel bandwidth and up to 400 MHz as a maximum channel bandwidth in FR2 by using the different data sub-carrier spacing (SCS) in Rel-15. In order to apply the same requirement for Band n257 and 258 to Table [X-X] in 3GPP TS 38.101-2 [x], Table 6-2 highlights in the extract of the table below.

Table 6-3: NR channel bandwidth in the frequency range between 24.25-29.5 GHz

| NR band |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band number | data SCS(kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n258 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

7       Common RF requirements for band n257 and n258

According to current study in RAN4 at least RF requirements summarized in Table 7-1 should be the band agnostic RF requirements for FR2 which are applicable for band n257 and n258 as well. Besides that, the RF requirements which are band specific but common between band n257 and n258 are gathered in this section.

Table 7-1: Summary on band agnostic of RF requirements for FR2

| BS TX side capture in TS 38.104 | BS RX side capture in TS 38.104 | UE TX side capture in TS 38.101-2 | UE RX side capture in TS 38.101-2 |
| --- | --- | --- | --- |
| 9.2 Radiated transmit power | 10.3 OTA reference sensitivity level | 6.3.1 Minimum output power | 7.4 Maximum input level |
| 9.3 OTA Base station output power | 10.5 OTA In-band selectivity and blocking | 6.4 Transmit signal quality |  |
| 9.4 OTA Output power dynamics | 10.7 OTA Receiver spurious emissions | 6.5.1 Occupied bandwidth |  |
| 9.5 OTA Transmit ON/OFF power | 10.9 OTA In-channel selectivity | 6.5.2.1 Spectrum emission mask:General |  |
| 9.6 OTA Transmitted signal quality |  |  |  |
| 9.7.2 OTA Occupied bandwidth |  |  |  |
| 9.7.5 OTA Transmitter spurious emissions |  |  |  |

The common RF requirements for Band n257 and n258 are listed in following sections with expected changes in TS 38.101-2 and 38.104 for UE and BS respectively.

7.1       UE specific

7.1.1      Transmitter characteristics

7.1.1.1     Adjacent Channel Leakage Ratio (ACLR)

For Band n257 and Band n258, and the general ACLR requirements is 17dB and more test parameters are shown in table 7.1.1.1-1:

Table 7.1.1.1-1: General ACLR requirements for Band n257 and Band n258

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258 | 17 dB | 17 dB | 17 dB | 17 dB |
| NR channel Measurement bandwidth | 47.52 MHz | 95.04 MHz | 190.08 MHz | 380.16 MHz |
| Adjacent channel centre frequency offset [MHz] | +50 / -50 | +100.0 / -100.0 | +200 / -200 | +400 / -400 |

7.1.2      Receiver characteristics

7.1.2.1     Adjacent Channel Selectivity (ACS)

For Band n257 and Band n258, the ACS requirements is 23dB and more parameter are shown in table 7.1.2.1-1, Table 7.1.2.1-2 and Table 7.1.2.1-3:

Table 7.1.2.1-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| ACS for band n257, n258 | dB | 23 | 23 | 23 | 23 |

Table 7.1.2.1-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB | REFSENS +35.5dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 | 100 / -100 | 200 / -200 | 400 / -400 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.3.1. NOTE 2: The REFSENS power level is specified in Table [XXXXXX]. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

Table 7.1.2.1-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258 | dBm | -46.5 | -46.5 | -46.5 | -46.5 |
| PInterferer | dBm | [-25] |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 | 100 / -100 | 200 / -200 | 400 / -400 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex X.X with one sided dynamic OCNG Pattern TDD as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

7.2	BS specific

7.2.1      Transmitter characteristics

7.2.1.1     Adjacent Channel Leakage Ratio (ACLR)

The BS OTA ACLR limit for Band n257 and n258 is defined based on TRP and specified ACLR1 in table 7.2.1.1-1. The OTA ACLR2 limit is not specified since the emission level at the ACLR2 range will be close to noise floor.

Table 7.2.1.1-1: Base Station OTA ACLR

| BS Channel bandwidth | BS adjacent channel centre frequency offset below the lower or above the upper Base Station RF bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| [50, 100, 200, 400, …] | BWChannel | NR of same BW | NR of same BW with SCS that provides largest transmission bandwidth configuration (BWConfig) | 28 (Note 1) |
| Note 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz |  |  |  |  |

Note: the BS OTA ACLR absolute limits are Band agnostic requirements in the frequency range between 24.25-29.5 GHz.

7.2.1.2     Spectrum emission mask

BS unwanted emissions for Band n257 and n258 shall not exceed the maximum levels specified in table 7.2.1.2-1 and table 7.2.1.2-2.

Table 7.2.1.2-1: SEM applicable for [PTx ≥ 35 dBm] in the frequency range 24.25 – 33.4 GHz

| Frequency offset from “edge of transmission” Δf | Limit | Measurement bandwidth |
| --- | --- | --- |
| 0 f < 10% of the total transmission bandwidth | -5 dBm | 1 MHz |
| 10% of the total transmission bandwidth f < OOB boundary | -13 dBm | 1 MHz |

Table 7.2.1.2-2: SEM applicable for [PTx < 35 dBm] in the frequency range 24.25 – 33.4 GHz

| Frequency offset from “edge of transmission” Δf | Limit | Measurement bandwidth |
| --- | --- | --- |
| 0 f < 10% of the total transmission bandwidth | [Max(PTx – 40 dB, -12 dBm)] | 1 MHz |
| 10% of the total transmission bandwidth f < OOB boundary | [Max(PTx – 48 dB, -20 dBm)] | 1 MHz |
| f ≥ OOB boundary | Spurious domain limits |  |

7.2.2      Receiver characteristics

7.2.2.1     Adjacent Channel Selectivity (ACS)

The BS OTA ACS for Band n257 and n258 is defined based on EIS and the OTA wanted and the interfering signal are specified in table 7.2.2.1-1 and table 7.2.2.1-2 for ACS.

Table 7.2.2.1-1: OTA ACS requirement in the frequency range 24.25 – 33.4 GHz

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] |
| --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6dB | EISREFSENS +[28.7 -SINR-IM] |

Table 7.2.2.1-2: OTA ACS interferer frequency offset in the frequency range 24.25 – 33.4 GHz

| BS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 | [24.29] | 50MHz DFT-S-OFDM NR signal 60kHz SCS |
| 100 | [24.31] | 50MHz DFT-S-OFDM NR signal 60kHz SCS |
| 200 | [24.29] | 50MHz DFT-S-OFDM NR signal 60kHz SCS |
| 400 | [24.31] | 50MHz DFT-S-OFDM NR signal 60kHz SCS |

8	NR band n257 specific issues

This section captures band specific RF requirements for n257.

8.1	UE specific

8.1.1	Transmitter characteristics

8.1.1.1	Transmitter power

8.1.1.1.1            Maximum output power

For FR2, a single power class is defined for handheld UEs, the power class value is defined as the minimum peak EIRP and is band dependent, and table 8.1.1.1.1-1 lists the value for n257.

Table 8.1.1.1.1-1: NR FR2 UE Power Class for n257

| NR band | Handheld Power Class Min Peak EIRP (dBm) |
| --- | --- |
| n257 | [21.2-25.2] |
| NOTE 1: minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power requirements on upper limit TRP and max allowed EIRP are defined as table 8.1.1.1.1-2 for n257.

Table 8.1.1.1.1-2: NR UE Maximum Output Power Limits for n257

| NR band | TRP Handheld (dBm) | EIRP (dBm) Handheld |
| --- | --- | --- |
| n257 | TBD | 43 |

8.1.1.1.2           Maximum output power with additional requirements

<The A-MPR will be added >

8.1.1.2	Additional spectrum emission mask

<The text will be added >

8.1.1.3	Spurious emission

8.1.1.3.1            Additional spurious emission

<The requirements limits will be added to protect EESS>

8.1.1.3.2            Spurious emission band UE co-existence

<The requirements limits will be added>

8.1.2	Receiver characteristics

8.1.2.1	 Reference sensitivity power level

The Reference sensitivity power level is specified as table 8.1.2.1-1 for n257.

Table 8.1.2.1-1: Reference sensitivity for n257

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | [-92.1 to -85.4 ] | [-89.1 to -82.4 ] |  | [-83.1 to -76.4 ] |

8.1.2.2	In-band blocking

In-band blocking reuqirements for n257 are specified as table 8.1.2.2-1.

Table 8.1.2.2-1: In band blocking requirements for n257

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257 | dBm | REFSENS + 42 | REFSENS + 42 | REFSENS + 42 | REFSENS + 42 |
| FIoffset | MHz | 100 / -100 NOTE 5 | 200 / -200 NOTE 5 | 400 / -400 NOTE 5 | 800 / -800 NOTE 5 |
| FInterferer | MHz | FDL_low – 75 to FDL_high + 75 | FDL_low – 150 to FDL_high + 150 | FDL_low – 300 to FDL_high + 300 | FDL_low – 600 to FDL_high + 600 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The REFSENS power level is specified in Table 8.1.2.1-1. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.X.X (QPSK, R=X/X) with one sided dynamic OCNG pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X NOTE 4: FIoffset is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FIoffset shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. |  |  |  |  |  |

8.2	BS specific

8.2.1	Radiated transmitter characteristics

<Text to be added>

8.2.2	Radiated receiver characteristic

<Text to be added>

9	NR band n258 specific issues

This section captures band specific RF requirement for n258.

9.1	UE specific

9.1.1	Transmitter characteristics

9.1.1.1	Transmitter power

9.1.1.1.1            Maximum output power

For FR2, a single power class is defined for handheld UEs, the power class value is defined as the minimum peak EIRP and is band dependent, and table 9.1.1.1.1-1 lists the value for n258.

Table 9.1.1.1.1-1: NR FR2 UE Power Class for n258

| NR band | Handheld Power Class Min Peak EIRP (dBm) |
| --- | --- |
| n258 | [21.2-25.2] |
| NOTE 1: minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power requirements on upper limit TRP and max allowed EIRP are defined as table 9.1.1.1.1-2 for n258.

Table 9.1.1.1.1-2: NR UE Maximum Output Power Limits for n258

| NR band | TRP Handheld (dBm) | EIRP (dBm) Handheld |
| --- | --- | --- |
| n258 | TBD | 43 |

9.1.1.1.2           Maximum output power with additional requirements

<The A-MPR will be added >

9.1.1.2	Additional spectrum emission mask

<The text will be added >

9.1.1.3	Spurious emission

9.1.1.3.1            Additional spurious emission

<The requirements limits will be added to protect EESS>

9.1.1.3.2            Spurious emission band UE co-existence

<The requirements limits will be added>

9.1.2	Receiver characteristics

9.1.2.1	Reference sensitivity power level

The Reference sensitivity power level is specified as table 9.1.2.1-1 for n258.

Table 9.1.2.1-1: Reference sensitivity for n258

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n258 | [-92.1 to -85.4 ] | [-89.1 to -82.4 ] |  | [-83.1 to -76.4 ] |

9.1.2.2	In-band blocking

In-band blocking reuqirements for n258 are specified as table 9.1.2.2-1.

Table 9.1.2.2-1: In band blocking requirements for n258

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n258 | dBm | REFSENS + 42 | REFSENS + 42 | REFSENS + 42 | REFSENS + 42 |
| FIoffset | MHz | 100 / -100 NOTE 5 | 200 / -200 NOTE 5 | 400 / -400 NOTE 5 | 800 / -800 NOTE 5 |
| FInterferer | MHz | FDL_low – 75 to FDL_high + 75 | FDL_low – 150 to FDL_high + 150 | FDL_low – 300 to FDL_high + 300 | FDL_low – 600 to FDL_high + 600 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.X.X with one sided dynamic OCNG Pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X. NOTE 2: The REFSENS power level is specified in Table 8.1.2.1-1. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.X.X (QPSK, R=X/X) with one sided dynamic OCNG pattern as described in Annex A.X.X.X and set-up according to Annex C.X.X NOTE 4: FIoffset is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FIoffset shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. |  |  |  |  |  |

9.2	BS specific

9.2.1	Radiated transmitter characteristics

### <Text to be added>

9.2.2	Radiated receiver characteristic

<Text to be added>

<End of TP>