3GPP TSG-RAN WG4 Meeting #86	R4-1804026

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia

Title: 					A-MPR for DC_(n)71B

Agenda Item: 			7.1.3.1.4

Document for:	Discussion

# 1	Introduction

This contribution presents A-MPR simulation results for DC_(n)71B contiguous intraband EN-DC configuration.

# 2	Discussion

## 2.1	Simulation assumptions

- PA calibration point: QPSK 100 RB DFT-s-OFDM signal with 0.5 dB MPR

- LTE LO leakage and IQ-Image = -25 dBc, NR LO leakage and IQ-Image = -28 dBc

- ALCR from 38.101-3, Table 6.5.3.2.4-1 (below)

- SEM from 38.101-3, Table 6.5.3.2.3.1-1 (according to our proposed correction [1]; below)

- NR general spurious limit from 38.101, Table 6.5.3.1-2

- LTE is having constant 20 dBm power and NR backoff is searched

Table 1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR | dBc | 30 |
| Measurement bandwidth |  | [0.95] ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

Table 2: Additional requirements

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

## 2.2	Results

Two different scenarios were simulated firstly a “contiguous case” where LTE and NR form a contiguous block and secondly a “non-contiguous” case where LTE and NR allocations start from outer edges of aggregated bandwidth. These two cases represent best and worst case scenarios from A-MPR point of view.

In both contiguous and non-contiguous case LTE is with 20 dBm power level and NR backoff is reported.

### 2.2.1	Edge case

In this study LTE and NR allocations are on the outer edges leaving a gap between until gap is filled as all combinations have been simulated.

Figure 1: LTE 10MHz + NR 10MHz, 15kHz, QPSK results

Figure 2: LTE 15MHz + NR 5MHz, 15kHz, QPSK results

Figure 3: LTE 5MHz + NR 15MHz, 15kHz, QPSK results

In case NR modem do not know the LTE allocation it has to assume worst case backoff which can be read from Y-axis. If NR modem knows the LTE allocation then it can use the correct backoff reported in the plots.

In all cases the required NR backoff is large or very large. It seems that strategy of keeping LTE at 20 dBm power has consequences for NR power as below the LTE channel, third-order emission power within some measurement bandwidth could be roughly approximated as P0_dBm + 2*P_LTE_dBm + P_NR_dBm, thus 1 dB decrease of LTE power would reduce third-order emissions as much as a 2 dB reduction in NR power. This means that if LTE and NR power are reduced same amount NR backoff would be smaller but this required that LTE and NR modems know each other’s allocations.

### 2.2.2	Contiguous case

In this study LTE and NR allocations are on the inner edges of the signal forming contiguous block apart from the gap between LTE and NR. All combinations have been simulated.

Figure 4: LTE 10MHz + NR 10MHz, 15kHz, QPSK results

Figure 5: LTE 15MHz + NR 5MHz, 15kHz, QPSK results

Figure 6: LTE 5MHz + NR 15MHz, 15kHz, QPSK results

In

Similarly as in non-contiguous case if NR modem do not know the LTE allocation it has to assume worst case backoff which can be read from Y-axis. If NR modem knows the LTE allocation then it can use the correct backoff reported in the plots.

For this contiguous allocation case there are many cases that NR does not need any backoff and for the cases that it does we have to compare the backoff for MPR as it is almost never 0 dB.

# 3	Conclusion

In this contribution we have presented NR backoff simulation results for DC_(n)71B when LTE has constant 20 dBm output power.

# 4	Reference

[1] R4-1802527, Correction to Additional spectrum emissions mask DC_(n)71B,	Nokia, RAN4#86

# Appendix

All simulation results for contiguous allocations  (both carriers have an allocation at the inner channel edge):

All simulation results for noncontiguous allocations (both carriers have an allocation at the outer channel edge):