3GPP TSG-RAN WG4 Meeting #86bis	R4-1804034

Melbourne, Australia, 16th – 20th Apr 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TS 36.307 addition of # of CC information

Agenda Item: 			4.2.6

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting CR’s for release independence is formation of uplink CA of Class C+C was agreed [1]. These CR’s introduced a new column for number of CCs but the information was left blank for other CA features than uplink interband CA for 2-bands. This contribution discusses how to define this pending information.

# 2	Discussion

We made a study what kind of CA bandwidth class combinations now exists in TS 36.101 and how many CC those contain in total per CA configuration, results are presented in Table 1-3. We can note that until now CA configuration of up to 5 CCs have been standardized and seems that all possible CA BW Class combinations have been standardized. It was anticipated that no more than 5 CCs are aggregated as RAN4 is just now starting to define configurations of more than 5 CCs. Hence we propose to include current number of CCs information to 36.307 to reflect the situation. We have prepared CRs to this meeting to do so [2]-[7].

Proposal 1: Number of aggregated CC is captured into release independence specifications as proposed in [2]-[7]

Once first more than 5 CC combinations are agreed then the release independence information for those should discussed and agreed, which may or may not lead to different conclusion for release applicability than 5 CC aggregation.

Table 1: Interband CA (without non-contiguous intraband CA)

| # of bands | Bands |  | # of CCs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | A | A | 2 |  |  |  |
|  | A | B | 3 |  |  |  |
|  | A | C | 3 |  |  |  |
|  | A | D | 4 |  |  |  |
|  | A | E | 5 |  |  |  |
|  | B | B | 4 |  |  |  |
|  | B | C | 4 |  |  |  |
|  | C | C | 4 |  |  |  |
|  | B | D | 5 |  |  |  |
|  | C | D | 5 |  |  |  |
| # of bands | Bands |  |  | # of CCs |  |  |
| 3 | A | A | A | 3 |  |  |
|  | A | A | B | 4 |  |  |
|  | A | A | C | 4 |  |  |
|  | A | B | C | 5 |  |  |
|  | A | A | D | 5 |  |  |
|  | A | C | C | 5 |  |  |
| # of bands | Bands |  |  |  | # of CCs |  |
| 4 | A | A | A | A | 4 |  |
|  | A | A | A | B | 5 |  |
|  | A | A | A | C | 5 |  |
| # of bands | Bands |  |  |  |  | # of CCs |
| 5 | A | A | A | A | A | 5 |

Table 2: Non-contiguous intraband CA

| # of bands | Bands |  | # of CCs |  |
| --- | --- | --- | --- | --- |
| 2 | A | A | 2 |  |
|  | A | B | 3 |  |
|  | A | C | 3 |  |
|  | A | D | 4 |  |
|  | A | E | 5 |  |
|  | C | C | 4 |  |
|  | C | D | 5 |  |
| # of bands | Bands |  |  | # of CCs |
| 3 | A | A | A | 3 |
|  | A | A | C | 4 |

Table 3: Combinations of interband CA and non-contiguous intraband CA

| # of bands | Bands |  | # of CCs |  |  |
| --- | --- | --- | --- | --- | --- |
| 2 | A | A-A | 3 |  |  |
|  | A | A-C | 4 |  |  |
|  | A | A-D | 5 |  |  |
|  | A | C-C | 5 |  |  |
|  | A | A-A-A | 4 |  |  |
|  | B | A-A | 4 |  |  |
|  | B | A-B | 5 |  |  |
|  | B | A-C | 5 |  |  |
|  | C | A-B | 5 |  |  |
|  | C | A-C | 5 |  |  |
|  | C | A-A | 4 |  |  |
|  | A-A | B | 4 |  |  |
|  | A-A | C | 4 |  |  |
|  | A-A | D | 5 |  |  |
|  | A-A | A-A | 4 |  |  |
|  | A-A | A-B | 5 |  |  |
|  | A-A | A-C | 5 |  |  |
| # of bands | Bands |  |  | # of CCs |  |
| 3 | A | A | A-A | 4 |  |
|  | A | A | A-B | 5 |  |
|  | A | A | A-C | 5 |  |
|  | A | B | A-A | 5 |  |
|  | A | C | A-A | 5 |  |
|  | A | A | A-A-A | 5 |  |
|  | A | A-A | A-A | 5 |  |
| # of bands | Bands |  |  |  | # of CCs |
| 4 | A | A | A | A-A | 5 |

# 3	Conclusion

In this contribution we have made two proposals.

Proposal 1: Number of aggregated CC is captured into release independence specifications as proposed in [2]-[7]

# 4	References

[1] R4-1803349, Introduction of 4UL CA into TS36.307, KDDI Corporation

[2] R4-1804035	TS 36.307 Rel-10

[3] R4-1804036	TS 36.307 Rel-11

[4] R4-1804037	TS 36.307 Rel-12

[5] R4-1804038	TS 36.307 Rel-13

[6] R4-1804039	TS 36.307 Rel-14

[7] R4-1804050	TS 36.307 Rel-15