3GPP TSG-RAN WG4 Meeting #86bits	R4-1804116

# Melbourne, Australia  April 16 – April 20, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on the interruption and delay requirements for V2X CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  |  | Date: |  | 2018-04-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The delay and interruption requirements for V2X CA are not unclairfied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the delay and interruption requirements for V2X CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The delay and interruption requirements for V2X CA will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Sections 13.7.2, 13.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

### 13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change

This sub-clause contains the requirements related to the interruptions on the V2X sidelink communication due to synchronization source change.

UE is allowed to drop V2V sidelink signal transmission or reception for up to 1 subframe when synchronization reference source is changed:

-	from GNSS

-	to Serving cell/PCell;

-	to SyncRef UE that is not synchronized to GNSS directly or in-directly;

-	from SyncRef UE that is synchronized to GNSS directly or in-directly

-	to Serving cell/PCell;

-	to SyncRef UE that is not synchronized to GNSS directly or in-directly;

-	from Serving cell/PCell

-	to GNSS

-	to SyncRef UE that is synchronized to GNSS directly or in-directly;

-	from SyncRef UE that is not synchronized to GNSS directly or in-directly

-	to GNSS;

-	to SyncRef UE that is synchronized to GNSS directly or in-directly;

UE is allowed to interruption any V2V sidelink signals including PSSCH, PSCCH, PSBCH and SLSS signals.

### 13.7.3	Interruption for V2X Sidelink Carrier Aggregation

When a component carrier is added or released for V2X carrier aggregation with sidelink transmission mode 3 using RRCConnectionReconfiguration that includes the V2X sidelink communication configuration message [2], the UE is allowed an interruption of up to 2 subframes on the cellular communication during the RRC reconfiguration procedure.

When a component carrier is added or released for V2X carrier aggregation with sidelink transmission mode 3 using RRCConnectionReconfiguration that includes the V2X sidelink communication configuration message [2], the UE is allowed an allowed to drop V2X sidelink transmission or reception for up to 2 subframes during the RRC reconfiguration procedure.

## 13.8	Reliability of GNSS signal

This clause contains requirements regarding reliability of GNSS signal for the UE capable of  V2X sidelink communication under the following additional condition:

-	The UE is configured or pre-configured with parameters for enabling the UE to acquire the GNSS synchronization.

If UE considers GNSS is a reliable synchronization reference, the UE shall meet timing accuracy requirement as specified in 12.2 and frequency accuracy requirement as specified in 6.5.1G of TS36.101. Otherwise, the UE shall be capable to select another synchronization reference source.

GNSS shall be considered reliable when the following side condition is fulfilled:

-	GNSS related side conditions given in Sections B.6.1.

< END OF CHANGE #1 >

< START OF CHANGE #2 >

## 13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation

The delay on component carrier addition/release for sidelink carrier aggregation in transmission mode 3 can be defined as the time period from the end of DL subframe with RRC configuration message until the moment when UE is ready for to perform V2X RX or TX transmission. The delay requirements are specified as follows:

- The delay time for single component carrier addition/release is up to [21] ms.

- The delay time for multiple component carrier addition/release is up to [20+N] ms, where N is the number of component carrier added/released.

< END OF CHANGE #2 >