3GPP TGS-RAN WG4 Meeting #86bis	R4-1804125
Melbourne, Australia, April 16 – April 20, 2018

Agenda item:	7.4.11.1

Source:	Intel Corporation, Apple Inc.

Title:	FR2 peak EIS requirement for handheld UE

Document for:	Approval

# 1	Introduction

The FR2 REFSENS requirement definition has been discussed recently [1-3]. Agreement on a single value for peak EIS has not been found yet, but a range was approved for both 28 and 39 GHz frequencies [3], with plans is to discuss and narrow it down in upcoming meetings. However, no progress was made during RAN4 AH-1801 and RAN4 #86.

This paper summarizes the latest agreements for FR2 REFSENS and details our proposed data-driven approach to finalize the minimum beam peak EIS requirement for handheld UE.

# 2	Discussion

## 2.1	Background

In RAN4 #84 [1], equation agreements were made to derive peak EIS level. As in power class, options were presented on how to define the requirement. To this end, companies presented their peak EIS values during RAN4 #84Bis [2], but no agreement was reached and companies requested time to discuss the numbers.

Agreements from RAN4 #84 [1]:

- Derive the REFSENS requirement based on alignment on peak EIS level equation and options listed in slide #6

  - Sensitivity = -174dBm(kT) + 10*log(Max. RX BW) + NF – Total Ant. gain – diversity gain + SNR + ILs

    - Total ant. gain = element gain + beamforming gain

    - ILs: Implementation Losses

    - Max. RX BW: Max. Received Bandwidth

- Detail REFSENS requirements will be discussed and decided

  - Option 1: Defined as beam peak EIS or

  - Option 2: Defined as a CDF EIS

    - [A %] EIS CDF: The percentile and # of points of CDF curve will be FFS.

  - Option 3 : Defined as Beam Peak and CDF EIS combinations

- Derive the REFSENS requirement based on alignment on peak EIS level equation and options listed in slide #6

  - Sensitivity = -174dBm(kT) + 10*log(Max. RX BW) + NF – Total Ant. gain – diversity gain + SNR + ILs

    - Total ant. gain = element gain + beamforming gain

    - ILs: Implementation Losses

    - Max. RX BW: Max. Received Bandwidth

- Detail REFSENS requirements will be discussed and decided

  - Option 1: Defined as beam peak EIS or

  - Option 2: Defined as a CDF EIS

    - [A %] EIS CDF: The percentile and # of points of CDF curve will be FFS.

  - Option 3 : Defined as Beam Peak and CDF EIS combinations

Meeting report for RAN4 #84Bis [2]:

- Chair asked if an average of the reported values -94. 2 could be taken as possible agreement for 50 MHz channel bandwidth for n257 and n258

  - Companies expressed needing time to discuss this number

- Chair asked if an average of the reported values -94. 2 could be taken as possible agreement for 50 MHz channel bandwidth for n257 and n258

  - Companies expressed needing time to discuss this number

Several companies presented their contributions for beam peak EIS during November’s RAN4 #85 meeting. Due to the significant variation in implementation losses, the peak EIS requirement could not be finalized. However, a range of values both at 28 and 39 GHz was approved in the way forward [3].

Agreements from RAN4 #85 [3]:

- For 28 GHz, the beam peak EIS range is [-92.1 to -85.4] dBm

- For 39 GHz, the beam peak EIS range is [-91.8 to -82.6] dBm

- Companies are encouraged to provide additional analysis to finalize beam peak EIS definition for both bands

- For 28 GHz, the beam peak EIS range is [-92.1 to -85.4] dBm

- For 39 GHz, the beam peak EIS range is [-91.8 to -82.6] dBm

- Companies are encouraged to provide additional analysis to finalize beam peak EIS definition for both bands

Since then, few contributions have been submitted to the last two meetings [4-7], and no progress has been made.

## 2.2	Analysis of reported beam peak EIS

Implementation loss has proven to be a controversial parameter in both peak EIRP and EIS derivations. A deeper analysis on implementation losses that includes measurements at 39 GHz is presented in our peak EIRP paper [8]. It underlines the impact the form-factor integration has on the overall implementation losses. The measurements presented conclude that by taking some integration aspects into account (glass layer), the form-factor integration loss is 5 dB at 39 GHz. These measurements did not include all the losses and impairments of a fully integrated package; once accounted for, the form-factor integration loss is expected to increase.

Table 1: Proposed beam peak EIS at 28 GHz [3,9,10]

|  | Intel Glass 1 | Intel Glass 2 | Intel alt. Glass 1 | Intel alt. Glass 2 | LGE | Huawei | MediaTek | Qualcomm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kTB/Hz [dBm] | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| 10log(Rx BW) [dB] | 76.99 | 76.99 | 76.99 | 76.99 | 76.77 | 76.99 | 76.99 | 76.99 |
| Effective realized antenna array gain [dB] | 8 | 8 | 8 | 8 | 10 | 9 | 7 | 9.5 |
| Diversity Gain [dB] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| SNR [dB] | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| NF [dB] | 10 | 10 | 10 | 10 | 10 | 10 | 9 | 10 |
| Total implementation loss [dB] | 9.6 | 10.1 | 9.5 | 10 | 8.5 | 7 | 6.7 | 8.0 |
| Form factor de-sense (FFD) | 1 | 1 | 1 | 1 | - | - | - | - |
| Sensitivity EIS [dBm] | -85.41 | -84.91 | -85.51 | -85.01 | -89.73 | -90.01 | -89.31 | -89.51 |

NOTE: Rx BW = 50 MHz

A summary of the reported minimum beam peak EIS for 28 GHz is found in Table 1. The table also includes values for different glass materials and designs found in [9], and updates made to relevant parameters in [9] and [10]. With the latest updates provided, we see that the numbers align a bit better. We will now examine the data using a statistical analysis to narrow the ranges.

Observation 1: Given it is a full system spec, we should use a statistical approach to finalize the minimum peak EIS requirement. This will ensure a reasonable integration requirement for handheld devices.

To illustrate and help us better analyse the reported data, Figure 1 shows a distribution of the minimum beam peak EIS reported results at 28 GHz, along with the distribution numbers.

| % | Min beam peak EIS |
| --- | --- |
| 0.0 | -90.01 |
| 0.1 | -89.81 |
| 0.2 | -89.64 |
| 0.3 | -89.49 |
| 0.4 | -89.35 |
| 0.5 | -87.41 |
| 0.6 | -85.49 |
| 0.7 | -85.42 |
| 0.8 | -85.17 |
| 0.9 | -84.98 |
| 1.0 | -84.91 |

Figure 1: Distribution of reported minimum beam peak EIS at 28 GHz

For 28 GHz, the agreed peak EIS range is from -92.1 to -85.4 dBm [3]. Since this limit will be imposed as FTA and regulatory limits (in some countries), we need to make sure that the number we choose inside this range allows for a reasonable device passing rate. If we look at the 90th percentile in Figure 1, its peak EIS value is -84.98 dBm, which is within the approved range.

Observation 2: The 90th percentile (representing 90% device passing rate) of the reported beam peak EIS values for handheld UE at 28 GHz is -84.98 dBm.

Proposal 1: Given the prior agreement on the range of minimum beam peak EIS values for consideration of the REFSENS requirement of handheld devices, we propose the value at 28 GHz and 50 MHz Rx BW to be -85.40 dBm.

We will now follow the same statistical analysis for the 39 GHz reported results, including those for two different glass materials and designs [9], and updates made in [9] and [10]. These results are summarized in the table below.

Table 2: Reported beam peak EIS at 39 GHz [3,9,10]

|  | Intel Glass 1 | Intel Glass 2 | Intel alt. Glass 1 | Intel alt. Glass 2 | LGE | Huawei | MediaTek | Qualcomm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kTB/Hz [dBm] | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| 10log(Rx BW) [dB] | 76.99 | 76.99 | 76.99 | 76.99 | 76.77 | 76.99 | 76.99 | 76.99 |
| Effective realized antenna array gain [dB] | 7.5 | 7.5 | 7.5 | 7.5 | 9 | 9 | 8.5 | 10.5 |
| Diversity Gain [dB] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| SNR [dB] | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| NF [dB] | 11 | 11 | 11 | 11 | 11 | 12 | 10.5 | 11.6 |
| Total implementation loss [dB] | 10.9 | 11.4 | 10.8 | 11.3 | 9.5 | 8 | 7.7 | 8.4 |
| Form factor de-sense (FFD) | 1 | 1 | 1 | 1 | - | - | - | - |
| Sensitivity EIS [dBm] | -82.61 | -82.11 | -82.71 | -82.21 | -86.73 | -87.01 | -88.31 | -88.51 |

NOTE: Rx BW = 50 MHz

The range of reported minimum beam peak EIS is about 6 dB. The figure below shows the distribution of the reported results at 39 GHz.

| % | Min beam peak EIS |
| --- | --- |
| 0.0 | -88.51 |
| 0.1 | -88.37 |
| 0.2 | -87.79 |
| 0.3 | -86.98 |
| 0.4 | -86.79 |
| 0.5 | -84.72 |
| 0.6 | -82.69 |
| 0.7 | -82.62 |
| 0.8 | -82.37 |
| 0.9 | -82.18 |
| 1.0 | -82.11 |

Figure 2: Distribution of reported beam peak EIS at 39 GHz

The approved range at 39 GHz is from -91.8 to -82.6 dBm [3]. The 90th percentile from the distribution in Figure 2, yields a peak EIS of -82.18 dBm, which is within the approved range.

Observation 3: The 90th percentile (representing 90% device passing rate) of the reported beam peak EIS values for handheld UE at 39 GHz is -82.18 dBm.

Proposal 2: Given the prior agreement on the range of beam peak EIS values for consideration of the REFSENS requirement, we propose the value at 39 GHz and 50 MHz Rx BW to be -82.6 dBm.

# 3	Conclusions

In this paper we presented our views on a suitable approach to finalize the beam peak EIS REFSENS requirement for handheld UEs. The following observations and proposals have been made:

Observation 1: Given it is a full system spec, we should use a statistical approach to finalize the minimum peak EIS requirement. This will ensure a reasonable integration requirement for handheld devices.

Observation 2: The 90th percentile (representing 90% device passing rate) of the reported beam peak EIS values for handheld UE at 28 GHz is -84.98 dBm.

Proposal 1: Given the prior agreement on the range of minimum beam peak EIS values for consideration of the REFSENS requirement of handheld devices, we propose the value at 28 GHz and 50 MHz Rx BW to be -85.40 dBm.

Observation 3: The 90th percentile (representing 90% device passing rate) of the reported beam peak EIS values for handheld UE at 39 GHz is -82.18 dBm.

Proposal 2: Given the prior agreement on the range of beam peak EIS values for consideration of the REFSENS requirement, we propose the value at 39 GHz and 50 MHz Rx BW to be -82.6 dBm.

# 4	References

- R4-1709140, “WF on REFSENS for mm-wave,” LG Electronics, Huawei, Sumitomo, Sony, Ericsson, Skyworks, Intel Corporation, KT, MediaTek, 3GPP RAN4 #84, August 2017

- R4-1712101, “ RAN4#84Bis Meeting report,” October 2017

- R4-1714448, “WF on FR2 REFSENS,” Intel Corporation, 3GPP RAN4 #85, November 2017

- R4-1800528, “EIS level of NR UE at mmWave, LGE, RAN4 AH-1801, January 2018

- R4-1800337, “Peak EIS for FR2 REFSENS,” Intel Corporation, RAN4 AH-1801, January 2018

- R4-1800289, “mmW UE EIS, a Resubmission,” Qualcomm, RAN4 AH-1801, January 2018

- R4-1801788, “Peak EIS for FR2 handheld UE type,” Intel Corporation, Apple, RAN4 #86, February 2018

- R4-1801786, “Peak EIRP for FR2 handheld UE type,” Intel Corporation, Apple, RAN4 #86, February 2018

- R4-1804123, “FR2 peak EIRP requirement for handheld UE,” Intel Corporation, Apple, RAN4 #86Bis, April 2018

- R4-1804589, “mmW UE EIS,”, Qualcomm Inc., RAN4 #86Bis, April 2018