3GPP TGS-RAN WG4 Meeting #86-Bis	R4-1804126
Melbourne, Australia, April 16 – April 20, 2018

Agenda item:	7.4.11.1

Source:	Intel Corporation

Title:	FR2 REFSENS for FWA UE

Document for:	Approval

# 1	Introduction

The FR2 power class and REFSENS requirements for handheld UEs have been heavily discussed over the last meetings. While ranges for both minimum peak EIRP and minimum beam peak EIS were agreed in [1] and [2], no consensus has been found to finalize the requirements [3-5]. During the AH-1801 meeting, a new UE type for fixed wireless access (FWA) devices was introduced [6] and in RAN4 #86 [7] an alignment table for peak EIS was approved. The plan for Rel-15 is to target power class and REFSENS requirements for both handheld and FWA UE types.

This paper summarizes the latest FR2 REFSENS agreements for FWA devices, and presents an example evaluation of the peak EIS requirement for approval.

# 2	Discussion

## 2.1	Background

With the deadline for Rel-15 fast approaching, definition of FR2 REFSENS requirements need to be concluded. Thus far, the discussions have focused on the requirements for handheld UE type. Even though a final peak EIS value has not been agreed on [8], a range of peak EIS was approved [2].

Meeting report for RAN4 #84Bis [8]:

- Chair asked if an average of the reported values -94. 2 could be taken as possible agreement for 50 MHz channel bandwidth for n257 and n258

  - Companies expressed needing time to discuss this number

Agreements from RAN4 #85 [2]:

- For 28 GHz, the beam peak EIS range is [-92.1 to -85.4] dBm

- For 39 GHz, the beam peak EIS range is [-91.8 to -82.6] dBm

Though the plan was to provide analysis to finalize the requirement, no new agreement or way forward was reached in AH-1801. However, a new UE type termed Fixed Wireless Access (FWA) was approved [6]. Agreements for the new UE type list determining the impact on UE requirements and aligning on parameters needed for peak EIS.

Once we are aligned on evaluations parameters, the plan is to target Rel-15 for both power class and REFSENS requirements.

Work plan for FWA [6]:

Lastly, during RAN4 #86 a table to align on relevant parameters for beam peak EIS derivation was approved [7].

## 2.2	REFSENS Requirements for FWA

Defining new UE types beyond handheld makes sense given the popularity and potential use case of additional devices operating in millimeter-wave. Specifically, fixed wireless access (FWA) devices are expected to have significant growth in 5G. These can provide broadband access to remote areas, thanks to their large coverage area and relatively low latency. The FCC has even adopted a general framework for the additional UE type across 28 and 39 GHz [9]. Because of this, RAN4 introduced and approved a new UE type for FWA, with a Rel-15 target for its requirements [6].

It is important to note that given the variability on potential use cases, form-factor size, and environment while in operation, we should align on how to separate and define the FWA UE type. This is addressed in [10], where a distinction is made between FWA devices on a stationary platform, and those in a moving platform while operating. The rest of this paper will focus on REFSENS requirements for FWA devices on a stationary platform.

An alignment table with relevant parameters used to derive the peak EIS requirement was approved in [7]. We use this content in Table 1 as an example evaluation for a FWA devices.

Table 1: Example of minimum beam peak EIS evaluation

|  | 28GHz | 39GHz | Comments |
| --- | --- | --- | --- |
| kTB/Hz [dBm] | -174 | -174 |  |
| 10log(Rx BW) [dB] | 76.99 | 76.99 |  |
| Effective realized antenna array gain [dB] | 14.0 | 13.5 | # elements increased from 4 to 16 includes roll-off |
| Diversity Gain [dB] | 0 | 0 |  |
| SNR [dB] | -1 | -1 |  |
| NF [dB] | 10 | 11 |  |
| Total implementation loss [dB] | 6.5 | 8.0 | Overall losses dropped |
| Sensitivity EIS [dBm] | -95.51 | -92.51 |  |

The higher power expected from FWA devices is mainly possible because of their larger form-factor. This enables different architecture implementations with more antenna elements in the array (larger gain) and provides the necessary room for greater separation between sensitive RF components, and non-ideal materials or reflectors that may otherwise affect radiation. The larger size also makes the overall integration easier and reduces the implementations losses. These differences are captured in Table 1 in the number of antennas and total implementation loss parameters.

Compared to our previously reported handheld numbers, the number of antennas increased to from 4 to 16, and the overall implementation losses dropped. After factoring these changes, we get a minimum beam peak EIS of -95.51 dBm for 28 GHz, and -92.51 dBm for 39 GHz. These numbers correspond to difference of about 10 dB compared to our results for handheld devices (-85.41 dBm for 28 GHz, -82.61 dBm for 39 GHz).

Observation 1: The example minimum beam peak EIS derivation for FWA devices on a stationary platform yields

-95.51 dBm for 28GHz, and -92.51 dBm for 39GHz. This represents a difference of about 10 dB from our handheld device values.

Proposal 1: Define the minimum beam peak EIS requirement for FWA on a stationary platform as -95.51 dBm for 28 GHz, and -92.51 dBm for 39 GHz.

We need to pay attention to any potential impact the new FWA UE type may have on RAN4 requirements. There will be implications on testability and spherical coverage studies, as measurements must now take into account the fixed nature of the device and typical surrounding materials. From a co-existence perspective, studies for +55dBm CPE have already been concluded with no issues reported, thus no additional work is needed [11].

The impact on UE requirements we discussed so far is not comprehensive. Additional topics are not precluded from future discussions. It is also important to note that there may be required features for handheld devices that can be made optional for FWA devices. These need to be discussed, agreed and documented.

Observation 2: Companies are encouraged to include additional topics of interest that may impact FWA requirements for discussion in upcoming meetings.

Proposal 2: We should revisit the required handheld device features and determine which ones can be defined as optional capabilities for FWA devices.

# 3	Conclusions

In this paper we presented initial REFSENS framework for fixed wireless access devices and discussed possible impact of the UE type on RAN4 requirements. We also an example table of peak EIS parameters, along with preliminary derivations. The following observations and proposals have been made:

Observation 1: The example minimum beam peak EIS derivation for FWA devices on a stationary platform yields

-95.51 dBm for 28 GHz, and -92.51 dBm for 39 GHz. This represents a difference of about 10 dB from our handheld device values.

Proposal 1: Define the minimum beam peak EIS requirement for FWA on a stationary platform as -95.51 dBm for 28 GHz, and -92.51 dBm for 39 GHz.

Observation 2: Companies are encouraged to include additional topics of interest that may impact FWA requirements for discussion in upcoming meetings.

Proposal 2: We should revisit the required handheld device features and determine which ones can be defined as optional capabilities for FWA devices.

# 4	References

- R4-1714445, “WF on pi/2 BPSK spectrum shaping and power class in FR2,” Intel Corporation, Apple, Qualcomm, IITH, LGE, Nokia, Huawei, Motorola Mobility, 3GPP RAN4 #85, November 2017

- R4-1714448, “WF on FR2 REFSENS,” Intel Corporation, 3GPP RAN4 #85, November 2017

- R4-1801106, “WF peak EIRP requirement for mmW,” Qualcomm,  RAN4 AH-1801, January 2018

- R4-1803372, “WF on output power requirements on FR2,” Qualcomm, Samsung, Verizon, AT&T, T-Mobile USA, NXP, DISH, Orange, ZTE, US Cellular, C Spire, Ericsson, Telstra, KT, Nokia, Vodafone, Mavenir, Telecom Italia, Reliance Jio, IITH, IITM, CEWiT, Tejas Networks, 3GPP RAN4 #86, February 2018

- R4-1803460, “WF on FR2 peak EIRP for handheld UE,” Intel, Xiaomi, OPPO, vivo, MediaTek, LGE, Apple, Qorvo, 3GPP RAN4 #86, February 2018

- R4-1801191, “WF on Fixed wireless access use case for FR2,” Intel Corporation, Verizon, January 2018

- R4-1801789, “On FR2 REFSENS for FWA devices,” Intel Corporation, RAN4 #86, February 2018

- R4-1712101, “RAN4#84Bis Meeting report,” October 2017

- FCC 16-89

- R4-1804124, “FR2 power class for FWA UE,” Intel Corporation, RAN4 #86-Bis, April 2018

- R4-1711861, “Summary of simulation results on Coexistence Studies for 55dBm CPE,” Ericsson, Qualcomm, Samsung, Verizon, Nokia, RAN4 #84bis, October 2017