3GPP TSG-RAN WG4#86Bis Meeting	R4-1804131

Melbourne, Australia, 16 – 20 April 2018

Agenda item:	6.29.2

Title: 	Simulation assumptions for 8Rx UE demodulation performance requirements

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN Plenary Meeting #78, the Rel.15 WI “UE requirements for LTE DL 8Rx antenna ports” has been approved and its WID is included in [1]. In the scope of the WID [1], there is part related to RAN4 UE demodulation/CSI performance tests as shown below:

- For UE demodulation/CSI requirements

  - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas;

    - Specify the static channel model;

  - Define test cases for the rank lower than or equal to 4.

    - Adopt the test cases studied in SI to define requirements.

  - Define test cases for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

In this contribution, we provide our initial views on the simulation assumptions for 8Rx UE PDSCH demodulation performance requirements.

# Discussion

In our parallel contribution [2], we made a proposal to define TM2, TM3 and TM9 test cases for 8Rx UE PDSCH demodulation performance requirements. In the following, we give the simulation assumption details for these test cases.

For TM2 and TM3 test cases, the simulation parameters are given in Table 1 below.

Table 1 Simulation parameters for TM2 and TM3 tests

| Test number | Transmission mode | Bandwidth and MCS | Reference channel | Propagation condition | Correlation matrix and antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | TM2 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA5 | 2x8 Low Medium correlation B, ULA |
| 2 | TM3 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA70 | 2x8 Low |

The uniform linear arrays (ULA) are applied for the antenna configuration at both eNodeB and UE sides. According to [3], the MIMO antenna correlation matrices and  at eNodeB and UE are given by Equations (1) and (2), respectively.

(1)

(2)

And the antenna correlation values “α” and “β” for different correlation levels are listed in Table 2. For “Low” and “Medium Correlation A”, the values has been defined in Table B.2.3.2-1 of TS36.101. Moreover, the 8Rx SI defined a new antenna correlation level “Medium Correlation B” to ensure that the 8Rx correlation level between adjacent Rx antennas is the same as 4Rx correlation level.

Table 2 The  and  parameters for ULA MIMO correlation matrices

| Correlation Model |  |  |
| --- | --- | --- |
| Low Correlation | 0 | 0 |
| Medium Correlation A | 0.3 | 0.3874 |
| Medium Correlation B | 0.3 | 0.005154 |

Note:

In addition, for the TM9 test case, the simulation parameters are proposed in Table 3 below.

Table 3 Simulation parameters for TM9 test

| Test number | Transmission mode | Bandwidth and MCS | Rank | Propagation condition | Correlation matrix and antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 3 | TM9 | 10 MHz 256QAM MCS table MCS=20 | Rank=4 | EPA5 | 8x8 Low ULA |

In summary, we propose that:

Proposal 1: Define 8Rx UE PDSCH demodulation performance tests for TM2, TM3 and TM9 modes as specified in Table 1 to Table 3.

# Conclusion

In this contribution, we provide our initial views on the simulation assumptions for 8Rx UE PDSCH demodulation performance requirements.

Proposal 1: Define 8Rx UE PDSCH demodulation performance tests for TM2, TM3 and TM9 modes as specified in Table 1 to Table 3.

# Reference

[1] R4-172842, “New WI proposal: UE requirements for LTE DL 8Rx antenna ports”, Huawei, HiSilicon, Dec. 2017

[2] R4-1804130, “Discussion on 8Rx UE demodulation performance requirements”, Intel Corporation, April 2018

[3] R4-1708538, “Way forward on 8x8 channel model”, Huawei, HiSilicon, Aug. 2017