3GPP TSG-RAN WG4 Meeting #86bis	R4-1804157

# Melbourne, Australia, Apri 16 – 20, 2018

Agenda item:	7.4.6.3

Source: 	Intel Corporation

Title: 	Measurement results of MPR for DC_(n)71B Intra-Band EN DC Scenario with single PA

Document for:	Discussion

# Introduction

The intra-Band DC_(n)71B EN DC scenario with single PA architecture is detailed here. Measurement results with required MPR for likely RB allocation scenarios are presented.

# Discussion

## Measurement Assumptions

- Both LTE and NR carriers use DFTs OFDM QPSK modulation with 15 kHz SCS.

- PA is calibrated for NR20 fully allocated (100RBs) DFTs OFDM QPSK modulated waveform with 1 dB MPR.

- LO leakage and IQ image were not taken into consideration.

- 16 different test cases including from full RB allocation to single RB allocation in both carrier to consider a range of scenarios are measured.

- ET PA + B71 Duplexer are measured.

- PA was in ET mode, which is always used for high output powers

- Assuming full knowledge on RB allocation and powers

The following is the measurement setup

Figure: Measurement Setup

With the baseline Testcase#1, for which the PA is calibrated – the DX output power is 24 dBm, with 2dB IL after DX assumed.

## Performance Criteria

- ACLR requirements from 38.101-3 from Table 6.5B.2.1.3-1

Table 6.5B.2.1.3-1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR | dBc | 30 |
| Measurement bandwidth |  | [0.95] ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

- SEM requirements of NR20 (same as LTE20) as per 38.101-1 Table 6.5.2.-1

Table 6.5.2.1.1-1: NR General spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -15 | -18 | -20 | -21 | -22 | -23 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 85-100 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  | -25 |  |

- Additional SEM requirements from 38.101-3 from Table 6.5B.2.1.2.1-1

  - Note, those are more relaxed than the general SEM

Table 6.5B.2.1.2.1-1: Additional requirements

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

- Spurious emissions, as per 38.101-1 and 36.101, as referenced in 38.101-3:

- Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than [2.69] GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than [5.2] GHz |  |  |  |

- Spurious emission band UE co-existence, based on 36.101.

  - Assuming same specs for NR and DC as for LTE

  - Focus was B12,29 and 71

  - Note that B12/29 specs are only valid outside of F_OOB

- Table 6.6.3.2-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |

## Measurement results

16 test cases were tested and summarized in the following table.  All test cases used DFT-s-QPSK, SCS = 15KHz.

| Testcases | Name | BW LTE | BW NR | RB LTE | RB NR | Carrier Power offset LTE/NR | MPR | Blocking factor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline NR20 Full Alloc | - | NR20 | - | Full Alloc | - | 1 | ACLR |
| 2 | Baseline DC | LTE10 | NR10 | Full Alloc | Full Alloc | 0dB | 2 | ACLR |
| 3 | Power imbalance DC | LTE10 | NR10 | Full Alloc | Full Alloc | 6dB | 4 | ACLR |
| 4 | Only NR | - | NR10 | - | Full Alloc | - | 0 |  |
| 5 | Only LTE | LTE10 | - | Full Alloc | - | - | 0 |  |
| 6 | "Devil Horns" (2x1RB at edges) | LTE10 | NR10 | 1@0 | 1@51 | 0dB | 0 |  |
| 7 | 20% allocation, CC center | LTE5 | NR15 | 5@10 | 15@32 | 0dB | 5 | ACLR |
| 8 | 20% allocation, CC center | LTE15 | NR5 | 15@30 | 5@10 | 0dB | 0 |  |
| 9 | 20% allocation, CC center | LTE10 | NR10 | 10@19 | 10@20 | 0dB | 1.2 | SEM |
| 10 | 20% allocation, CC outer edge | LTE5 | NR15 | 5@0 | 15@64 | 0dB | 0 |  |
| 11 | 20% allocation, CC outer edge | LTE15 | NR5 | 15@0 | 5@20 | 0dB | 1.3 | SEM |
| 12 | 20% allocation, CC outer edge | LTE10 | NR10 | 10@0 | 10@42 | 0dB | 0 |  |
| 13 | 2x1RB R4-1802243 case 1 | LTE10 | NR10 | 1@3 | 1@1 | 0dB | 6.1 | SEM |
| 14 | 2x1RB R4-1802243 case 2 | LTE10 | NR10 | 1@10 | 1@35 | 0dB | 3.5 | SEM |
| 15 | 2x1RB R4-1802243 case 3 | LTE10 | NR10 | 1@15 | 1@15 | 0dB | 5.1 | ACLR |
| 16 | 2x1RB R4-1802243 case 6 | LTE10 | NR10 | 1@1 | 1@49 | 0dB | 0 |  |

Based on the measurement results the main observations are:

Observation 1: General SEM & ACLR are the main blocking factors.

Observation 2: B71 UE Coexistence is not a blocking factor due to high Duplexer attenuation.

Observation 3: B12/29 are not blocking factors because not valid within F_OOB, also see 2).

Observation 4: PSD imbalance will require more power backoff that equal PSD scenario.

It is also noted that we do not consider self desense, since it was never before considered for MPR.

# Conclusion

We evaluated required MPR by using DFT-s-OFDM QPSK waveform to identify a few typical test cases by measurements. It shows that in general, IM products generated by small RB blocks in some allocations will need higher MPR values, and also PSD imbalance needs extra backoff than equal PSD cases. We have following observations:

Observation 1: General SEM & ACLR are the main blocking factors.

Observation 2: B71 UE Coexistence is a not blocking factor due to high Duplexer attenuation.

Observation 3: B12/29 are not blocking factors because not valid within F_OOB, also see 2).

Observation 4: PSD imbalance will require more power backoff that equal PSD scenario.

# References