3GPP TSG-RAN WG4 Meeting #86bis	R4-1804243

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	7.6.2.5.2.2

Source: 	Nokia, Nokia Shanghai Bell, NTT DOCOMO, INC, ZTE

Title: 	TP to TR 38.817-02: NR CACLR requirement for non-continuous allocation in FR2 (9.7.3.3)

Document for:	Approval

# 1.	Introduction

The way forward on NR CACLR requirement for non-continuous allocation was agreed in RAN4 AH-1801 [1], where the requirement in FR2 is FFS.

This contribution provides a proposal to specify the NR CACLR requirement for non-continuous allocation in FR2, and the corresponding text proposal to TR 38.817-02 [2].

# 2.	Discussion

The same channel bandwidth with the wanted signal is assumed as the adjacent channel bandwidth for ACLR requirement in FR2. Currently there are 4 different channel bandwidths (50, 100, 200 and 400 MHz) defined for NR in FR2. This means that there are 16 different combinations of channel bandwidth at the edges of the gap. As discussed in [3], different gap sizes for the CACLR requirement need to be applied for different combinations of NR channel bandwidth. Thus 16 different gap sizes need to be considered for the CACLR requirement, which would greatly increase the test complexity of the requirement.

On the other hand, it has been agreed to apply the 5 MHz adjacent channel carrier to NR channel bandwidth ≤20 MHz and the 20 MHz adjacent channel carrier to NR channel bandwidth >20 MHz for NR CACLR requirement for non-continuous allocation in FR1 [1]. It is proposed to adopt the same approach in FR2 with the same maximum NR channel bandwidth to adjacent channel bandwidth ratio of 4:

Proposal: Apply the 50 MHz adjacent channel carrier to NR channel bandwidth <200 MHz and the 200 MHz adjacent channel carrier to NR channel bandwidth ≥200 MHz for OTA CACLR requirement in FR2.

Based on above proposal, the CACLR requirement and the ACLR requirement in non-contiguous spectrum in FR2 can be specified as shown below in tables 1 and 2.

# Table 1: BS type 2-O ACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted [MHz] | Sub-block gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | Wgap ≥ 100 (Note 5) Wgap ≥ 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| 200, 400 | Wgap ≥ 400 (Note 6) Wgap ≥ 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50, 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200, 400 MHz. |  |  |  |  |  |

# Table 2: BS type 2-O CACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted [MHz] | Sub-block gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | 50 ≤ Wgap < 100 (Note 5) 50 ≤ Wgap < 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| 200, 400 | 200 ≤ Wgap < 400 (Note 6) 200 ≤ Wgap < 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50, 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200, 400 MHz. |  |  |  |  |  |

# 3.	Conclusion

This contribution has provided a proposal to specify the NR CACLR requirement for non-continuous allocation in FR2, and the corresponding text proposal to TR 38.817-02 is provided below.

Proposal: Apply the 50 MHz adjacent channel carrier to NR channel bandwidth <200 MHz and the 200 MHz adjacent channel carrier to NR channel bandwidth ≥200 MHz for OTA CACLR requirement in FR2.

# 4.	Text Proposal

<Start of change>

- 9.7.3.3	AAS radiated ACLR requirement for FR2

For the frequency ranging from 24.24 GHz to 33.4 GHz, the OTA ACLR1 limit is 28 dB.

For the frequency ranging from 37 GHz to 52.6 GHz, the OTA ACLR1 limit is 26 dB.

The OTA ACLR2 limit is not specified since the emission level at the ACLR2 range will be close to noise floor.

The same BS channel bandwidth with the wanted signal can be assumed as the adjacent channel bandwidth.

The transmission bandwidth configuration of wanted channel (it would differ depending on SCS and/or BS channel bandwidth) should be used as a measurement bandwidth for the wanted signal power measurement.

Maximum transmission bandwidth configuration of the BS channel bandwidth (between subcarrier spacing) specified in Release 15 should be used as a measurement bandwidth for adjacent channel power measurement, i.e. the measurement bandwidth should also apply to future releases regardless of whether new SU is introduced or not.

The minimum radiated requirement for 2-O NR BS for FR2 is specified in Table 9.7.3.3-1.

Table 9.7.3.3-1: BS type 2-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| Note 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. Note 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). Note 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz Note 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |  |

It has also been agreed to define absolute ACLR limits for FR2, thus the following is agreed. In case the absolute ACLR requirement is not compatible with Japanese regulation, a regional requirement for Japan can be introduced.

Table 9.7.3.3-2: BS type 2-O ACLR absolute limit

| NR BS class | ACLR absolute limit |
| --- | --- |
| Wide-area BS | -13 dBm/MHz |
| Medium-range BS | -20 dBm/MHz |
| Local-area BS | -20 dBm/MHz |

It has also been agreed to apply the 50 MHz adjacent channel carrier to NR channel bandwidth <200 MHz and the 200 MHz adjacent channel carrier to NR channel bandwidth ≥200 MHz for OTA CACLR requirement in FR2. Accordingly, the CACLR requirement and the ACLR requirement in non-contiguous spectrum in FR2 are specified as shown below in tables 9.7.3.3-3 and 9.7.3.3-4.

# Table 9.7.3.3-3: BS type 2-O ACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted [MHz] | Sub-block gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | Wgap ≥ 100 (Note 5) Wgap ≥ 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| 200, 400 | Wgap ≥ 400 (Note 6) Wgap ≥ 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50, 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200, 400 MHz. |  |  |  |  |  |

# Table 9.7.3.3-4: BS type 2-O CACLR limit in non-contiguous spectrum

| BS channel bandwidth of lowest/highest NR carrier transmitted [MHz] | Sub-block gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | CACLR limit |
| --- | --- | --- | --- | --- | --- |
| 50, 100 | 50 ≤ Wgap < 100 (Note 5) 50 ≤ Wgap < 250 (Note 6) | 25 MHz | 50 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| 200, 400 | 200 ≤ Wgap < 400 (Note 6) 200 ≤ Wgap < 250 (Note 5) | 100 MHz | 200 MHz NR (Note 2) | Square (BWConfig) | 28 (Note 3) 26 (Note 4) |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz. NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz. NOTE 5: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 50, 100 MHz. NOTE 6: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 200, 400 MHz. |  |  |  |  |  |

<End of change>

# References

[1]	R4-1801020, “WF on NR CACLR requirement for non-continuous allocation”, Nokia, Nokia Shanghai Bell, ZTE, NTT DOCOMO.

[2]	R4-1802143, “TR 38.817-02 v0.7.0: General aspects for BS RF for NR”, Ericsson.

[3]	R4-1800529, “Further discussion on NR CACLR requirement”, ZTE.