3GPP TSG-RAN WG4 Meeting #86bis	R4-1804251

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	7.7.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Measurement uncertainty and test tolerance for BS Type 1-O OTA sensitivity and REFSENS conformance test

Document for:	Approval

# 1.	Introduction

Measurement uncertainty of testing equipment for NR BS were discussed in RAN4#86, and the way forward was agreed [1].

This contribution provides our proposals on the measurement uncertainty and test tolerance for BS Type 1-O OTA sensitivity and REFSENS conformance test.

# 2.	Discussion

The measurement uncertainty of testing equipment for AAS BS OTA sensitivity conformance test currently recorded in TR 37.842 [2] is provided below in Table 1.

# Table 1: Test system specific measurement uncertainty values for the EIS test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 1.22 | 1.25 |
| Compact Antenna Test Range | 1.33 | 1.40 |
| One Dimensional Compact Range Chamber | 1.29 | 1.43 |
| Near Field Test Range | 1.24 | 1.24 |
| Common maximum accepted test system uncertainty | 1.3 | 1.4 |

And the test tolerance for AAS BS OTA sensitivity conformance test currently specified in TS 37.145-2 [3] is provided below in Table 2.

Table 2: Derivation of Test Requirements (OTA receiver tests)

| Test | Minimum Requirement in 3GPP TS 37.105 [6] | Test Tolerance | Test Requirement in the present document |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See 3GPP TS 37.105 [6], subclause 10.2 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz | Formula: Declared Minimum EIS + TT |

Since the nature and procedure of BS Type 1-O OTA sensitivity and REFSENS conformance tests are similar to those of AAS BS OTA sensitivity conformance test, and the same test equipment would likely be used for these conformance tests, the current measurement uncertainty and test tolerance for AAS BS OTA sensitivity conformance test can be reused for BS Type 1-O OTA sensitivity and REFSENS conformance tests. Therefore, we propose to adopt the measurement uncertainty and test tolerance provided below in Tables 1 and 2, respectively, for BS Type 1-O OTA sensitivity and REFSENS conformance tests.

# 3.	Conclusion

This contribution has provided our proposals on the measurement uncertainty and test tolerance for BS Type 1-O OTA sensitivity and REFSENS conformance test.

Proposal: To adopt the measurement uncertainty and test tolerance provided above in Tables 1 and 2, respectively, for BS Type 1-O OTA sensitivity and REFSENS conformance tests.

# References

[1]	R4-1803546, “WF on MU of testing equipment for AAS BS and NR BS”, NTT DOCOMO, Huawei, NEC, Keysight.

[2]	3GPP TR 37.842 v13.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA); Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

[3]	3GPP TS 37.145-2 v14.4.0, “Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing”.