3GPP TSG-RAN WG4 Meeting #86bis	R4-1804253

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	7.7.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Manufacturer declarations for BS Type 1-O OTA REFSENS conformance test

Document for:	Approval

# 1.	Introduction

This contribution provides our proposals on the manufacturer declarations for NR BS Type 1-O OTA REFSENS conformance test.

# 2.	Discussion

The manufacturer declarations for AAS BS OTA sensitivity conformance test currently specified in TS 37.145-2 [1] is provided below in Table 1.

Table 1: Manufacturer declarations for AAS BS OTA sensitivity conformance test

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D10.1 | OSDD identifier | A unique identifier for the OSDD. |
| D10.2 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D10.1. NOTE 2: As each identified OSDD has a declared minimum EIS value (D10.6), multiple operating band can be only be declared if they have the same minimum EIS declaration. |
| D10.3 | OSDD RAT support | RAT(s) supported by the OSDD for each supported operating band, declared for every OSDD identified in D10.1. NOTE 3: If the OSDD supports multiple RAT's with different minimum EIS value (D10.6) if all other parameters are the same then different EIS values for different RATS and signal BW's may be declared for an OSDD. |
| D10.4 | OTA sensitivity E-UTRA supported channel BW | The E-UTRA channel BW's supported by each OSDD. |
| D10.5 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD |
| D10.6 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared for every supported RAT and E-UTRA supported channel BW for the OSDD. Declared per OSDD identified in D10.1. NOTE 4: If the AAS BS is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.9) for each OSDD. |
| D10.8 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD |
| D10.9 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.8). NOTE 5: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. |
| D10.10 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.11). |
| D10.11 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. |

Since NR BS Type 1-O OTA REFSENS requirement is intended to ensure the OTA reference sensitivity level for a declared OTA REFSENS RoAoA for a specified NR reference measurement channel, there is no need for declarations corresponding to D10.3, D10.5, D10.6, D10.8 and D10.10. Otherwise, the current manufacturer declarations for AAS BS OTA sensitivity conformance test can be reused for NR BS Type 1-O REFSENS conformance test, with the term ‘sensitivity’ changed to ‘REFSENS’. Therefore, we propose to adopt the manufacturer declarations provided below in Table 2 for NR BS Type 1-O OTA REFSENS conformance test.

Table 2: Manufacturer declarations for NR BS Type 1-O OTA REFSENS conformance test

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D11.1 | OSDDREFSENS identifier | A unique identifier for the OSDDREFSENS. |
| D11.2 | OSDDREFSENS operating band support | Operating band supported by the OSDDREFSENS, declared for every OSDDREFSENS identified in D11.1. NOTE 1: As each identified OSDDREFSENS has a minimum EISREFSENS value, multiple operating band can be only be declared if they have the same minimum EISREFSENS. |
| D11.3 | OTA REFSENS NR supported channel BW | The NR BS channel BW's supported by each OSDDREFSENS. |
| D11.5 | Receiver target reference direction REFSENS Range of Angle of Arrival | The REFSENS RoAoA associated with the receiver target reference direction (D11.6) for each OSDDREFSENS. |
| D11.6 | Receiver target reference direction | For each OSDDREFSENS an associated direction inside REFSENS RoAoA (D11.5). |
| D11.7 | Conformance test directions | For each OSDDREFSENS four conformance test directions. 1) The direction determined by the maximum φ value achievable inside the REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the REFSENS RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the REFSENS RoAoA, while φ value being the closest possible to the receiver target reference direction. |

# 3.	Conclusion

This contribution has provided our proposals on the manufacturer declarations for NR BS Type 1-O OTA REFSENS conformance test.

Proposal: To adopt the manufacturer declarations provided above in Table 2 for NR BS Type 1-O OTA REFSENS conformance test.

# References

[1]	3GPP TS 37.145-2 v14.4.0, “Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing”.