3GPP TSG-RAN WG4 Meeting #86-Bis					R4-1804360

Melbourne, AU, 16th – 20th April, 2018

Source:	Verizon, Ericsson, Qualcomm, Samsung, Nokia

Title:	TP for TR 37.863-01-01 NR intra-band contiguous n261 CA

Agenda item:			7.1.3.7

Document for:	Approval

# 1. Introduction

Rel-15 LTE-NR CA combinations are listed in the latest WID [1]. This contribution is a text proposal for TR 37.863-01-01 to add NR intra-band contiguous n261 CA.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 6.X	NR intra-band contiguous n261 CA

### 6.X.1	Operating bands for NR

Table 6.X.1-1: NR band combination of NR band

| NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for NR CA

### Table 6.X.2-1: NR CA configurations and bandwidth combination fallback group defined for contiguous intra-band CA

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS [KHz] | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Fallback group |
|  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_n261B |  | 60 | 100, 200 | 200 |  |  |  | 400 | 1 |
|  |  | 120 | 200, 400 | 400 |  |  |  | 800 |  |
| CA_n261C |  | 120 | 400 | 200, 400 | 200 |  |  | 1000 |  |
|  |  |  | 200, 400 | 200, 400 | 400 |  |  | 1200 |  |
| CA_n261D |  | 60/120 | 200 | 50, 100, 200 |  |  |  | 400 | 2 |
| CA_n261E |  | 60 | 100, 200 | 100, 200 | 200 |  |  | 600 |  |
|  |  | 120 | 400 | 50, 100 | 50, 100 |  |  |  |  |
| CA_n261F |  | 60 | 100, 200 | 100, 200 | 100, 200 | 100, 200 |  | 800 |  |
|  |  | 120 | 400 | 50, 100 | 50, 100, 200 | 50, 100, 200 |  |  |  |
| CA_n261O |  | 60/120 | 50, 100 | 50, 100 |  |  |  | 200 | 4 |
| CA_n261P |  | 60/120 | 50 | 50, 100 | 50, 100 |  |  | 300 |  |
|  |  |  | 100 | 50, 100 | 50, 100 |  |  |  |  |
|  |  |  | 200 | 50 | 50 |  |  |  |  |
| CA_n261Q |  | 60/120 | 50 | 50, 100, 200 | 50, 200, 100 | 200, 100, 50 |  | 400 |  |
|  |  |  | 100 | 50, 100, 200 | 50, 200, 100 | 200, 100, 50 |  |  |  |
|  |  |  | 200 | 50,100 | 50, 100 | 100, 50 |  |  |  |

### Table 6.X.2-2: NR CA configurations and bandwidth combination fallback group defined for contiguous intra-band CA

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | SCS [KHz] | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Fallback group |
|  |  |  | CBW for carrier [MHz] | CBW for carrier [MHz] | CBW for carrier [MHz] | CBW for carrier [MHz] | CBW for carrier [MHz] | CBW for carrier [MHz] | CBW for carrier [MHz] | CBW for carrier [MHz] |  |  |
| CA_n261G |  | 60/120 | 100 | 50, 100 |  |  |  |  |  |  | 200 | 3 |
| CA_n261H |  | 60/120 | 100 | 100 | 50, 100 |  |  |  |  |  | 300 |  |
| CA_n261I |  | 60/120 | 100 | 100 | 100 | 50, 100 |  |  |  |  | 400 |  |
|  |  |  | 200 | 50, 100 | 50 | 50 |  |  |  |  |  |  |
| CA_n261J |  | 60/120 | 100 | 100 | 100 | 100 | 50, 100 |  |  |  | 500 |  |
| CA_n261K |  | 60/120 | 100 | 100 | 100 | 100 | 100 | 50, 100 |  |  | 600 |  |
| CA_n261L |  | 60/120 | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 |  | 700 |  |
| CA_n261M |  | 60/120 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 50, 100 | 800 |  |

----- Unchanged sections omitted -----

# Reference

[1]	RP-180536, “Revision of WI: New Radio Access Technology”, NTT DOCOMO, INC.