3GPP TSG RAN WG4 Meeting #86bis	R4-1804463

Melbourne, Australia, 16 – 20 April 2018

Source:		Rohde & Schwarz

Title:	On TRP Measurement Grids for mm-wave

Agenda Item:		7.12.2

Document for:	Approval

# Introduction

Different options regarding the selection of a measurement grid for TRP measurements have been discussed in previous meetings. The WF on NR MU and test tolerance from 3GPP TSG-RAN WG4 AH-1801 lists several options [1]. TRP measurement uncertainties for constant density measurement grids with different number of measurement points have been already discussed in [2].

This contribution goes one step back and compares constant step size measurement grids and constant density measurement grids with the help of the simulated 8 x 2 patch antenna array pattern from [3].

# Discussion

## Evaluation Setup

The same pattern of a simulated 8 x 2 patch antenna array as in [3] has been used for the subsequent analysis. Table 1 and Table 2 show the adapted equations from [4] that are used to calculate the UE antenna patterns.

Table 1: Single Antenna Element Radiation Pattern

| Antenna element horizontal radiation pattern | , Am =30 dB |
| --- | --- |
| Horizontal half-power beamwidth of single element | according to wanted element gain (e.g. 260° for 5 dBi element gain) |
| Antenna element vertical radiation pattern | , SLAv =30 dB |
| Vertical half-power beamwidth of single array element | according to wanted element gain (e.g. 130º for 5 dBi element gain) |
| Array element radiation pattern |  |
| Element gain without antenna losses | GE,max = 1.5 dBi or 5 dBi |

Table 2: Composite Antenna Array Radiation Pattern

| Composite array radiation pattern in dB | the super position vector is given by: the weighting is given by: |
| --- | --- |
| Antenna array configuration (Row×Column) | 8 × 2 |
| Horizontal radiating element spacing dh/λ | 0.5 |
| Vertical radiating element spacing dv/λ | 0.5 |

This contribution only focuses on measurement grids for in-band TRP measurements. Measurement grids for spherical coverage measurements (i.e. EIRP CDF) are discussed in contribution [3].

In order to assess the reproducibility of TRP measurements and the corresponding measurement uncertainty of the different measurement grids, the relative orientation of the DUT and the measurement grid was altered randomly. The statistics in terms of TRP variation for each measurement grid are derived from a set of 10000 random orientations.

Figure 1 shows the composite antenna pattern of an 8 x 2 patch antenna array with an average antenna element gain of 5 dBi where the beam is steered in boresight direction. This EIRP pattern with a TRP value of 0 dBm is used throughout the subsequent analyses.

Figure 1: Radiation pattern of 8 x 2 patch antenna array steering the beam in boresight direction.

## Measurement Results

Figure 2 shows a histogram of TRP measurement results for a constant step size measurement grid with a step size of 15 degrees (i.e. 264 measurement points). The histogram of the TRP values is derived from 10000 random orientations of the antenna array relative to the measurement grid. The mean and the standard deviation of the TRP results are in an acceptable range. Nevertheless, some outliers are present almost 1 dB below the expected TRP value of 0 dBm.

Observation 1:
Constant step size measurement grids produce outliers which underestimate the real TRP value even in cases where the statistics show an acceptable mean value and standard deviation.

8

Figure 2: Histogram of TRP measurement results for a constant step size measurement grid
with a step size of 15 degrees.

Figure 3 shows a similar histogram for the same measurement grid with 15 degrees step size. In this case the peak of the antenna array pattern is directed to one pole of the measurement grid. The histogram of TRP values in this case is derived from 10000 random orientations while keeping the beam peak always directed to one pole of the measurement grid. Therefore, only random orientations along the axis of the poles are introduced. This approach reduces the standard deviation of the TRP results almost completely. Nevertheless, this approach introduces a significant offset of 0.95 dB when comparing the mean TRP value of the histogram to the expected TRP value of 0 dBm.

Figure 3: Histogram of TRP measurement results for a constant step size measurement grid
with 15 degrees step size, where the beam peak is always directed to one pole of the measurement grid.

Figure 4 shows a histogram for a constant step size measurement grid with 10 degrees step size. Similar to Figure 3, the peak of the antenna array pattern is always directed to one pole of the measurement grid for all random orientations along the remaining rotational axis. The decreased step size of the measurement grid also decreases the offset of the measured TRP values from 0.95 dB in Figure 3 to 0.29 dB in Figure 4.

Observation 2:
When the peak beam is directed to the pole of a constant step size measurement grid, the TRP value is underestimated. The difference between the real TRP and the measured value depends on the antenna pattern of the DUT and the step size of the measurement grid.

Figure 4: Histogram of TRP measurement results for a constant step size measurement grid
with 10 degrees step size, where the beam peak is always directed to one pole of the measurement grid.

Figure 5 shows another histogram for a measurement grid with a step size of 15 degrees (i.e. 264 measurement points). In this case the peak of the antenna array is always directed to one measurement point at the equator of the constant step size measurement grid. This approach reduces the standard deviation and the offset significantly. Other than being a well understood grid for omnidirectional antenna patterns, the constant step size grid has the advantage that tests could be optimized for speed, e.g., with the use of continuously moving positioners. On the other hand, beams with lobes with larger EIRPs in unintentional (side) directions will experience higher uncertainties [3]. For constant density grids, the intentional or unintentional beam peak directions do not need to be known before starting the TRP measurement.

Observation 3:
When the beam peak is directed to the equator of a constant step size measurement grid, the measurement uncertainty decreases significantly.

Figure 5: Histogram of TRP measurement results for a constant step size measurement grid
with 15 degrees step size, where the beam peak is always directed to the equator of the measurement grid.

Figure 6 shows a histogram of 10000 TRP measurements using a constant density measurement grid with 264 measurement points for comparison. The measurement grid was derived using the approach described in [5]. In this simulation, the orientation between the antenna array and the measurement grid was completely random (i.e. no restrictions in terms of rotational axis was applied). The results show that the variation of the measured TRP values when using a constant density measurement grid is below the variation of a constant step size grid with the same number of measurement points. The decreased measurement uncertainty is achieved without any knowledge of the beam peak direction or restrictions in terms of the rotational axis for the random orientation of the DUT and the measurement grid.

Observation 4:
Constant density measurement grids provide a lower measurement uncertainty compared to constant step size measurement grids with the same number of measurement points.

Figure 6: Histogram of TRP measurement results for a constant density measurement grid
with 264 measurement points, where the orientation of the DUT is completely random.

As outlined above, the minimum number of measurements points yield different MUs, especially with different grid types. It is suggested to leave the number of measurement points up to the system vendor but have the vendors assess the MU for the chosen measurement grid type and number of measurement points for antenna arrays similar to the one evaluated.

Proposal 1: The minimum number of measurement points to determine the TX and RX beam peak direction is TBD (and to be confirmed by the next meeting); vendors have to assess the MU for the chosen measurement grid type and number of measurement points for antenna arrays similar to the one evaluated.

# Conclusion

This contribution compared TRP variations of constant step size measurement grids and constant density measurement grids with the help of a simulated NR reference antenna architecture. Additionally, possibilities to reduce the TRP measurement uncertainties of constant step size measurement grids have been discussed. In order to derive the required statistics and the corresponding measurement uncertainties of the different measurement grids, a set of 10000 orientations have been tested for each measurement grid.

The contribution helps to select a feasible option from the WF on NR MU and test tolerance [1].

The following observations have been made:

Observation 1:
Constant step size measurement grids produce outliers which underestimate the real TRP value even in cases where the statistics show an acceptable mean value and standard deviation.

Observation 2:
When the peak beam is directed to the pole of a constant step size measurement grid, the TRP value is underestimated. The difference between the real TRP and the measured value depends on the antenna pattern of the DUT and the step size of the measurement grid.

Observation 3:
When the beam peak is directed to the equator of a constant step size measurement grid, the measurement uncertainty decreases significantly.

Observation 4:
Constant density measurement grids provide a lower measurement uncertainty compared to constant step size measurement grids with the same number of measurement points.

The following proposals have been made:

Proposal 1: The minimum number of measurement points to determine the TX and RX beam peak direction is TBD (and to be confirmed by the next meeting); vendors have to assess the MU for the chosen measurement grid type and number of measurement points for antenna arrays similar to the one evaluated.

# References

- R4-1800745, CATR, Anritsu, Intel, “WF on NR MU and test tolerance”, 3GPP TSG RAN WG4 Meeting #AH-1801, San Diego, US, 22 - 26 January 2018

- R4-1802332, Rohde & Schwarz, “On Constant Density TRP Measurement Grids for mm-wave”, 3GPP TSG RAN WG4 Meeting #86, Athens, Greece, 26 February - 2 March 2018

- R4-1804464, Rohde & Schwarz, “On spherical coverage/CDF Measurement Grids for mm-wave”, 3GPP TSG RAN WG4 Meeting #86bis, Melbourne, Australia, 16 – 20 April 2018

- 3GPP TR 37.842 V13.2.0 (2017-03), Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA); Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

- J. J. Thomson, “On the structure of the atom”, Phil. Mag., 7, 237 – 265, 1904