3GPP TSG-RAN WG4 Meeting #86bis		R4-1804616

Melbourne, Australia, 16 - 20 April 2018

Source: 	NTT DOCOMO, INC.

Title: 	PH report mapping table for NR

Agenda item:	7.9.14.2

Document for:	Discussion

- Introduction

At the last RAN4 meeting, field length for PH was discussed in RAN2 and RAN4, and RAN2 agreed that a 6-bits table is used for NR PH and RAN4 agreed that a 6-bits mapping table for PH for both FR1 and FR2 are feasible, which is the same number of bits as in LTE [1][2]. Although PH report mapping range for LTE is from -23 to 40 with 1dB step, NR would need to expand this range because of the larger maximum number of RBs than that in LTE and larger Pcmax,c for mm-wave, and so on. In this contribution, we provide our views on PH report mapping table for NR.

- Discussion

LTE PH report mapping table is defined as table 1, where value range is defined from -23 to 40 with 1dB step. In NR case, PH report range would need to be expanded because the maximum number of RBs and Pcmax,c for mm-wave are larger than those in LTE. Since it was agreed that the number of reported values was 64 (i.e., by using 6-bits) in [2], which is the same number as LTE, the coarser resolution i.e., larger step size would need to be considered in order to cover the wide PH report range necessary in NR. In case that PH report range for NR is expanded from LTE range, the coarser resolution should be applied to the large PH values since the error of the reported value from real value is relatively small compared with the error in the small PH values. Therefore, 1dB resolution should be kept for small PH values.

Observation 1: In NR case, PH range would need to be expanded because maximum number of RBs and Pcmax,c for mm-wave are larger than those in LTE.

Proposal 1: In case that PH reportrange for NR is expanded from that in LTE, the coarser resolution should be applied to the large PH values, and 1 dB resolution should be kept for small PH values.

Table 1: Power headroom report mapping for LTE

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -23 PH -22 |
| POWER_HEADROOM_1 | -22 PH -21 |
| POWER_HEADROOM_2 | -21 PH -20 |
| POWER_HEADROOM_3 | -20 PH -19 |
| POWER_HEADROOM_4 | -19 PH -18 |
| POWER_HEADROOM_5 | -18 PH -17 |
| POWER_HEADROOM_57 | 34 PH 35 |
| POWER_HEADROOM_58 | 35 PH 36 |
| POWER_HEADROOM_59 | 36 PH 37 |
| POWER_HEADROOM_60 | 37 PH 38 |
| POWER_HEADROOM_61 | 38 PH 39 |
| POWER_HEADROOM_62 | 39 PH 40 |
| POWER_HEADROOM_63 | PH ≥ 40 |

In order to define the PH report mapping table for NR, RAN4 has to define the possible maximum and minimum PH values first. Regarding minimum PH value, we think that we could refer the principle in [3], i.e., about -28dB could be considered as minimum PH value. Regarding maximum value of PH, we need further discussion considering power class for FR2, which is under discussion in RF agenda.

Table 2 shows how to define PH report mapping table based on our proposal 1. The maximum and minimum PH values highlighted by red are just example.

Table 2: Power headroom report mapping for NR (Just an example)

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -28 PH -27 |
| POWER_HEADROOM_1 | -27 PH -26 |
| POWER_HEADROOM_2 | -26 PH -25 |
| POWER_HEADROOM_3 | -25 PH -24 |
| POWER_HEADROOM_4 | -24 PH -23 |
| POWER_HEADROOM_5 | -23 PH -22 |
| POWER_HEADROOM_54 | 26  PH  27 |
| POWER_HEADROOM_55 | 27  PH  28 |
| POWER_HEADROOM_56 | 28  PH  30 |
| POWER_HEADROOM_57 | 30  PH  32 |
| POWER_HEADROOM_58 | 32  PH  34 |
| POWER_HEADROOM_59 | 34  PH  36 |
| POWER_HEADROOM_60 | 36  PH  38 |
| POWER_HEADROOM_61 | 38  PH  40 |
| POWER_HEADROOM_62 | 40  PH  42 |
| POWER_HEADROOM_63 | PH ≥ 42 |

1dB step

1dB step

2dB step

2dB step

- Conclusion

In this contribution, we provided our view on PH report mapping table. Our observations and proposals are as follows:

Observation 1: In NR case, PH range would need to be expanded because maximum number of RBs and Pcmax,c for mm-wave are larger than those in LTE.

Proposal 1: In case that PH report range for NR is expanded from that in LTE, the coarser resolution should be applied to the large PH values, and 1 dB resolution should be kept for small PH values.

- Reference

[1] R2-1803863, “LS on field length of Pcmax,c and PH”, NTT DOCOMO

[2] R4-1803498, “Reply LS on field length of Pcmax,c and PHR”, NTT DOCOMO

[3] R4-1800663, “Discussion on LS from RAN2 on PHR mapping table for FR1 and FR2”, Huawei, HiSilicon