3GPP TSG-RAN WG4 Meeting #86bis			R4-1804633

Melbourne, Australia, 16 – 20 April 2018

Title:	[DRAFT] LS response on CRS muting

Response to:	R1-1706734

Release:	Rel-15

Work Item:	LTE_eMTC4-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

Cc:

Contact Person:

#### Name:	Kazuyoshi Uesaka

Tel. Number:

E-mail Address:	kazuyoshi.uesaka (at) ericsson.com

Attachments:	None

1. Overall Description:

RAN WG4 would like to thank RAN WG1 for the LS on CRS muting for Rel-15 LTE-MTC. RAN WG4 have discussed the minimum amount of CRS to be maintained in time and frequency.

For UE demodulation perspective, RAN WG4 has concluded the necessary amount of CRS(s) for X PRB(s) in frequency domain and Y subframes in time domain as follows:

- Frequency domain

  - X=1 PRB for Cat-M1 (1.4MHz UE RF bandwidth)

  - X=0 PRBs for Cat-M2 (5MHz UE RF bandwidth)

- Time domain

  - Cool-down period (after the transmission)

    - Y=1 subframe for CRS-based PDSCH transmission at the end of the last narrowband transmission (both CE Mode A and CE Mode B)

    - Y=0 for other cases

  - Warm-up period (before the transmission)

    - Y=1 subframe before the target MPDCCH and PDSCH transmission (both CE Mode A and CE Mode B)

    - Y=0 for other cases

For RRM perspective, RAN WG4 has concluded the necessary amount of CRS(s) as follows:

- Warm-up/Cool-down

  - Warmup period before active time of DRX (e.g. DRX ON, MIB, paging, SIBx etc):

    - 1 DL subframe

  - Cool down period before active time of DRX:

    - No additional CRS transmission is necessary

- eNB should inform the UE whether CRS muting is enabled or not in the serving cell in both RRC_IDLE and RRC_CONNECTED states

  - In addition, the eNB also informs the UE about the number of PRBs in the centre of the cell bandwidth when CRS muting is enabled, which are:

    - 6 PRBs, or

    - 24 PRBs

- Band scanning/initial cell search

  - When UE is pre-provisioned with EARFCN and its geographical information, UE may assume there is no full cell bandwidth CRS transmissions.

  - When UE is not pre-provisioned with EARFCN and its geographical information, UE shall assume full cell bandwidth CRS transmission in 1 DL subframe every [20] ms.

2. Actions:

To RAN1 group.

ACTION: 	RAN WG4 kindly asks RAN WG1 to take our conclusion into account.

To RAN2 group.

ACTION: 	RAN WG4 kindly asks RAN WG2 to take our conclusion into account and introduce the necessary signalling.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #87	21st – 25th May 2018		Busan, South Korea.

TSG-RAN WG4 Meeting #88 	20th – 24th August 2018	Göteborg, Sweden.