3GPP TSG-RAN WG4 Meeting #86bis															R4-1804634

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:		6.18.3

Source:				Ericsson

Title:					FRC of REFSENS for eMTC UE subPRB transmission

Document for:		Approval

# 1	Introduction

RAN4#86 agreed to add the REFSENS requirement for sub-PRB allocation at BS RF, and define a new FRC for REFSENS for sub-PRB allocation at BS RF [1].

This contribution proposes the FRC for PUSCH with subPRB transmission according to the RAN1 agreements below.

Agreement in RAN1#91 Confirm the above WA and choose option 1 with the following changes: - For Sub-PRB allocation, only the following are supported: - 6 subcarriers with SC-FDMA QPSK modulation, at least for CE Mode A - FFS: CE Mode B - 3 subcarriers with SC-FDMA QPSK modulation - 3 subcarriers with SC-FDMA Pi/2 BPSK modulation - The Pi/2 rotation is performed across SC-FDMA symbols - Use only 2 adjacent subcarriers out of the 3 allocated subcarriers with DFT-spread of length 2 - FFS: which 2 subcarriers out of the 3 allocated subcarriers are used - Working assumption: The 2 used subcarriers shall be fixed per cell in specification - FFS: semi-statically configured - FFS: Frequency hopping case Agreements in RAN1#92 Agreement - For a 2 of 3 sub-carrier pi/2 BPSK transmission, the SC-FDMA symbols over which the DMRS is mapped shall provide a PAPR that is roughly the same as the PAPR of the SC-FDMA symbols not carrying DMRS. - Cross-correlation properties for the DM-RS need to be considered. - Note that Alt. 1 and Alt. 2 may or may not meet the above requirement - Alt. 1: Each DMRS symbol is length-2 BPSK with DFT-S-OFDM - Alt. 1: is the same modulation as data - Alt. 2: Each DMRS symbol is mapped to three sub-carriers - Other alternative(s) are not precluded Agreement - For 6 sub-carriers QPSK allocations, 2 non-overlapping allocations per PRB, i.e., {0, 1, 2, 3, 4, 5} and {6, 7, 8, 9, 10, 11}, will be defined - For 3 sub-carrier QPSK allocations and 2 of 3 sub-carrier pi/2 BSPK allocations, 4 non-overlapping allocations per PRB, i.e., {0, 1, 2}, {3, 4, 5}, {6, 7, 8} and{9, 10, 11}, will be defined - FFS: Which of the above sub-PRB allocations are granted by DCI Agreement - The candidates for FDD are Proposal #1, #2 and #4 for CE mode A and Proposal#1, #2, and #3 for CE mode B and down-select the candidates at RAN1#92bis - Proposal #1:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 8 ms - Proposal #2:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 6 ms - Proposal #3:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 16 ms. - Proposal #4:RU Size of 2 of 3 sub-carriers pi/2 BPSK option = 4 ms FFS whether the above can be reused for TDD or not

# 2	FRC of REFSENS for subPRB PUSCH transmission

Figure 1 illustrates the current RAN1 assumption for 2 out of 3 sub-carriers (sub-PRB) Pi/2 transmitter. From the FRC design point of view, it is equivalent to the BPSK modulation with 2 sub-carriers (1 bit/symbol).

Figure 1	Signal flow of 2 out of 3 sub-carriers Pi/2 PBSK transmitter.

Since one transport block transmission with 2 sub-carriers with 1ms is inefficient from resource utilization, RAN1 is discussing to introduce a ‘resource unit’ where one transport block is transmitted over several subframes, which is similar to NB-IoT UL transmission. As shown in the agreement above, for each CE Mode, there are 3 options in RAN1 for the resource unit length: 8ms, 6ms and 4ms for CE Mode A, and 16ms, 8ms, and 6ms for CE Mode B.

RAN1 keeps one DMRS symbol is transmitted every slot (0.5ms) per subcarrier. This means 12 of 14 OFDM symbols per sub-carrier per subframe (1ms) can be used for PUSCH transmission in the case of normal CP. In the case of 8ms RU length, the channel bits per RU is given as 192 bits (=12 symbols/sub-carrier/1ms * 2 sub-carriers * 8ms * 1bit/symbol).

Table 1 shows our proposal for FRC per RU length candidates. Since the FRC is used for REFSENS simulation and REFSENS is specifed with no repetition case, the proposed FRCs are based on the current RAN1 agreement on CE Mode A. Since RAN4 usually uses 1/3 of coding rate for REFSENS, we set the closet TBS according to the existing specification considering 24 bits CRC and 12 bits tail bits. Note RAN1 is still discussing the MCS/TBS table for sub-PRB PUSCH transmission.

According to our analysis, we can specify FRC with 1RU for Proposals 1 and 2. However we may need 2RUs for Proposal 4 to keep TBS larger than 16 bits.

Depending on the RAN1 agreement, we propose to choose one (or more) FRC(s) for evaluation of PUSCH REFSENS with sub-PRB transmission.

Table 1	Proposed FRC for PUSCH REFSENS with sub-PRB transmission.

| Reference channel | Axx-1 | Axx-2 | Axx-3 |
| --- | --- | --- | --- |
| Length of RU (ms) | 8ms (Proposal 1) | 6ms (Proposal 2) | 4ms (Proposal 4) |
| Tx time | 8ms | 6ms | 8ms |
| Allocated resource blocks | 1 | 1 | 1 |
| Number of subcarriers | 2 out of 3 | 2 out of 3 | 2 out of 3 |
| Allocated resource blocks | 6 | 6 | 6 |
| DFT-OFDM Symbols per subframe | 12 | 12 | 12 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK |
| Code rate | 1/3 | 1/3 | 1/3 |
| MCS index | TBD | TBD | TBD |
| Payload size (bits) | [40] | [24] | [40] |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 |
| Total number of bits per resource unit | 192 | 144 | 192 |
| Total symbols per resource unit | 192 | 144 | 192 |

# 3	References

- R4-1802543, “subPRB feature impact on BS RF”, Ericsson.

- R1-1803010, “efeMTC Key Issue Summary for PUSCH Sub-PRB Allocation”, Sierra Wireless.