3GPP TSG-RAN WG4 Meeting #86bis	R4-1804644

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	6.19.4.1

Source:	Ericsson

Title:	Simulation results of PDSCH demodulation for 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#86 agreed with the test parameters for PDSCH demodulation with 1024QAM as follow [1].

| Transmission Mode | TM4, TM9 |
| --- | --- |
| Antenna configurations | 4 x 2 low, FFS 4 x 4 low |
| Tx EVM | Option 1: 2% Option 2: 1.5% RAN4 should consider the feasibility of test equipment |
| MCS | TBD, coding rate < 0.85, based on RAN1 new MCS table (select lower MCS for 1024QAM) |
| MIMO layer | 2-layer (FSS 1-layer optional) |
| Propagation condition | EPA5 |
| Bandwidth | 10MHz for FDD and TDD |
| Reference receiver | LMMSE or MMSE-IRC |

In this contribution we provide the initial simulation results for evaluation.

# 2	Simulation results

There are mainly two open issues for PDSCH test with 1024QAM: one for Tx EVM and another one is TBS. Figure 1 shows our ideal simulation result with PDSCH 50PRB TM4 2 layers with 2 CRS ports and 4 CRS ports. We set TxEVM to 1.5% and 2.0% as listed in WF, and set TBS options to [52752, 55056, 57336, 59256, 61664], which is agreed in RAN1 for 1024QAM with 50PRB.

Figure 1	Simulation results for PDSCH with 1024QAM.

Table 1	SNR [dB] achieving 70% of maximum throughput.

|  | 2 CRS ports |  | 4 CRS ports |  |
| --- | --- | --- | --- | --- |
|  | Tx EVM=1.5% | Tx EVM=2.0% | Tx EVM=1.5% | Tx EVM=2.0% |
| TBS=52752bits | 33.4 | 33.8 | 30.0 | 30.5 |
| TBS=55056bits | 34.7 | 35.0 | 31.2 | 31.7 |
| TBS=57336bits | 37.3 | 37.9 | 33.5 | 34.5 |
| TBS=59256bits | 36.7 | 37.2 | 33.7 | 34.9 |
| TBS=61664bits | 38.2 | 39.0 | 36.1 | 38.0 |

Table 1 summarizes the SNR [dB] to achieve 70% of the maximum throughput. It is observed that, with TxEVM=1.5%, the required SNR for the lowest TBS (=52752 bits) is 33.4dB with 2 CRS ports and 30.0dB for 4 CRS ports. Considering the additional margin due to RF impairments, we propose to set the lowest TBS=52752bits for PDSCH demodulation requirements.

Proposal 1: Set the lowest MCS (e.g., TBS=52752bits with 50PRB) for PDSCH demodulation test for 1024QAM.

Regarding the Tx EVM options, with TBS=52752, the performance improvement from 2% to 1.5% is 0.4dB for 2 CRS ports and 0.5dB for 4 CRS ports, respectively. Since the required SNR test point is anyway very high (>30dB), we don’t see any help if we set 1.5%. Also in the last meeting, test equipment vendors concerted the Tx EVM of 1.5% especially in higher frequency band such as B42. Therefore we propose to assume TxEVM=2.0% to derive PDSCH demodulation requirement with 1024QAM.

Proposal 2: Assume TxEVM=2.0% to derive the PDSCH demodulation requirements for 1024QAM.

# References

- R4-1803106 “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon.