3GPP TSG-RAN WG4 Meeting #86bis	R4-1804645

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	6.19.4.2

Source:	Ericsson

Title:	Simulation results of SDR test for 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#86 agreed with the test parameters for SDR test with 1024QAM as follow [1].

- Common parameters

  - TM3

  - 2 x 2 (for 2-layer transmission per CC, and 4 x 4 (for 4-layer transmission per CC)

    - For 4x4, the definition of 1024QAM capable UE category should be considered (to address the concern on high SNR, the MCS can be further study)

  - CFI=1

- Tx EVM assumption:

  - Option 1: 2%

  - Option 2: 1.5%

  - RAN4 should consider the feasibility of test equipment

- 85% throughput

In this contribution we provide the initial simulation results for evaluation.

# 2	Simulation results

There are mainly two open issues: one for Tx EVM and another one is TBS. Figure 1 shows our ideal simulation result with PDSCH 50PRB TM3 2x2 rank 2. We set TxEVM to 1.5% and 2.0% as listed in WF, and set TBS options to [52752, 55056, 57336, 59256, 61664], which is agreed in RAN1 for 1024QAM with 50PRB.

Figure 1	Simulation result of SDR with 1024QAM.

Table 1	SNR [dB] achieving 85% of maximum throughput.

|  | Tx EVM=1.5% | Tx EVM=2.0% |
| --- | --- | --- |
| TBS=52752bits | 27.0 | 27.4 |
| TBS=55056bits | 27.6 | 27.6 |
| TBS=57336bits | 29.5 | 29.9 |
| TBS=59256bits | 29.4 | 29.5 |
| TBS=61664bits | 30.5 | 31.6 |

Table 1 summarizes the SNR [dB] to achieve 85% of maximum throughput. It is observed that even with the lowest TBS, the required SNR to achieve 85% of maximum throughput is 27.0dB. Since it is for SDR test, maybe we should test with larger TBS. One possible option is TBS=57336bits because it corresponds to the effective coding rate of 0.8 with CFI=1 and 2 CRS ports, and the required SNR level is same as PDSCH demodulation test under fading condition [2].

Proposal 1: Set TBS=57336 for SDR test with 10MHz (50PRB).

Regarding the TxEVM, the maximum difference between 1.5% and 2.0% is 1.1dB at TBS=61664bits. For lower TBS such as TBS=57336, however, the performance difference regarding Tx EVM is 0.4dB. Moreover in the last meeting, test equipment vendors concern the 1.5% EVM especially for higher frequency band (e.g. B42). Therefore we propose to assume Tx EVM = 2.0% to derive the PDSCH SDR requirements for 1024QAM.

Proposal 2: Assume TxEVM=2.0% to derive the PDSCH SDR requirements for 1024QAM.

# References

- R4-1803106 “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon.

- R4-1804645 “Simulation results of PDSCH demodulation for 1024QAM”, Ericsson.