3GPP TSG-RAN WG4 #86bis	R4-1804985

Melbourne, Australia 16th April –20th April 2018

Source:	Ericsson

Title:	EVM Equalizer Requirements for FR2

Agenda item:	7.6.2.4.2.1

Document for:	Discussion

# 1.	Introduction

In order to complete all core requirements, the EVM normative annex would need also to be completed.  For FR2 EVM requirement is still required to be settled.  During RAN4#85 meeting EVM level for FR2 was set with a range of values in square brackets with the intension to finalize the full EVM requirements.  For FR2 requirements specifics on the Phase Tracking Reference Signal (PT-RS) will need to be considered as part of the EVM requirement under the equalizer specifics. In this contribution we propose an EVM equalizer that takes into account the common phase error (CPE) and employs PT-RS to estimate and compensate for it.

In a companion contribution [2] a draft CR is proposed to implement these changes.

# 2.	Discussion

The Phase Tracking Reference Signal (PT-RS) is designed to be used for estimating one of the two effects of phase noise which is the CPE. The estimated CPE is afterwards used to compensate for the common phase error that is experienced by all the subcarriers across the allocated band.  The other aspect to consider is the residual after CPE compensation, the ICI, is left unattended such that at low subcarrier spacing and higher carrier frequencies, the performance degradation is significant. It is known that phase noise grows a function of carrier frequency. Given these aspects, the PT-RS is used to compensate for distortions as a results of phase noise and improve signal quality would be essential for determining the EVM requirement.

EVM measurement reference point occurs after phase correction has taken place, as illustrated in the figure below.  In FR2 a PT-RS is part of the CPE estimation and then compensation.  It therefore makes sense to take care of a defined PT-RS configuration in order to achieve a minimum requirement for CPE estimation and compensation.  If a common PT-RS configuration is not defined, the risk is an EVM level during the measurement could fail the minimum requirement.

# Figure E.1-1: Reference point for EVM measurement [2]

## 2.1 DM-RS and PR-RS for EVM measurement

The following DM-RS and PT-RS patterns shown in Figure 2 are considered for frequency response and CPE estimation which are later used for proper equalization of the symbol used in EVM measurement.

PTRS every OFDM symbol                   PTRS every 2nd OFDM symbol                PTRS every 4th OFDM symbol

Figure 2a: TYPE-1 Pattern front loaded DM-RS (1 DM-RS symbol).

PTRS every OFDM symbol                    PTRS every 2nd OFDM symbol                PTRS every 4th OFDM symbol

Figure 2b: TYPE-1 Pattern with 1+1 DM-RS (2 DM-RS symbol).

PDCCH     DM-RS       PT-RS     PDSCH

In NR, several PT-RS patterns are supported where each pattern has different density in time (number of occupied OFDM symbols). In Figure 2, only two of the proposed DM-RS patterns are showed. In Figures 2a. and 2b, DR-RS patterns of Type-1 with one and two symbols DM-RSs respectively are shown together with PT-RS patterns of different time densities. Given one of the three PT-RS patterns, a frequency density is chosen on PRB level. That is, PT-RS every PRB or 2nd PRB or 4th … etc.

An accurate EVM measurement requires good frequency response and CPE estimation performance. In FR2, phase noise is deemed to be the dominant source of RF distortion and therefore the need for CPE compensation is essential. In addition to the RF distortion, higher MCSs, are also sensitive to channel estimation performance at the receiver and therefore requires a denser DM-RS pattern, shown in Figure 2b, compared to lower MCSs, where at low speeds, a single symbol (front loaded only) DM-RS pattern, shown in Figure 2a, is sufficient. Therefore, for a consistent EVM measurement, estimation of the frequency response of the EVM equalizer employing the pattern in Figure 2b, that is a denser DM-RS density of 2 symbols and the densest PR-RS, which is pattern 1, would be sufficient for a more accurate and consistent EVM measurement.

## 2.2 Link Level Simulation Results

In a previous contribution [3] the effect of phase noise and the impact of phase noise compensation was presented.  The impact to the overall link performance was studied for chosen EVM levels considered from PN and also from other transmitter impariments.

For the case of 64 QAM and 60 kHz SCS the following illustrates around 40% degradation in throughput without any CPE estimation and compensation taken into account.

Figure 3: 64 QAM, TDL_A channel with 300ns delay spread, 60kHz SCS

If the lower values in the overall range currently defined in TS 38.104 of the current proposed values are to be considered for EVM level, whilst requiring the same level of throughput; then for accurate EVM measurement a PT-RS pattern specified in section 2.1 would be required.

Observation 1:  PT-RS pattern is specified in Figure 2b with PT-RS pattern every OFDM symbol would provide the most accurate CPE estimation and compensation.

The values shown in Table 1 currently consider two values.  The lower end value is the EVM level considered for each MCS in LTE and also for NR FR1.  In these scenarios the PN is minimal.  However, at FR2 frequencies the contribution due to PN in the form of CPE is higher.  The EVM due to PN is estimated below in the its own column in addition to the EVM experienced by other contributions experienced by the transceiver such as clipping, PA nonlinearities, etc. The two values are then considered as root squared summation to arrive at the upper limit.  The following section, section 2.2, outlines how CPE estimation and compensation would need to be considered if the lower limit EVM level is to be considered.  However, even if perfect CPE compensation has been impleted the residual effect of ICI would be present and as such be reflected in the overall EVM level.

Table 1: EVM Budget Derivation

| Modulation scheme for PDSCH | EVM [%] contribution to PN | EVM [%] used in LTE | Overall range for Required EVM [%] |
| --- | --- | --- | --- |
| QPSK | 8% | 17.5% | [17.5 – 19]% |
| 16QAM | 6% | 12.5% | [12.5 – 14]% |
| 64QAM | 4% | 8% | [8 – 9]% |

## 2.2 Common Phase Error (CPE) Estimation and Compensation

Using the PT-RS pattern desnity suggested in section 2.1, at a minimum the CPE estimation and compensation would need to be considered. First, initial estimate of the complex equalization coefficients (frequency response) need to be obtained using the DMRS symbols. Afterwards, since these estimated coefficients will also contain phase rotation due to the CPE, estimation and compensation of the CPE needs to follow.

Setting the post FFT measured signal at subcarrier f and OFDM symbol time t as

Zt,f=I(t,f)H(t,f)ejθ(t)                                          (1)

where

I(t,f) is the corresponding ideal signal reconstructed by the measurement equipment in accordance with relevant Tx models,

Ht,f=a(t,f)ejφ(t,f) is the complex coefficient frequency response of the Tx chain and

θ(t) is the common phase error (CPE), that rotates all the subcarriers of the OFDM symbol at time t.

The equalized symbol is therefore given by

The frequency response coefficients employing the DMRS symbols would then be obtained by

Ht',f'ejθt'=at',f'ejφt',f'ejθt'=Zt',f'It',f'              (2)

where t'∈Tdmrs is a member of the set of time indices of  OFDM symbols containing DMRS symbols, Tdmrs, and f'∈Fdmrs is a member of the set of subcarriers assigned for DMRS symbols, Fdmrs. Note that the CPE, θt', is  independent of f'since it is common to subcarriers. Averaging over DMRS symbols in time would result in the amplitude response as well as the phase response in which the CPE is combined

af'=t'∈Tdmrsat',f'Ndmrs

(3)

φf'+θdmrs=t'∈Tdmrsφt',f'+θt'Ndmrs

where Ndmrs is the number of reference symbol time-domain locations, i.e. Ndmrs=|Tdmrs| and θ is the CPE, θt' averaged over  t'∈Tdmrs. Smoothing and interpolation steps described in annex F.3.4 of [1] would then result in the equalization coefficients for each subcarreries   a(f) and φf+θ such that an estimated complex coefficient frequency response given by

Ht,f=Ht,fejθ=a(t,f)ejφ(t,f)ejθ		              (4)

is obtained. Given the above estimate of the complex coefficients containing the CPE, estimate of the CPE can be obtained using the PT-RS symbols. Similar to (2), frequency response (equalization) coefficients at the PT-RS symbols positions is obtained as

Hptrst'',f''=Ht'',f''ejθt''=at'',f''ejφt'',f''ejθt''=Zt'',f''It'',f''    (5)

where Hptrst'',f'', is the complex frequency response parameters at the PT-RS symbol positions, t'∈Tptrs is a member of the set of time indices of  OFDM symbols containing PT-RS symbols, Tptrs, and f'∈Fptrs is a member of the set of subcarriers assigned for PT-RS symbols, Fptrs. Estimates of the amplitude and phase responses at the PT-RS positions are then obtained by averaging over time positions of the PTRS symbols

af''=t''∈Tptrsat'',f''Nptrs

(6)

φf''+θptrs=t''∈Tdmrsφt'',f''+θt''Nptrs

where Nptrs is the number of PT-RS time symbol locations, i.e. Nptrs=|Tptrs| and θptrs is the CPE, θt'' averaged over  t''∈Tptrs  such that the CPE phase rotated frequency response can be written as

Hptrsf''=Hf''ejθptrs=af''ejφf''ejθptrs.                    (7)

Estimate of the CPE, θ(t), at OFDM symbol time, t, can then be obtained from the expression

θt''=argf''Hptrst'',f'' H*t'',f''                                   (8)

where H*t'',f'' is complex conjugate of estimate of the frequency response in (4). The equalized symbol after CPE compensation is then given by

Z't,f= Z(t,f)Ht,fe-jθ(t)   .                                              (9)

It should be noted depending how how fast the phase noise is varying over time, PT-RS patterns 1, 2 or 4, shown in Figure 1, can be implemented. The denser the PT-RS patter employed, the more accurate the CPE estimation is. For EVM measurement, a more accurate CPE measurement is needed and therefore it would be best to employ PT-RS pattern 1.

# 3.	Conclusions

In conclusion, this contribution takes phase noise into account in the design of EVM equalizer for FR2. Phase noise is the dominant source of RF distortion in FR2 and therefore proper CPE estimation and compensation needs to be integrated in the EVM equalizer. DMRS based frequency response estimation is first implemented as proposed in TS 36.104 [1]. Since the estimated frequency response will contain influence of the CPE, the PT-RSs are initially employed to initially determine the frequency response coefficients at the PT-RS positions which are then used to estimate the CPE as proposed by (8). The estimated CPE is then used to derotate the equalized symbol.

# 4.	References

3GPP TR 36.104 “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception” v14.3.0 (2017-04)

R4-1804984, “Draft CR to TS 38.104: Update to Annex C.1, C.6 Estimation of TX chain amplitude and frequency response parameters”, Ericsson

R4-1713558, “Link Level Simulation Results for EVM FR2”, Ericsson