3GPP TSG-RAN WG4 Meeting #86bis                 	R4-1805080

Melbourne, Australia, April 16th – 20th 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining issues for SSB based CSI-RS

Agenda item:	7.9.10.1.2

Document for:	Discussion

# 1	Introduction

In RAN4#86, RAN4 has further discussed the open issues in RLM requirements, and the remaining open issues are captured in [1]. In RAN#79, the earlier RAN4 agreement to introduce CSI-RS based RLM requirements in Rel-15 was confirmed, so requirements for both SSB and CSI-RS based RLM will be discussed in RAN4.

From [1] the remaining open issues that are specific for CSI-RS based RLM include

- Evaluation period - Companies are encouraged to provide simulation results based on R4-1801301, to conclude on the RLM requirements - Hypothetical PDCCH parameters - FFS

In this paper, we will provide our views on evaluation period and reference CORESET for PDCCH parameters for CSI-RS based RLM.

2	Discussion

## 2.1	Evaluation period requirements

For SSB based RLM, RAN4 conducted simulation to check how many samples are needed by UE to accurately measure SNR. Conclusion is that 5 samples are needed at Qin condition and 10 at Qout condition. For CSI-RS based RLM, we have provided our simulation results [2] for the SNR measurement accuracy with different SNR points, measurement BW and RS density.

In this paper we will provide additional simulation results to find out the reasonable side conditions and number of samples for evaluation period for CSI-RS based RLM. In our companion paper for PDCCH performance evaluation, we have given our suggestions on the PDCCH parameters and the corresponding Qout and Qin level. Following our proposal,

- Qout would be around -10.6dB for AWGN and -7.3dB for fading channels.

- Qin would be around -4.4dB for AWGN and -0.6dB for fading channels.

In the following discussion, we will focus on the results with SNR -11dB for Qout and -6dB for Qin. For the accuracy, as the parameter design targets >5dB gap between Qout and Qin, we will focus on absolute accuracy of 3dB.

Table 1 shows the SNR estimation accuracy for 15kHz SCS and AWGN for below cases.

- PRB = 24, D = 1

- PRB = 24, D = 3

- PRB = 96, D = 1

- PRB = 96, D = 3

Table 1: SNR estimation accuracy for 15kHz SCS and AWGN

| PRB = 24, D = 1 | PRB = 24, D = 3 |
| --- | --- |
| PRB = 96, D =1 | PRB = 96, D = 3 |

From the results, it can be seen that in order to get 3dB accuracy at -11dB SNR condition, we need

- PRB = 24, D = 1: more than 20 samples

- PRB = 24, D = 3: 10 samples

- PRB = 96, D = 1: 10 samples

- PRB = 96, D = 3: 5 samples

From the results, it can be seen that in order to get 3dB accuracy at -6dB SNR condition, we need just 5 samples for all cases.

In  Table 2 we show the SNR estimation accuracy for 120kHz SCS and CDL-C channel. The same observations can be made. Actually we have observed the same for all combinations of SCS and propagation channels, but due to space limit, we cannot put all the simulation results in the paper.

Since the BW and density of CSI-RS for RLM is configured by network, it is then a question for which configurations RAN4 should define UE requirements. Based on our simulation results, our suggestion is that the requirements are defined for two cases below since they lead to reasonable measurement burden at the UE. The required number of samples are same for the two cases, i.e. 5 samples at Qin and 10 samples at Qout.

- PRB = 24, D = 3

- PRB = 96, D = 1

Table 2: SNR estimation accuracy for 120kHz SCS and CDL-C (30ns, 30km/h)

| PRB = 24, D = 1 | PRB = 24, D = 3 |
| --- | --- |
| PRB = 96, D =1 | PRB = 96, D = 3 |

Evaluation period requirements for CSI-RS based RLM are defined based on 5 samples at Qin and 10 samples at Qout for the below conditions

- 	PRB = 24, D = 3

-	PRB = 96, D = 1

## 2.2	Reference CORESET for PDCCH parameters

For SSB based RLM, the hypothetical PDCCH parameters are mainly based on RMSI CORESET. This is reasonable since SSB is most likely to be transmitted using cell level beams, and can better represent a common control channel performance. CSI-RS is, on the other hand, UE specific, so the parameters like SCS and BW of PDCCH may need to follow some UE specific CORESET configuration.

According to RAN1, for each BWP at most 3 CORESETs can be configured, and we think one of the configured CORESETs can be used as the reference for hypothetical PDCCH for RLM. In RAN4#86, some options are proposed by companies, including

  - Option 1: Based on Coreset has QCL relationship with CSI-RS

  - Option 2: Based on first Coreset within default activated BWP

  - Option 3: Based on RMSI coreset (same as for SSB based RLM)

In our view, option 1 may not work. Network may configure up to 3 CORESETs, and for each CORESET there is a TCI state which indicates the source RS which UE can assume to be QCL’d with the DMRS of the CORESET, i.e. the source RS is the beam used for PDCCH. However, the source RS of a CORESET may or may not be a CSI-RS for RLM. In other words, a specific RLM-RS may not be the source RS in the TCI state of any configured CORESET, i.e. we cannot find a CORESET that is QCL’d with the CSI-RS for RLM. On the other hand, it may also happen that different CSI-RS resources for RLM may be indicated as the source RS for different CORESETs, and which should be used to derive the hypothetical PDCCH parameters is still unclear.

Option 3, as discussed above, is not proper for CSI-RS based RLM. Therefore, our preference is option 2, i.e. the hypothetical PDCCH parameters are simply derived from the first CORESET in the active BWP. Actually, network would ensure the configured RLM-RS for RLM are QCL’d with the CORESET, otherwise the link quality monitored on the CSI-RS cannot represent the performance of the real PDCCH.

The hypothetical PDCCH parameters for CSI-RS based RLM are derived based on first CORESET in the active BWP.

3	Conclusion

In this paper we provided our views on the remaining issues for CSI-RS based RLM.

Proposal 1: Evaluation period requirements for CSI-RS based RLM are defined based on 5 samples at Qin and 10 samples at Qout for the below conditions

Proposal 2: The hypothetical PDCCH parameters for CSI-RS based RLM are derived based on first CORESET in the active BWP.

References

- R4-1803477, WF on RLM requirements for NR, Nokia, Nokia Shanghai Bell

- R4-1802401, Simulation results for CSI-RS based RLM measurements, Nokia, Nokia Shanghai Bell