3GPP TSG-RAN WG4 Meeting #86	 R4-1805113

# Melbourne, Australia, 16-20 April, 2018

Agenda Item:	7.12.1.

Source: 	Sony, Ericsson

Title: 	UE Grey Box Approach for FR 2 OTA tests

Document for:	Approval

Introduction

It can be time consuming to discover at which direction in space a UE offer its beam peaks from FR2 OTA measurements. At R4 #84 it was decided to adopt the “black box approach” described in [1] meaning no information about antenna positioning within the UE needs to be provided by manufacturers prior to qualification measurements. This approach does however not prevent applicants from providing their UE: s beam peak direction(s).

Proposal

It is here proposed the requirement specification TS 38.101-2 to demand manufacturers to declare their UE: s TX and RX beam peak directions respectively according to the “Grey box” approach in the below table. Thereby a UE can be characterized more quickly and precise with respect to its OTA parameters. If its peak beam direction is aligned well with the conditions required by a testing chamber in fact the power class parameter minimum EiRP max can be demanded with a tighter tolerance than by the black box approach, i.e.: a higher value and is due to the lower MU expected.

Please note that this proposal is not in conflict with the agreed black box approach since no information about antenna designs nor their detailed positions is required.

| UE approach | Idea | RAN 4 decision | Effects on OTA testing |  | Ref. |
| --- | --- | --- | --- | --- | --- |
| White box | Antenna locations in UE provided by manufacturer | Stopped | Low Measurement Uncertainty due to accurate positioning in QZ |  |  |
| Black box | Out of the box testing | Approved | Worse MU, larger risk of not detecting best EiRP and EiS |  | [1] |
| Pink box | Limited sector in space only testing | Noted | Quicker EiRP/EiS or FoM testing at a typical user position |  | [2] |
| Grey box | Manufacturers to declare TX and RX beam peak directions resp. | This paper | Quick and accurate beam peak tests. Beam correspondence information automatically provided |  | [3] |

Table 1. Overview of FR 2 UE OTA testing approaches discussed at recent RAN 4 meetings. (QZ=quiet zone in anechoic chambers in which a UE under test is located).

Conclusion

It is proposed UE manufacturers to declare at which direction in space their UE: s obtain TX and RX beam peak responses respectively. The lower MU expected compared to the black box approach offers min. peak EiRP to be specified at a higher value. Hence also the beam correspondence spatial components are provided, all with the objective of speeding FR 2 OTA tests up.

References

[1]	R4-1708200, #84, R&S, Black Box vs. White Box Testing for NR.

[2]	R4-1709718, NR ad-hoc #3 2017, Sony, Pink Box Approach for Rapid Range 2 UE´s OTA Performance testing.

[3]	R4-1802336, #86, R&S, On Peak Directions for FR 2.