3GPP TSG-RAN WG4 Meeting #86bis	R4-1805325
Melbourne, Australia April 16-20, 2018

Agenda item:	7.4.8.2

Source:	Apple Inc., Intel Corporation

Title:	Impact of Peak EIRP on Network Performance

Document for:	Discussion

# 1	Introduction

During the RAN #73 meeting, the study item on New Radio access technology [1] was finalized with its outcome captured in TR 38.803 [2]. During RAN #75 the New Radio Work Item was initiated [3]. With an agreement to define and test RF requirements for NR mm-wave over the air (OTA), the discussion on the minimum peak EIRP requirement has evolved over a number of agreements.

In RAN4#85, a WF on power class in FR2 was agreed in [4]:

Agreements on peak EIRP:

- The handheld UE peak EIRP range is defined as follows and will be captured in TS38.101-2 in this meeting

  - For 28 GHz: [21.2-25.2] dBm

  - For 39 GHz: [19.4-23.7] dBm

Most recently, in RAN4#86, two WFs on power class in FR2 were proposed in [8] and [9]. They proposed to use following peak EIRP values:

Proposals for Rel. 15:

  - R4-1803460 [8]:

    - Use the 50th percentile value of reported data for min peak EIRP definition

    - For 28GHz = 22.0 dBm

    - For 39GHz = 20.0 dBm

  - R4-1803372 [9]:

    - For 28GHz:

      - Min peak EIRP is 23.2 dBm

      - 50%-tile requirement for EIRP CDF is 13.8dBm, 9.4 dB down from peak (above)

    - For 39GHz :

      - Min peak EIRP is 21 dBm

      - 50%-tile requirement for EIRP CDF is FFS

During the last several RAN4 meetings, we have provided several simulation results evaluating the impact of peak EIRP and EIRP CDF on the network performance.  A brief summary of our previous contributions is provided below:

RAN#84bis:

- We provided our view on UE output power (peak EIRP) budget calculation including realistic losses [10]. In summary, we found that realistic form-factor, implementation and manufacturing constraints significantly reduce the peak EIRP.

RAN4#85:

- We provided simulation results in [11] showing the impact of peak EIRP on UL throughput and outage performance of indoor office and dense urban scenarios when simulation parameters were based on TR38.803. We showed that for aforementioned scenarios the mean UL throughput loss is less than 2% when peak EIRP is varied between 22 dBm and 30 dBm. For the same scenarios, we showed that outage performance was also comparable for the same range of peak EIRP.

- We also shared our views on the EIRP CDF for a modern form factor in [5] and compared it to a form factor in which the radiating elements see largely plastic. Our results showed that when the phone model employs features which are common to the modern cellular phone, including a full display OLED/LCD and a metal bezel, spherical CDF performance is significantly compromised relative to that of a generally plastic and benign form factor.

RAN4#AH-1801:

- We showed further simulation results on the outage and throughput performance of the urban macro scenario using the EIRP CDF in [7]. We showed that the outage probability of the urban macro scenario stays under 5% when the ISD is smaller than 200 m and peak EIRP is within the 21.2-25.2 dBm range [7].

In this paper, we provide simulation results comparing the throughput and outage performance when peak EIRP takes values of 22 dBm and 23 dBm. We simulated the indoor office, dense urban, and urban macro deployment scenarios using the updated assumptions provided in the WF [6]. For each scenario, we also compare the network performance using antenna pattern with CDF presented in [5] with that of the 3GPP antenna pattern from TR 38.803.

# 2	Simulation Results

In RAN4#AH-1801, companies provided views on the relevant parameters and scenarios in TR 38.803 and necessary modifications to address the spherical coverage performance as summarized in the WF [6]. To be consistent with the spherical performance simulations, we use the same updated simulation assumptions from [6] in this paper. The goal is to compare the throughput and outage performance of all three network deployment scenarios (indoor office, dense urban, and urban macro) at 30 GHz when the peak EIRP takes values of 22 dBm and 23 dBm. As mentioned above, we use both of our proposed EIRP CDF from [5] and the EIRP CDF introduced in TR 38.803 to compare the network performance for the same range of peak EIRP values. Figure 1 demonstrates the resulting CDF by application of a spatial mask [7] to the baseline antenna pattern in TR 38.803. The resulting model is a good approximation of the CDF presented by us in [5].

Figure 1: EIRP CDF.

## 2.1	Outage Performance

We simulated the coverage performance for the indoor, dense urban, and urban macro scenarios using the minimum peak EIRP values of 22 dBm and 23 dBm on the uplink. We defined the outage as a SNR below -10 dB and compared the outage probability of our CDF [5] with that of the CDF defined in TR 38.803.  We observed that there is no outage in the indoor office scenario when the peak EIRP is 22 dBm or 23 dBm for both CDF profiles.

Observation 1: In the indoor office scenario, there is no outage when the peak EIRP is 22 dBm or 23 dBm.

The indoor-outdoor ratios of 0%, and 20% were simulated for the dense urban and urban macro scenarios. Per the WF [6], we simulated ISDs of 200 m and 400 m. The outage performance for the dense urban and urban macro scenarios are described below.

### 2.1.1	Dense Urban Scenario

Figures 2(a) and 2(b) show the outage performance of the dense urban scenario when the masked CDF [5] and the original CDF from TR 38.803 are used respectively. We observe that there is no outage when all the users are outdoor and the UE peak EIRP is 22-23 dBm. Moreover, we observe the difference in the outage performance between EIRP 22 or 23 dBm is always less than 3% for both CDFs even when all users are indoor.

- (b)

Figure 2: Outage Probability in the Dense Urban Scenario

Observation 2: In the dense urban deployment scenario, there is no outage when all the users are outdoor and peak EIRP is 22 or 23 dBm.

Observation 3: In the dense urban deployment scenario, the difference in the outage performance between 22 or 23 dBm EIRP is always less than 3% for both CDFs even when all users are indoor.

Observation 4: In the dense urban deployment scenario, peak EIRP does not influence outage performance as much as the antenna pattern assumption itself.

### 2.1.2	Urban Macro Scenario

Figures 3(a) and 3(b) show the outage performance of the urban macro scenario for UE peak EIRP values of 22 dBm or 23 dBm and the realistic masked CDF [5] and the original CDF from TR 38.803 are used respectively. We observe that for both CDFs and peak EIRP values, the outage probability is less than 1% when ISD is 200 m and all users are outdoor. Further, the difference in the outage probabilities associated with peak EIRP values of 22 dBm or 23 dBm is always less than 3% for ISDs of 200 m and 400 m.

(a)                                                                                                   (b)

Figure 3: Outage Probability in Urban Macro Scenario

Observation 5: In the urban macro deployment, the outage probability is less than 1% when ISD is 200 m, all users are outdoor and peak EIRP is 22 or 23 dBm.

Observation 6: In the urban macro deployment, the difference in the outage probabilities associated with peak EIRP values of 22 dBm or 23 dBm is always less than 3% for ISDs of 200 m and 400 m.

## 2.2	UL Throughput Performance

We simulated UL throughput performance for the indoor, dense urban, and urban macro scenarios when the peak EIRP takes values of 22 or 23 dBm. We compared the mean and 5%-tile UL throughput performance of the masked CDF presented in [5] with that of the CDF defined in TR 38.803.

### 2.2.1 Indoor Office Scenario

For the indoor office scenario, we simulated UL throughput of the masked CDF [5] and the CDF derived from the TR 38.803 antenna pattern when the peak EIRP takes values of 22 or 23 dBm. Figures 4(a) and 4(b) show the percentage of UL throughput loss between peak EIRP of 22 dBm and 23 dBm. We observe that the mean UL throughput loss when peak EIRP is 22 dBm is less than 3% and 2% for the masked CDF in [5] and TR 38.803 antenna pattern respectively. The 5%-tile UL throughput loss is less than 1% for both CDFs.

Observation 7: For the indoor office scenario, the mean UL throughput loss for peak EIRP of 22 dBm is less than 3% and 2% for the realistic masked CDF and the CDF derived from TR 38.803 antenna pattern respectively.

- (b)

Figure 4: UL Throughput Loss(%) in Indoor Office Scenario

### 2.2.2 Dense Urban Scenario

For the dense urban scenario, we simulated UL throughput of the masked CDF [5] and the CDF derived from the TR 38.803 antenna pattern when the peak EIRP takes values of 22 or 23 dBm. We simulated indoor-outdoor user ratios of 0% and 20%. Figures 5(a) and 5(b) show the percentage of UL throughput loss between peak EIRP of 22 dBm and 23 dBm. We observe that the mean UL throughput loss when peak EIRP is 22 dBm is less than 2% and 1% for the masked CDF [5] and the CDF derived from TR 38.803 respectively. We also observe that the 5%-tile UL throughput loss for peak EIRP 22 dBm when all users are outdoor, is less than 2% for both the realistic masked CDF and the CDF derived from the TR 38.803.

(a)                                                                                             (b)

Figure 5: UL Throughput Loss(%) in Dense Urban Scenario

Observation 8: In the dense urban deployment, the mean UL throughput loss for peak EIRP 22 dBm is less than 2% and 1% for the realistic masked CDF and the CDF derived from TR 38.803 respectively.

Observation 9: In the dense urban deployment, the 5%-tile UL throughput loss for peak EIRP 22 dBm when all users are outdoor, is less than 2% for both the realistic masked CDF and the CDF derived from t TR 38.803.

### 2.2.3	Urban Macro Scenario

For the urban macro scenario, we simulated UL throughput of the CDF [5] and the CDF derived from the TR 38.803 antenna pattern when the peak EIRP takes values of 22 or 23 dBm. We simulated indoor-outdoor user ratios of 0% and 20%. Figures 6(a) and 6(b) show the percentage of UL throughput loss between peak EIRP of 22 dBm and 23 dBm. We observe that the mean UL throughput loss for peak EIRP 22 dBm is less than 5% for both the masked CDF [5] and the CDF derived from TR 38.803. We also observe that the 5%-tile UL throughput loss for peak EIRP 22 dBm when the ISD is 200 m and all users are outdoor, is less than 8% and 6% for the masked CDF [5] and the CDF derived from the TR 38.803 respectively. Furthermore, we observe that the 5%-tile UL throughput loss for peak EIRP 22 dBm when the ISD is 400 m and all users are outdoor is more than 11% for both the masked CDF [5] and the CDF derived from TR 38.803. Finally, we note that in the UMa deployment, the UL throughput performance of the two CDFs is comparable. We can therefore conclude that the spherical CDF is not critical to the relative UL throughput performance when comparing EIRP values of 22 and 23 dBm.

(a)                                                                                             (b)

Figure 6: UL Throughput in Urban Macro Scenario

Observation 10: In the urban macro deployment, the mean UL throughput loss for peak EIRP 22 dBm is less than 5% for both the realistic masked CDF and the CDF derived from TR 38.803.

Observation 11: In the urban macro deployment, the 5%-tile UL throughput loss for peak EIRP 22 dBm when ISD is 200 m and all users are outdoor, is less than 8% and 6% for the realistic masked CDF and the CDF derived from TR 38.803 respectively.

Observation 12: In the urban macro deployment, the 5%-tile UL throughput loss for peak EIRP 22 dBm when ISD is 400 m and all users are outdoor, is more than 11% for both the realistic masked CDF and the CDF derived from the TR 38.803.

Observation 13: In the urban macro deployment, the spherical CDF is not critical to the relative UL throughput performance when comparing EIRP values of 22 and 23 dBm.

## 2.3	DL Throughput Performance

We note that the DL throughput of the network is not in general a function of the peak EIRP value in UL.

Observation 14: In all deployment scenarios, there is no DL throughput loss between peak EIRP values of 22 or 23 dBm.

# 3	Conclusions

In this contribution, we provided simulation results on the throughput and outage performance for indoor, dense urban, and urban macro scenarios when the peak EIRP takes values of 22 or 23 dBm.

Based on the simulation results, the following observations are made:

Observation 1: In the indoor office scenario, there is no outage when the peak EIRP is 22 dBm or 23 dBm.

Observation 2: In the dense urban deployment scenario, there is no outage when all the users are outdoor and peak EIRP is 22 or 23 dBm.

Observation 3: In the dense urban deployment scenario, the difference in the outage performance between EIRP 22-23 dBm is always less than 3% for both CDFs even when all users are indoor.

Observation 4: In the dense urban deployment scenario, peak EIRP does not influence outage performance as much as the antenna pattern assumption itself.

Observation 5: In the urban macro deployment, the outage probability is less than 1% when ISD is 200 m and all users are outdoor and peak EIRP takes values of 22 or 23 dBm.

Observation 6: In the urban macro deployment, the difference in the outage probabilities associated to peak EIRP values of 22 dBm and 23 dBm is always less than 3% for ISD of 200 m and 400 m.

Observation 7: For the indoor office scenario, the mean UL throughput loss of peak EIRP = 22 dBm is less than 3% and 2% for the realistic masked CDF and the CDF derived from TR 38.803 antenna pattern respectively.

Observation 8: In the dense urban deployment, the mean UL throughput loss of peak EIRP = 22 dBm is less than 2% and 1% for the realistic masked CDF and the CDF derived from the TR 38.803 respectively.

Observation 9: In the dense urban deployment, the 5%-tile UL throughput loss of peak EIRP = 22 dBm when all users are outdoor, is less than 2% for both realistic masked CDF and the CDF derived from the TR 38.803.

Observation 10: In the urban macro deployment, the mean UL throughput loss of peak EIRP = 22 dBm is less than 5% for both the realistic masked CDF and the CDF derived from the TR 38.803.

Observation 11: In the urban macro deployment, the 5%-tile UL throughput loss of peak EIRP = 22 dBm when ISD is 200 m and all users are outdoor, is less than 8% and 6% for the realistic masked CDF and the CDF derived from the TR 38.803 respectively.

Observation 12: In the urban macro deployment, the 5%-tile UL throughput loss of peak EIRP = 22 dBm when ISD is 400 m and all users are outdoor, is more than 11% for both the realistic masked CDF and the CDF derived from the TR 38.803.

Observation 13: In the urban macro deployment, the spherical CDF is not critical to the relative UL throughput performance when comparing EIRP values of 22 and 23 dBm.

Observation 14: In all deployment scenarios, there is no DL throughput loss between peak EIRP values of 22-23 dBm.

Based on observations above, we found that the network performance in terms of the outrage, mean UL throughput loss, 5%-tile UL throughput loss, and DL throughput loss are at least very close if not equal when the peak EIRP takes values of 22 dBm and 23 dBm. We therefore suggest to use the smaller peak EIRP value to reduce the burden on UE vendors. Furthermore, as reported in [12], increasing the minimum peak EIRP from 22 dBm to 23 dBm reduces the passing rate from 50% to 25% per reports provided by UE vendors. It is therefore suggested to define the minimum peak EIRP value of 22 dBm for the 28 GHz band. Accordingly, it is suggested to define the minimum peak EIRP value of 20 dBm for the 39 GHz band.

Proposal 1: Use the minimum peak EIRP value of 22 dBm for the 28 GHz band.

Proposal 2: Use the minimum peak EIRP value of 20 dBm for the 39 GHz band.

# 4	References

- RP-161596, “Study on New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #73, September 2016.

- TR 38.803, “Study on New Radio Access Technology RF and co-existence aspects,” Version 2.0.0, 3GPP RAN #75, March 2017.

- RP-170855, “Work Item on New Radio (NR) Access Technology,” NTT DOCOMO, 3GPP RAN #75, March 2017.

- R4-1714445, “WF on pi/2 BPSK spectrum shaping and power class in FR2”, Intel Corporation, Apple, Qualcomm, IITH, LGE, Nokia, Huawei, Motorola Mobility, 3GPP RAN4 #85, November 2017.

- R4-1713850,	“Consideration of EIRP spherical coverage requirement”, Apple Inc., Intel Corporation, 3GPP RAN4 #85, November 2017.

- R4-1801198, “WF on network performance analysis for spherical coverage”, Samsung, RAN4#AH-1801, January 2018.

- R4-1800285,	“Assumptions for Simulating Spherical Coverage Performance”, Apple Inc., Intel Corporation, RAN4#AH-1801, January 2018.

- R4-1803460,	“WF on FR2 peak EIRP for handheld UE”, Intel, Xiaomi, OPPO, vivo, MediaTek, [Huawei], LGE, Apple, Qorvo, RAN4 #86, February 2018.

- R4-1803372, "WF on output power requirements on FR2", Qualcomm, et.al., RAN4 #86, February 2018.

- R4-1711406, “UE power class proposals for mm-wave”, Apple Inc., Intel Corporation, 3GPP RAN4 #84bis, October 2017

- R4-1714003, “Network performance impact of practical peak EIRP”, Apple Inc., Intel Corporation, 3GPP RAN4 #85, November 2017.

- R4-1801786, “Peak EIRP for FR2 handheld UE type”, Intel Corporation, Apple Inc., 3GPP RAN4 #86, February 2018.