3GPP TSG-RAN WG4 Meeting #86bis	R4-1805390

Melbourne, Vic, Australia, 16th – 20th April 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on TRP approximation errors – names and definitions

Agenda item:	6.26.3.1.4

Document for:	Discussion

# 1	Introduction

During the last RAN4#86 meeting, the WF on OTA TRP conformance requirements [1] and the WF on TR structure [2] were approved. The former outlined a list of open issues to be addressed in forthcoming meetings and the latter provided a skeleton for capturing estimation methods, OTA test methods and measurement uncertainty of TRP.

In [1][2], the term “TRP estimation accuracy” is used without a proper definition, which is open to different interpretations during discussions. Furthermore, such a term has not been agreed in RAN4. To have a common understanding, RAN4 needs to agree on the name and definition of TRP estimation accuracy. This document attempts to provide such an unambiguous name and definition.

# 2	Discussion

## 2.1 TRP approximations

In Rel-15 TS 37.105 [3], TRP-based OTA requirements can be divided into two groups:

- In band

  - OTA BS maximum output power

  - OTA ACLR

  - OTA spectrum emission mask

  - OTA operating band unwanted emissions

- Out of band

  - OTA TX spurious emissions

  - OTA RX spurious emissions

From Rel-15 TS 37.105 [3], TRP is defined as the total power radiated by the antenna; i.e., TRP is the actual power radiated by the antenna of DUT.

In the far field region, the TRP of the DUT is obtained by integrating EIRP patterns of the DUT over the whole spherical surface, that is

TRPactual= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ					(W)										(1)

In a test chamber, the definite integral of TRP in Equation (1) is approximated by summations of discrete EIRP data samples that are measured on a grid in two spherical coordinates, the elevation angle θ∈[0,  π] and the azimuth angle ϕ∈[0, 2π]. Examples of numerical integration techniques include Riemann sum, Simpson rule, Trapezoid rule, etc.

Let us assume a spherical equal angle (also known as uniform) sampling grid, then θ and ϕ are discretized into N and M equally spaced angular subintervals. Let n and m be the indices used to denote the nth θ and mth ϕ angles, respectively. Then the width of each subinterval in the θ- and ϕ-angle is defined as

Δθ= θn- θn-1= π-0N     					n∈[0,N]																(2a)

∆ϕ= ϕm- ϕm-1= 2π-0M  				m∈[0,M]															(2b)

Using the numerical integration technique for approximating the definite integral of ϕ in Equation (1), we get

TRPapprox≈ 14πθ=0πΔϕ2m=0M-1EIRPθ,ϕm+EIRP(θ,ϕm+1)sinθdθ										(3)

Equation (3) can be rearranged as

TRPapprox= 14πθ=0π∆ϕm=1M-1EIRPθ,ϕm+ ∆ϕ2EIRPθ,ϕ0+EIRP(θ,ϕMsinθdθ					(4)

Since ϕ0=0 and ϕM=2π, EIRPθ,ϕ0=EIRP(θ,ϕM), Equation (4) can be rewritten as

TRPapprox= Δϕ4πθ=0πm=0M-1EIRP(θ,ϕm)sinθdθ																(5)

Following the same principle as for the ϕ integral, the definite integral of θ can be approximated as

TRPapprox = ∆ϕ4π∆θn=1N-1m=0M-1EIRP(θn,ϕm)sinθn+ ∆θ2m=0M-1EIRP(θ0,ϕm)sinθ0+ m=0M-1EIRP(θN,ϕj)sinθN  																						(6)

Since sin(θ0=0) = sin(θN=π), Equation (6) becomes

TRPapprox=∆ϕΔθ4πn=1N-1m=0M-1EIRP(θn,ϕm)sinθn																(7)

Using Equations (2a) and 2(b), Equation (7) becomes

TRPapprox=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn																(8)

In the test chamber, each EIRPθn,ϕm is the sum of two orthogonally polarized components, EIRPco(θn,ϕm) and EIRPx(θn,ϕm), which correspond to the polarized components of the DUT transmit antenna, respectively.

In [5], we showed that the absolute error as a result of using the approximating technique is the difference between the value of the definite integral and the numerical result, that is

TRPactual- TRPapprox=  ϵabs>0,   ∀N,M                where 0<N<∞ and  0<M<∞   				(9)

Representing the absolute error as a relative error

TRPactual-TRPapproxTRPactual=ϵrel>1, ∀N,M                where 0<N<∞ and  0<M<∞    						(10)

TRPapprox= TRPactual as N and M approach ∞, that is

limN,M→(∞,∞)TRPapprox= TRPactual																				(11)

Observation 1: for every positive integer number N and M, there exists an absolute error ϵabs>0 in approximating TRPactual.

Observation 2: the error strongly depends on N and M, i.e., ϵ→0 when N→∞ and M→∞.

Observation 3: the error is one contributor to TRP measurement uncertainty; relative error ϵrel is preferred.

As there is more than one sampling grid candidate that can be used to numerically approximate the TRP integral, the relative error should be based on the spherical equal angle sampling grid.

Observation 4: if RAN4 is to specify ϵrel then it should be based on the TRP approximation in Equation (8) which is derived from the spherical equal angle sampling grid rather than other grids such as equal area sampling grid.

Observation 5: the spherical equal angle sampling grid should serve as the baseline grid.

## Name and definition

The name used in [1][2] is “TRP estimation accuracy” but it is not defined. Thus, we assume it is the error attributed to approximating the actual TRP by numerical integration as discussed in Section 2.1.

In addition, the following names are already used in Rel-15 TR 37.843 [4]:

- TRP accuracy

- EIRP accuracy

TRP and EIRP accuracy are related to the AAS BS output power requirements only while TRP estimation accuracy is used for all TRP-based OTA requirements (including four in band and two out of band requirements).

“TRP accuracy” could easily be misinterpreted as “TRP estimation accuracy” and vice versa, causing confusion, since both terms contain the word “accuracy”. We propose the following unambiguous name and definition:

Proposal:

| Name | Definition |
| --- | --- |
| TRP estimation error | The difference between the actual TRP and numerically approximated TRP |

# 3	Conclusions

We have discussed the issue of approximating TRP using a numerical technique. Our observation is as follows:

Observation 1: for every positive integer number N and M, there exists an absolute error ϵabs>0 in approximating TRPactual.

Observation 2: the error strongly depends on N and M, i.e., ϵ→0 when N→∞ and M→∞.

Observation 3: the error is one contributor to TRP measurement uncertainty; relative error ϵrel is preferred.

Observation 4: if RAN4 is to specify ϵrel then it should be based on the TRP approximation formula given in Equation (8) which is derived from the spherical equal angle sampling grid rather than other grids such as equal area sampling grid.

Observation 5: the spherical equal angle sampling grid should serve as the baseline grid.

Based on the above observations, we propose the name and definition for the error in TRP estimation:

Proposal:

| Name | Definition |
| --- | --- |
| TRP estimation error | The difference between the actual TRP and numerically approximated TRP |

# References

- R4-1803392, WF on OTA TRP conformance requirements, Ericsson

- R4-1803388, WF on TR structure for Section 10, Ericsson

- TS 37.105 v15.0.0: AAS BS transmission and reception (Release 15)

- TR 37.843 v15.0.0, RF requirement background for AAS BS radiated requirements (Release 15)

- R4-1701833, On accuracy of approximating TRP, Nokia, Alcatel-Lucent Shanghai Bell