3GPP TSG-RAN WG4 Meeting #86bis	R4-1805391

Melbourne, Vic, Australia, 16th – 20th April 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On spherical equal angle sampling grid for numerical TRP approximations

Agenda item:	6.26.3.1.4

Document for:	Discussion

# 1	Introduction

During the last RAN4#86 meeting, the WF on OTA TRP conformance requirements [1] was approved, which outlined a list of open issues to be addressed. One of the open issues is numerical integration methods for TRP estimation.

In our companion document [2], a definition and an alternative name for “TRP estimation error” is provided. This document discusses numerical methods that apply on a spherical equal angle (also known as spherical uniform) sampling grid for TRP approximations.

# 2	Discussion

For OTA measurements, the DUT is positioned in the center of a sphere. In the far field region, the TRP of the DUT is obtained by integrating radiated power (i.e., EIRP) patterns of the DUT over the whole spherical surface

TRPactual= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ					(W)										(1)

As discussed in [2], various numerical integration techniques can be applied to approximate the definite integral of TRP in Equation (1).

Spherical equal angle sampling grid:

A spherical sampling grid is defined using the spherical coordinate system. A point in space is represented by (r, θ,ϕ), where r is the radius, θ∈[0, π] is the elevation angle and ϕ∈0,2π is the azimuth angle.

With the spherical equal angle sampling grid, the elevation angle interval [0, π] and azimuth angle interval 0,2π are discretized into a numeral grid, consisting of N and M equally spaced angular subintervals, respectively. Let n and m be the indices used to denote the nth θ and mth ϕ angles, respectively. Then, the width of each subinterval in the θ- and ϕ-angle is defined as

Δθ= θn- θn-1= π-0N=constant      		n∈[0,N]															(2a)

∆ϕ= ϕm- ϕm-1= 2π-0M=constant  		m∈[0,M]														(2b)

As shown in [2], the numerical approximation of Equation (1) is

TRPapprox=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn																(3)

In the test chamber, each EIRPθn,ϕm is the sum of two orthogonally polarized components, EIRPco(θn,ϕm) and EIRPx(θn,ϕm), which correspond to the co- and cross-polarization of the DUT transmit antennas, respectively.

In Equation (1), the total number of sampled points generated using the equal angle sampling grid is

Top=(N-1)×M																									(4)

Figure 1 shows the distribution of points on the spherical surface generated using the equal angle sampling grid. The distribution of each angle in the θ-ϕ plane.

Figure 1: Spherical equal angle sampling grid

Figure 2: Distribution of angles in the θ-ϕ plane.

Observation 1: Spherical equal angle sampling grid does not generate uniformly distributed points on the sphere but they are clustered around the poles at θ=0 (North Pole) and θ=π (South Pole).

In TR 37.843 [3], the numerical approximation for the integral in Equation (1) is

TRPapprox≈π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn																(5)

In Equation (5), the total number of sampled points is

Tnp=N×M																											(6)

Since sin(θ0=0)=sinθN=π=0, data samples at points (θ0=0, ϕ) and (θ0=π, ϕ) are redundant. Equation (5) includes the points at (θ0=0, ϕ).

Observation 2: Equation (5) which is the numerical approximation formula for TRP is not optimized.

In addition to the spherical equal angle sampling grid, grid spacing that is non-uniform can be applied to approximate the TRP integral in Equation (1).

Observation 3: the spherical equal angle sampling grid should serve as the baseline grid.

# 3	Conclusions

We have discussed the approximation of TRP using numerical techniques which are based on the spherical equal angle sampling grid. The resulting optimum formula for TRP approximations is

TRPapprox=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn

and the total number of sampled points generated using the equal angle sampling grid is

Top=(N-1)×M

Based on our analysis, the following observations are made:

Observation 1: Spherical equal angle sampling grid does not generate uniformly distributed points on the sphere but they are clustered around the poles at θ=0 (North Pole) and θ=π (South Pole).

Observation 2: Equation (5) (which is the numerical approximation formula for TRP in TR 37.843) is not optimized.

Observation 3: the spherical equal angle sampling grid should serve as the baseline grid.

# References

- R4-1803392, WF on OTA TRP conformance requirements, Ericsson

- R4-1805390, Discussion on TRP approximation errors – names and definitions, Nokia, Nokia Shanghai Bell

- TR 37.843 v15.0.0, RF requirement background for AAS BS radiated requirements (Release 15)