3GPP TSG-RAN WG4 Meeting #86bis	R4-1805392

Melbourne, Vic, Australia, 16th – 20th April 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On spherical equal area sampling grid for numerical TRP approximations

Agenda item:	6.26.3.1.4

Document for:	Discussion

# 1	Introduction

During the last RAN4#86 meeting, the WF on OTA TRP conformance requirements [1] was approved, which outlined a list of open issues to be addressed. One of the open issues is numerical integration methods for TRP estimation.

In our companion document [2], a definition and an alternative name for “TRP estimation error” is provided.

This document discusses a spherical equal area sampling grid which is used to generate sampling points for the numerical integration of TRP. Refer to [3], for discussions on numerical TRP approximation that is based on the spherical equal angle sampling grid.

# 2	Discussion

For OTA measurements, the DUT is positioned in the center of a sphere. In the far field region, the TRP of the DUT is obtained by integrating radiated power (i.e., EIRP) patterns of the DUT over the whole spherical surface

TRPactual= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ					(W)										(1)

In [3], we investigated the spherical equal angle sampling grid that was used for the numerical TRP approximation and made the following observation:

Observation 1: Spherical equal angle sampling grid does not generate uniformly distributed points on the sphere but they are clustered around the poles at θ=0 (North Pole) and θ=π (South Pole).

Spherical equal area sampling grid:

With the spherical equal area sampling grid, the spherical surface is partitioned into N equal area regions. Let n be the index for the nth region and there is one point (θn,ϕn) located in the center of each region. The TRP integral in Equation (1) can be approximated as

TRPapprox≈1Nn=1NEIRP(θn,ϕn)  																				(2)

In the test chamber, each EIRPθn,ϕn is the sum of two orthogonally polarized components, EIRPco(θn,ϕn) and EIRPx(θn,ϕn), which correspond to the co- and cross-polarization of the DUT transmit antennas.

As shown in Figure 1, the distribution of points on the spherical surface is uniform but the pattern of θ and ϕ angles is irregular. There exist several algorithms for generating equal area points on the spherical surface. One such algorithm is the UV projection.

Figure 1: Spherical equal area sampling grid

# 3	Conclusions

We have discussed the approximation of TRP based the spherical equal area sampling grid. The numerical TRP approximation is

TRPapprox≈1Nn=1NEIRP(θn,ϕn)

# References

- R4-1803392, WF on OTA TRP conformance requirements, Ericsson

- R4-1805390, Discussion on TRP approximation errors – names and definitions, Nokia, Nokia Shanghai Bell

- R4-1805391, On spherical equal angle sampling grid for TRP estimation, Nokia, Nokia Shanghai Bell

- TR 37.843 v15.0.0, RF requirement background for AAS BS radiated requirements (Release 15)