3GPP TSG-RAN WG4 Meeting #86bis	R4-1805395

Melbourne, Vic, Australia, 16th – 20th April 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	CATR set up for BS rated output power of OTA AAS BS and NR BS type 1-O

Agenda item:	6.26.3.1.4

Document for:	Discussion

# 1	Introduction

During the last RAN4#86 meeting, the WF on OTA TRP conformance requirements [1] was approved, which outlined a list of open issues to be addressed in forthcoming meetings. One such open issue is the OTA test methods for TRP-based OTA requirements.

This document considers a typical CATR test system set up for measuring radiated BS rated output power of OTA AAS BS and NR BS type 1-O. The proposed test methodology will serve a baseline for further optimization.

# 2	Discussion

## 2.1 	OTA BS rated output power

Referring to Rel-15 TS 37.105 and TS 38.104, the rated carrier output power of the OTA AAS BS and NR BS type 1-O is shown in Tables 1 and 2, respectively.

Table 1: OTA AAS rated output power

| OTA AAS BS class | PRated,c,TRP |
| --- | --- |
| Wide Area BS | (NOTE) |
| Medium Range BS | ≤ 47 dBm |
| Local Area BS | ≤ 33 dBm |
| NOTE: There is no upper limit for the PRated,c,TRP of the Wide Area Base Station. |  |  |

Table 2: NR BS type 1-O rated output power

| BS class | Prated,c,TRP |
| --- | --- |
| Wide Area BS | (note) |
| Medium Range BS | ≤ + 47 dBm |
| Local Area BS | ≤ + 33 dBm |
| NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station. |  |

## 2.2	CATR test set up for radiated BS rated output power

TR 37.842 outlines four OTA test systems which are used for testing the EIRP accuracy requirement:

- Indoor anechoic Chamber

- Compact Antenna Test Range

- One Dimensional Compact Range Chamber

- Near Field Test Range

Figure 1 depicts one example of a typical CATR test system set up.

PM/SG

Figure 1: A typical CATR system set up for OTA BS rated output power measurements

# Stage 1 - Calibration:

1)	Path loss calibration C→A:

a)	Place a SGH antenna in the position of the DUT.

b)   Measure SGH (or other calibrated reference antenna) reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment such as signal generator, spectrum analyser and power meter) to obtain ΓSGH.

c)	Measure cable loss from point C to input of SGH, call this LC↔SGH which is the equivalent of 20log|S21| from the use of a network analyser.

d)	Calculate the combined total path loss from C→A by using the following expression:

-	LSGHcal = LC,SGH + 10log(1 - |ΓSGH|2) - GSGH;

-	where10log1-ΓSGH2  10log(1 - |ΓSGH|2)  is the compensation for SGH connector return loss, GSGH  is the known gain of the reference SGH.

2)	Connect SGH and C↔A cable.

3)	To remove polarization(s) mismatch between range antenna (labelled as feeder antenna in diagram) and SGH use positions to position the SGH in the beam peak direction of range antenna.

4)	Measure path loss C→B with network analyser LC→B = 20log|S21|.

5)	Calculate the test path loss compensation factor.  This is the total path loss between A↔B using the results from step 1d and 4. L = LSGHcal LSGHcal  - LC→B.

where ΓSGH is the reflection coefficient (or mismatch) seen at the SGH connector (S11 with a network analyser).

# Stage 2 - Measurement:

- Set up OTA AAS BS/NR BS type 1-O (DUT) in place of SGH from the calibration stage.

2)    Configure OTA AAS BS/NR BS type 1-O output power according to the manufacturer’s declared rate power.

3)	 Set the DUT to transmit a single beam according to the test signal according to E-TM1.1 at 5 MHz bandwidth configuration.

- Set the spectrum analyser centre frequency, start frequency (fstart) and stop frequency (fstop) to transmit carrier frequency, FBW_RF,low and FBW_RF,high, respectively.

- Set the number of trace points in a sweep by the spectrum analyser to ≥2 ×spanBWchannel, where span= fstop-fstart.

- Configure the sweep time of the spectrum analyser to less than the transmission duration; for non-continuous transmission, employ the spectrum analyser gated sweep (i.e., gate triggered in such a way that the analyser only sweeps when the DUT is transmitting).

- Select the spectrum analyser power detection mode to power averaging.

- Select a suitable spherical sampling grid (e.g., spherical equal area sampling grid, etc.) and determine the total number of spherical points T = FFS.

9)  Set the DUT position to the first test point from the set of T points in the sampling grid and align with the direction of range antenna.

10) Measurements should be made with the measurement antenna oriented in both orthogonal polarizations. If the measurement antenna supports single polarization, then it should be oriented in one polarization at a time.

11) Record two orthogonal polarized mean power levels (Pmeas,p1, Pmeas,p2) and DUT angular position in list L.

12)	 Move the DUT to the next test point and repeat Steps 10-11 until all T spherical points are completed.

13) For each Pmeas sample in L, calculate and record EIRP in L, where EIRP = Pmeas + LA→B  dB.

14) Calculate TRP (using the formula for the sampling grid selected in Step 8 using the EIRP samples in L.

# 3	Conclusions

This document has investigated the CATR test system set up for measuring radiated BS rated output power for OTA AAS BS, and NR BS type 1-O. The proposed test method should serve as a baseline for further optimization.

# References

- R4-1803392, WF on OTA TRP conformance requirements, Ericsson