3GPP TSG-RAN WG4 Meeting #86bis	R4-1805408

Melbourne, Vic, Australia, 16th – 20th April 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to draft CR TR 37.843 - CATR set up for OTA TX spurious and EMC emissions

Agenda item:	6.26.3.1.5

Document for:	Approval

# 1	Introduction

During the last RAN4#86 meeting, the WF on OTA TR structure for Section 10 [1] was approved, which outlined the skeleton for capturing OTA directional and TRP conformance requirements.

This document contributes to a TP to TR 37.843, which describes a test method for OTA TX spurious and EMC emission measurements in a CATR chamber. Constructive feedback is encouraged.

# 2	Discussion

According to TR 37.843 and TR 38.817-02, radiated TX spurious emissions can no longer be differentiated from EMC emissions for over the air testing. As a result, one single OTA test system set up can be used for testing both the OTA TX spurious and EMC emission requirements simultaneously.

The range of TX spurious and EMC emission frequency is shown in Figure 1.

ΔfOBUE

ΔfOBUE

EMC/Spurious emission limits

EMC/Spurious emission limits

12.75 GHz

30 MHz

Figure 1: Radiated TX spurious and EMC emission frequency range

In [2], we discussed a comprehensive test procedure for TX spurious and EMC emission measurements in one single CATR set up. The proposed test procedure serves as a baseline procedure. As spurious and EMC emission measurements are extremely time consuming, efficient techniques (such as pre-scan, peak EIRP and fast TRP measurements) can be applied. We can always fall back on the baseline procedure if necessary.

# 3	Conclusions

It is proposed to agree on the TP to TR 37.843.

# 4	References

- R4-1803388, WF on TR structure for Section 10, Ericsson

- R4-1805397, CATR set up for TX spurious and EMC emissions of OTA AAS BS and NR BS type 1-O, Nokia, Nokia Shanghai Bell

# 5	Text proposal

TR 37.843 V15.0.0

--------------Start of text proposal-------------

# 10.3.1		Measurement uncertainty for TRP requirements

Based on the definition of the TRP measurement, the measurement uncertainty of the TRP requirement relies on the measurement uncertainty derivation for the directional requirements, with consideration of the additional uncertainty contributors, depending on the OTA test range and sampling grid.

#### 10.3.1.4	OTA TX spurious emissions

##### 10.3.1.4.1	Compact antenna test range

##### 10.3.1.4.1.1	Description

Refer to TR 37.842.

##### 10.3.1.4.1.2	Test method limitations and scope

Refer to TR 37.842. The bandwidth of range antenna reflector may restrict the range of spurious frequency that can be tested.

##### 10.3.1.4.1.3	Procedure

Stage 1 - Calibration:

Refer to TR 37.842 for EIRP measurements. The range of spurious emission frequency should also be included in the calibration stage.

# Stage 2 - Measurement:

- Set up OTA AAS BS/NR BS type 1-O (DUT) in place of SGH from the calibration stage.

2)   Configure OTA AAS BS/NR BS type 1-O output power according to the manufacturer’s declared rated power for a single beam.

3)	 Set the DUT to transmit a single beam according to the test signal according to E-TM1.1 at 5 MHz bandwidth configuration.

- Set the spectrum analyser start frequency (fstart) and stop frequency (fstop) to the lowest frequency (fl) and highest (fh)  operating frequency of the measurement antenna, where the measurement antenna bandwidth = fh-fl.

- Set the spectrum analyser resolution bandwidth in accordance with the MBW requirement as specified in TS 37.105 or TS 38.104.

- Set the number of trace points in a sweep by the spectrum analyser to ≥2 ×spanMBW, where span= fstop-fstart.

- Configure the sweep time of the spectrum analyser to less than the transmission duration; for non-continuous transmission; employ the spectrum analyser gated sweep.

- Select the spectrum analyser power detection mode to power averaging.

- Select a suitable spherical sampling grid (e.g., spherical equal area sampling grid, etc.) and determine the total number of spherical points T = FFS.

10)  Set the DUT position to the first test point from the set of T points in the sampling grid and align with the direction of range antenna.

11) Measurements should be made with the measurement antenna oriented in both orthogonal polarizations. If the measurement antenna supports single polarization, then it should be oriented in one polarization at a time.

12) Record two orthogonal polarized mean power levels (Pmeas,p1, Pmeas,p2), spurious emission frequency and DUT angular position for each measurement bandwidth in list L.

13)	 Move the DUT to the next test point and repeat Steps 11-12 until all T spherical points are completed.

14) For each Pmeas sample in L, calculate and record EIRP in L, where EIRP = Pmeas + LA→B  dB.

15) Calculate TRP (using the formula for the sampling grid selected in Step 9) for each measurement bandwidth using the EIRP samples in L.

16) If the measurement antenna is unable to cover the entire spurious frequency range, install the next measurement antenna appropriate to the frequency range under measurement and repeat Steps 4 to 15.

--------------end of text proposal-------------