3GPP TSG-RAN WG4 #86bis	R4-1805495

Melbourne, April, 16 – April 20, 2018

Agenda Item:	6.20.6.2

Source:	Ericsson

Title:	Simulation assumptions for sPUCCH and sPUSCH

Document for:	Approval

# Introduction

In this contribution, simulation assumptions for sPUSCH and sPUCCH are proposed.

# Simulation assumption for sPUSCH

Table 1: Simulation assumption

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| Fraction of maximum throughput | 70 % |
| Maximum number of HARQ transmissions | 4 |
| DMRS configuration | IFDMA is not configured |
| DMRS sharing pattern | Option 1: - RDD DD DD RD DD RDD Option 2: - RDD DD¦R RD DD¦R RD RDD |
| Number of HARQ processes | 16 |
| RV sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| Modulation | 16QAM |

# Table A.4-1 FRC parameters for performance requirements (16QAM 3/4)

| Reference channel | Ax-1 |  |  | Ax-2 |  | Ax-3 |  | Ax-4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated resource blocks | 24 |  |  | 48 |  | 72 |  | 100 |  |
| DFT-OFDM Symbols per subframe | 1 | 2 |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Modulation | 16QAM |  |  | 16QAM |  | 16QAM |  | 16QAM |  |
| Code rate | 3/4 |  |  | 3/4 |  | 3/4 |  | 3/4 |  |
| Payload size (bits) | 872 |  | 1736 | 1736 | 3496 | 2536 | 5160 | 3624 | 7224 |
| Transport block CRC (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Total number of bits per sub-frame | 1152 |  | 2304 | 2304 | 4608 | 3456 | 6912 | 4800 | 9600 |
| Total symbols per sub-frame | 288 |  | 576 | 576 | 1152 | 864 | 1728 | 1200 | 2400 |

Note: This FRC may be revisited in case any problems are identified for the FRC.

# Simulation assumption for sPUCCH

In Table 1, simulation assumption is given.

Table 1: Simulation assumption for sPUCCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| SPUCCH formats | Format 1a, format 4 |
| Number of bits per PRB for format 4 | 3 bits per 2 PRB |

# Conclusion

In this contribution, simulation assumptions are proposed for sPUSCH and sPUCCH.