3GPP TSG-RAN WG4 Meeting #86bis                                                             R4-1805786

Melbourne, AU, 16th – 20th April, 2018

Title:	LS on the UE capability of maxUplinkDutyCycle for NR FR1 power class 2 UE

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

Name:	Xiaoran ZHANG

E-mail Address:	zhangxiaoran@chinamobile.com

1. Overall Description:

RAN4 has discussed how to solve the potential SAR (Specific Absorption Rate) issue for FR1 power class 2 UE on Band n41 n77 n78 n79.

It is agreed to introduce a UE capability of maxUplinkDutyCycle which indicates the maximum percentage of uplink transmission time that can be scheduled within [TBD]ms to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies.

For a FR1 power class 2 capable UE operating on Band n41 n77 n78 n79, when the percentage of uplink transmission time scheduled by the network within an evaluation period [TBD]ms is larger than its capability, i.e., maxUplinkDutyCycle, the requirements for FR1 power class 2 are not applicable, and the corresponding requirements for a FR1 power class 3 UE shall apply.

The capability of maxUplinkDutyCycle is reported by UE as a per band capability. The granularity is 10%. For FR1 power class2 UE, the minimum value is 50%, and if the capability is absent, 50% shall be applied. The detailed signalling design is up to RAN2.

2. Actions:

To RAN1/ RAN2 WG:

RAN4 kindly asks RAN2 to take the above agreement into consideration and design the capability signalling.

3. Date of Next RAN4 Meetings:

TSG-RAN4 Meeting #87	                 	May 21 – 25, 2018		Busan, KR

TSG-RAN4 Meeting #88	  	August 20 – 24, 2018		Gothenburg, SE