3GPP TSG-RAN WG4 Meeting #86bis	R4-1805865

Melbourne, Vic, Australia, 16th – 20th April 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Way forward on TRP measurement grids

Agenda item:	6.26.3.1.4

Document for:	Approval

# 1	Introduction

During the 3GPP RAN4 #86bis meeting, contributions [1]-[5] on the issue of spherical grids, which can be applied to numerically approximate the definite TRP integral, were presented and discussed.

This way forward aims at capturing any agreements and open issues at this meeting in an attempt to make progress following online discussions.

# 2	Discussion

## 2.1 	Background

In Rel-15 TS 37.105, TRP requirements are:

- In band

  - OTA BS maximum output power

  - OTA ACLR

  - OTA spectrum emission mask

  - OTA operating band unwanted emissions

- Out of band

  - OTA TX spurious emissions

  - OTA RX spurious emissions

## 2.2	Terms and definitions

Currently, there is currently not a common term and definition for specifying errors attributed to approximating the definite TRP integral.

Referring to [1], TRP error is estimated as

TRPError=TRPReference-TRPUsed sampling grid

Referring to [3], the proposal is

| Name | Definition |
| --- | --- |
| TRP estimation error | The difference between the actual TRP and numerically estimated TRP |

In the above definitions, TRPUsed sampliong grid corresponds to the numerically approximated TRP and TRPReference is equivalent to the actual TRP when Δθ→0 and ∆ϕ→0.

TRP estimation error = TRPtheoretical- TRPapprox

Instantaneous power density vector associated with the EM field at a given point is

P=E ×H

In the far field region, the TRP is obtained by integrating EIRP patterns of the DUT over the whole spherical surface, that is

TRPtheoretical= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

Using the numerical integration technique, the definite TRP integral can be approximated as

TRPapprox=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn

Proposal 1: TRP systematic error is defined as the relative difference between theoretical TRP and numerically estimated TRP.

Proposal 1b: The following terms are defined:

TRPReference : the theoretical TRP

TRPEstimate : the numerically approximated TRP

TRP systematic error	: The difference between the theoretical TRP and numerically estimated TRP

## 2.2 	Measurement uncertainty

For each test chamber type, total measurement uncertainty budget for each of the OTA TRP requirements can be derived from the existing EIRP measurement uncertainty tables in TR 37.842 as follows:

Total uncertainty budget consists of measurement uncertainty per point and TRP estimation error

If a far field test chamber were used the measurement uncertainty per point would be equivalent to the EIRP measurement uncertainty, however TRP measurements may also be done in the near field where the measurement uncertainty per point cannot be described as EIRP MU.

Proposal 2: Total uncertainty budget consists of measurement uncertainty per point and TRP system error

It is not clear of the statistical relationship between the measurement uncertainty per point and the TRP systematic error how the values are added (i.e. rms or arithmetic sum) is FFS.

Open issue 1: statistical relationship and appropriate method of addition of measurement uncertainty per point and the TRP systematic error.

Companies are encouraged to submit proposals for total uncertainty budget with TRP systematic error values for each of the OTA TRP requirements to the RAN4 #87 meeting. Different spherical grids (e.g., equal area grid) [1]-[5] can be used to derive the TRP estimation error for determining the total uncertainty budget.

## 2.3 	Spherical grids

TRP measurements consist of spatially averaging of power over a full sphere. Various full spherical grids can be used for TRP measurements. For unwanted emission requirements, optimized or/and fast TRP measurement techniques on top of the full spherical grid, which can be used to reduce test duration, are FFS.

Proposal 3: Optimized TRP measurement techniques on top of the full spherical grids are FFS.

# 3	Way forward

Based on the above discussion, the following proposals are agreed:

Proposal 1: TRP systematic error is defined as the relative difference between theoretical TRP and numerically estimated TRP.

Proposal 1b: The following terms are defined:

TRPReference : the theoretical TRP

TRPEstimate : the numerically approximated TRP

TRP systematic error	: The difference between the theoretical TRP and numerically estimated TRP

Proposal 2: Total uncertainty budget consists of measurement uncertainty per point and TRP systematic error

Open issue 1: statistical relationship and appropriate method of addition of measurement uncertainty per point and the TRP systematic error.

Companies are encouraged to submit proposals for total uncertainty budget with TRP systematic error values for each of the OTA TRP requirements for the RAN4 #87 meeting. Different spherical grids and methods can be used to derive the TRP estimation error for determining the total uncertainty budget.

Proposal 3: Optimized TRP measurement techniques on top of the full spherical grids are FFS.

# References

- R4-1805107, Overview of In-band TRP uncertainty versus sampling grid, MVG Industries

- R4-1805047, TP for TR 37.843: TRP test method for unwanted emissions using measurements on a spherical, Ericsson

- R4-1805390, Discussion no TRP approximation errors – names and definitions, Nokia, Nokia Shanghai Bell

- R4-1805391, On spherical equal angle sampling grid for numerical TRP approximations, Nokia, Nokia Shanghai Bell

- R4-1805392, On spherical equal area sampling grid for numerical TRP approximations, Nokia, Nokia Shanghai Bell