3GPP TSG-RAN WG4 Meeting #86bis	R4-1805878

Melbourne, Australia, 16 – 20 April 2018

Agenda Item:	7.7.4.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TS 38.141-2: NR BS OTA REFSENS conformance test (7.3)

Document for:	Approval

# 1.	Introduction

This contribution provides a text proposal to the NR BS draft TS 38.141-2 [1] to specify the NR BS OTA REFSENS conformance test.

# 2.	Text Proposal

<Start of change>

## 7.3	OTA Reference sensitivity level

### 7.3.1	Definition and applicability

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel. The OTA REFSENS requirement shall apply to each supported polarization, under the assumption of polarization matching.

### 7.3.2	Minimum Requirement

For BS type 1-O the minimum requirement is in 3GPP TS 38.104 [1], subclause 10.3.2.

For BS type 2-O the minimum requirement is in 3GPP TS 38.104 [1], subclause 10.3.3.

### 7.3.3	Test Purpose

The test purpose is to verify that the BS can meet the throughput requirement for a specified measurement channel at the EISREFSENS level and the range of angles of arrival within the OTA REFSENS RoAoA.

### 7.3.4	Method of test

#### 7.3.4.1	Initial conditions

Test environment:

-	Normal: see 3GPP TS 38.141-1 [x], annex B.2.

RF channels to be tested:

-	B, M and T; see subclause 4.9.x.

Directions to be tested:

- OTA REFSENS receiver target reference direction (see table 4.6-x, Dx.x),

- OTA REFSENS conformance test directions (see table 4.6-x, Dx.x).

#### 7.3.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D1.1.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction of the BS according to the OTA REFSENS RoAoA for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in clause 5.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in subclause 7.3.5.4.

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in subclause 7.3.5.

9)	Measure:

-	Throughput according to annex x for each supported polarization.

10)	Repeat steps 3 to 9 for all OTA REFSENS conformance test directions of the BS (see table 4.6-x, Dx.x).

For multi-band capable FR1 BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.3.5	Test Requirements

#### 7.3.5.1	General

The FR1 EISREFSENS level is the conducted REFSENS requirement value offset by ΔOTAREFSENS. The test requirement is calculated from the EISREFSENS level offset by the EISREFSENS Test Tolerance specified in subclause 4.1.

#### 7.3.5.2	Test Requirements for BS type 1-O

For each measured carrier, the throughput measured in step 9 of subclause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex TS 38.104 [2] A.1 with parameters specified in tables 7.3.5.2-1 to 7.3.5.2-3.

Table 7.3.5.2-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -101.7 - ΔOTAREFSENS + [1.3 dB] | -101.7 - ΔOTAREFSENS + [1.4 dB] | -101.7 - ΔOTAREFSENS + TBD |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -101.8 - ΔOTAREFSENS + [1.3 dB] | -101.8 - ΔOTAREFSENS + [1.4 dB] | -101.8 - ΔOTAREFSENS + TBD |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -98.9 - ΔOTAREFSENS + [1.3 dB] | -98.9 - ΔOTAREFSENS + [1.4 dB] | -98.9 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -95.3 - ΔOTAREFSENS + [1.3 dB] | -95.3 - ΔOTAREFSENS + [1.4 dB] | -95.3 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -95.6 - ΔOTAREFSENS + [1.3 dB] | -95.6 - ΔOTAREFSENS + [1.4 dB] | -95.6 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -95.7 - ΔOTAREFSENS + [1.3 dB] | -95.7 - ΔOTAREFSENS + [1.4 dB] | -95.7 - ΔOTAREFSENS + TBD |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -96.7 - ΔOTAREFSENS + [1.3 dB] | -96.7 - ΔOTAREFSENS + [1.4 dB] | -96.7 - ΔOTAREFSENS + TBD |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -96.8 - ΔOTAREFSENS + [1.3 dB] | -96.8 - ΔOTAREFSENS + [1.4 dB] | -96.8 - ΔOTAREFSENS + TBD |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -93.9 - ΔOTAREFSENS + [1.3 dB] | -93.9 - ΔOTAREFSENS + [1.4 dB] | -93.9 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -90.3 - ΔOTAREFSENS + [1.3 dB] | -90.3 - ΔOTAREFSENS + [1.4 dB] | -90.3 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -90.6 - ΔOTAREFSENS + [1.3 dB] | -90.6 - ΔOTAREFSENS + [1.4 dB] | -90.6 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -90.7 - ΔOTAREFSENS + [1.3 dB] | -90.7 - ΔOTAREFSENS + [1.4 dB] | -90.7 - ΔOTAREFSENS + TBD |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.2-3: Local Area BS EISREFSENS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| 5, 10, 15, 25, 30 | 15 | G- FR1-A1-1 | -93.7 - ΔOTAREFSENS + [1.3 dB] | -93.7 - ΔOTAREFSENS + [1.4 dB] | -93.7 - ΔOTAREFSENS + TBD |
| 5, 10, 15, 25, 30 | 30 | G- FR1-A1-2 | -93.8 - ΔOTAREFSENS + [1.3 dB] | -93.8 - ΔOTAREFSENS + [1.4 dB] | -93.8 - ΔOTAREFSENS + TBD |
| 10, 15, 25, 30 | 60 | G- FR1-A1-3 | -90.9 - ΔOTAREFSENS + [1.3 dB] | -90.9 - ΔOTAREFSENS + [1.4 dB] | -90.9 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 15 | G- FR1-A1-4 | -87.3 - ΔOTAREFSENS + [1.3 dB] | -87.3 - ΔOTAREFSENS + [1.4 dB] | -87.3 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 | -87.6 - ΔOTAREFSENS + [1.3 dB] | -87.6 - ΔOTAREFSENS + [1.4 dB] | -87.6 - ΔOTAREFSENS + TBD |
| 20, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 | -87.7 - ΔOTAREFSENS + [1.3 dB] | -87.7 - ΔOTAREFSENS + [1.4 dB] | -87.7 - ΔOTAREFSENS + TBD |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

#### 7.3.5.2	Test Requirements for BS type 2-O

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in the corresponding table and TS 38.104 [2] annex A.1 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the FR2 OTA REFSENS RoAoA.

EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50MHZ BS channel bandwidth. EISREFSENS_50M itself is not a requirement and although it is based on a a reference measurement channel with 50MHz BS channel bandwidth it does not imply that BS has to support 50MHz BS channel bandwidth.

For wide area BS, EISREFSENS_50MC is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For medium range BS, EISREFSENS_50M is an integer value in the range -91 to -114 dBm. The specific value is declared by the vendor.

For local area BS, EISREFSENS_50M is an integer value in the range -86- to -109 dBm. The specific value is declared by the vendor.

Table 10.3.3-1 FR2 OTA Reference sensitivity requirement

| BS channel Bandwidth [MHz] [MH | Sub-carrier spacing [kHz] | FRC | EISREFSENS level (dBm) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + TBD |
| 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + TBD |
| 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + TBD |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified in Table 10.3.3-2. |  |  |  |

<End of change>

# References

[1]	R4-1711983, “TS 38.141-2 v0.0.1”, Huawei.