3GPP TSG-RAN WG4 Meeting #86-bis                              R4-1805894

# Melbourne, Australia, 16th – 20th April 2018

Agenda item:	7.12.1

Source:	MVG Industries

Title:	TP to TR 38.810 – Full package for Near Field Test Range (NFTF)

Document for:	Approval

# 1	Introduction

This contribution is a text proposal for the full package of the NFTF to be included in TR 38.810 v2.0.0 [1]. It shall be noted that all the technical contents have been already agreed through e-mail reflector, R4-1803573. The resubmission was needed in order to fix issues found during the implementation of TP into 38.810 v2.0.0.

# 2	Proposal

--------------------------------------START PROPOSAL---------------------------------------

### 5.2.4		Near field to far field transform (NFTF)

The NFTF method computes the metrics defined in Far Field by using the Near Field to Far Field transformation.

#### 5.2.4.1		Description

The NFTF measurement setup of UE RF characteristics for f > 6 GHz is capable of centre and off centre of beam measurements and is shown in Figure 5.2.5.1-1:

Figure 5.2.5.1-1: Typical NFTF measurement setup of EIRP/TRP measurements

# The key aspects of the Near Field test range are:

- Radiated Near Field UE beam pattern are measured and based on the NFtoFF mathematical transform, the final metric such as EIRP is the same as the metric for the baseline setup

- A positioning system such as the angle between the dual-polarized measurement/link antenna and the DUT has at least two axes of freedom and maintains a polarization reference

- For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1UL configuration, an LTE link antenna is used to provide the LTE link to the DUT

- The LTE link antenna provides a stable LTE signal without precise path loss or polarization control

-	The applicability criteria of the NFTF setup are:

-	The DUT radiating aperture is D ≤ 5 cm

- Either a single radiating aperture, multiple non-coherent apertures or multiple coherent apertures DUTs can be tested

- If multiple antenna panels that are phase coherent are defined as a single array, the criterion on DUT radiating aperture applies to this single array

- D is based on the MU assessment in Annex B.1.4.3

- If the uncertainties can be further optimized, the MU may be reduced or D may be increased

-	A manufacturer declaration on the following elements is needed:

-	Manufacturer declares antenna array size

-   EIRP, TRP, and spurious emissions metrics can be tested.

#### 5.2.4.2	Parameter mapping to RF requirements

<Editor’s note: clause content is FFS>

#### 5.2.4.3	Testing and calibration aspects

##### 5.2.4.3.1			Calibration Measurement Procedure

Calibration accounts for the various factors affecting the measurements of the EIRP. These factors include components such as range length path loss, cable losses, gain of the receiving antenna, etc. Each measured data point for radiated power is transformed from a relative value in dB to an absolute value in dBm. For doing that the total path loss from the DUT to the measurement receiver, named L path loss is calibrated out. The calibration measurement is usually done by using a reference antenna with known gain. This approach is based on the so called gain-comparison method. Figure 5.2.5.3-1 shows the typical configuration for measuring path loss.

# Figure 5.2.5.3-1: NFTF – Typical setup for path loss measurement

The Lpath loss can be determined from the power into the reference antenna by adding the gain of the reference antenna:

# Piso = Pref + GrefPIsotropic Radiator=Preference antenna+Greference antenna

so that:

# Lpath loss = Pref + Gref - PtestLpath loss=Preference antenna+ Greference antenna- Ptest equipment

In order to determine Pref, a cable reference measurement is performed in order to calibrate out the A, and B pathsLtest equipment-receiver. Assuming that the power at the source is fixed, it can be showed that:

# Pref-Ptes=Prec'-Prec

Where Prec Preceiver and Prec' are the power measured at the receiver during the calibration measurement with the reference antenna and the power measured at the receiver during the cable reference measurement respectively. Lpath loss Lpath loss is then given by:

Lpath loss = Gref + Prec' - Prec

##### 5.2.4.3.2			EIRP Measurement Procedure

# The TX beam peak direction is found with a 3D EIRP scan (separately for each orthogonal polarization) with a grid that is TBD. The TX beam peak direction is where the maximum total component of EIRP is found.

1)	Connect the SS (System Simulator) to the DUT through the measurement antenna with polarization reference PolMeas to form the TX beam towards the previously determined TX beam peak direction and respective polarization.

2)	Lock the beam toward that direction for the entire duration of the test.

3)	Perform a 3D pattern measurement (amplitude and phase) with the DUT sending a modulated signal.

4)	Determine the EIRP for both polarization towards the TX beam peak direction by using a Near Field to Far Field transform.

5)	Calculate total EIRP = EIRP + EIRP

##### 5.2.4.3.3			TRP Measurement Procedure

- Connect the SS to the DUT through the measurement antenna with polarization reference PolMeas to form the TX beam towards the previously determined TX beam peak direction and respective polarization.

- Lock the beam toward that direction for the entire duration of the test.

3)	Perform a 3D pattern measurement (amplitude and phase) with the DUT sending a modulated signal.

4)	For each measurement point on the TBD grid, determine the EIRP for both polarization by using a Near Field to Far Field transform.

- The TRP value for the constant step size measurement grids are calculated using

TRP=π2NMi=1N-1j=0M-1EIRPθθi,ϕj+EIRPϕθi,ϕjsinθi

Where N is the number of angular intervals in the nominal theta range from 0 to π and M is the number of angular intervals in the nominal phi range from 0 to 2 π . The TRP values for the constant density grids are calculated using:

TRP=1Ni=0N-1EIRPθθi,ϕi+EIRPϕθi,ϕi

where N is the number of measurement points.

--------------------------------------END OF PROPOSAL--------------------------------------

--------------------------------------START PROPOSAL---------------------------------------

Table 5.3-2: Overview of test method applicability for permitted test methods

| DUT category |  | Direct Far Field (DFF) | Indirect Far Field (IFF) | Near Field to far field transform (NFTF) | TBD |
| --- | --- | --- | --- | --- | --- |
| Category 1 |  | Yes | Yes | Yes | TBD |
| Category 2 |  | Yes | Yes | Yes | TBD |
| Category 3 |  | No | Yes | No | TBD |
|  | NOTE: A positive indication means that applicability exists for at least one RF test cases for the given UE category |  |  |  |  |

--------------------------------------END OF PROPOSAL--------------------------------------

--------------------------------------START PROPOSAL---------------------------------------

Annex B

## B.1.4		NFTF setup

B.1.4.1	Uncertainty budget calculation principle

The uncertainty tables should be presented with two stages:

-	Stage 1: the calibration of the absolute level of the DUT measurement results is performed by means of using a calibration antenna whose absolute gain is known at the frequencies of measurement

-	Stage 2: the actual measurement with the DUT as either the transmitter or receiver is performed.

The MU budget should comprise of a minimum 5 headings:

1)	The uncertainty source,

2)	Uncertainty value,

3)	Distribution of the probability,

4)	Divisor based on distribution shape,

5)	Calculated standard uncertainty (based on uncertainty value and divisor).

### B.1.4.2	Uncertainty budget format

Table B.1.4.2-1: Uncertainty contributions for EIRP and TRP measurement

| UID | Description of uncertainty contribution | Details in paragraph |
| --- | --- | --- |
|  | Stage 2: EIRP Near Field Radiation Pattern Measurement and EIRP Near Field DUT power measurement |  |
| 1 | Axis Alignment | B.1.4.4.1 |
| 2 | Probe XPD | B.1.4.4.2 |
| 3 | Probe Polarization Amplitude and Phase | B.1.4.4.3 |
| 4 | Probe Array Uniformity | B.1.4.4.4 |
| 5 | Probe Pattern Effect | B.1.4.4.5 |
| 6 | Multiple Reflections: Coupling between Measurement Antenna and DUT | B.1.4.4.6 |
| 7 | Quality of the Quiet Zone | B.1.4.4.7 |
| 8 | Phase curvature | B.1.4.4.24 |
| 9 | Measurement Distance | B.1.4.4.8 |
| 10 | NF to FF truncation | B.1.4.4.9 |
| 11 | Mismatch of receiver chain | B.1.4.4.10 |
| 12 | Uncertainty of the RF power measurement equipment | B.1.4.4.11 |
| 13 | Amplifier uncertainties | B.1.4.4.12 |
| 14 | Phase Recovery Non-Linearity over signal bandwidth | B.1.4.4.13 |
| 15 | Phase Drift and Noise | B.1.4.4.14 |
| 16 | Leakage and Crosstalk | B.1.4.4.15 |
| 17 | Random uncertainty | B.1.4.4.16 |
|  | Stage 1: Calibration measurement |  |
| 18 | Uncertainty of the Network Analyzer | B.1.4.4.17 |
| 19 | Amplifier uncertainties | B.1.4.4.18 |
| 20 | Mismatch of receiver chain | B.1.4.4.19 |
| 21 | Mismatch in the connection of the calibration antenna | B.1.4.4.20 |
| 22 | Measurement Distance | B.1.4.4.21 |
| 23 | Quality of the Quiet Zone for Calibration Process | B.1.4.4.22 |
| 24 | Uncertainty of the absolute gain of the calibration antenna | B.1.4.4.23 |

### B.1.4.3	Uncertainty assessment

The uncertainty assessment table is organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D, and the uncertainty assessment has been derived for the case of D = 5 cm

-	The uncertainty assessment for EIRP and TRP, assuming D = 5 cm, is provided in Table B.1.4.3-1:

Table B.1.4.3-1: Uncertainty assessment for EIRP and TRP measurement (D = 5 cm)

| UID | Description of uncertainty contribution | Uncertainty Value | Distribution of the probability | Divisor |  | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
|  | Stage 2: EIRP Near Field Radiation Pattern Measurement and EIRP Near Field DUT power measurement |  |  |  |  |  |
| 1 | Axis Alignment | 0 | Normal | 2 | 0 |  |
| 2 | Probe XPD | 0 | Normal | 2 | 0 |  |
| 3 | Probe Polarization Amplitude and Phase | 0.11 | Normal | 2 | 0.055 |  |
| 4 | Probe Array Uniformity (if not corrected for above) | 0 | Normal | 2 | 0 |  |
| 5 | Probe Pattern Effect (need if not pure FF) | 0 | Normal | 2 | 0 |  |
| 6 | Multiple Reflections: Coupling Measurement Antenna and DUT | 0 | Normal | 2 | 0 |  |
| 7 | Quality of the Quiet Zone (NOTE 2) | 1.5 | Actual | 1 | [1.5] |  |
| 8 | Phase curvature | 0 | Actual | 1 | [0] |  |
| 9 | Measurement Distance (no impact) | 0 | Rectangular | 1.73 | 0 |  |
| 10 | NF to FF truncation | 0.006 | Normal | 2 | [0.003] |  |
| 11 | Mismatch of receiver chain (NOTE 3) | 1.3 | Actual | 1 | [1.3] |  |
| 12 | Uncertainty of the RF power measurement equipment | 2.16 | Normal | 2 | [1.08] |  |
| 13 | Amplifier uncertainties | 2 | Normal | 2 | 1 |  |
| 14 | Phase Recovery Non-Linearity over signal bandwidth (NOTE 5) | 0 | Normal | 2 | [0] |  |
| 15 | Phase Drift and Noise | 0.04 | Normal | 2 | [0.02] |  |
| 16 | Leakage and Crosstalk | 0 | Normal | 2 | 0 |  |
| 17 | Random uncertainty | 0.4 | Rectangular | 1.73 | [0.23] |  |
|  | Stage 1: Calibration measurement |  |  |  |  |  |  |
| 18 | Uncertainty of the Network Analyzer | 0.4 | Normal | 2 | 0.21 |  |
| 19 | Amplifier uncertainties | 0 | Normal | 2 | 0 |  |
| 20 | Mismatch of receiver chain | 0 | U-shaped | 1.41 | 0 |  |
| 21 | Mismatch in the connection of the calibration antenna | 0.0987 | U-shaped | 1.41 | 0.07 |  |
| 22 | Measurement Distance | 0 | Rectangular | 1.73 | 0 |  |
| 23 | Quality of the Quiet Zone for Calibration Process (NOTE 2) | 1.5 | Actual | 1 | [1.5] |  |
| 24 | Uncertainty of the absolute gain of the calibration antenna | 1.6 | Normal | 2 | [0.80] |  |
| EIRP Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [5.92] |  |
| TRP Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [5.04] |  |
| NOTE 1: The impact of phase variation on EIRP is FFS NOTE 2: The quality of quiet zone is different for EIRP and TRP. For TRP, the standard uncertainty is [1dB]; for EIRP, the standard uncertainty of quiet zone is [1.5dB]. NOTE 3: The analysis was done only for the case of operating at max output power, in-band, non-CA NOTE 4: The assessment assumes maximum DUT output power. NOTE 5: The Phase Recovery Non-Linearity over signal bandwidth is FFS |  |  |  |  |  |  |

### B.1.4.4	Measurement error contribution descriptions

#### B.1.4.4.1	Axes Alignment

Includes the following mechanical alignment errors:

- The uncertainty related with the lateral displacement between the horizontal and vertical axes of the DUT positioner.

- The differences from 90° of the angle between the horizontal and vertical axes.

- The horizontal mis-pointing of the horizontal axis to the probe reference point for Theta=0°.

These mechanical errors can result in sampling the field on a non-ideal sphere. This uncertainty can be considered to have a normal distribution.

#### B.1.4.4.2	Probe XPD

Refer to B.1.1.4.10 [7].  If the Probe Polarization Amplitude and Phase is measured and corrected for then this uncertainty term can be considered to be zero.

#### B.1.4.4.3	Probe Polarization Amplitude and Phase

The amplitude and phase of the probe polarization coefficients should be measured. This uncertainty is assumed to have a normal distribution.

#### B.1.4.4.4	Probe Array Uniformity (for multi -probe systems only)

This is the uncertainty due to the fact that different probes are used for each physical position. Different probes have different radiation patterns. Generally, the probe array is calibrated so that the uniformity of the probes is achieved.   This uncertainty term must be considered if the amplitude and phase of each probe is not identical or corrected for.  This uncertainty is assumed to have a normal distribution

#### B.1.4.4.5	Probe Pattern Effect

The probe/s pattern/s is assumed to be known so that the DUT measurement in near field can be corrected when performing the near field to far field transform.   If the probe pattern is known, then the uncertainty term is zero.  There is no direct dependence between the DUT pattern and the probe pattern in near field measurements. This uncertainty is assumed to have a normal distribution.

#### B.1.4.4.6	Multiple Reflections: Coupling Measurement Antenna and DUT

The multiple reflections occur when a portion of the transmitted signal is reflected form the receiving antenna back to the transmitting antenna and re-reflected by the transmitting antenna back to the receiving antenna. This uncertainty can be determined by multiple measurements of the DUT when at different distance from the probes. This uncertainty is assumed to have a normal distribution.

#### B.1.4.4.7	Quality of the Quiet Zone

See B.1.1.4.3

#### B.1.4.4.8	Measurement Distance

See B.1.1.4.2

#### B.1.4.4.9	NF to FF truncation

The measured near field is expanded using a finite set of spherical modes. The number of modes is linked to number of samples. The filtering effect generated by the finite number of modes can improve measurement results by removing signals from outside the physical area of the DUT. Care must be taken in order to make sure the removed signals are not from the DUT itself. This term also includes the uncertainty related to the scan area truncation.  This uncertainty is usually negligible.  This uncertainty is assumed to have a normal distribution.

#### B.1.4.4.10		Mismatch of receiver chain

See B.1.1.4.4

#### B.1.4.4.11		Uncertainty of the RF power measurement equipment

See B.1.1.4.6.

#### B.1.4.4.12		Amplifier uncertainties

See B.1.1.4.8

#### B.1.4.4.13		Phase Recovery Non-Linearity over signal bandwidth

This uncertainty originates from the non-linearity of the phase recovery for wide band signal. The phase recovery can be due to either phase non-linearity of the receiver and/or the DUT itself. The method to quantify the non-linarites is FFS.

#### B.1.4.4.14		Phase Drift and Noise

This uncertainty is due to the noise level and drift of the test range and should be determined or measured at the DUT location. The noise level is usually measured with a Spectrum Analyzer. This uncertainty is assumed to have a normal distribution.

#### B.1.4.4.15		Leakage and Crosstalk

This uncertainty can be addressed by measurements on the actual system setup. The leakage and crosstalk cannot be separated from the random amplitude and phase errors so that the relative importance should be determined. This uncertainty is assumed to have a normal distribution.

#### B.1.4.416		Random uncertainty

See B.1.1.4.9

#### B.1.4.4.17		Uncertainty of the Network Analyzer

See B.1.1.4.12

#### B.1.4.4.18		Amplifier Uncertainties

See B.1.1.4.8

#### B.1.4.4.19		Mismatch of receiver chain

See B.1.1.4.4

#### B.1.4.4.20		Mismatch in the connection of the calibration antenna

See B.1.1.4.4

#### B.1.4.4.21		Measurement Distance

See B.1.1.4.17

#### B.1.4.4.22		Quality of the Quiet Zone for Calibration Process

See B.1.1.4.18

#### B.1.4.4.23		Uncertainty of the absolute gain of the calibration antenna

See B.1.1.4.5.

#### B.1.4.4.24		Phase curvature

See B.1.1.4.7

--------------------------------------END OF PROPOSAL--------------------------------------

# 4	References

[1] RP-180181, 3GPP TR 38.810 v2.0.0, Feb 2018

# [2] R4-1803572 TP to TR 38.810 on Measurement Uncertainty of Mismatch