3GPP TSG-RAN WG4 meeting #86bis	R4-1806009
Melbourne, Australia, 16 April – 20 April, 2018

Title:	draft LS reply to RAN1 on P_0 ranges on UL power control

Reply to:	R1-1721608

Release:	Rel-15

Work Item:	NR_newRAT

Source:	ZTE Corporation

To:	RAN1

Cc:	RAN2

Contact Persons:

#### Name:               CAO Aijun

#### E-mail Address: cao.aijun@zte.com.cn

Attachments: 	None

1. Overall Description:

RAN4 thanks RAN1 for the LS on P0 range on UL power control. RAN4 has had discussions both online and offline in the most recent several WG meetings, and at the request of some operator, a reply LS R4-1803466 was agreed via email approval after RAN4#86 even though RAN4 study on the P0 range has not been finalized.

RAN4 studies further on X and Y values, and the feasible lowest value for P0 range and the range of preamble received target power:

- Due to operators’ demand for some special deployment, e.g., DAS, the maximum P0 (Y) for PRACH should be -60 dBm, changed from -80 dBm shown in the previous reply LS.

- Two options for X

  - Option 1: 71 dB

  - Option 2: 76 dB

- With 2 dB step size unchanged, RAN2 needs to increase the range of preamble received target power to cover the range based on bullets (1) and (2). .

- For P0 range of PUSCH/PUCCH/SRS, RAN4 also finds a lower minimum useful SNR is possible in NR than that in LTE (i.e., -5 dB), thus a lower value than -126 in the term “-126 - X” in the RAN1 LS may be applicable.

- RAN4 will conduct further study and finalize the overall range of P0 for UL power control in RAN4 #87.

RAN4 kindly asks RAN1/RAN2 to consider the above in the future work for the P0 range and the range of preamble received target power described in R1-1721608.

2. Actions:

To: RAN WG1/WG2

ACTION:

RAN4 kindly asks RAN1/RAN2 to consider the above in the future work for the P0 range and the range of preamble received target power described in R1-1721608.

3. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #87	 21 - 25 May 2018                                         Busan, KR