3GPP TSG-RAN WG4 Meeting #87 	R4-1806184

# Busan, South Korea, May 21 – 25, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-2 on the power class requirement for FR2 handheld UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., Intel Corporation, vivo, Xiaomi, OPPO, SGS Wireless, TCL, Spreadtrum, Asus, Kyocera |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Agreements on peak EIRP and spherical coverage have been reached and can be captured in the specification as components of the FR2 power class requirement for NR FR2 handheld UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaces the range of peak EIRP values with a single value for FR2 handheld UEs Introduces the spherical coverage requirement as a component of the UE maximum output power requirement for FR2 handheld UEs Various editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The power class requirement for FR2 handheld UEs would not be defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-2 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of change>

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The values listed on the table below are for handheld UE, defined as minimum peak EIRP.

Table 6.2.1-1: NR FR2 UE Power Class

| NR band | Handheld Power Class Min Peak EIRP (dBm) |
| --- | --- |
| n257 | 22.5 |
| n258 | 22.5 |
| n260 | 20.5 |
| n261 | 22.5 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1-2 below. The maximum allowed EIRP is derived from regulatory requirements [8].

Table 6.2.1-2: NR UE Maximum Output Power Limits

| NR band | TRP Handheld (dBm) | EIRP (dBm) Handheld |
| --- | --- | --- |
| n257 | TBD | 43 |
| n258 | TBD | 43 |
| n260 | TBD | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the indicated region around the UE (such as sphere, hemisphere, etc.) is defined as the spherical coverage requirement and is found in Table 6.2.1-3 below.

Table 6.2.1-3: NR FR2 UE Spherical Coverage

| NR band | Handheld Power Class Min EIRP at 50%-tile CDF (dBm) |
| --- | --- |
| n257 | 8.0 |
| n258 | 8.0 |
| n260 | 2.5 |
| n261 | 8.0 |
| NOTE 1: Minimum EIRP at 50%-tile CDF is defined as the lower limit without tolerance NOTE 2: For handheld UEs the applicable region is the full sphere |  |

### 6.2.2	UE maximum output power for modulation / channel bandwidth

Detailed content of the subclause is TBD.

### 6.2.3	UE maximum output power with additional requirements

Detailed content of the subclause is TBD.

### 6.2.4	Configured transmitted power

Detailed content of the subclause is TBD.

<end of change>