3GPP TSG-RAN WG4 Meeting #87	R4-1806310

Busan, South Korea, 21 – 25 May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5772 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on interruption for SCell activation and deactivation with sTTI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_sTTIandPT-Core |  |  |  |  |  |  | Date: |  | 2018-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The interruption location for SCell activation and deactivation in section 7.7 is not modified with STTI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the interruption location for SCell activation and deactivation modified with STTI in section 7.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interruption location for SCell activation and deactivation is not modified with STTI in section 7.7. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change>>

### 7.7.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell. The requirements in this section are applicable for E-UTRA FDD, E-UTRA TDD and E-UTRA TDD-FDD carrier aggregation.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in subframe n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command as specified in [17] for the SCell being activated no later than in subframe n+Nact_known provided the following conditions are met for the SCell:

-	During the period equal to max(5 measCycleSCell, 5 DRX cycles) before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SCell being activated remains detectable according to the cell identification conditions specified in section 8.3.3.2,

-	SCell being activated also remains detectable during the SCell activation delay according to the cell identification conditions specified in section 8.3.3.2 where

Nact_known =24;

Nact_known =23 if the activation command is transmitted on the PDSCH with 1ms TTI and shortened processing time;

Nact_known =22 if the activation command is transmitted on the PDSCH with 1 slot TTI or with subslot TTI;

Otherwise upon receiving the SCell activation command in subframe n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command as specified in [17] for the SCell being activated no later than in subframe n+Nact_unknown provided the SCell can be successfully detected on the first attempt where

Nact_unknown =34;

Nact_unknown =33 if the activation command is transmitted on the PDSCH with1ms TTI and shortened processing time;

Nact_unknown =32 if the activation command is transmitted on the PDSCH with 1 slot TTI or with subslot TTI;

If there is no reference signal received for the CSI measurement over the delay corresponding to the minimum requirements specified above, then the UE shall report corresponding valid CSI for the activated SCell on the next available uplink reporting resource after receiving the reference signal.

The SCell activation delay specified in this section can be extended with each SRS carrier based switching to any carrier occuring during the SCell activation procedure.

If there are no uplink resources for reporting the valid CSI in subframe n+Nact_known or n+Nact_unknown or uplink transmission is interrupted due to SRS carrier based switching then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

The valid CSI is based on the UE measurement and corresponds to any CQI value specified in [3] with the exception of CQI index = 0 (out of range) provided:

-	the conditions in section 7.7 are met over the entire SCell activation delay and

-	the conditions for CQI reporting defined in Section 7.2.3 of [3] are met.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [17] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

The PCell interruption specified in section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+9 when PCell belongs to E-UTRA FDD.

Note: The PCell interruption due to SCell activation and deactivation shall not occur before

-	subframe n+ 5 if the deactivation command is transmitted on the PDSCH with 1ms TTI and 4 subframe HARQ processing time;

-	subframe n+ 4 if the deactivation command is transmitted on the PDSCH with 1ms TTI and 3 subframe HARQ processing time;

-	subframe n+ 3 if the deactivation command is transmitted on the PDSCH with 1 slot TTI;

-	subframe n+ 2 if the deactivation command is transmitted on the PDSCH with subslot TTI.

The above specification applies to all the statements on the location of PCell interruptin due to SCell activation and deactivation in section 7.7.

The PCell interruption specified in section 7.8.2 shall not occur before subframe n+5 and not occur after subframe n+11 when PCell belongs to E-UTRA TDD.

Starting from the subframe specified in section 4.3 of [3] and until the UE has completed the SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for the SCell.

## << End of Change>>