3GPP TSG-RAN WG4 Meeting #87                                                                R4-1806319

Busan, KR, 21th – 25th May, 2018

Agenda item:	6.21.4.1

Source:	Intel Corporation

Title:	PDSCH Demodulation requirements under fading conditions with 1024QAM

Document for:	Discussion

- Introduction

In RAN4#86bis demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM were discussed and a way forward [1] was approved. In this paper we provide simulation results and discuss PDSCH demodulation requirements in fading conditions with 1024QAM.

- Discussion

For PDSCH demodulation requirements with 1024QAM in fading channel conditions the following common parameters were agreed upon:

| Parameters | Value |
| --- | --- |
| Bandwidth | 10MHz |
| Channel model | EPA5 |
| CFI (OFDM symbol for control channel) | 1 |
| Duplex mode | FDD |

In order to select the parameters for test cases companies were encouraged to provide results for the following options for further evaluation:

| No. | TM | PMI feedback (pre-coding) | MIMO layer | MCS | Antenna config. | Tx EVM |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TM4 | Wideband PMI Option 1: Feedback (PUSCH 3-1, Reporting interval =1) Option 2: Random | Op1: 1-layer Op2: 2-layer | MCS#23 (with 52752 bits) FFS: other MCS for 1-layer | 4x2 Low | Op1: 1.5% Op2: 2% |
| 2 | TM4 | Wideband PMI Option 1: Feedback (PUSCH 3-1, Reporting interval =1) Option 2: Random | Op1: 1-layer Op2: 2-layer | MCS#23 (with 52752 bits) FFS: other MCS for 1-layer | 4x4 Low | Op1: 1.5% Op2: 2% |
| 3 | TM9 | Wideband PMI Option 1: Feedback Option 2: Random | Op1: 1-layer Op2: 2-layer | MCS#23 (with 52752 bits) FFS: other MCS for 1-layer | 4x2 Low | Op1: 1.5% Op2: 2% |
| 4 | TM9 | Wideband PMI Option 1: Feedback Option 2: Random | Op1: 1-layer Op2: 2-layer | MCS#23 (with 52752 bits) FFS: other MCS for 1-layer | 4x4 Low | Op1: 1.5% Op2: 2% |

## Fading Channel Tests in TM4

In TM4 performance was evaluated with the following parameters:

- PMI: Random ,Wideband PMI

- # Rx Antenna: 2Rx, 4Rx

- MIMO Layers: 1,2

- MCS: 23

- # Subframes PDSCH transmission :8

- Tx EVM: 1.5%, 2%

- Rx EVM: 0

Figure 1: Performance in TM4 with 2Rx

With 2 Rx antennas with TM4, all cases achieve maximum throughput except with 2 layers using random PMI.

Observation #1: 4x2, 2 layers with random PMI doesn’t achieve max TP

Figure 2: Performance in TM4 with 4Rx

In TM4 with 4Rx antenna all cases achieve max TP.

In TM4 across all cases the delta in performance between Tx EVM of 1.5, 2% is not significant in EPA5 channel with no Rx EVM.

Observation #2: In TM4 with no Rx EVM, the performance delta between Tx EVM of 1.5 and 2% is not significant

To have sufficient test coverage and limit the number of tests introduced for 1024QAM introduce 4Rx test in TM4 with 2 layer using wideband PMI.

Proposal #1: For PDSCH demodulation test in fading channel with 4Rx introduce test in TM4 with 2 layer using wideband PMI

## Fading Channel Tests in TM9

In TM9 performance was evaluated with the following parameters:

- PMI: Random ,Wideband PMI

- # Rx Antenna: 2Rx, 4Rx

- MIMO Layers: 1,2

- MCS: 23

- # Subframes PDSCH transmission :8

- Tx EVM: 1.5%, 2%

- Rx EVM: 0

Figure 3: Performance in TM9 with 2Rx

With 2 Rx antennas in TM9, 2 layers tests cases don’t achieve max TP using random or wideband PMI.

Observation #3: TM9 with 2 layers doesn’t achieve max TP with random or wideband PMI

Figure 4: Performance in TM9 with 4Rx

With 4Rx antenna all cases achieve max TP.

In TM9 in some cases the delta in performance between Tx EVM of 1.5, 2% is up to 1dB in EPA5 channel with no Rx EVM.

Observation #4: In TM9 with no Rx EVM, the performance delta between Tx EVM of 1.5 and 2% is almost 1dB in some cases

To have sufficient test coverage and limit the number of tests introduced for 1024QAM introduce 2Rx test in TM9 with 1 layer using random PMI.

Proposal #2: For PDSCH demodulation test in fading channel with 2Rx introduce test in TM9 with 1 layer using random PMI

## Effect of Rx EVM

In demodulation performance alignment Rx EVM is not usually considered and included as part of RF impairments model. Rx EVM introduces non-linear distortion resulting in a noise floor such that the max configured throughput is not achieved. The effect of practical Rx EVM in combination with Tx EVM is captured in the figure below.

Figure 5: Effect of Rx EVM

As shown in figure 5 above Rx EVM introduces non-linear performance degradation and limits the max achievable throughput and should be considered in deriving performance requirements.

Observation #5: Rx EVM introduces non-linear performance degradation and limits the max achievable throughput

Proposal #3: Consider practical Rx EVM while deriving demodulation performance requirements

- Conclusion

In this paper we have presented simulation results with 1024QAM in fading channels to derive PDSCH demodulation requirements. Our observations and proposals are summarized as follows:

Observation #1: 4x2, 2 layers with random PMI doesn’t achieve max TP

Observation #2: In TM4 with no Rx EVM, the performance delta between Tx EVM of 1.5 and 2% is not significant

Observation #3: TM9 with 2 layers doesn’t achieve max TP with random or wideband PMI

Observation #4: In TM9 with no Rx EVM, the performance delta between Tx EVM of 1.5 and 2% is almost 1dB in some cases

Observation #5: Rx EVM introduces non-linear performance degradation and limits the max achievable throughput

Proposal #1: For PDSCH demodulation test in fading channel with 4Rx introduce test in TM4 with 2 layer using wideband PMI

Proposal #2: For PDSCH demodulation test in fading channel with 2Rx introduce test in TM9 with 1 layer using random PMI

Proposal #3: Consider practical Rx EVM while deriving demodulation performance requirements

- References

- R4-1805959, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon, Anritsu, Rohde & Schwarz