3GPP TSG-RAN WG4 Meeting #87                                                                R4-1806322

Busan, KR, 21th – 25th May, 2018

Agenda item:	7.10.4.4

Source:	Intel Corporation

Title:	Discussion on Beam Failure Recovery for Beam Management

Document for:	Discussion

- Introduction

In RAN4#86bis Beam Failure Detection and Recovery for Beam Management were discussed. A WF [1] on Beam Management was approved and the following was agreed on Beam Failure Recovery:

- FFS to clarify the definition of beam failure and whether to introduce the beam failure detection requirements in RAN4. - Companies are encouraged to investigate: - Evaluation period for beam link failure detection with/without DRX - SSB-based and CSI-RS based and CSI-RS+SSB based - Whether it is feasible to reuse RLM out-of-synch framework or new sets of parameter values are needed - PDCCH configuration parameters - UE is not assumed to perform BFR outside its active DL BWP - FFS whether and how to define the new beam identification requirements.

In this paper we present our views on Beam Failure Recovery.

- Discussion

In the context of Beam Management, RAN4 shall specify requirements for

- L1 RSRP Measurement

- Beam Failure Recovery

As described in Section 6 of TS38.213, Beam Failure Recovery constitutes 2 parts

- Beam Failure Detection (BFD)

- Beam Identification

A set of resources  is configured to monitor the link quality - Beam-Failure-Detection-RS-ResourceConfig. The UE. The UE monitors only those resources that are QCL with DM-RS of PDCCH monitored by the UE and reports a Beam Failure Instance to higher layer if all resources report link quality worse than Qout,LR. The lower bound on the reporting periodicity is 2ms.

In case UE detects beam failure, it needs to identify a set of beams and indicate them to the gNB. A set of resources is configured as a list of RS corresponding to candidate beams that the UE shall measure L1-RSRP and report the IDs corresponding to the set that has RSRP greater than Qin,LR.

Hypothetical PDCCH

BFD is similar to out-of-sync detection in RLM and the same framework as out-of-sync detection could be used. The configured resource set could consist of CSI-RS and/or SS/PBCH blocks. The evaluation shall be based on hypothetical PDCCH BLER, with same BLEROut.

Proposal #1: Re-use the framework as Out-of-Sync detection for Beam Failure Detection

The PDCCH transmission parameters for BFD determination shall be the same as out-of-sync. If the BFD-RS is CSI-RS, the PDCCH parameters corresponding to table 1 below shall be used and if it is SSB, the PDCCH parameters corresponding to Table 2 below shall be used for hypothetical PDCCH BLER.

Table 1: PDCCH transmission parameters for BFD (CSI-RS)

| Attribute | Value for BLER pair#0 | Value for BLER pair#1 |
| --- | --- | --- |
| DCI format | 1-0 | TBD |
| Number of control OFDM symbols | Same as the number of symbols of CORESET QCL with CSI-RS |  |
| Aggregation level (CCE) | 8 |  |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | [3 or 4]dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | [3 or 4]dB |  |
| Bandwidth (MHz) | Same as the number of PRBs of CORESET QCL with CSI-RS |  |
| Sub-carrier spacing (kHz) | Same as the SCS of CORESET QCL with CSI-RS |  |
| DMRS precoder granularity | Same as DMRS precoder granularity of CORESET QCL with CSI-RS |  |
| REG bundle size | Same as REG bundle size of CORESET QCL with CSI-RS |  |
| CP length | Same as the CP length of CORESET QCL with CSI-RS |  |
| Mapping from REG to CCE | Same as REG to CCE mapping of CORESET QCL with CSI-RS |  |

Table 2: PDCCH transmission parameters for BFD (SSB)

| Attribute | Value for BLER pair#0 | Value for BLER pair#1 |
| --- | --- | --- |
| DCI format | 1-0 | TBD |
| Number of control OFDM symbols | Same as the number of symbols of RMSI CORESET |  |
| Aggregation level (CCE) | 8 |  |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | [3 or 4]dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | [3 or 4]dB |  |
| Bandwidth (MHz) | Same as the number of PRBs of RMSI CORESET |  |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Same as the CP length of RMSI CORESET |  |
| Mapping from REG to CCE | Distributed |  |

Proposal #2: Hypothetical PDCCH transmission parameters for Beam Failure detection shall be same as out-of-sync for RLM

Evaluation Period for BFD

The reporting interval of beam failure is dependent on the periodicity of CSI-RS/ SSB and lower bounded by 2ms. The evaluation period should be ≥ the reporting interval. The evaluation period for BFD would depend on the desired measurement accuracy of SINR over the BFD-RS. Since BFD is desired to be faster than RLF evaluation, the evaluation period should be smaller than that for Out-of-Sync.

Observation #1: The evaluation period for BFD depends on the accuracy of SINR measurement on BFD-RS

Proposal #3: Further study the accuracy requirements needed for SINR measurement for BFD

Proposal #4: Evaluation period for BFD shall be less than that for Out-of-Sync

Beam Identification

Beam Identification requires the UE to measure the RSRP on the configured set of RSs of candidate beams. As beam identification is a part of BFR, it would be essential to have tests introduced to test beam identification.

Proposal#5: RAN4 introduces beam identification requirements

The requirements for L1 RSRP are under discussion. Any measurement accuracy requirements introduced for L1 RSRP might not be sufficient to test beam identification. In [2] we proposed a methodology to test beam detection accuracy for configured CSI-RS resource set. Such beam detection accuracy could also be used for beam identification.

Proposal#6: Introduce beam detection measurement accuracy tests for Beam Identification

There are a few key aspects for L1-RSRP measurement discussed in [2] that would apply to RSRP measurement for beam identification as well.

- Number of samples

- Tx Beam configuration

- CSI-RS density

Proposal#7: Further study the number of samples, CSI-RS density, Tx beam configuration for beam identification

- Conclusion

In this paper we have provided our views on Beam Failure Detection and Beam Identification framework and requirements and have the following proposals:

Proposal #1: Re-use the framework as Out-of-Sync detection for Beam Failure Detection

Proposal #2: Hypothetical PDCCH transmission parameters for Beam Failure detection shall be same as out-of-sync for RLM

Proposal #3: Further study the accuracy requirements needed for SINR measurement for BFD

Proposal #4: Evaluation period for BFD shall be less than that for Out-of-Sync

Proposal#5: RAN4 introduces beam identification requirements

Proposal#6: Introduce beam detection measurement accuracy tests for Beam Identification

Proposal#7: Further study the number of samples, CSI-RS density, Tx beam configuration for beam identification

- References

- R4-1805530, “Way Forward on Beam Management”, Ericsson

- R4-180xxxx, “Discussion about CSI-RS based L1-RSRP for beam detection”, Intel Corporation