3GPP TR 36.715-02-01 V0.4.0 (2018-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE Advanced inter-band Carrier Aggregation for 2DL/1UL

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	9

4.1	TR Maintenance	9

5	5 Band Carrier Aggregation with Single UL: Specific Band Combination Part	9

5.1	CA_20A-40A_1UL_BCS1	9

5.1.1 Operating band for CA	9

5.1.3 	UE co-existence studies	10

5.1.4 	∆TIB and ∆RIB values	10

5.1.5 	REFSENS	10

5.2	CA_38A-40A_1UL_BCS1	11

5.2.1 Operating band for CA	11

5.2.2 Channel bandwidths per operating band for CA	11

5.2.3 	UE co-existence studies	11

CA_38A-40A is TDD-TDD CA combination and the co-existence studies could be found in TR 36.852-13.	11

5.2.4 	∆TIB and ∆RIB values	11

5.3 CA_1A-7A_1UL_BCS1	12

5.3.1	Channel bandwidths per operating band for CA	12

5.3.2	Co-existence studies	12

5.3.4	REFSENS requirements	12

5.4 CA_7A-20A_1UL_BCS2	12

5.4.1	Channel bandwidths per operating band for CA	13

5.4.2	Co-existence studies	13

5.4.3	ΔTIB and ΔRIB values	13

5.4.4	REFSENS requirements	13

5.5 CA_2DL_8A-20A_1UL_BCS2	13

5.5.1 Operating bands for CA	13

5.5.2	Channel bandwidths per operating band for CA	13

5.5.3  Co-existence studies	13

5.5.4	 ΔTIB,c and ΔRIB,c values	13

5.6 CA_34A-39A	14

5.6.1	Operating bands for CA	14

5.6.2	Channel bandwidths per operating band for CA	14

5.6.3	Co-existence studies	14

5.6.4 ΔTIB,c and ΔRIB,c values	15

5.7 CA_34A-41A	16

5.7.1	Operating bands for CA	16

5.7.2	Channel bandwidths per operating band for CA	16

5.7.3	Co-existence studies	16

5.7.4 ΔTIB,c and ΔRIB,c values	17

5.8	CA_40A-43A_BCS0	18

5.8.1 Operating bands for CA	18

5.8.2	Channel bandwidths per operating band for CA	18

5.8.3   Co-existence studies	18

5.8.4	∆TIB,c and ∆RIB,c values	19

5.8.5	REFSENS requirements	19

5.9	CA_42A-43A_BCS0	19

5.9.1 Operating bands for CA	19

5.9.2	Channel bandwidths per operating band for CA	20

5.9.3   Co-existence studies	20

5.9.4	∆TIB,c and ∆RIB,c values	20

5.9.5	REFSENS requirements	21

5.10	CA_8A-32A_BCS0	21

5.10.1 Operating bands for CA	21

5.10.2	Channel bandwidths per operating band for CA	21

5.10.3   Co-existence studies	21

5.10.4	∆TIB,c and ∆RIB,c values	22

5.11	CA_8A-38A_BCS0	23

5.11.1 Operating bands for CA	23

5.11.2	Channel bandwidths per operating band for CA	23

5.11.3   Co-existence studies	23

5.11.4	∆TIB,c and ∆RIB,c values	23

5.12	CA_66A-70A_BCS0	24

5.12.1	Channel bandwidths per operating band for CA	24

5.12.2	UE co-existence studies	24

5.12.3	ΔTIB,c and ΔRIB,c values	25

5.12.4	REFSENS requirements	25

5.13	CA_11A-26A_BCS0	26

5.14	CA_4A-71A_BCS0	28

5.14.1	Channel bandwidths per operating band for CA	28

5.14.2	Co-existence studies	28

5.14.4	REFSENS requirements	29

5.15	CA_66A-71A_BCS0	29

5.15.1	Channel bandwidths per operating band for CA	29

5.15.2	Co-existence studies	29

5.15.4	REFSENS requirements	30

5.16	CA_2DL_2A-14A_1UL_BCS0	30

5.16.3	Co-existence studies	31

5.17	CA_2DL_14A-66A_1UL_BCS0	31

5.17.3	Co-existence studies	32

5.18	CA_2DL_14A-30A_1UL_BCS0	33

5.18.2	Co-existence studies	33

5.19	CA_2A-71A	34

5.19.1	LTE-Advanced Carrier Aggregation of Band 2 and Band 71 (1 UL)	34

5.19.1.1	 List of specific combination issues	34

5.19.1.1.1	Channel bandwidths per operating band for CA	34

5.19.2	Coexistence	34

6.2.3	∆TIB and ∆RIB values	34

5.19.4	MSD	35

5.20	CA_2DL_26A-48A_1UL_BCS0	35

5.20.3	Co-existence studies	35

5.21	CA_2DL_28A-38A_1UL_BCS0	36

5.21.3	Co-existence studies	37

5.22	CA_2DL_12A-46A-1UL_BCS0	38

5.22.1	Channel bandwidths per operating band for CA	38

5.22.2	UE Co-existence studies	38

5.22.3	∆TIB,c and ∆RIB,c values	38

5.22.4	REFSENS requirements	39

5.23	CA_70A-71A_BCS0	39

5.23.1	Channel bandwidths per operating band for CA	39

5.23.2	UE co-existence studies	39

5.23.3	ΔTIB,c and ΔRIB,c values	40

5.23.4	REFSENS requirements	41

5.24	LTE-Advanced Carrier Aggregation of Band 2 and Band 71 (1 UL)	42

5.24.1.1	 List of specific combination issues	42

5.24.1.1.1	Channel bandwidths per operating band for CA	42

5.24.1.1.2	Coexistence	42

6.2.1.1.3	∆TIB and ∆RIB values	42

5.24.1.1.4.1	Band 2 MSD	43

5.24.1.1.4.2	Band 71 MSD	45

5.25 CA_1A-32A_BCS0	49

5.25.1 Operating bands for CA	49

5.25.2	Co-existence studies	49

5.25.3 ΔTIB,c and ΔRIB,c values	50

5.25.4	REFSENS requirements	50

5.26	CA_2DL_48A-71A_1UL_BCS0	50

5.26.1	Channel bandwidths per operating band for CA	50

5.26.2	Co-existence studies	50

5.26.3	ΔTIB,c and ΔRIB,c values	51

5.26.4	REFSENS requirements	51

5.27	CA_2DL_46A-71A_1UL_BCS0	51

5.27.1	Channel bandwidths per operating band for CA	51

5.27.2	Co-existence studies	51

5.27.3	ΔTIB,c and ΔRIB,c values	52

5.27.4	REFSENS requirements	52

5.28	CA_32A-43A_1UL_BCS0	52

5.28.1 Operating band for CA	52

5.28.3 	UE co-existence studies	53

5.28.4 	∆TIB and ∆RIB values	53

5.28.5 	REFSENS	53

5.29	CA_20A-43A_1UL_BCS0	54

5.29.1 Operating band for CA	54

5.29.3 	UE co-existence studies	54

5.29.4 	∆TIB and ∆RIB values	54

5.29.5 	REFSENS	55

There is no specific REFSENs requirements.	55

5.30	CA_32A-42A_1UL_BCS0	55

5.30.1 Operating band for CA	55

5.30.3 	UE co-existence studies	55

5.30.4 	∆TIB and ∆RIB values	56

5.30.5 	REFSENS	56

5.31	CA_1A-43A_1UL_BCS0	56

5.31.1 Operating band for CA	56

5.31.3 	UE co-existence studies	57

5.31.4 	∆TIB and ∆RIB values	57

5.31.5 	REFSENS	57

5.32	CA_3A-43A_1UL_BCS0	58

5.32.1 Operating band for CA	58

5.32.3 	UE co-existence studies	58

5.32.4 	∆TIB and ∆RIB values	58

5.32.5 	REFSENS	59

There is no specific REFSENs requirements.	59

5.33	CA_2DL_25A-46A-1UL_BCS0	59

5.33.2	Channel bandwidths per operating band for CA	59

5.33.3	UE Co-existence studies	59

5.33.4	∆TIB,c and ∆RIB,c values	60

5.33.5	Reference sensitivity	60

5.34	CA_3A-18A_BCS0	60

5.34.1	Operating bands for CA	60

5.34.2	Channel bandwidths per operating band for CA	61

5.34.3	Co-existence studies	61

5.34.4	∆TIB and ∆RIB values	62

5.34.5	REFSENS requirements	62

5.35	CA_2DL_46A-48A_1UL_BCS0	62

5.35.1	Channel bandwidths per operating band for CA	62

5.35.2	Co-existence studies	62

5.35.3	ΔTIB,c and ΔRIB,c values	63

5.35.4	MSD	63

5.36	CA_2DL_5A-28A_1UL_BCS0	64

5.36.3	Co-existence studies	64

5.37	CA_8A-27A	65

5.37.1	Operating band for CA	65

5.37.3	UE Co-existence studies	65

5.37.4	∆TIB,c and ∆RIB,c values	66

5.X	CA_xA-yA_BCSn	67

5.X.1	Operating bands for CA	67

5.X.2	Channel bandwidths per operating band for CA	67

5.X.3	Co-existence studies	67

5.X.4	∆TIB and ∆RIB values	67

5.X.5	REFSENS requirements	67

5.X.6	Blocking requirements	67

Annex A: Change history	68

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for 2DL/1UL Inter-band Carrier Aggregation under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address 2DL/1UL Inter-band Carrier Aggregation requirements for the Rel-15 band combinations in Table 1-1.

Table 1-1: Release 15 2DL/1UL inter-band carrier aggregation combinations

| CA combination | REL independent from |
| --- | --- |
| CA_20A-40A | 12 |
| CA_38A-40A | 10 |
| CA_1A-7A | 10 |
| CA_7A-20A | 10 |
| CA_8A-20A | 10 |
| CA_34A-39A | 10 |
| CA_34A-41A | 10 |
| CA_40A-43A | 10 |
| CA_42A-43A | 10 |
| CA_8A-32A | 10 |
| CA_8A-38A | 12 |
| CA_66A-70A | 10 |
| CA_11A-26A | 10 |
| CA_4A-71A | 10 |
| CA_66A-71A | 10 |
| CA_2A-14A | 10 |
| CA_14A-66A | 10 |
| CA_14A-30A | 10 |
| CA_2A-71A | 10 |

This TR contains a band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-171026,	“New WID on LTE Advanced inter-band CA Rel-15 for 2DL/1UL”

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [2] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [2].

# 4	Background

The present document is a technical report for 2DL/1UL Inter-band Carrier Aggregation under Rel-15 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	5 Band Carrier Aggregation with Single UL: Specific Band Combination Part

## 5.1	CA_20A-40A_1UL_BCS1

### 5.1.1 Operating band for CA

Table 5.X.1-1: Inter-band CA operating bands for CA_20-40

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_20-40 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

5.1.2 Channel bandwidths per operating band for CA

Table 5.1.2-1: E-UTRA CA configuration and bandwidth combination sets for 1UL CA_20A-40A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_20A-40A | - | 20 |  |  | Yes | Yes | Yes |  | 35 | 1 |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

### 5.1.3 	UE co-existence studies

CA_20A-40A has harmonic mixing issue, and the corresponding co-existence studies could be found in TR 36.852-13 for this CA combination.

### 5.1.4 	∆TIB and ∆RIB values

For CA_20A-40A_1UL_BCS1, the same requirements on ∆TIB and ∆RIB of CA_20A-40A_1UL_BCS0 case can apply. Therefore, the TIB,c and RIB,c values are shown in table 5.X.4-1 and in table 5.X.4-2.

Table 5.1.4-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_20A-40A | 20 | 0.3 |
|  | 40 | 0.3 |

Table 5.1.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_20A-40A | 20 | 0 |
|  | 40 | 0 |

### 5.1.5 	REFSENS

For the REFSENS of B20 for CA_20A-40A-40A_1UL_BCS1 due to harmonic mixing issue, the exceptions are shown in Table 5.X.5-1 and Table 5.X.5-2.

Table 5.1.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_20A-40A15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. … NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 5.1.5-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_20A-40A3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| NOTE3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. |  |  |  |  |  |  |  |  |

## 5.2	CA_38A-40A_1UL_BCS1

### 5.2.1 Operating band for CA

Table 5.2.1-1: Inter-band CA operating bands for CA_38-40

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_38-40 | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 5.2.2 Channel bandwidths per operating band for CA

Table 5.2.2-1: E-UTRA CA configuration and bandwidth combination sets for 1UL CA_38A-40A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_38A-40A | - | 38 |  |  |  | Yes | Yes | Yes | 40 | 1 |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

### 5.2.3 	UE co-existence studies

### CA_38A-40A is TDD-TDD CA combination and the co-existence studies could be found in TR 36.852-13.

### 5.2.4 	∆TIB and ∆RIB values

The same requirements on ∆TIB and ∆RIB of CA_38A-40A_1UL_BCS0 case can apply for this case. Therefore, the TIB,c and RIB,c values are shown in table 5.X.4-1 and in table 5.X.4-2.

Table 5.2.4-1: TIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_38A-40A | 38 | 04 |
|  | 40 | 04 |
| … NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.2.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_38A-40A | 38 | 0.54 |
|  | 40 | 0.54 |
| … NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

## 5.3 CA_1A-7A_1UL_BCS1

### 5.3.1	Channel bandwidths per operating band for CA

Table 5.3.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Ma3imum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.3.2	Co-existence studies

The analysis has been done in CA_1A-7A_BCS0, No further analysis is needed.

### 5.3.3	ΔTIB and ΔRIB values

The UE architecture is same as that of CA_1A-7A_BCS0. Same values of TIB,c and ΔRIB,c can be re-used.

### 5.3.4	REFSENS requirements

REFSENS remain unchanged.

## 5.4 CA_7A-20A_1UL_BCS2

### 5.4.1	Channel bandwidths per operating band for CA

Table 5.4.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_7A-20A | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 2 |
|  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.4.2	Co-existence studies

The analysis has been done in CA_7A-20A_BCS0/CA_7A-20A_BCS1, No further analysis is needed.

### 5.4.3	ΔTIB and ΔRIB values

The UE architecture is same as that of CA_7A-20A_BCS0/ CA_7A-20A_BCS1. No different values of TIB,c and ΔRIB,c would be needed.

### 5.4.4	REFSENS requirements

REFSENS remain unchanged.

## 5.5 CA_2DL_8A-20A_1UL_BCS2

### 5.5.1 Operating bands for CA

Table 5.5.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-20 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

### 5.5.2	Channel bandwidths per operating band for CA

Table 5.5.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-20A | 8 |  |  | Yes | Yes |  |  | 30 | 2 |
|  | 20 |  |  |  | Yes | Yes | Yes |  |  |

### 5.5.3  Co-existence studies

Co-existence already studied when BCS0 for CA_8A-20A was introduced.

### 5.5.4	 ΔTIB,c and ΔRIB,c values

Insertion loss values already defined when BCS0 for CA_8A-20A was introduced.

## 5.6 CA_34A-39A

### 5.6.1	Operating bands for CA

Table 5.6.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_34-39 | 34 | 2010 MHz | -- | 2025 MHz | 2010 MHz |  | 2025 MHz | TDD |
|  | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |

### 5.6.2	Channel bandwidths per operating band for CA

Table 5.6.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_34A-39A | 34 |  |  | yes | yes | yes |  | 35 | 0 |
|  | 39 |  |  |  | yes | yes | yes |  |  |

### 5.6.3	Co-existence studies

The harmonic frequencies are observed in Table 5.6.3-1:

Table 5.6.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 4020 | 4050 | 6030 | 6075 |
| 39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 3760 | 3840 | 5640 | 5760 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 34 and Band 39 DL carriers can be calculated as shown in table 5.6.3-2 below:

Table 5.6.3-2: Band 34 and Band 39 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1880 | 1920 | 2010 | 2025 |
| 2nd order harmonics frequency range (MHz) | 3760 | 3840 | 4020 | 4050 |
| 3rd order harmonics frequency range (MHz) | 5640 | 5760 | 6030 | 6075 |
| 2nd order IMD products | (f2_low – f1_high) | (f2_high – f1_low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 90 | 145 | 3890 | 3945 |
| 3rd order IMD products | f2_low – 2*f1_high | f2_high – 2*f1_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limits (MHz) | 1830 | 1735 | 2100 | 2170 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 5770 | 5865 | 5900 | 5970 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 1865 | 1935 | 1970 | 2065 |
| 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency limits (MHz) | 1865 | 1935 | 1990 | 2045 |

It can be seen from table 5.6.3-2 that:

-	the 2nd harmonics of BS transmitting in Bands 34 and 39 may fall into BS receive band 43

-	and 3rd harmonics of BS transmitting in Bands 34 and 39 may fall into BS receive band 46

-	the 3rd IMD products may fall into BS receive Bands 1, 2, 3, 4, 9, 10, 23, 25, 33, 34, 35, 36, 37, 39, 46, 47, 65 and 66.

If the BS only transmits up to 15 MHz and 20 MHz DL in Band 34 and Band 39 respectively as stated in Table 5.6.2-1, the 3rd IMD products may still fall into the BS receive band of the Bands 1, 23, 34, 36 and 65 as shown in the last row in table 5.6.3-2.

In addition, it should be noted that Bands 2, 4, 9, 10, 23, 25, 33, 35, 36, 37, 46, 47, 65 and 66 are not intended for use in the same geographical area as Bands 34 and 39. This leaves Bands 1, 3, 34, and 39 to consider for IMD products.

It is recommended that Bands 34 and 39 BS transmitters do not share the same antenna with band of Bands 1, 3, 34 and 39 BS receivers so that the antenna PIM will not cause Bands 1, 3, 34, and 39 BS receiver desensitization.

### 5.6.4 ΔTIB,c and ΔRIB,c values

For TDD inter-band CA_B34_B39 UE supporting 2DL +1UL without simultaneous Rx/Tx, the TIB,c and RIB values are given in Table 5.x.4-1 and 5.x.4-2.

Table 5.6.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_34A-39A | 34 | 01 |
|  | 39 | 01 |
| Note1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRAN band and without simultaneous Rx/Tx |  |  |

Table 5.x.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_34A-39A | 34 | 0.21 |
|  | 39 | 0.21 |
| Note1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRAN band and without simultaneous Rx/Tx |  |  |

## 5.7 CA_34A-41A

### 5.7.1	Operating bands for CA

Table 5.7.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_34-41 | 34 | 2010 MHz | -- | 2025 MHz | 2010 MHz |  | 2025 MHz | TDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 5.7.2	Channel bandwidths per operating band for CA

Table 5.7.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_34A-41A | 34 |  |  | yes | yes | yes |  | 35 | 0 |
|  | 41 |  |  |  |  |  | yes |  |  |

### 5.7.3	Co-existence studies

The harmonic frequencies are observed in Table 5.7.3-1:

Table 5.7.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 4020 | 4050 | 6030 | 6075 |
| 41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 4992 | 5380 | 7488 | 8070 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 34 and Band 41 DL carriers can be calculated as shown in table 5.7.3-2 below:

Table 5.7.3-2: Band 34 and Band 41 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2010 | 2025 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 4020 | 4050 | 4992 | 5380 |
| 3rd order harmonics frequency range (MHz) | 6030 | 6075 | 7488 | 8070 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 471 | 680 | 4506 | 4715 |
| 3rd order IMD products | f2_low – 2*f1_high | f2_high – 2*f1_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limits (MHz) | 1554 | 1330 | 2967 | 3370 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 6516 | 6740 | 7002 | 7405 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 1816 | 2219 | 2481 | 2705 |
| 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency limits (MHz) | 1990 | 2045 | 2481 | 2705 |

It can be seen from table 5.7.3-2 that:

-	the 2nd harmonics of BS transmitting in Bands 34 and 39 may fall into BS receive band 46

-	the 3rd IMD products may fall into BS receive Bands 1, 2, 7, 11, 21, 23, 25, 33, 34, 35, 36, 37, 38, 41, 45 and 65.

If the BS only transmits up to 15 MHz and 20 MHz DL in Band 34 and Band 41 respectively as stated in Table 5.7.2-1, the 3rd IMD products may still fall into the BS receive band of the Bands 7, 23, 34, 38, 41 and 65 as shown in the last row in table 5.7.3-2.

In addition, it should be noted that Bands 2, 7, 11, 21, 23, 25, 33, 35, 36, 37, 38, 45 and 65 are not intended for use in the same geographical area as Bands 34 and 41. This leaves Bands 1, 34, and 41 to consider for IMD products.

It is recommended that Bands 34 and 41 BS transmitters do not share the same antenna with band of Bands 1, 34 and 41 BS receivers so that the antenna PIM will not cause Bands 1, 34, and 41 BS receiver desensitization.

### 5.7.4 ΔTIB,c and ΔRIB,c values

For TDD inter-band CA_34A_41A UE supporting 2DL +1UL without simultaneous Rx/Tx, the TIB,c and RIB values are given in Table 5.x.4-1 and 5.x.4-2.

Table 5.x.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_34A-41A | 34 | 01 |
|  | 41 | 01 |
| Note1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRAN band and without simultaneous Rx/Tx |  |  |

Table 5.x.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_34A-41A | 34 | 0.21 |
|  | 41 | 0.21 |
| Note1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRAN band and without simultaneous Rx/Tx |  |  |

## 5.8	CA_40A-43A_BCS0

### 5.8.1 Operating bands for CA

Table 5.8.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_40-43 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |

### 5.8.2	Channel bandwidths per operating band for CA

Table 5.8.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_40A-43A | 40 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

### 5.8.3   Co-existence studies

Table 5.8.3-1 gives the harmonic frequency limits for Band 40 and Band 43 2DL CA. As shown in the table, UL carrier of CA_40A-43A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.8.3-1: DL harmonics frequency limits for CA of Band 40 and Band 43

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2300 | 2400 | 3600 | 3800 |
| 2nd order DL harmonics frequency range (MHz) | 4600 to 4800 |  | 7200 to 7600 |  |
| 3rd order DL harmonics frequency range (MHz) | 6900 to 7200 |  | 10800 to 11400 |  |

As shown in Table 5.8.3-2, the harmonic frequencies of Band 40 and Band 43 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.8.3-2: UL harmonics frequency limits for CA of Band 40 and Band 43

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 |
| 43 | 3600 | 3800 | 3600 | 3800 | 7200 | 7600 | 10800 | 11400 |

### 5.8.4	∆TIB,c and ∆RIB,c values

For 2DL/1UL CA_2DL_40A-43A_1UL_BCS0, the TIB,c and RIB,c  values are shown in table 6.x.4-1 and table 6.x.4-2, respectively.

# Table 6.x.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_40A-43A | 40 | 04 |
|  | 43 | 0.54 |
| NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

# Table 6.x.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_40A-43A | 40 | 0.44 |
|  | 43 | 0.54 |
| NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.8.5	REFSENS requirements

No MSD issue for this CA_2DL_40A-43A_1UL_BCS0, since the CA bands are synchronized.

## 5.9	CA_42A-43A_BCS0

### 5.9.1 Operating bands for CA

Table 5.9.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_42-43 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |

### 5.9.2	Channel bandwidths per operating band for CA

Table 5.9.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_42A-43A | 42 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

### 5.9.3   Co-existence studies

Table 5.9.3-1 gives the harmonic frequency limits for Band 42 and Band 43 2DL CA. As shown in the table, UL carrier of CA_42A-43A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.9.3-1: DL harmonics frequency limits for CA of Band 42 and Band 43

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 3400 | 3600 | 3600 | 3800 |
| 2nd order DL harmonics frequency range (MHz) | 6800 to 7200 |  | 7200 to 7600 |  |
| 3rd order DL harmonics frequency range (MHz) | 10200 to 10800 |  | 10800 to 11400 |  |

As shown in Table 5.9.3-2, the harmonic frequencies of Band 42 and Band 43 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.9.3-2: UL harmonics frequency limits for CA of Band 42 and Band 43

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 42 | 3400 | 3600 | 3400 | 3600 | 6800 | 7200 | 10200 | 10800 |
| 43 | 3600 | 3800 | 3600 | 3800 | 7200 | 7600 | 10800 | 11400 |

### 5.9.4	∆TIB,c and ∆RIB,c values

For 2DL/1UL CA_2DL_42A-43A_1UL_BCS0, the TIB,c and RIB,c  values are shown in table 5.9.4-1 and table 5.9.4-2, respectively. It is common understanding that the single carrier for Band 42 and Band 43 share the same RF chain and filter component, thus the inter-band CA for 42A-43A can be operated as intra-band non-contiguous CA for the frequency range covering Band 42+43. Thus theΔTIB,c and ΔRIB,c will be 0 which is same as single carrier.

# Table 5.9.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_42A-43A | 42 | 04 |
|  | 43 | 04 |
| NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

# Table 5.9.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_42A-43A | 42 | 04 |
|  | 43 | 04 |
| NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.9.5	REFSENS requirements

## No MSD issue for this CA_2DL_42A-43A_1UL_BCS0, since the CA bands are synchronized.

## 5.10	CA_8A-32A_BCS0

### 5.10.1 Operating bands for CA

Table 5.10.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-32 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 32 | N/A |  |  | 1452 MHz | – | 1496 MHz | FDD |

### 5.10.2	Channel bandwidths per operating band for CA

Table 5.10.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-32A | 8 |  | Yes | Yes | Yes |  |  | 30 | 0 |
|  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

### 5.10.3   Co-existence studies

Table 5.10.3-1 gives the harmonic frequency limits for Band 8 and Band 32 2DL CA. As shown in the table, UL carrier of CA_8A-32A_BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.10.3-1: DL harmonics frequency limits for CA of Band 8 and Band 32

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 1452 | 1496 |
| 2nd order DL harmonics frequency range (MHz) | 1850 to 1920 |  | 2904 to 2992 |  |
| 3rd order DL harmonics frequency range (MHz) | 2775 to 2880 |  | 4356 to 4488 |  |

As shown in Table 5.10.3-2, the harmonic frequencies of Band 8 and Band 32 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.10.3-2: UL harmonics frequency limits for CA of Band 8 and Band 32

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 |
| 32 | N/A | N/A | 1452 | 1496 | N/A | N/A | N/A | N/A |

### 5.10.4	∆TIB,c and ∆RIB,c values

The CA_8A-32A belongs to the Low-Mid band combination class, in which the dTib =0.3/dRib =0 were defined for the low band. For the Mid band, due to the extended high-low type of diplexers may be used, thus the dTib =0.4/dRib =0 may apply for the Mid band e.x. Band 11. However for the CA_8A-32A, Band 32 is downlink only. So there is no need to define dTib for Band 32.

For two simultaneous DL and only one UL, the TIB,c and RIB,c  values are shown in table 5.10.4-1 and table 5.10.4-2, respectively.

# Table 5.10.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-32A | 8 | 0.3 |

# Table 5.10.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8A-32A | 8 | 0 |
|  | 32 | 0 |

## 5.11	CA_8A-38A_BCS0

### 5.11.1 Operating bands for CA

Table 5.11.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-38 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 5.11.2	Channel bandwidths per operating band for CA

Table 5.11.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-38A | 8 |  |  | Yes | Yes |  |  | 30 | 0 |
|  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

### 5.11.3   Co-existence studies

Table 5.11.3-1 gives the harmonic frequency limits for Band 8 and Band 38 2DL CA. As shown in the table, UL carrier of CA_8A-38A_BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.11.3-2: DL harmonics frequency limits for CA of Band 8 and Band 38

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 2570 | 2620 |
| 2nd order DL harmonics frequency range (MHz) | 1850 to 1920 |  | 5140 to 5240 |  |
| 3rd order DL harmonics frequency range (MHz) | 2775 to 2880 |  | 7710 to 7860 |  |

As shown in Table 5.11.3-2, the harmonic frequencies of Band 8 and Band 38 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.11.3-3: UL harmonics frequency limits for CA of Band 8 and Band 38

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |

### 5.11.4	∆TIB,c and ∆RIB,c values

Since there is no specific co-existence issue identified, the Low- High band combinations class dTib and dRib value can apply to the 2DL/1UL CA_8A-38A.

For two simultaneous DL and only one UL, the TIB,c and RIB,c  values are shown in table 5.11.4-1 and table 5.11.4-2, respectively.

# Table 5.11.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-38A | 8 | 0.3 |
|  | 38 | 0.3 |

# Table 5.11.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_8A-38A | 8 | 0 |
|  | 38 | 0 |

## 5.12	CA_66A-70A_BCS0

### 5.12.1	Channel bandwidths per operating band for CA

Table 5.12.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_66-70 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.12.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_66A-70A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_66A-70A | 66 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 70 |  |  | Yes | Yes | Yes |  |  |  |

### 5.12.2	UE co-existence studies

UE harmonics are shown in table below.

Table 5.12.2-1 UE harmonics

| UE UL frequencies | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| Frequency [MHz] | 1710 | 1780 | 1695 | 1710 |
| 2nd harmonic [MHz] | 3420 | 3560 | 3390 | 3420 |
| 3rd harmonic [MHz] | 5130 | 5340 | 5085 | 5130 |
| 4th harmonic [MHz] | 6840 | 7120 | 6780 | 6840 |
| 5th harmonic [MHz] | 8550 | 8900 | 8475 | 8550 |

There are no harmonic issues for UE.

### 5.12.3	ΔTIB,c and ΔRIB,c values

Triplexer data is collected in table below.

Table 5.12.3-1 Insertion loss for CA_66A-70A

The average worst case B66 IL is 3.3dB for TX and 2.7dB for RX. The average worst case B70 IL is 3.3dB for TX and 2.1dB for RX. The difference to respective single band duplexer IL is <1dB for TX and <0.5dB for RX. Thus, the generic 3GPP framework values for can be used for both bands.

For the UE which supports CA_66A-70A, the ΔTIB,c and ΔRIB,c  are defined for applicable bands in Table 5.12.3-2 and 5.12.3-3 respectively.

Table 5.12.3-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66A-70A | 66 | 0.5 |
|  | 70 | 0.5 |

Table 5.12.3-3: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66A-70A | 66 | 0 |
|  | 70 | 0 |

### 5.12.4	REFSENS requirements

Isolation values are collected in table below.

Table 5.12.4-1 Isolation for CA_66A-70A

We note that one of the vendors provided information for both conditions, one provided isolation for typical only, and one provided information for worst case only. We have calculated averages for typical conditions and worst case conditions separately. These averages are not directly comparable with each other because they use different data mixes.

TX isolation needs to be above 50dB to avoid MSD for this band while 50dB RX isolation is sufficient. Both the typical and worst case averages show all isolation values are at least 52.75dB. According to our analysis this should be sufficient with the assumptions below to warrant no MSD is needed for B66, B70, and B66+B70 CA.

The following additional assumptions were used in our analysis:

RX IP2					50dB

Phase noise				-156dBc/Hz

B66/B70 FE TX IL	        5.3dB

B66 RX FE IL			  4.9dB

B70 RX FE IL			  4.0dB

PA noise at RX 			-128dBm/Hz

Conclusion: No MSD requirement is needed for B66, B70, and B66+B70 CA when UE supports CA_66A-70A

The impact of UE supporting both B2+B66 and B66+B70 was also evaluated.

It was found out that one option is to have parallel B2+B66 quadplexer and B66+B70 triplexer. The outcome of the study would not change, i.e no MSD is needed. UE RF front-end would be a more complex with B66 duplicated across two components.

Another option would be to modify B2+B66 quadplexer filters to accommodate B70 as well. In this case, the quadplexer would in practice support B2, B66, B70 and any inter-band CA of these. B2 out-of-band blocking capabilities in this option would be worse making it less attractive because the quadplexer would offer attenuation for B2 RX starting 30MHz away from B2 RX upper edge. One quadplexer using an advanced was found to have no impact to the outcome of the study i.e no MSD is needed while another quadplexer using a more conventional technology was found to be unfeasible due to excessive insertion losses.

## 5.13	CA_11A-26A_BCS0

### 5.13.1	Operating bands for CA

# Table 5.13.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| CA_11-26 | 11 | 1427.9 MHz | – | 1447.9 MHz |  | 1475.9 MHz | – | 1495.9 MHz |  | FDD |  |
|  | 26 | 814 MHz | – | 849 MHz |  | 859 MHz | – | 894 MHz |  | FDD |  |

### 5.13.2	Channel bandwidths per operating band for CA

# Table 5.13.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_11A-26A | 11 |  |  |  | Yes | Yes |  |  | 25 | 0 |
|  | 26 |  |  |  | Yes | Yes | Yes |  |  |  |

### 5.13.3	Co-existence studies

Table 5.13.3-1 gives the UL to DL harmonic products for CA band 11 + band 26 with 1UL.

# Table 5.13.3-1: Band 11 and Band 26 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 814 | 849 |
| 2nd order harmonics frequency range (MHz) | 2855.8 to 2895.8 |  | 1628 to 1698 |  |
| 3rd order harmonics frequency range (MHz) | 4283.7 to 4343.7 |  | 2442 to 2547 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 11 and band 26 DL carriers is calculated and shown in table 5.13.3-2 below:

# Table 5.13.3-2: Band 11 and Band 26 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1475.9 | 1495.9 | 859 | 894 |
| 2nd order harmonics frequency range (MHz) | 2951.8 to 2991.8 |  | 1718 to 1788 |  |
| 3rd order harmonics frequency range (MHz) | 4427.7 to 4487.7 |  | 2577 to 2682 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 581.9 to 636.9 |  | 2334.9 to 2389.9 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 2057.8 to 2132.8 3810.8 to 3885.8 |  | 222.1 to 312.1 3193.9 to 3283.9 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1460.9 to 1510.9 |  | 849 to 904 |  |

It can be seen from table 5.13.3-2 that any harmonics or IMD products do not fall into own band 11 and band 26.

### 5.13.4	∆TIB and ∆RIB values

For inter-band CA_11-26, same requirements on values for ∆TIB,c and ∆RIB,c can be applied as per 2UL-2DL case.  Although requirements for UL CA_11A-26A have not been specified yet, those for similar combinations can be re-used such as CA_11A-18A.  Therefore, for two simultaneous DL, the TIB,c and RIB,c values are shown in table 5.13.4-1 and in table 5.13.4-2:

Table 5.13.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_11A-26A | 11 | 0.3 |
|  | 26 | 0.3 |

Table 5.13.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_11A-26A | 11 | 0 |
|  | 26 | 0 |

### 5.13.5	REFSENS requirements

Void

### 5.13.6	Blocking requirements

Void

## 5.14	CA_4A-71A_BCS0

### 5.14.1	Channel bandwidths per operating band for CA

Table 5.14.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_4A-71A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.14.2	Co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 4 and Band 71 UL carriers can be calculated as shown in Table 5.14.2-1 below. None of the products falls into the B4 or B71 UE receiving band. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.14.2-1: Band 4 and Band 71 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 4 | 1710 | 1755 | 2110 | 2155 | 3420 | 3510 | 5130 | 5265 |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |

### 5.14.3	ΔTIB and ΔRIB values

CA_4A-71A is formed by diplexing a carrier in Band 4 with a carrier in Band 71.  No harmonic terms overlap with downlink in Band 4 from uplink in Band 71.  The diplexer relaxations TIB = 0.3 dB, RIB = 0 dB apply with no MSD or reference sensitivity exceptions.

Table 5.14.3-1: ΔTIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_4A-71A | 4 | 0.3 |
|  | 71 | 0.3 |

Table 5.14.3-2: ΔRIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_4A-71A | 4 | 0 |
|  | 71 | 0 |

### 5.14.4	REFSENS requirements

REFSENS remain unchanged.

## 5.15	CA_66A-71A_BCS0

### 5.15.1	Channel bandwidths per operating band for CA

Table 5.15.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_66A-71A | 66 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.15.2	Co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 66 and Band 71 UL carriers can be calculated as shown in Table 5.15.2-1 below. None of the products falls into the B66 or B71 UE receiving bands. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.15.2-1: Band 66 and Band 71 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |

### 5.15.3	ΔTIB and ΔRIB values

CA_66A-71A is formed by diplexing a carrier in Band 66 with a carrier in Band 71.  No harmonic terms overlap with downlink in Band 66 from uplink in Band 71.  The diplexer relaxations TIB = 0.3 dB, RIB = 0 dB apply with no MSD or reference sensitivity exceptions.

Table 5.15.3-1: ΔTIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66A-71A | 66 | 0.3 |
|  | 71 | 0.3 |

Table 5.15.3-2: ΔRIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66A-71A | 66 | 0 |
|  | 71 | 0 |

### 5.15.4	REFSENS requirements

REFSENS remain unchanged.

## 5.16	CA_2DL_2A-14A_1UL_BCS0

### 5.16.1	Operating bands for CA

Table 5.16.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-14 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |

### 5.16.2	Channel bandwidths per operating band for CA

Table 5.16.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-14A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  | 14 |  |  | Yes | Yes |  |  |  |  |

### 5.16.3	Co-existence studies

Table 5.16.3-1 gives the harmonic frequency limits for Band 2 and Band 14 2DL CA. As shown in the table, UL carrier of CA_2A-14A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.16.3-1: DL harmonics frequency limits for CA of Band 2 and Band 14

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 768 | 1850 | 1910 |
| 2nd order DL harmonics frequency range (MHz) | 1516 to 1536 |  | 3700 to 3820 |  |
| 3rd order DL harmonics frequency range (MHz) | 2274 to 2304 |  | 5550 to 5730 |  |

As shown in Table 5.16.3-2, the harmonic frequencies of Band 2 and Band 14 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.16.3-2: UL harmonics frequency limits for CA of Band 2 and Band 14

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |
| 14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 |

### 5.16.4	ΔTIB,c and ΔRIB,c values

Table 5.16.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-14A | 2 | 0.3 |
|  | 14 | 0.3 |

Table 5.16.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-14A | 2 | 0 |
|  | 14 | 0 |

### 5.16.5	 MSD

No MSD values need to be defined for this combination.

## 5.17	CA_2DL_14A-66A_1UL_BCS0

### 5.17.1	Operating bands for CA

Table 5.17.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_14-66 | 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 5.17.2	Channel bandwidths per operating band for CA

Table 5.17.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_14A-66A | 14 |  |  | Yes | Yes |  |  | 30 | 0 |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.17.3	Co-existence studies

Table 5.17.3-1 gives the harmonic frequency limits for Band 14 and Band 66 2DL CA. As shown in the table, UL carrier of CA_14A-66A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.17.3-1: DL harmonics frequency limits for CA of Band 14 and Band 66

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 768 | 1710 | 1780 |
| 2nd order DL harmonics frequency range (MHz) | 1516 to 1536 |  | 3420 to 3560 |  |
| 3rd order DL harmonics frequency range (MHz) | 2274 to 2304 |  | 5130 to 5340 |  |

As shown in Table 5.17.3-2, the harmonic frequencies of Band 14 and Band 66 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.17.3-2: UL harmonics frequency limits for CA of Band 14 and Band 66

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 |
| 66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |

### 5.17.4	ΔTIB,c and ΔRIB,c values

Table 5.17.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_14A-66A | 14 | 0.3 |
|  | 66 | 0.3 |

Table 5.17.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_14A-66A | 14 | 0 |
|  | 66 | 0 |

### 5.17.5	 MSD

No MSD values need to be defined for this combination.

## 5.18	CA_2DL_14A-30A_1UL_BCS0

### 5.18.1		Channel bandwidths per operating band for CA

Table 5.18.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_14A-30A | - | 14 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |

### 5.18.2	Co-existence studies

As shown in Table 5.18.2-1, the UL harmonic frequencies of Band 14 and Band 30 are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.18.2-1: UL harmonics frequency limits for CA of Band 2, Band 14 and Band 30

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 14 | 788 | 798 | 1576 | 1596 | 2364 | 2394 |
| 30 | 2305 | 2315 | 4610 | 4630 | 6915 | 6945 |

### 5.18.3	ΔTIB,c and ΔRIB,c values

Table 5.18.3-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_14A-30A | 14 | 0.3 |
|  | 30 | 0.3 |

Table 5.18.3-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_14A-30A | 14 | 0 |
|  | 30 | 0 |

### 5.18.4	 MSD

No MSD values need to be defined for this combination.

## 5.19	CA_2A-71A

### 5.19.1	LTE-Advanced Carrier Aggregation of Band 2 and Band 71 (1 UL)

Table 5.19.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA operating Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE transmit / BS receive |  |  | UE receive / BS transmit |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-71 | 2 | 1850 MHz | – | 1980 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz |  |

#### 5.19.1.1		List of specific combination issues

##### 5.19.1.1.1	Channel bandwidths per operating band for CA

Table 5.19.1.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-71A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  | 71 |  |  | Yes | Yes |  |  |  |  |

##### 5.19.2	Coexistence

Table 5.19.1.1.2-1: Band 2 and Band 71 UL harmonics

|  |  |  |  |  | 3rd Harmonic |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 5790 | 5970 | 5550 | 5730 |
| 71 | 663 | 698 | 617 | 652 | 1851 | 1956 | 1989 | 2094 |

The uplink of Band 71 is 663 - 698 MHz for which third harmonic products fall within 1989 – 2094 MHz, while the downlink of Band 2 is 1930 - 1990 MHz.  Thus, it can be seen that there is potential for third harmonic interference from uplink in Band 71 to downlink in Band 2.  However, the potential is limited to edge channels since the overlap is only 1 MHz from 1989 – 1990 MHz.  Only an uplink allocation in the lowest edge of Band 71 paired with simultaneous downlink in the uppermost edge of Band 2 could be impacted.  Moreover, due the guard bands inherent in the definition of the transmission bandwidth configuration for LTE, there may not be any direct overlap at all.  However, there may still be some effect due to spectral spreading of the third harmonic even if direct overlap is averted.

Another potential mechanism for harmonic interference is mixing of the uplink in Band 2 with the third harmonic of the Rx LO.  In this case, the frequency range impacted is much larger.  The third harmonic of Rx extends from 1851 to 1956 MHz while the uplink in Band 2 ranges from 1850 to 1910 MHz.  Therefore, there is nearly complete overlap between these two ranges.  Depending upon the particular frequency assignments and allocation of PRB’s for uplink within Band 2 and downlink within Band 71, there may be interference from this mechanism.

##### 6.2.3	∆TIB and ∆RIB values

As described in clause 5.19.1.1.2 above, the overlap between the third harmonic of the uplink in Band 71 and the downlink in Band 2 is small.  Therefore, the number of deployment scenarios that may be impacted are also small and may possibly be overcome by network scheduling or some other means.  Consequently, a harmonic trap filter is not assumed to be present and therefore not accounted for in the TIB and RIB values.  The agreed values of 0.3 dB for TIB and 0 dB for RIB for high/low band combinations is shown below.

Table 5.19.1.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-71A | 2 | 0.3 |
|  | 71 | 0.3 |

# Table 5.19.1.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_2A-71A | 2 | 0 |
|  | 71 | 0 |

##### 5.19.4	MSD

It is expected that MSD will be specified for the two types of harmonic interference described above.  MSD is captured by means of the refsens exception table for harmonics.  The MSD analysis is TBD.

## 5.20	CA_2DL_26A-48A_1UL_BCS0

### 5.20.1	Operating bands for CA

Table 5.20.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_26-48 | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

### 5.20.2	Channel bandwidths per operating band for CA

Table 5.20.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_26A-48A | 26 |  | Yes | Yes | Yes |  |  | 30 | 0 |
|  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.20.3	Co-existence studies

Table 5.20.3-1 gives the harmonic frequency limits for Band 26 and Band 48 2DL CA. As shown in the table, UL carrier of CA_26A-48A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.20.3-1: DL harmonics frequency limits for CA of Band 26 and Band 48

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 859 | 894 | 3550 | 3700 |
| 2nd order DL harmonics frequency range (MHz) | 1718 to 1788 |  | 7100 to 7400 |  |
| 3rd order DL harmonics frequency range (MHz) | 2577 to 2682 |  | 10650 to 11100 |  |

As shown in Table 5.20.3-2, the harmonic frequencies of Band 26 and Band 48 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.20.3-2: UL harmonics frequency limits for CA of Band 26 and Band 48

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 |
| 48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

### 5.20.4	ΔTIB,c and ΔRIB,c values

Table 5.20.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_26A-48A | 26 | 0.3 |
|  | 48 | 0.3 |

Table 5.20.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_26A-48A | 26 | 0 |
|  | 48 | 0 |

### 5.20.5	 MSD

No MSD values need to be defined for this combination.

## 5.21	CA_2DL_28A-38A_1UL_BCS0

### 5.21.1	Operating bands for CA

Table 5.21.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_28-38 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 5.21.2	Channel bandwidths per operating band for CA

Table 5.21.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_28A-38A | 28 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.21.3	Co-existence studies

Table 5.21.3-1 gives the harmonic frequency limits for Band 28 and Band 38 2DL CA. As shown in the table, UL carrier of CA_28A-38A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.21.3-1: DL harmonics frequency limits for CA of Band 28 and Band 38

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 2570 | 2620 |
| 2nd order DL harmonics frequency range (MHz) | 1516 to 1606 |  | 5140 to 5240 |  |
| 3rd order DL harmonics frequency range (MHz) | 2274 to 2409 |  | 7710 to 7860 |  |

As shown in Table 5.21.3-2, the harmonic frequencies of Band 28 and Band 38 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.21.3-2: UL harmonics frequency limits for CA of Band 28 and Band 38

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |

### 5.21.4	ΔTIB,c and ΔRIB,c values

Table 5.21.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_28A-38A | 28 | 0.3 |
|  | 38 | 0.3 |

Table 5.21.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_28A-38A | 28 | 0 |
|  | 38 | 0 |

### 5.21.5	 MSD

No MSD values need to be defined for this combination.

## 5.22	CA_2DL_12A-46A-1UL_BCS0

### 5.22.1	Channel bandwidths per operating band for CA

Table 5.22.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_12A-46A | - | 12 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |

### 5.22.2	UE Co-existence studies

Table 5.22.3-3 summarizes frequency ranges where harmonics occur due to Band 12 and Band 46 CA with 1 UL.

Table 5.22.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 12 | 699 | 716 | 1398 | 1432 | 2097 | 2148 |
| 46 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |

It can be seen from Table 5.22.2-1 that there is no harmonic interference for CA_12A-46A towards own receive bands.

### 5.22.3	∆TIB,c and ∆RIB,c values

For two simultaneous DLs and one UL of band 12 and Band 46 for 1UL/2DL CA_12A-46A, the TIB,c and RIB,c  values are shown in table 5.22.3-1 and  table 5.22.3-2, respectively.

dTib values for bands 5 and 12 are taken from existing CA_13A-46A.

Table 5.22.3-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_12A-46A | 12 | 0 |
|  | 46 | 0 |

dRib values for bands 5 and 12 are taken from existing CA_5A-12A.

Table 5.22.0-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_12A-46A | 12 | 0 |
|  | 46 | 0 |

### 5.22.4	REFSENS requirements

REFSENS requriement for CA_12A-46A is presented in Table 5.22-4-1

Table 5.22.4-1: REFSENS requriements for CA_12A-46A

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_12A-46A | 12 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |

## 5.23	CA_70A-71A_BCS0

### 5.23.1	Channel bandwidths per operating band for CA

Table 5.23.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_70-71 | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz |  |

Table 5.23.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_70A-71A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_70A-71A | 70 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.23.2	UE co-existence studies

UE harmonics are shown in table below.

Table 5.23.2-1 UE harmonics

| UE UL frequencies | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| Frequency [MHz] | 1695 | 1710 | 663 | 698 |
| 2nd harmonic [MHz] | 3390 | 3420 | 1326 | 1396 |
| 3rd harmonic [MHz] | 5085 | 5130 | 1989 | 2094 |
| 4th harmonic [MHz] | 6780 | 6840 | 2652 | 2792 |
| 5th harmonic [MHz] | 8475 | 8550 | 3315 | 3490 |

Third harmonic from B71 hits on top of B70 DL. This is captured in MSD section.

### 5.23.3	ΔTIB,c and ΔRIB,c values

For the UE which supports CA_70A-71A, the ΔTIB,c and ΔRIB,c  are defined for applicable bands in Table 5.23.3-2 and 5.23.3-3 respectively.

Table 5.23.3-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_70A-71A | 70 | 0.3 |
|  | 71 | 0.6 |

Table 5.23.3-3: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_70A-71A | 70 | 0 |
|  | 71 | 0 |

### 5.23.4	REFSENS requirements

Table 5.23.4-1:  Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to

harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

The UL allocations to verify harmonic MSD are as follows:

Table 5.23.4-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

## 5.24	LTE-Advanced Carrier Aggregation of Band 2 and Band 71 (1 UL)

Table 5.24.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA operating Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UE transmit / BS receive |  |  | UE receive / BS transmit |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz |  |

#### 5.24.1.1		List of specific combination issues

##### 5.24.1.1.1	Channel bandwidths per operating band for CA

Table 5.24.1.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-71A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 71 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  | 71 |  |  | Yes | Yes |  |  |  |  |

##### 5.24.1.1.2	Coexistence

Table 5.24.1.1.2-1: Band 2 and Band 71 UL harmonics

|  |  |  |  |  | 3rd Harmonic |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 1930 | 1990 | 5790 | 5970 | 5550 | 5730 |
| 71 | 663 | 698 | 617 | 652 | 1851 | 1956 | 1989 | 2094 |

The uplink of Band 71 is 663 - 698 MHz for which third harmonic products fall within 1989 – 2094 MHz, while the downlink of Band 2 is 1930 - 1990 MHz.  Thus, it can be seen that there is potential for third harmonic interference from uplink in Band 71 to downlink in Band 2.  However, the potential is limited to edge channels since the overlap is only 1 MHz from 1989 – 1990 MHz.  Only an uplink allocation in the lowest edge of Band 71 paired with simultaneous downlink in the uppermost edge of Band 2 could be impacted.  Moreover, due the guard bands inherent in the definition of the transmission bandwidth configuration for LTE, there may not be any direct overlap at all.  However, there may still be some effect due to spectral spreading of the third harmonic even if direct overlap is averted.

Another potential mechanism for harmonic interference is mixing of the uplink in Band 2 with the third harmonic of the Rx LO.  In this case, the frequency range impacted is much larger.  The third harmonic of Rx extends from 1851 to 1956 MHz while the uplink in Band 2 ranges from 1850 to 1910 MHz.  Therefore, there is nearly complete overlap between these two ranges.  Depending upon the particular frequency assignments and allocation of PRB’s for uplink within Band 2 and downlink within Band 71, there may be interference from this mechanism.

##### 6.2.1.1.3	∆TIB and ∆RIB values

As described in clause 5.24.1.1.2 above, the overlap between the third harmonic of the uplink in Band 71 and the downlink in Band 2 is small.  Therefore, the number of deployment scenarios that may be impacted are also small and may possibly be overcome by network scheduling or some other means.  Consequently, a harmonic trap filter is not assumed to be present and therefore not accounted for in the TIB and RIB values.  The agreed values of 0.3 dB for TIB and 0 dB for RIB for high/low band combinations is shown below.

Table 5.24.1.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-71A | 2 | 0.3 |
|  | 71 | 0.3 |

# Table 5.24.1.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_2A-71A | 2 | 0 |
|  | 71 | 0 |

##### 5.24.1.1.4.1	Band 2 MSD

For the case of 3rd harmonic of uplink in Band 71 interfering with downlink in Band 2, the harmonic interference level at Tx antenna port over the frequency range of the Band 2 receive channel was measured.  The measurement was conducted for both full allocation and single RB and is summarized below.  Given that the output is referenced to the antenna port, the obtain a value at the PA output, the Band 71 Tx front-end insertion loss assumed to be 5 dB should be added.

Table 5.24.1.1.4.1-1.  Measured harmonic noise integrated over Rx channel.

| Tx channel BW (MHz) | Rx channel BW (MHz) | Measured harmonic noise at Tx antenna output (dBm) |  |
| --- | --- | --- | --- |
|  |  | 1 RB | Full Allocation |
| 5 | 5 | -38.6 | -39.0 |
| 5 | 10 | -44.1 | -44.2 |
| 5 | 15 | -44.6 | -44.6 |
| 5 | 20 | -44.6 | -44.6 |
| 10 | 5 | -46.9 | -46.6 |
| 10 | 10 | -44.5 | -46.8 |
| 10 | 15 | -44.5 | -47.0 |
| 10 | 20 | -44.5 | -47.4 |
| 15 | 5 | -47.0 | -48.1 |
| 15 | 10 | -47.0 | -47.4 |
| 15 | 15 | -44.4 | -47.5 |
| 15 | 20 | -44.5 | -47.6 |
| 20 | 5 | -46.7 | -49.1 |
| 20 | 10 | -46.9 | -47.9 |
| 20 | 15 | -46.5 | -47.7 |
| 20 | 20 | -44.5 | -47.7 |

Based on these noise measurements, the MSD can be computed.  It was agreed that a harmonic trap filter would not be assumed at the output of the Band 71 PA.  Therefore, the filtering of the 3rd harmonic comes primarily from the Band 71 duplexer and the low/high diplexer.  Other contributions to harmonic, such as switch, harmonic generation within Band 71 and Band 2 duplexers, etc., are not included in the calculation for simplicity.  Lastly, the assumption of 67 dB PCB isolation is adopted in this analysis.

A Band 71 filter datasheet indicates that the duplexer Tx attenuation in the frequency range of the 3rd harmonic is 40 dB and is used here.  The low/high diplexer provides an additional attenuation of 25 dB.  The front-end loss in Band 2 Rx is assumed to be 5.5 dB due to the high insertion loss of the B2+B4 quadplexer.  With these assumptions, the following results are obtained using the 5 MHz + 5 MHz as an example.  The MSD analysis is based on an MRC combining algorithm assuming that the interference is correlated across primary and diversity receivers.

Table 5.24.1.1.4.1-2.  Example MSD analysis for 5 MHz + 5 MHz scenario

|  | Parameter | Primary | Diversity |
| --- | --- | --- | --- |
| B71 PA H3 | -33.6 | -33.6 |  |
| B71 duplexer | 40 | -73.6 |  |
| Diplexer | 25 | -98.6 |  |
| Antenna isolation | 10 | -98.6 | -108.6 |
| PCB PA to LNA | 67 | -100.6 | -100.6 |
| B2 Rx FE loss | 5.5 | -95.1 | -95.1 |
| Composite |  | -93.4963 | -94.9102 |
| MSD | -4.62 |  |  |

Following the same procedure and assumptions, the other bandwidth combinations were analyzed with a MSD summary provided in the table below

Table 5.24.1.1.4.1-3.  Summary of MSD for all aggressor and victim bandwidths

| Band 71 CBW | Band 2 CBW | MSD |
| --- | --- | --- |
| 5 | 5 | 4.6 |
| 5 | 10 | 1.0 |
| 5 | 15 | 0.6 |
| 5 | 20 | 0.5 |
| 10 | 5 | 1.2 |
| 10 | 10 | 1.0 |
| 10 | 15 | 0.7 |
| 10 | 20 | 0.6 |
| 15 | 5 | 1.1 |
| 15 | 10 | 0.6 |
| 15 | 15 | 0.7 |
| 15 | 20 | 0.6 |
| 20 | 5 | 1.2 |
| 20 | 10 | 0.6 |
| 20 | 15 | 0.4 |
| 20 | 20 | 0.6 |

The reference sensitivity can be defined as follows where the maximum MSD for a given Band 2 channel bandwidth is applied.  The 5 MHz channel bandwidth requires an amendment when the Band 71 Tx bandwidth is also 5 MHz since the MSD is significantly larger for this case.  Also, it is noted that the MSD only applies when the channels are located in proximity of  the lower edge of Band 71 and the upper edge of Band 2.

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-71AX | 2 |  |  | -93.4 | -94 | -92.5 | -91.4 | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |
| CA_2A-71AY | 2 |  |  | -96.8 | -94 | -92.5 | -91.4 | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 |  |
| NOTE X: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE Y: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-71A | 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

##### 5.24.1.1.4.2	Band 71 MSD

The UE reference architecture used in this analysis is shown in Figure 5.24.1.1.4.2-1 where single antenna topology has been assumed between B2 and B71. Mid/Low band diplexers are put in both primary and diversity paths.

Figure 5.24.1.1.4.2-1 Reference architecture for CA_2A-71A B71 Rx harmonic mixing MSD analysis

The B2 UL signal propagation path numbering as shown in Figure 2-1 is detailed as below:

1: UE transceiver B2 TX output to B71 primary RX LNA input

2: B2 PA output to B71 primary RX LNA input

3: B2 UL signal coupling due to TXM mid-band port to low-band port

4: Direct signal path from B2 UL to B71 primary-RX LNA input

5: UE transceiver B2 TX output to B71 diversity RX LNA input

6: B2 PA output to B71 diversity RX LNA input

7: Direct signal path from B2 UL to B71 diversity-RX LNA input

B71 duplexer RX 3rd harmonic rejection data from two component vendors are also shown in Figure 5.24.4.1.1.4.2-2. The RX 3rd harmonic rejection can vary from 23 to 42 dB. We propose 37 dB as the calculation assumption.

Figure 5.24.4.1.1.4.2-2 B71 Duplexer RX 3rd harmonic rejection data

Considering that discrete front-end components are still widely used on phones, and that numerous bands/CAs with relatively dense component placement could introduce lots of coupling paths, we propose 67dB PCB isolation as B71 MSD calculation assumption. On the other hand, Thanks to the continual advancement in integrated circuit design, the transceiver’s low-band RX front-end 3rd order harmonic rejection can be improved to 30 dB.

Table 5.24.4.1.1.4.2-1 summarizes a set of typical receiver performance parameters for MSD analysis. Table 5.24.4.1.1.4.2-2 presents the link analysis to calculate the interference level in both main and diversity Rx paths as well as the combined REFSENS level based on uncorrelated MRC. Table 2-3 shows the MSD contribution percentage of each individual propagation path number.

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation Primary <-> Diversity | 15 | dB |
| TXM port isolation from B2 to B71 | 35 | dB |
| Diplexer isolation at B2 | 25 | dB |
| PCB isolation from B2 PA output to B71 RX LNA input | 67 | dB |
| B71 DUP Ant to RX port rejection at 1850~1910MHz | 37 | dB |
| B71 Diversity path SAW rejection at 1850 – 1910 MHz | 35 | dB |
| LPF rejection at 1850 – 1910 MHz | 35 | dB |
| B71 Primary RX front-end loss | 4 | dB |
| B71 Diversity RX front-end loss | 3 | dB |
| B71 Diversity path switch + PCB trace loss | 1 | dB |
| Transceiver B2 TX port to B71 RX port isolation | 55 | dB |
| RX 3rd order harmonic rejection | 30 | dB |
| SNR requirement for QPSK | -1 | dB |

Table 5.24.4.1.1.4.2-1 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity | Unit |
| Transceiver B2 output power | 0 | 0 | dBm |
| TX power at B2 PA output | 27 | 27 | dBm |
| B2 TX power at B2 DUP-ant port | 25 | 25 | dBm |
| B2 Tx power at antenna port | 23 | 8 | dBm |
| Path number 4 at B71 primary-RX LNA input (with LPF) | -73 | NA | dBm |
| Path number 4 at B71 primary-RX LNA input (w/o LPF) | -38 | NA | dBm |
| Path number 7 at B71 diversity-RX LNA input (with LPF) | NA | -88 | dBm |
| Path number 7 at B71 diversity-RX LNA input (w/o LPF) | NA | -53 | dBm |
| PCB Coupling Path |  |  |  |
| Parameter | Main | Diversity |  |
| Path number 1 and 5 at LNA input | -55 | -55 | dBm |
| Path number 2 and 6 at LNA input | -40 | -40 | dBm |
| Path number 3 (with LPF) at LNA input | -82 | NA | dBm |
| Path number 3 (w/o LPF) at LNA input | -47 | NA | dBm |
| Combined |  |  |  |
| Total interference power due to B2 TX at B71 LNA input (with LPF) | -39.9 | -39.9 | dBm |
| Total interference power due to B2 TX at B71 LNA input (w/o LPF) | -35.5 | -39.7 | dBm |
| Tx power after Rx mixing (referred to antenna, with LPF) | -66.9 | -67.9 | dBm |
| Tx power after Rx mixing (referred to antenna, w/o LPF) | -62.5 | -67.7 | dBm |
| Combined B71 REFSENS (with LPF) | -70.4 |  | dBm |
| Combined B71 REFSENS (w/o LPF) | -68.8 |  | dBm |

Table 5.24.4.1.1.4.2-2 Link analysis for REFSENS calculation

Table 5.24.4.1.1.4.2-3 MSD contribution percentage of each individual path due to B2 TX signal

Here are some observations from the MSD analysis:

Observation 1: PCB isolation (path number 2 and 6) is the dominant factor of B71 MSD level for both primary RX and diversity RX paths.

Observation 2: On primary path (path number 4), LPF on B71 UL/DL common path can also reduce its UL 3rd harmonic side lobe falling to high range of B2 RX. However, since the MSD of B71 TX 3rd harmonic side lobe falling to B2 RX high range is relative small, the LPF can be put on B71 RX path only to avoid degradation of B71 TX power due to LPF insertion loss.

Observation 3: On diversity path, (comparing path number 7 of with/without LPF tables) whether LPF exists or not does not affect MSD a lot on this path.

Observation 4: The worst bandwidth combination for RX 3rd order mixing of the CA combination will happen on Band 2 UL bandwidth at 5MHz, no matter what B71 DL bandwidth is.

Based on assumption of LPF on B71 primary-RX path only, B71 REFSENS exceptions in CA_2A-71A_1UL due to B71 Rx 3rd order harmonic mixing

Table 5.24.4.1.1.4.2-4 Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-71A15,16 | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 5.24.4.1.1.4.2-5 Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-71A | 2 |  |  | 25 | 50 | 50 | 50 | FDD |

## 5.25 CA_1A-32A_BCS0

### 5.25.1 Operating bands for CA

Table 5.25.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-32 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 32 | N/A |  |  | 1452 MHz | – | 1496 MHz | FDD |

Table 5.25.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink band | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-32A | 1 | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.25.2	Co-existence studies

Table 2.1.3-1 shows the frequency ranges for 2nd and 3rd harmonics for Band 1 UL carriers. It can be seen that the UL harmonic frequencies of Band 1 do not fall into any UE receive bands of interest in the DL.

Table 5.25.2-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 32 | N/A |  | 1452 | 1496 | N/A |  |  |  |

### 5.25.3 ΔTIB,c and ΔRIB,c values

For the UE which supports CA_1A-32A the ΔTIB,c and ΔRIB,c are defined for applicable bands in Table 5.25.3-1 and 5.25.3-2 respectively.

Table 5.25.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-32A | 1 | 0.5 |
|  | 32 | N/A |

For the UE that supports CA_1A-32A the ΔRIB,c is proposed for applicable bands in Table 2.1.4-2:

Table 5.25.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-32A | 1 | 0 |
|  | 32 | 0 |

### 5.25.4	REFSENS requirements

There are no MSD requirements for CA_1A-32A.

## 5.26	CA_2DL_48A-71A_1UL_BCS0

### 5.26.1	Channel bandwidths per operating band for CA

Table 5.26.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz |  | 10 MHz | 15 MHz |  |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_48A-71A |  | 48 |  |  |  | Yes | Yes |  | Yes | Yes |  | 40 | 0 |
|  |  | 71 |  |  |  | Yes | Yes |  | Yes | Yes |  |  |  |

### 5.26.2	Co-existence studies

Table 5.26.2-1 summarizes frequency ranges where harmonics occur due to Band 48 and Band 71 CA with 1 UL.

Table 5.26.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |

It can be seen from Table 5.26.2-1 that there is no harmonic impact of band 48 and band 71 into its own receiving bands.

### 5.26.3	ΔTIB,c and ΔRIB,c values

For the UE which supports CA_48A-71A the ΔTIB,c and ΔRIB,c are defined for applicable bands in Table 5.26.3-1 and 5.26.3-2 respectively.

Table 5.26.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_48A-71A | 48 | 0.3 |
|  | 71 | 0.3 |

Table 5.26.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_48A-71A | 48 | 0 |
|  | 71 | 0 |

### 5.26.4	REFSENS requirements

There is no need to define reference sensitivity requirements for CA_48A-71A.

## 5.27	CA_2DL_46A-71A_1UL_BCS0

### 5.27.1	Channel bandwidths per operating band for CA

Table 5.27.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz |  | 10 MHz | 15 MHz |  |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_46A-71A |  | 46 |  |  |  |  |  |  |  | Yes |  | 40 | 0 |
|  |  | 71 |  |  |  | Yes | Yes |  | Yes | Yes |  |  |  |

### 5.27.2	Co-existence studies

Table 5.27.2-1 summarizes frequency ranges where harmonics occur due to Band 46 and Band 71 CA with 1 UL.

Table 5.27.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 |

It can be seen from Table 5.27.2-1 that there is no harmonic impact of band 46 and band 71 into its own receiving bands.

### 5.27.3	ΔTIB,c and ΔRIB,c values

For the UE which supports CA_46A-71A the ΔTIB,c and ΔRIB,c are defined for applicable bands in Table 5.27.3-1 and 5.27.3-2 respectively.

Table 5.27.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_46A-71A | 71 | 0 |

Table 5.27.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_46A-71A | 71 | 0 |

### 5.27.4	REFSENS requirements

Reference sensitivity requirements for CA_46A-71A that need to be added to Table 7.3.1A-0eA are specified in Table 5.27.4-1.

Table 5.27.4-1: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_46A-71A | 46 |  |  |  |  |  | -90 | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | TDD |

## 5.28	CA_32A-43A_1UL_BCS0

### 5.28.1 Operating band for CA

Table 5.28.1-1: Inter-band CA operating bands for CA_32-43

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_32-43 | 32 | N/A |  |  | 1452 MHz | – | 1496MHz | FDD |
|  | 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |

5.28.2 Channel bandwidths per operating band for CA

Table 5.28.2-1: E-UTRA CA configuration and bandwidth combination sets for 1UL CA_32A-43A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_32A-43A | - | 32 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

### 5.28.3 	UE co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 43 UL carriers can be calculated as shown in Table 5.28.3-1 below. None of the products falls into the B32 UE receiving band. For UE coexistence the single UL is already considered for Band 43 in TS 36.101.

Table 5.28.3-1: Band 32 and Band 43 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 32 | N/A |  | 1452 | 1496 | N/A |  | N/A |  |
| 43 | 3600 | 3800 | 3600 | 3800 | 7200 | 7600 | 10800 | 11400 |

### 5.28.4 	∆TIB and ∆RIB values

For CA_32A-43A_1UL_BCS0, it is a MB/VHB carrier combination, therefore the TIB,c and RIB,c values are shown in table 5.28.4-1 and in table 5.28.4-2.

Table 5.28.4-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_32A-43A | 43 | 0.8 |

Table 5.28.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_32A-43A | 43 | 0.5 |

### 5.28.5 	REFSENS

For the REFSENS of CA_32A-43A_1UL_BCS0 with a SDL band, the exceptions are shown in Table 5.28.5-1.

Table 5.28.5-1: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_32A-43A | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |

## 5.29	CA_20A-43A_1UL_BCS0

### 5.29.1 Operating band for CA

Table 5.29.1-1: Inter-band CA operating bands for CA_20-43

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_20-43 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |

5.29.2 Channel bandwidths per operating band for CA

Table 5.29.2-1: E-UTRA CA configuration and bandwidth combination sets for 1UL CA_20A-43A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_20A-43A | - | 20 |  |  | Yes |  |  |  | 25 | 0 |
|  |  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

### 5.29.3 	UE co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 20 and Band 43 UL carriers can be calculated as shown in Table 5.29.3-1 below. None of the products falls into the B20 or B43 UE receiving band. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.29.3-1: Band 20 and Band 43 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 |
| 43 | 3600 | 3800 | 3600 | 3800 | 7200 | 7600 | 10800 | 11400 |

### 5.29.4 	∆TIB and ∆RIB values

For CA_20A-43A_1UL_BCS0, it is a LB/VHB carrier combination, therefore the TIB,c and RIB,c values are shown in table 5.29.4-1 and in table 5.29.4-2.

Table 5.29.4-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_20A-43A | 20 | 0.3 |
|  | 43 | 0.8 |

Table 5.29.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_20A-43A | 20 | 0 |
|  | 43 | 0.5 |

### 5.29.5 	REFSENS

### There is no specific REFSENs requirements.

## 5.30	CA_32A-42A_1UL_BCS0

### 5.30.1 Operating band for CA

Table 5.30.1-1: Inter-band CA operating bands for CA_32-42

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_32-42 | 32 | N/A |  |  | 1452 MHz | – | 1496MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

5.30.2 Channel bandwidths per operating band for CA

Table 5.30.2-1: E-UTRA CA configuration and bandwidth combination sets for 1UL CA_32A-42A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_32A-42A | - | 32 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

### 5.30.3 	UE co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 42 UL carrier can be calculated as shown in Table 5.30.3-1 below. None of the products falls into the B32 UE receiving band. For UE coexistence the single UL is already considered for band 42 in TS 36.101.

Table 5.30.3-1: Band 32 and Band 42 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 32 | N/A |  | 1452 | 1496 | N/A |  | N/A |  |
| 42 | 3400 | 3600 | 3400 | 3600 | 6800 | 7200 | 10200 | 10800 |

### 5.30.4 	∆TIB and ∆RIB values

For CA_32A-42A_1UL_BCS0, it is a MB/VHB carrier combination, therefore the TIB,c and RIB,c values are shown in table 5.30.4-1 and in table 5.30.4-2.

Table 5.30.4-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_32A-42A | 42 | 0.8 |

Table 5.30.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_32A-42A | 42 | 0.5 |

### 5.30.5 	REFSENS

For the REFSENS of CA_32A-42A_1UL_BCS0 with a SDL band, the exceptions are shown in Table 5.30.5-1.

Table 5.30.5-1: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_32A-42A | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |

## 5.31	CA_1A-43A_1UL_BCS0

### 5.31.1 Operating band for CA

Table 5.31.1-1: Inter-band CA operating bands for CA_1-43

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-43 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |

5.31.2 Channel bandwidths per operating band for CA

Table 5.31.2-1: E-UTRA CA configuration and bandwidth combination sets for 1UL CA_1A-43A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-43A | - | 1 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

### 5.31.3 	UE co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 1 and Band 43 UL carriers can be calculated as shown in Table 5.31.3-1 below. None of the products falls into the B1 or B43 UE receiving band. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.31.3-1: Band 1 and Band 43 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 43 | 3600 | 3800 | 3600 | 3800 | 7200 | 7600 | 10800 | 11400 |

### 5.31.4 	∆TIB and ∆RIB values

For CA_1A-43A_1UL_BCS0, it is a HB/VHB carrier combination, therefore the TIB,c and RIB,c values are shown in table 5.31.4-1 and in table 5.31.4-2.

Table 5.31.4-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-43A | 1 | 0.3 |
|  | 43 | 0.8 |

Table 5.31.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-43A | 1 | 0 |
|  | 43 | 0.5 |

### 5.31.5 	REFSENS

There is no specific REFSENs requirements.

## 5.32	CA_3A-43A_1UL_BCS0

### 5.32.1 Operating band for CA

Table 5.32.1-1: Inter-band CA operating bands for CA_3-43

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-43 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |

5.32.2 Channel bandwidths per operating band for CA

Table 5.32.2-1: E-UTRA CA configuration and bandwidth combination sets for 1UL CA_3A-43A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-43A | - | 3 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [6] when operating in single carrier mode.

### 5.32.3 	UE co-existence studies

The 2nd and 3rd order harmonics caused in the UE by transmitting of Band 3 and Band 43 UL carriers can be calculated as shown in Table 5.32.3-1 below. None of the products falls into the B3 or B43 UE receiving band. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.32.3-1: Band 3 and Band 43 UL harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 |
| 43 | 3600 | 3800 | 3600 | 3800 | 7200 | 7600 | 10800 | 11400 |

### 5.32.4 	∆TIB and ∆RIB values

For CA_3A-43A_1UL_BCS0, it is a HB/VHB carrier combination, therefore the TIB,c and RIB,c values are shown in table 5.32.4-1 and in table 5.32.4-2.

Table 5.32.4-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-43A | 3 | 0.3 |
|  | 43 | 0.8 |

Table 5.32.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-43A | 3 | 0 |
|  | 43 | 0.5 |

### 5.32.5 	REFSENS

### There is no specific REFSENs requirements.

## 5.33	CA_2DL_25A-46A-1UL_BCS0

### 5.33.1 	Operating bands for CA

Table 5.33.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_25-46 | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

### 5.33.2	Channel bandwidths per operating band for CA

Table 5.33.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_25A-46A | - | 25 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  | Yes |  | Yes |  |  |

### 5.33.3	UE Co-existence studies

Table 5.33.3-1 summarizes frequency ranges where harmonics occur due to Band 25 and Band 46 CA with 1 UL.

Table 5.33.3-1: Band 25 and Band 46 UL harmonics

| BS UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1915 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3700 | 3830 | 10300 | 11850 |
| 3rd order harmonics frequency range (MHz) | 5550 | 5745 | 15450 | 17775 |

It can be seen from Table 5.33.3-1 that there is harmonic interference for CA_25A-46A towards own receive bands with the 3rd harmonic of B25 being in the B46 range.

### 5.33.4	∆TIB,c and ∆RIB,c values

Since the frequency range for B25 is similar to that of B2, ∆TIB,c and ∆RIB,c values from CA_2A-46A can be used.  For two simultaneous DLs and one UL of band 25 and Band 46 for 1UL/2DL CA_25A-46A, the TIB,c and RIB,c  values are shown in table 5.33.4-1 and  table 5.33.4-2, respectively.

ΔTIB,c value for band 25 is taken from existing CA_2A-46A, and for band 46 are taken from existing CA_12A-46A.

Table 5.33.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_25A-46A | 25 | 0 |
|  | 46 | 0 |

ΔRIB,c value for band 25 is taken from existing CA_2A-46A, and for band 46 are taken from existing CA_12A-46A.

Table 5.33.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_25A-46A | 25 | 0 |
|  | 46 | 0 |

### 5.33.5	Reference sensitivity

Since 3rd harmonic interference is potentially present in this CA configuration, a reference sensitivity exception is necessary to avoid desensitization.  The REFSENS value for Band 46 is the same as for CA_2A-46A since Band 2 and Band 25 are very similar.  Due to the specifications for CA_2A-46A but adjusting for a different single carrier reference sensitivity in Band 25 compared to Band 2, the following is proposed.

Table 5.33.5-1: REFSENS requirements for CA_25A-46A

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_25A-46A | 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 46 |  |  |  | -93 |  | --90 | TDD |

## 5.34	CA_3A-18A_BCS0

### 5.34.1	Operating bands for CA

# Table 5.34.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-18 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |

### 5.34.2	Channel bandwidths per operating band for CA

# Table 5.34.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-18A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 18 |  |  | Yes | Yes | Yes |  |  |  |

### 5.34.3	Co-existence studies

The harmonic frequencies are observed in Table 5.34.3-1:

Table 5.6.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 3610 | 3760 | 5415 | 5640 |
| 18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 1720 | 1750 | 2580 | 2625 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 3 and Band 18 DL carriers can be calculated as shown in table 5.6.3-2 below

# Table 5.34.3-2: Band 3 and Band 18 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 860 | 875 | 1805 | 1880 |
| 2nd harmonics frequency limits (MHz) | 1720 | 1750 | 3610 | 3760 |
| 3rd harmonics frequency limits (MHz) | 2580 | 2625 | 5415 | 5640 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 930 | 1020 | 2665 | 2755 |
| 3rd order IMD products | ¦2*f1_low – f2_high¦ | ¦2*f1_high – f2_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 160 | 55 | 2735 | 2900 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 3525 | 3630 | 4470 | 4635 |
| 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low –f2_BWmax¦ | ¦f1_high + f2_BW max¦ | ¦f2_low – f1_BW max¦ | ¦f2_high + f1_BW max¦ |
| IMD frequency limits (MHz) | 760 | 975 | 1785 | 1900 |

It can be seen from table 5.34.3-2 that:

-	the 2nd harmonics of BS transmitting in Bands 3 and 18 may fall into BS receive band 3, 10, 43, 66

-	and 3rd harmonics of BS transmitting in Bands 3 and 18 may fall into BS receive band 38, 41

-	the 3rd IMD products may fall into BS receive Bands 2, 5, 6, 8, 13, 14, 18, 19, 20, 25, 26, 27, 39, 41, 42, 43 and 44

### 5.34.4	∆TIB and ∆RIB values

For CA_3A-18A_BCS0, the same requirements on ∆TIB and ∆RIB of CA_3A-19A_BCS0 case can apply. Therefore, the TIB,c and RIB,c values are shown in table 5.34.4-1 and in table 5.34.4-2.

Table 5.34.4-1: IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-18A | 3 | 0.3 |
|  | 18 | 0.3 |

Table 5.34.4-2: R IB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-18A | 3 | 0 |
|  | 18 | 0 |

### 5.34.5	REFSENS requirements

No change in B3 or B18 REFSENS requirements are needed, as the 1UL does not cause harmonic distortion to the own receiving bands.

## 5.35	CA_2DL_46A-48A_1UL_BCS0

### 5.35.1	Channel bandwidths per operating band for CA

Table 5.35.1-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_46A-48A | 46 |  |  |  |  |  | Yes | 40 | 0 |
|  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.35.2	Co-existence studies

Table 5.35.2-1 summarizes frequency ranges where harmonics occur due to Band 46 and Band 48 CA with 1 UL.

Table 5.35.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 46 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

. . Table 5.35.2-1 shows there are no harmonic issues for this combination.

So we can conclude that there is no issue on both DL/UL harmonic interference.

### 5.35.3	ΔTIB,c and ΔRIB,c values

For the UE which supports CA_46A-48A the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 5.35.3-1 and 5.35.3-2 respectively assuming separate antenna architecture without HTF as was assumed CA_42A-46A. Hence the delta values are chosen by similar existing combination of CA_42A-46A

Table 5.35.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_46A-48A | 46 | 0.0 |
| CA_46A-48A | 48 | 0.8 |

Table 5.35.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_46A-48A | 48 | 0.5 |
| CA_46A-48A | 46 | 0 |

### 5.35.4	MSD

Table 5.4.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 48)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_46A-48A | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |

Due to insufficient cross band isolation between Band 46 and Band 48, MSD needs to be specified. The MSD analysis was conducted when CA_42A-46A was discussed. Because of similarity of CA_46A-48A to CA_42_46A, we apply the MSD specified for CA_42A-46A to that for CA_46A-48A.

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"

## 5.36	CA_2DL_5A-28A_1UL_BCS0

# 5.36.1	Operating bands for CA

# Table 5.36.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_5-28 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

# 5.36.2	Channel bandwidths per operating band for CA

# Table 5.36.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_5A-28A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.36.3	Co-existence studies

Table 5.36.3-1 gives the harmonic frequency limits for Band 5 and Band 28 2DL CA. As shown in the table, UL carrier of CA_5A-28A _BCS0 is not located at 2nd order harmonics or 3rd order harmonics of DL carriers.

# Table 5.36.3-1: DL harmonics frequency limits for CA of Band 5 and Band 28

| DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 869 | 894 | 758 | 803 |
| 2nd order DL harmonics frequency range (MHz) | 1738 to 1788 |  | 1516 to 1606 |  |
| 3rd order DL harmonics frequency range (MHz) | 2607 to 2682 |  | 2274 to 2409 |  |

As shown in Table 5.361.3-2, the harmonic frequencies of Band 5 and Band 28 in UL are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

# Table 5.36.3-2: UL harmonics frequency limits for CA of Band 5 and Band 28

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 |

### 5.36.4	ΔTIB,c and ΔRIB,c values

# Table 5.36.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_5A-28A | 5 | 0.5 |
|  | 28 | 0.5 |

# Table 5.36.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_5A-28A | 5 | 0 |
|  | 28 | 0 |

### 5.36.5	 MSD

No MSD values need to be defined for this combination.

## 5.37	CA_8A-27A

### 5.37.1	Operating band for CA

Table 5.37.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8A-27A | 8 | 880 MHz | -- | 915 MHz | 925 MHz |  | 960 MHz | FDD |
|  | 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |

5.37.2	Channel bandwidths per operating band for CA

Table 5.37.2-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_8A-27A | 8 |  |  | yes | yes |  |  | 20 | 0 |
|  | 27 |  |  | yes | yes |  |  |  |  |

### 5.37.3	UE Co-existence studies

Table 5.37.3-1 summarizes frequency ranges where harmonics occur due to Band 8 and Band 27 CA with 1 UL.

Table 5.37.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1 760 | 1 830 | 2 640 | 2 745 | 1 850 | 1 920 | 1 775 | 2 880 |
| 27 | 807 | 824 | 852 | 869 | 1 614 | 1 648 | 2 421 | 2 472 | 1 704 | 1 738 | 2 556 | 2 607 |

It can be seen from Table 5.37.3-1 that there is no harmonic interference for CA_8A-27A towards own receive bands.

Table 5.37.3-2: Band 8 and Band 27 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 852 | 869 |
| 2nd order harmonics frequency range (MHz) | 1 850 | 1 920 | 1 704 | 1 738 |
| 3rd order harmonics frequency range (MHz) | 2 775 | 2 880 | 2 556 | 2 607 |
| 2nd order IMD products | (f2_low – f1_high) | (f2_high – f1_low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 56 | 108 | 1 777 | 1 829 |
| 3rd order IMD products | (f2_low – 2*f1_high) | (f2_high – 2*f1_low) | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 981 | 1 068 | 744 | 813 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 2 702 | 2 789 | 2 629 | 2 698 |
| 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
|  | 908 | 977 | 817 | 904 |
| 3rd order IMD products (Considering Max BW) | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency limits (MHz) | 915 | 970 | 842 | 879 |

It can be seen from Table 5.37.3-2 that the 2nd harmonics of BS transmitting in Band 8 may fall into the BS receive band of Bands 2, 25, 33, 35,37 and 39 and none of 3rd harmonics may fall into the any E-UTRA operating BS receive band. In addition 2nd harmonics of BS transmitting in Band 27 may fall into the BS receive band of Bands 3, 4, 10 and 3rd harmonics may fall into BS receive band of Bands 7, 38 and 41.

Meanwhile the 2nd IMD products caused by BS supporting carrier aggregation of Band 8 and Band 27 may fall into the BS receive band of Band 3, 9 and 3rd IMD products may fall into the BS receive band of Band 5, 6, 8, 13, 14, 18, 19, 20, 26, 27, 28, 41 and 44. Note that the calculation in Table 5.37.3-2 (except the last row) assumes the BS is transmitting with the whole 35 MHz DL frequency of Band 8 and the whole 17 MHz DL frequency of Band 27.

If the BS is only transmitting an up to 10 MHz DL in Band 8 and 10 MHz in Band 27 as stated in the WIDs, the 3rd IMD products will not fall into the BS receive band of the Band 6, 8, or 18

It should be noted that Bands 2, 4, 10, 13, 14, 18, 19, 20, 25, 33, 35, 37, 38 and 39 are not intended for use in the same geographical area as Bands 8 and 27. Therefore, the focus here will be on the harmonics and IMD falling into Bands 3, 5, 7, 9, 26, 27, 28, 41 and 44. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 3, 5, 7, 8, 9, 26, 27, 28, 41 or 44 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 8 and 27 BS transmitters do not share the same antenna with Band 3, 5, 7, 8, 9, 26, 27, 28, 41 or 44 BS receiver

### 5.37.4	∆TIB,c and ∆RIB,c values

Following relaxations are allowed for the UE which supports inter-band carrier aggregation of Band 8 and Band 27.

Table 5.37.4-1: ΔTIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_8A-27A | 8 | 0.8 |
|  | 27 | 0.8 |

Table 5.37.4-2: ΔRIB,c for 2DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_8A-27A | 8 | 0.3 |
|  | 27 | 0.3 |

## 5.38	CA_7A-30A_1UL_BCS0

### 5.38.1	Channel bandwidths per operating band for CA

Table 5.38.1-1: Supported E-UTRA bandwidths per CA configuration for 2DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7A-30A | - | 7 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |

### 5.38.2	UE Co-existence studies

Table 5.38.2-1 summarizes frequency ranges where harmonics occur due to Band 7 and Band 30 with 1 UL.

Table 5.38.2-1: Impact of 1 UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 5240 | 5380 | 7860 | 8070 |
| 30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | 4700 | 4720 | 7050 | 7080 |

It can be seen from Table 5.38.2-1 that none of the harmonic products falls into the B7 or B30 UE receiving band..

### 5.38.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are shown in Table 5.38.3-1 and Table 5.38.3-2, respectively. The combination of Band 7 and Band 30 is a High-High band combination. The framework for HH band combination can apply for this combination, i.e.  TIB is 0.5 dB and RIB is 0 dB.

Table 5.38.3-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-30A | 7 | 0.5 |
|  | 30 | 0.5 |

Table 5.38.3-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7A-30A | 7 | 0 |
|  | 30 | 0 |

### 5.38.4	REFSENS requirements

There is no specific REFSENS requirement.

## 5.39 CA_2DL_1A-41A_BCS1

#### 5.39.1 Operating bands for CA

Table 5.39.1-1: 2DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-41A1 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| NOTE 1: For a UE that signals support of BCS1 for CA_1A-41A (see Table 5.39.1-2), requirements for PCell in Band 1 or Band 41 are applicable. Otherwise, requirements only apply for PCell in Band 1. |  |  |  |  |  |  |  |  |

### 5.39.1 Channel bandwidths per operating band for CA

Table 5.39.1-2: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink band | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-41A | 1, 41 | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.39.2	Co-existence studies

Table 5.39.2-1 shows the frequency ranges for 2nd and 3rd harmonics for Band 1 and 41 UL carriers. It can be seen that the UL harmonic frequencies of Band 1 and 41 do not fall into any UE receive bands of interest in the DL.

Table 5.39.2-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |

### 5.39.3 ΔTIB,c and ΔRIB,c values

For the UE that supports CA_2DL_1A-41A_BCS1, ΔTIB,c is given here for applicable bands in Table 6.X.3-1:

Table 5.39.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-41A | 1 | 0.5 |
|  | 41 | 0.5 |

For the UE that supports CA_2DL_1A-41A_BCS1, ΔRIB,c is given for applicable bands in Table 6.X.3-2:

Table 5.39.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-41A | 1 | 0 |
|  | 41 | 0 |

### 5.39.4 REFSENS requirements

There are no MSD or reference sensitivity exceptions for CA_2DL_1A-41A_BCS1.

## 5.X	CA_xA-yA_BCSn

### 5.X.1	Operating bands for CA

<Text will be added.>

### 5.X.2	Channel bandwidths per operating band for CA

<Text will be added.>

### 5.X.3	Co-existence studies

<Text will be added.>

### 5.X.4	∆TIB and ∆RIB values

<Text will be added.>

### 5.X.5	REFSENS requirements

<Text will be added if it’s necessary.>

### 5.X.6	Blocking requirements

<Text will be added if it’s necessary.>

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2017-08 | 3GPP RAN4#84 |  |  |  | Initial TR skeleton |  | 0.0.1 |
| 2017-9 | 3GPP RAN4#84 |  |  |  | R4-1707047,R4-1707048,R4-1707235,R4-1707236,R4-1707272,R4-1707845,R4-1707846,R4-1707847,R4-1707848,R4-1708021,R4-1708022,R4-1708023,R4-1708024,R4-1708025,R4-1708026,R4-1708027,R4-1708028 |  | 0.0.2 |
| 2017-11 | 3GPP RAN4#84bis |  |  |  | R4-1710273,R4-1710659,R4-1710660,R4-1710844,R4-1710845,R4-1710952,R4-1711463 |  | 0.1.1 |
| 2017-11 | 3GPP RAN4#85 |  |  |  |  |  | 0.1.2 |