3GPP TSG-RAN WG4 Meeting RAN4#87                                                 R4-1806442

Busan,Korean, 21 -25 May, 2018

Agenda Item:   7.12.1.3

Source:        Samsung

Title:          Overview of UE demodulation performance requirements

Document for:  Discussion

Introduction

A dedicated specification was assigned for NR UE performance requirements as 38.101-4. The target competition time for Rel-15 NR performance part is RAN#82 in Dec 2018. In this contribution, preliminary views for UE demodulation requirements were provided.

Discussion

In Rel-15, under NR WI, there are several physical layer features which related to baseband processing, and potential performance requirements required to cover such features.

Numerologies and CHBW

For LTE, only 15 kHz SCS and 6 (1.4MHz, 3MHz, 5MHz ~20MHz) Channel bandwidths were introduced. In LTE, all the channel bandwidths were supported by UE, and dedicated test cases were introduced to verify UE supported all the CHBWs (Test number 9, 10, 11,12,14,15 in table 8.2.1.1.1-2 of TS36.101). Beside of dedicated test cases covering CHBWs from 3MHz to 20MHz, almost all other test cases specified in LTE were based on 10MHz CHBW to ensure test coverage since almost all LTE bands support 10MHz.

For NR in Rel-15, 15 kHz, 30 kHz and 60 kHz supported for data channel transmission in FR1 and 60 kHz, 120 kHz supported in FR2. The Channel bandwidth sets are pending on numerologies and frequency ranges. For FR1, 5MHz ~50MHz supported for 15 kHz, for 30 kHz/60kHz numerologies, upper to 100MHz. For FR2, 50MHz ~200MHz for 60kHz, upper to 400MHz for 120kHz cases. Overall, we have 40 combinations of channel bandwidth and SCSs across FR1 and FR2 in total (in LTE, we only have 6 combos). It’s impractical to verify all the combinations. We need to pick up some combinations to introduce dedicated test cases which used to verify UE supporting different numerologies and Channel bandwidths.

Ob1: It’s impractical to introduce demodulation test cases covering all the combinations of numerologies and channel bandwidths (total 40 combos) in early stage.

Meanwhile we also need to select proper configuration of numerology and channel bandwidth as generic test set-up for normal performance requirements. In order to maximize test coverage and maintain performance requirements in a band agonistic manner, such generic combination {numerology and channel bandwidth} should be applicable for UE capability and all NR bands.

For supported numerologies, such agreements agreed in previous RAN4 meeting:

- FR1: 15kHz, 30kHz for sub 1GHz; 15kHz, 30kHz and 60kHz (optional) for above 1GHz

- FR2: 60kHz, 120kHz

Regarding channel bandwidths supported by UE, all the channel bandwidths combos of FR1 in current specification mandatory supported by UE and for FR2, upper to 200MHz mandatory supported by UE.

As refer to table “5.3.5-1 Channel Bandwidths for Each NR band” in TS38.101-1 v15.1.0, we can observe that for FR1:

- All bands support the combination of {15 kHz/30kHz, 20 MHz} except band 76 and band 79.

- Band 76 only support the combination of {15kHz, 5MHz}

- Band 79 start with minimum CHBW 40MHz

And for FR2, all the bands in TS38.101-2 support below combinations:

- 60kHz: 50MHz, 100MHz, 200MHz

- 120kHz: 50MHz, 100MHz, 200MHz, 400MHz (optional)

With above observations, we proposed to use below combination {numerology and Channel bandwidth} as generic test set-up for UE performance requirements:

Proposal 1: Using below combination {numerology and Channel bandwidth} as generic test set-up for UE performance requirements

- FR1: 30kHz, 20MHz

- FR2: 60kHz, 100MHz

Also in order to ensure test coverage for specific bands and verify UE processing capability to support different combinations of numerologies and CHBW, below combinations proposed as dedicated test cases for numerologies and CHBWs.

Proposal 2: Introduce test cases for below combination {numerology and Channel bandwidth} as dedicated test cases for verify UE processing capability to support different combinations

- FR1: {30kHz, 20MHz} basic, {15kHz, 5MHz} ,{60kHz, 20 MHz}, {30kHz, 40MHz}

- FR2: 60kHz, 100MHz basic, {120kHz, 200MHz}

Modulation and coding rate

For NR DL, QPSK, 16QAM, 64QAM and 256QM were supported. In LTE, QPSK 1/3, 16QAM 1/2, 64QAM 3/4 and 256 QAM 4/5 were used as normal combinations. In NR, such combinations still can be candidate normal configurations. Meanwhile, we also see the TBS determination methodology in RAN1 is different than what specified in LTE, now is more aligned with RAN4 rules for TBS selection in LTE phase, which try to ensure target code rate is more close to actual code rate considering actual overhead for PDSCH transmission. Taking LTE experience, FRC tables defined in annex were quite huge and accumulated (217 pages for FRC in current LTE spec), we may need to consider how to introduce FRC in NR with more efficiency and future proof way.

Ob2: Reusing LTE modulation and coding rate combinations of QPSK 1/3, 16QAM 1/2, 64QAM 3/4 and 256 QAM 4/5 (FR1 only) as candidate options for NR.

Ob3: RAN4 need to consider optimization of introducing FRC into specs.

Transmission schemes and layers

In NR Rel-15, only one transmission scheme introduced with DMRS based transmission (transmission scheme 1). From RAN1 perspective, upper to 8 layers for SU-MIMO and upper 12 layers supported for MU-MIMO. Meanwhile from RAN4 agreements for UE RF architecture, 4 receiver ports with upper to 4 layers in FR1 and 2 receiver ports with upper to 2 layers supported.

Ob4: Demodulation test cases need to cover upper to 2/4 layers with 2Rx and 4Rx in Rel-15.

Channel model & Doppler shift

Basically, TDL mode captured in TR 38.901 can be used for introducing performance test cases in NR. For NR, large frequency ranges covered across FR1 and FR2, Doppler shift need to be considered with proper frequency ranges. As we calculated below for FR1 with different operating ranges:

| UE moving speed | 5km/h | 30km/h | 120km/h |
| --- | --- | --- | --- |
| FDD band Operating ranges (800MHz ~2GHz) | 3.7Hz ~ 9Hz | 22Hz ~ 56Hz | 89Hz ~ 222Hz |
| TDD band Operating ranges (1.5G~5GHz) | 7Hz ~23Hz | 42Hz ~ 139Hz | 167Hz ~555Hz |

Proposal 3: For FR1 FDD, such Doppler shift recommended 5Hz, 30Hz, 100Hz, for FR1 TDD, Doppler shift as 15Hz, 90Hz and 360Hz

Based on above analysis, list of candidate test cases were proposed below:

- Table 1 below list test cases for FR1 FDD mode (10 cases +2/4 receiver ports)

- Table 2 below list test cases for FR1 TDD mode (10 cases +2/4 receiver ports)

- Table 3 below list test cases for FR2 TDD mode (5 cases+ 2 receiver ports)

Table 1: List of test cases for FR1 FDD mode

| Test number | Bandwidth | SCS | MCS & Coding rate | Propagation Condition | MIMO | Duplex mode | Number of Layer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 MHz | 15kHz | 16QAM 1/2 | TDL-B(100ns), 30Hz | 1x2 1x4 | FDD | 1 |
| 2 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 30Hz | 1x2 1x4 | FDD | 1 |
| 3 | 40 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 30Hz | 1x2 1x4 | FDD | 1 |
| 4 | 20MHz | 60kHz | 16QAM 1/2 | TDL-B(100ns), 30Hz | 1x2 1x4 | FDD | 1 |
| 5 | 20 MHz | 30kHz | QPSK 1/3 | TDL-C(300ns), 100 Hz | 1x2 1x4 | FDD | 1 |
| 6 | 20MHz | 30kHz | 64QAM 3/4 | TDL-A(30ns), 5Hz | 1x2 1x4 | FDD | 1 |
| 7 | 20MHz | 30kHz | 256QAM 4/5 | TDL-A(30ns), 5Hz | 1x2 1x4 | FDD | 1 |
| 8 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 30Hz | 2x2 2x4 | FDD | 2 |
| 9 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 30Hz | 4x2 4x4 | FDD | 2 |
| 10 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 30Hz | 4x4 | FDD | 4 |

Table 2: List of test cases for FR1 FDD mode

| Test number | Bandwidth | SCS | MCS & Coding rate | Propagation Condition | MIMO | Duplex mode | Number of Layer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 MHz | 15kHz | 16QAM 1/2 | TDL-B(100ns), 90Hz | 1x2 1x4 | TDD | 1 |
| 2 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 90Hz | 1x2 1x4 | TDD | 1 |
| 3 | 40 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 90Hz | 1x2 1x4 | TDD | 1 |
| 4 | 20MHz | 60kHz | 16QAM 1/2 | TDL-B(100ns), 90Hz | 1x2 1x4 | TDD | 1 |
| 5 | 20 MHz | 30kHz | QPSK 1/3 | TDL-C(300ns), 360 Hz | 1x2 1x4 | TDD | 1 |
| 6 | 20MHz | 30kHz | 64QAM 3/4 | TDL-A(30ns), 15Hz | 1x2 1x4 | TDD | 1 |
| 7 | 20MHz | 30kHz | 256QAM 4/5 | TDL-A(30ns), 15Hz | 1x2 1x4 | TDD | 1 |
| 8 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 90Hz | 2x2 2x4 | TDD | 2 |
| 9 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 90Hz | 4x2 4x4 | TDD | 2 |
| 10 | 20 MHz | 30kHz | 16QAM 1/2 | TDL-B(100ns), 90Hz | 4x4 | TDD | 4 |

Table 3: List of test cases for FR2 TDD mode

| Test number | Bandwidth | SCS | MCS & Coding rate | Propagation Condition | MIMO | Duplex mode | Number of Layer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 MHz | 60kHz | 16QAM 1/2 | FFS | 1x2 | TDD | 1 |
| 2 | 200 MHz | 120kHz | 16QAM 1/2 | FFS | 1x2 | TDD | 1 |
| 3 | 100 MHz | 60kHz | QPSK 1/3 | FFS | 1x2 | TDD | 1 |
| 4 | 100 MHz | 60kHz | 64QAM 3/4 | FFS | 1x2 | TDD | 1 |
| 5 | 100 MHz | 60kHz | 16QAM 1/2 | FFS | 1x2 | TDD | 2 |

Conclusion

In this contribution, preliminary views for UE demodulation requirements were provided.

Ob1: It’s impractical to introduce demodulation test cases covering all the combinations of numerologies and channel bandwidths (total 40 combos) in early stage.

Ob2: Reusing LTE modulation and coding rate combinations of QPSK 1/3, 16QAM 1/2, 64QAM 3/4 and 256 QAM 4/5 (FR1 only) as candidate options for NR.

Ob3: RAN4 need to consider optimization of introducing FRC into specs.

Ob4: Demodulation test cases need to cover upper to 2/4 layers with 2Rx and 4Rx in Rel-15.

Proposal 1: Using below combination {numerology and Channel bandwidth} as generic test set-up for UE performance requirements

- FR1: 30kHz, 20MHz

- FR2: 60kHz, 100MHz

Proposal 2: Introduce test cases for below combination {numerology and Channel bandwidth} as dedicated test cases for verify UE processing capability to support different combinations

- FR1: {30kHz, 20MHz} basic, {15kHz, 5MHz} ,{60kHz, 20 MHz}, {30kHz, 40MHz}

- FR2: 60kHz, 100MHz basic, {120kHz, 200MHz}

Proposal 3: For FR1 FDD, such Doppler shift recommended 5Hz, 30Hz, 100Hz, for FR1 TDD, Doppler shift as 15Hz, 90Hz and 360Hz

Based on analysis, list of candidate test cases were summarized in table 1, table 2 and table 3 for FR1 FDD, FR1 TDD and FR2 separately.

Reference

[1] R4-1805549, “Way Forward on NR UE performance requirements”, Intel

[2] R1-1805752, “LS on NR UE feature list”

[3] TS38.101-1 V15.1.0

[4] TS38.101-2 V15.1.0

Annex

Table 5.3.5-1 Channel Bandwidths for Each NR band in 38.101-1

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
| n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |
| n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |
| n70 | 15 | Yes | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |
| n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n75 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n76 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
| n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
| n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |
|  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |
| n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-1: Channel bandwidths for each NR band from TS38.101-1

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n258 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n260 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n261 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |