3GPP TSG-RAN WG4 Meeting #87                     	R4-1806547

Busan, Korea, May 21st – 25th, 2018

Agenda item:            7.3.2.1

Source:	MediaTek Inc.

Title:	On the clarification of in the same carrier

Document for:          Discussion

# 1	Introduction

In this paper, following the RAN4 #86bis agreement [1], we discuss the definition of “in the same carrier” to clarify the requirement in the specification

# 2	Discussion

For the LTE-NR coexistence in UL sharing from UE perspective, there is one switching time requirement defined in the feature list [2] as in Table 1. The condition for supporting this requirement - “Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective” - is not well defined and it will affect the UE implementation

Table 1 Feature description of Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective

| WI | # | Feature group | Components | Prerequisite feature groups | Need for gNB t | Consequences if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Remarks | Responsible WG | Recommendation for TSG-RAN | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1-11 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Support of switching type between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. Type 1: ~0us Type 2: <20us | 1-10 | Yes | UE does not support UL subcarrier alignment between LTE and NR for EN-DC with LTE-NR coexistence in UL | Type 3 (per band combination) | No Need | Applicable only to FR1 |  | This feature is the switching time between LTE UL and NR UL in the same carrier Per band combination signalling (Type 3) UE Capability signalling elements. 1: ~0us switching type. 2: <20us switching type. | RAN4 |  |  |

In the RAN4 #86 meeting, additional condition of same carrier between LTE and NR is added to clarify the requirement:

Agreements Keep the name of feature “Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective”. Add the clarifications in the remark column to clarify this feature is the switching time between LTE UL and NR UL in the same carrier

However, “in the same carrier” is not well defined in the specification.

- RAN4 to conclude the definition of “in the same carrier” in next meeting RAN4#87 for the following scenarios

  - Between NR and LTE

  - Within one RAT

- RAN4 to conclude the definition of “shared carrier” in next meeting RAN4#87

In RAN4 #86bis, the WF [1] is agreed to clarify the wording in the specification

Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective must be ~0us or 20us [3] under the condition of “in the same carrier”. For such short switching time, considering the UE implementation, UE RF setting for LTE and NR must be almost the same. Since UE RF requirement will determined by the carrier center frequency and range, it is reasonable that LTE carrier and NR carrier must have the same frequency range in the “same carrier” scenario. Otherwise, longer switching time is needed.

Proposal 1: Regarding LTE and NR in the same carrier for UL sharing from UE perspective, the carrier frequency range of LTE (CarrierLTE) and the carrier frequency range of NR (CarrierNR) are the same; i.e., the bandwidth and position of CarrierLTE and CarrierNR are the same.

NR

LTE

Carrier bandwidth

Agreement:

Proposal 1: UE requirements for FDM based ULSUP are developing assuming simultaneous LTE and NR UL transmissions on the shared UL carrier used in EN-DC.

Proposal 3: No UE switching time is allowed for FDM based ULSUP as UE is required to support simultaneous LTE and NR UL transmissions on the shared UL carrier.

In RAN4-#86bis meeting, there are discussion and agreement including the wording “shared carrier”

So far, there is no clear definition of shared carrier. Based on existed agreements, we have the following proposals

Proposal 2: “Shared carrier” is a carrier with the frequency range (frequency position and bandwidth) that is dynamically or semi-statically shared (TDM, FDM or mixed TDM/FDM) by more than 1 cell (The cells can be from different RAT, ex. LTE cell + NR cell).

Proposal 3: In the scenario of LTE-NR coexistence in UL sharing from UE perspective, the carrier is shared by more than 1 RAT, i.e. LTE and NR cell.

# 3	Conclusions

In the contribution, we provide our view on the definition of “in the same carrier”. We have the following proposals:

Proposal 1: Regarding LTE and NR in the same carrier for UL sharing from UE perspective, the carrier frequency range of LTE (CarrierLTE) and the carrier frequency range of NR (CarrierNR) are the same; i.e., the bandwidth and position of CarrierLTE and CarrierNR are the same.

Proposal 2: “Shared carrier” is a carrier with the frequency range (frequency position and bandwidth) that is dynamically or semi-statically shared (TDM, FDM or mixed TDM/FDM) by more than 1 cell (The cells can be from different RAT, ex. LTE cell + NR cell).

Proposal 3: In the scenario of LTE-NR coexistence in UL sharing from UE perspective, the carrier is shared by more than 1 RAT, i.e. LTE and NR cell.

# 4	Reference

# [1] R4-1805987, “Wayforward on clarification of in the same carrier”, Mediatek

# [2] R4-1803564, “Updated UE feature list”

# [3] R4-1711738, WF on LTE/NR UL subcarrier alignment, Huawei, HiSilicon, Vodafone, Deutsche Telekom, Orange, CATR, China Mobile, Etisalat, China Telecom, MediaTek, LGE, Vivo