3GPP TGS-RAN WG4 Meeting #87	R4-1806573
Busan, South Korea, May 21 – 25, 2018

Agenda item:	7.5.7.1

Source:	Intel Corporation

Title:	FR2 peak EIRP requirement for FWA

Document for:	Approval

# 1	Introduction

FR2 power class agreements have evolved over the last months. The parameters needed to derive the peak EIRP for handheld devices were agreed in [1] and an agreement for the min peak EIRP requirements was captured in [2]. In the AH-1801 meeting, a new UE type targeting Rel-15 for fixed wireless access (FWA) devices was introduced [3]. Later in RAN4 #86, the parameters table for FWA peak EIRP calculations was discussed and approved [4]. During our last meeting (RAN4 #86-Bis) it was agreed that RRM aspects for FWA will be the same as handheld in Rel-15 [5].

This paper summarizes the latest agreements for FWA devices and focuses on deriving values for the minimum peak EIRP requirements of FWA devices for approval.

# 2	Discussion

## 2.1	Background

FR2 power class requirements target Rel-15. The main focus in FR2 power class discussions has been handheld UE requirements, with its minimum peak EIRP agreed during our last meeting [2]. The remaining requirement to be finalized for this UE type is spherical coverage. Fixed wireless access (FWA) UE type was approved during AH-1801 and its requirements also target the Rel-15 scope [3].

Agreements from RAN4 #86-Bis [2]:

Agreement:

- For Rel-15

  - For 28GHz

    - Min peak EIRP is 22.4 dBm

    - 50%-tile requirement for EIRP CDF is FFS

  - For 39GHz

    - Min peak EIRP is 20.6 dBm

    - 50%-tile requirement for EIRP CDF is FFS

Note: for 50%-tile, the values are FFS.

Work plan for FWA [3]:

To align on the evaluation parameters needed to derive min peak EIRP for the new UE type, a table was approved in RAN4 #86 [4]. Finally, in RAN4 #86-Bis the RRM core requirements were agreed to remain the same for the FWA UE type in Rel-15. The work plan to finalize FWA requirements by Rel-15 was reiterated.

Agreement:

- RRM core requirement in TS38.133 needs no change due to introducing FWA UE type (with 55dBm peak EIRP limit) in Rel-15.

- Introduction of FWA does not introduce any additional RRM core requirement.

- FWA UE has the same RRM core requirements as the handhold UE at least in Rel-15.

## 2.2	Defining UE Types in FR2

The discussion outcome of how to divide and define new UE types was captured in [6]. A summary table with four UE types was included in this WF. Potential aspects to distinguish UE types include the anticipated power, max TRP and spherical coverage needs. It is important to make sure there is alignment on these parameters before choosing how to differentiate UE types. As more UE types are defined, their power class requirements will also need defining. Given the short amount of time left for Rel-15, a release independent approach was included in [6] to finalize any remaining requirements for new UE types. The rest of the paper will focus on deriving the min peak EIRP requirement for FWA devices on a stationary platform.

Observation 1: In case more time is needed to finalize the FWA requirements, we may follow a release independent approach.

## 2.3	Power class requirements for FWA

The table below details relevant architecture parameters needed to determine minimum peak EIRP [4]. We will use the data found in Table 1 to derive the minimum peak EIRP values for FWA devices on a stationary platform.

Table 1. FWA parameters for minimum peak EIRP evaluation

| Parameter | Unit | Freq. range 24.25-29.5 GHz | Freq. range 37.0-40.0 GHz | Comments |
| --- | --- | --- | --- | --- |
| Pout per element | dBm | 14.0 | 14.0 |  |
| # of antennas in array |  | 16 | 16 | Previously 4 for handheld |
| Total conducted power per polarization | dBm | 26.0 | 26.0 | Previously 20 dBm |
| Avg. antenna element gain | dBi | 4.0 | 4.0 |  |
| Antenna roll-off loss vs frequency | dB | -2.0 | -2.5 |  |
| Realized antenna array gain | dBi | 14.0 | 13.5 |  |
| Polarization gain | dB | 2.80 | 2.80 |  |
| Mismatch and transmission line loss including load pull | dB | -3.00 | -3.50 |  |
| Beam forming loss (phase shifter and amplitude error) | dB | -0.50 | -0.50 |  |
| Finite beam table | dB | -0.25 | -0.25 |  |
| Beam forming loss (one beam table fits all) | dB | -0.25 | -0.25 |  |
| Form-factor integration losses | dB | -3.50 | -4.50 |  |
| Total implementation loss (worst-case) | dB | -7.50 | -9.00 |  |
| Peak EIRP (Minimum) | dBm | 35.30 | 33.30 |  |

The derived values for FWA in Table 1 yield higher minimum peak EIRP values in great part because of the larger form-factor. This enables implementations with more antenna elements and allows for greater separation between sensitive RF components and high dielectric constant/lossy materials and reflectors that may otherwise affect radiation. The larger size also makes the overall integration easier and reduces the implementation losses. These differences are captured in the number of antennas, total conducted power and total implementation loss parameters. Compared to our handheld numbers, the number of antennas increased from 4 to 16, thus increasing the total conducted power by 6 dB. The overall implementation loss parameter has been reduced from the handheld value, highlighting the integration challenges of smaller form-factors. Once all parameters are factored in, we get a min peak EIRP of 35.30 dBm for 28 GHz, and 33.30 dBm for 39 GHz. These numbers correspond to a min peak EIRP increase of 15 dB compared to our numbers for handheld devices (20.2 dBm for 28 GHz, 18.4 dBm for 39 GHz).

Observation 2: The derived minimum peak EIRP values for FWA devices on a stationary platform are 35.30 dBm for 28GHz, and 33.30 dBm for 39GHz. This represents a 15 dB increase from our handheld device values.

Proposal 1: Define the minimum peak EIRP requirement for FWA devices on a stationary platform as 35.30 dBm for 28 GHz, and 33.30 dBm for 39 GHz.

We need to be mindful of the potential impact the new FWA UE type may have on RAN4 requirements. There will certainly be implications on testability and spherical coverage studies, as measurements must now take into account the fixed nature of the device, use case and typical surrounding materials or environment. From a co-existence perspective, no additional work is needed as previous studies for +55dBm CPE have been concluded with no issues reported [7]. Therefore, we should discuss what to include from these results in the FWA requirements.

Observation 3: RAN4 should discuss and agree on what FWA requirements to include from the completed co-existence study.

# 3	Conclusions

In this paper we derived the power class minimum peak EIRP requirement for FWA devices. The following observations and proposals have been made:

Observation 1: In case more time is needed to finalize the FWA requirements, we may follow a release independent approach.

Observation 2: The derived minimum peak EIRP values for FWA devices on a stationary platform are 35.30 dBm for 28GHz, and 33.30 dBm for 39GHz. This represents a 15 dB increase from our handheld device values.

Proposal 1: Define the minimum peak EIRP requirement for FWA devices on a stationary platform as 35.30 dBm for 28 GHz, and 33.30 dBm for 39 GHz.

Observation 3: RAN4 should discuss and agree on what FWA requirements to include from the completed co-existence study.

# 4	References

- R4-1706935, “WF on power class framework for mmWave,” Intel Corporation, Huawei, LGE, 3GPP RAN4 AH NR#2, June 2017

- R4-180xxxx, “RAN4 #86-Bis Meeting report,” 3GPP RAN4, April 2018

- R4-1801191, “WF on Fixed wireless access use case for FR2,” Intel Corporation, Verizon, 3GPP RAN4 AH-1801, January 2018

- R4-1801787, “On FR2 power class for FWA devices,” Intel Corporation, RAN4 #86, February 2018

- R4-1805920, “WF on FR2 FWA related requirements,” Intel Corporation, Samsung, Verizon, LGE, 3GPP RAN4 #86-Bis, April 2018

- R4-1805926, “How to Distinguish UE Types in FR2,” Samsung, LGE, Intel Corporation, Verizon, 3GPP RAN4 #86-Bis, April 2018

- R4-1711861, “Summary of simulation results on Coexistence Studies for 55dBm CPE,” Ericsson, Qualcomm, Samsung, Verizon, Nokia, RAN4 #84bis, October 2017