3GPP TR 36.715-00-02 V0.5.0 (2018-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE Advanced xDL/2UL Carrier Aggregation with x=3,4,5 (Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices.

Keywords

LTE-Radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – FRANCE

Tel. : +33 4 92 94 42 00 Fax : +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	11

1	Scope	12

2	References	22

3	Definitions, symbols and abbreviations	23

3.1	Definitions	23

3.2	Symbols	23

3.3	Abbreviations	23

4	Background	23

5	xDLs/2UL Carrier Aggregation: General part	24

5.1	UE RF specific	24

5.1.1	UE general architecture	24

5.1.2	General treatment of ∆TIB and ∆RIB values	24

5.1.3	Fallback CA mode and mandatory support of all paired 2UL CA configurations	27

5.1.4	General TX/RX requirements	27

5.1.4.1 Transmitter requirements	27

5.1.4.2 Receiver requirements	28

5.2	RRM specific	40

6	3DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	40

6.1	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 32 with 2 ULs	40

6.1.1	List of specific combination issues	40

6.1.1.1	Channel bandwidths per operating band for CA	40

6.1.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-32A	40

6.1.1.3	MSD	42

6.1.1.4	∆TIB and ∆RIB values	42

6.2	LTE Advanced Carrier Aggregation: Band 3 and Band 20 and Band 32 with 2 ULs	42

6.2.1	List of specific combination issues	42

6.2.1.1	Channel bandwidths per operating band for CA	42

6.2.1.2	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_3A-20A-32A	43

6.2.1.3	MSD	44

6.2.1.4	∆TIB and ∆RIB values	44

6.3	LTE Advanced Carrier Aggregation: Band 7 and Band 20 and Band 32 with 2 ULs	44

6.3.1	List of specific combination issues	44

6.3.1.1	Channel bandwidths per operating band for CA	44

6.3.1.2	Co-existence studies for LTE-A UL CA_7A-20A and DL CA_7A-20A-32A	44

6.3.1.3	MSD	46

6.3.1.4	∆TIB and ∆RIB values	46

6.4	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 with 2 ULs	46

6.4.1	List of specific combination issues	46

6.4.1.1	Channel bandwidths per operating band for CA	46

6.4.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41C	47

6.4.1.3	MSD	49

6.4.1.4	∆TIB and ∆RIB values	49

6.5	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 with 2 ULs	49

6.5.1	List of specific combination issues	49

6.5.1.1	Channel bandwidths per operating band for CA	49

6.5.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41A-42A	50

6.5.1.3	MSD	52

6.5.1.4	∆TIB and ∆RIB values	52

6.6	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 with 2 ULs	53

6.6.1	List of specific combination issues	53

6.6.1.1	Channel bandwidths per operating band for CA	53

6.6.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-28A-41A	53

6.6.1.3	MSD	55

6.6.1.4	∆TIB and ∆RIB values	55

6.7	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 42 with 2 ULs	55

6.7.1	List of specific combination issues	56

6.7.1.1	Channel bandwidths per operating band for CA	56

6.7.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41A-42A	56

6.7.1.3	MSD	57

6.7.1.4	∆TIB and ∆RIB values	57

6.8	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 with 2 ULs	58

6.8.1	List of specific combination issues	58

6.8.1.1	Channel bandwidths per operating band for CA	58

6.8.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A	58

6.8.1.3	MSD	60

6.8.1.4	∆TIB and ∆RIB values	60

6.9	LTE Advanced Carrier Aggregation: Band 7 and Band 7 and Band 8 with 2 ULs	60

6.9.1	List of specific combination issues	60

6.9.1.1	Channel bandwidths per operating band for CA	60

6.9.1.2	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_7A-7A-8A	60

6.9.1.3	MSD	62

6.9.1.4	∆TIB and ∆RIB values	62

6.10	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 40 with 2 ULs	62

6.10.1 Operating bands for CA	62

6.10.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-40A	62

6.10.1.3	MSD	64

6.10.1.4	∆TIB and ∆RIB values	64

6.11	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 with 2 ULs	64

6.11.1 	Operating bands for CA	64

6.12	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 19 with 2 ULs	65

6.12.1 	Operating bands for CA	65

6.13	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 21 with 2 ULs	65

6.13.1 	Operating bands for CA	65

6.14	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 42 with 2 ULs	66

6.14.1 	Operating bands for CA	66

6.15	LTE Advanced Carrier Aggregation: Band 1 and Band 42 and Band 42 with 2 ULs	67

6.15.1 	Operating bands for CA	67

6.16	LTE Advanced Carrier Aggregation: Band 3 and Band 42 and Band 42 with 2 ULs	67

6.16.1 	Operating bands for CA	67

6.17	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 26 with 2 ULs	68

6.17.1	List of specific combination issues	68

6.17.1.1	Channel bandwidths per operating band for CA	68

6.17.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-26A	68

6.17.1.3	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-7A-26A	70

6.17.1.4	MSD	70

6.17.1.5	∆TIB and ∆RIB values	71

6.18	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 26 with 2 ULs	71

6.18.1	List of specific combination issues	71

6.18.1.1	Channel bandwidths per operating band for CA	71

6.18.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-26A	71

6.18.1.3	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_3A-7A-26A	73

6.18.1.4	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_3A-7A-26A	75

6.18.1.5	MSD	76

6.18.1.6	∆TIB and ∆RIB values	76

6.19	LTE Advanced Carrier Aggregation: Band 7 and Band 7 and Band 26 with 2 ULs	76

6.19.1	List of specific combination issues	76

6.19.1.1	Channel bandwidths per operating band for CA	76

6.19.1.2	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_7A-7A-26A	76

6.19.1.3	MSD	77

6.19.1.4	∆TIB and ∆RIB values	77

6.20	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 7 with 2ULs	78

6.20.1	List of specific combination issues	78

6.20.1.1	Channel bandwidths per operating band for CA	78

6.20.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7C	78

6.20.1.3	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-7C	80

6.20.1.4	 MSD	80

6.20.1.5	 ∆TIB and ∆RIB values	80

6.21	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 28 with 2ULs	81

6.21.1	List of specific combination issues	81

6.21.1.1	Channel bandwidths per operating band for CA	81

6.21.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-7A-28A	81

6.21.1.3	 MSD	83

6.21.1.4	 ∆TIB and ∆RIB values	83

6.22	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 18 with 2 ULs	83

6.22.1	List of specific combination issues	83

6.22.1.1	Channel bandwidths per operating band for CA	83

6.22.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-18A	83

6.22.1.3	Co-existence studies for LTE-A UL CA_1A-18A and DL CA_1A-3A-18A	85

6.22.1.4	Co-existence studies for LTE-A UL CA_3A-18A and DL CA_1A-3A-18A	86

6.22.1.5	MSD	88

6.22.1.6	∆TIB and ∆RIB values	88

6.23	LTE Advanced Carrier Aggregation: Band 3 and Band 11 and Band 18 with 2 ULs	88

6.23.1	List of specific combination issues	88

6.23.1.1	Channel bandwidths per operating band for CA	88

6.23.1.2	Co-existence studies for LTE-A UL CA_3A-11A and DL CA_3A-11A-18A	89

6.23.1.3	Co-existence studies for LTE-A UL CA_3A-18A and DL CA_3A-11A-18A	90

6.23.1.4	Co-existence studies for LTE-A UL CA_11A-18A and DL CA_3A-11A-18A	91

6.23.1.5	MSD	93

6.23.1.6	∆TIB and ∆RIB values	93

6.24	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 20 with 2 ULs	94

6.24.1	List of specific combination issues	94

6.24.1.1	Channel bandwidths per operating band for CA	94

6.24.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-20A	94

6.24.1.3	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-3A-20A	95

6.24.1.4	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_1A-3A-20A	96

6.24.1.5	MSD	97

6.24.1.6	∆TIB and ∆RIB values	97

7	 4DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	97

7.1	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 20 and Band 32 with 2 ULs	97

7.1.1	List of specific combination issues	97

7.1.1.1	Channel bandwidths per operating band for CA	97

7.1.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-20A-32A	98

7.1.1.3	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_3A-7A-20A-32A	99

7.1.1.4	Co-existence studies for LTE-A UL CA_7A-20A and DL CA_3A-7A-20A-32A	101

7.1.1.5	MSD	102

7.1.1.6	∆TIB and ∆RIB values	102

7.2	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 42 and Band 42 with 2 ULs	102

7.2.1	List of specific combination issues	102

7.2.1.1	Channel bandwidths per operating band for CA	102

7.2.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41A-42C	103

7.2.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_1A-41A-42C	104

7.2.1.4	MSD	105

7.2.1.5	∆TIB and ∆RIB values	105

7.3	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 41 and Band 42 with 2 ULs	105

7.3.1	List of specific combination issues	105

7.3.1.1	Channel bandwidths per operating band for CA	105

7.3.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41C-42A	105

7.3.1.3	MSD	107

7.3.1.4	∆TIB and ∆RIB values	107

7.4	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 40 and Band 40 with 2 ULs	107

7.4.1 Operating bands for CA	107

7.4.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-40C	107

7.4.1.3	MSD	109

7.4.1.4	∆TIB and ∆RIB values	109

7.5	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 and Band 8 with 2 ULs	109

7.5.1	List of specific combination issues	109

7.5.1.1	Channel bandwidths per operating band for CA	109

7.5.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-7A-8A	109

7.5.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-7A-7A-8A	110

7.5.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-7A-7A-8A	111

7.5.1.5	MSD	112

7.5.1.6	∆TIB and ∆RIB values	113

7.6	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 8 with 2 ULs	113

7.6.1	List of specific combination issues	113

7.6.1.1	Channel bandwidths per operating band for CA	113

7.6.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A-8A	113

7.6.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-3A-7A-8A	114

7.6.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-3A-7A-8A	115

7.6.1.5	MSD	116

7.6.1.6	∆TIB and ∆RIB values	117

7.7	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 with 2 ULs	117

7.7.1	List of specific combination issues	117

7.7.1.1	Channel bandwidths per operating band for CA	117

7.7.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A-7A	117

7.7.1.3	MSD	118

7.7.1.4	∆TIB and ∆RIB values	118

7.8	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 and Band 42 with 2 ULs	118

7.8.1	List of specific combination issues	119

7.8.1.1	Channel bandwidths per operating band for CA	119

7.8.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41C-42A	119

7.8.1.3	MSD	121

7.8.1.4	∆TIB and ∆RIB values	121

7.9	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 and Band 42 with 2 ULs	121

7.9.1	List of specific combination issues	121

7.9.1.1	Channel bandwidths per operating band for CA	121

7.9.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41A-42C	121

7.9.1.3	MSD	123

7.9.1.4	∆TIB and ∆RIB values	123

7.10	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 41 with 2 ULs	123

7.10.1	List of specific combination issues	123

7.10.1.1	Channel bandwidths per operating band for CA	123

7.10.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-28A-41C	123

7.10.1.3	MSD	125

7.10.1.4	∆TIB and ∆RIB values	125

7.11	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 42 with 2 ULs	125

7.11.1	List of specific combination issues	125

7.11.1.1	Channel bandwidths per operating band for CA	125

7.11.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-28A-41A-42A	125

7.11.1.3	MSD	127

7.11.1.4	∆TIB and ∆RIB values	127

7.12	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 19 with 2 ULs	127

7.12.1	List of specific combination issues	127

7.12.1.1	Operating bands for CA	127

7.12.1.2	Channel bandwidths per operating band for CA	127

7.12.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-19A	128

7.12.1.4	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-3A-19A	128

7.12.1.5	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-3A-19A	128

7.12.1.6	MSD	128

7.13	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 21 with 2 ULs	128

7.13.1	List of specific combination issues	128

7.13.1.1	Operating bands for CA	128

7.13.1.2	Channel bandwidths per operating band for CA	128

7.13.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-21A	129

7.13.1.4	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-3A-21A	129

7.13.1.5	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-3A-21A	129

7.13.1.6	MSD	129

7.14	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 42 with 2 ULs	129

7.14.1	List of specific combination issues	129

7.14.1.1	Operating bands for CA	129

7.14.1.2	Channel bandwidths per operating band for CA	129

7.14.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-42A	130

7.14.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-3A-42A	130

7.14.1.5	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-3A-42A	130

7.14.1.6	MSD	130

7.15	LTE Advanced Carrier Aggregation: CA_1A-42A-42C with 2 ULs	130

7.15.1	List of specific combination issues	130

7.15.1.1	Operating bands for CA	130

7.15.1.2	Channel bandwidths per operating band for CA	130

7.15.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42A-42C	130

7.15.1.4	MSD	131

7.16	LTE Advanced Carrier Aggregation: CA_1A-42D with 2 ULs	131

7.16.1	List of specific combination issues	131

7.16.1.1	Operating bands for CA	131

7.16.1.2	Channel bandwidths per operating band for CA	131

7.16.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42D	131

7.16.1.4	MSD	131

7.17	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 19 and Band 21 with 2 ULs	131

7.17.1	List of specific combination issues	132

7.17.1.1	Operating bands for CA	132

7.17.1.2	Channel bandwidths per operating band for CA	132

7.17.1.3	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-3A-19A-21A	132

7.17.1.4	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-3A-19A-21A	132

7.17.1.5	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_3A-3A-19A-21A	132

7.17.1.6	MSD	132

7.18	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 42 and Band 42 with 2 ULs	132

7.18.1	List of specific combination issues	133

7.18.1.1	Operating bands for CA	133

7.18.1.2	Channel bandwidths per operating band for CA	133

7.18.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-3A-42C	133

7.18.1.4	MSD	133

7.19	LTE Advanced Carrier Aggregation: CA_3A-42A-42C with 2 ULs	133

7.19.1	List of specific combination issues	133

7.19.1.1	Operating bands for CA	133

7.19.1.2	Channel bandwidths per operating band for CA	134

7.19.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42A-42C	134

7.19.1.4	MSD	134

7.20	LTE Advanced Carrier Aggregation: CA_3A-42D with 2 ULs	134

7.20.1	List of specific combination issues	134

7.20.1.1	Operating bands for CA	134

7.20.1.2	Channel bandwidths per operating band for CA	134

7.20.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42D	134

7.20.1.4	MSD	135

7.21	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 26 with 2 ULs	135

7.21.1	List of specific combination issues	135

7.21.1.1	Channel bandwidths per operating band for CA	135

7.21.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-26A	135

7.21.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-26A	137

7.21.1.4	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_1A-3A-7A-26A	139

7.21.1.5	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-3A-7A-26A	141

7.21.1.6	MSD	141

7.21.1.7	∆TIB and ∆RIB values	142

7.22	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 7 and Band 26 with 2 ULs	142

7.22.1	List of specific combination issues	142

7.22.1.1	Channel bandwidths per operating band for CA	142

7.22.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-7A-26A	142

7.22.1.3	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-7A-7A-26A	144

7.22.1.4	MSD	144

7.22.1.5	∆TIB and ∆RIB values	145

7.23	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 and Band 26 with 2 ULs	145

7.23.1	List of specific combination issues	145

7.23.1.1	Channel bandwidths per operating band for CA	145

7.23.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-7A-26A	145

7.23.1.3	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_3A-7A-7A-26A	147

7.23.1.4	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_3A-7A-7A-26A	149

7.23.1.5	MSD	149

7.23.1.6	∆TIB and ∆RIB values	150

7.24	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 7 with 2ULs	150

7.24.1	List of specific combination issues	150

7.24.1.1	Channel bandwidths per operating band for CA	150

7.24.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7C	150

7.24.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7C	152

7.24.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7C	154

7.24.1.5	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-3A-7C	156

7.24.1.6	 MSD	156

7.24.1.7	 ∆TIB and ∆RIB values	156

7.25	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 7 with 2ULs	157

7.25.1	List of specific combination issues	157

7.25.1.1	Channel bandwidths per operating band for CA	157

7.25.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C-7A	157

7.25.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3C-7A	159

7.25.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3C-7A	161

7.25.1.5	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C-7A	163

7.25.1.6	 MSD	163

7.25.1.7	 ∆TIB and ∆RIB values	163

7.26	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 with 2ULs	164

7.26.1	List of specific combination issues	164

7.26.1.1	Channel bandwidths per operating band for CA	164

7.26.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3C-7C	164

7.26.1.3	Co-existence studies for LTE-A UL CA_3C and DL CA_3C-7C	164

7.26.1.4	Co-existence studies for LTE-A UL CA_7C and DL CA_3C-7C	165

7.26.1.5	 MSD	165

7.26.1.6	 ∆TIB and ∆RIB values	165

8	 5DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	166

8.1	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs	166

8.1.1	List of specific combination issues	166

8.1.1.1	Channel bandwidths per operating band for CA	166

8.1.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41C-42C	166

8.1.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_1A-41C-42C	167

8.1.1.4	MSD	168

8.1.1.5	∆TIB and ∆RIB values	168

8.2	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs	168

8.2.1	List of specific combination issues	168

8.2.1.1	Channel bandwidths per operating band for CA	168

8.2.1.2	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-41C-42C	168

8.2.1.3	MSD	169

8.2.1.4	∆TIB and ∆RIB values	169

8.3	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs	169

8.3.1	List of specific combination issues	169

8.3.1.1	Channel bandwidths per operating band for CA	169

8.3.1.2	Co-existence studies for LTE-A UL CA_42C and DL CA_28A-41C-42C	169

8.3.1.3	MSD	170

8.3.1.4	∆TIB and ∆RIB values	170

8.4	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 42 and Band 42 with 2 ULs	170

8.4.1	List of specific combination issues	170

8.4.1.1	Channel bandwidths per operating band for CA	170

8.4.1.2	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-28A-41A-42C	170

8.4.1.3	MSD	171

8.4.1.4	∆TIB and ∆RIB values	171

8.5	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 19 and Band 21 with 2 ULs	171

8.5.1	List of specific combination issues	171

8.5.1.1	 Operating bands for CA	171

8.5.1.2	 Channel bandwidths per operating band for CA	172

8.5.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-19A-21A	172

8.5.1.4	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-3A-19A-21A	172

8.5.1.5	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-3A-19A-21A	172

8.5.1.6	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-3A-19A-21A	172

8.5.1.7	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-3A-19A-21A	172

8.5.1.8	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_1A-3A-3A-19A-21A	172

8.5.1.9	MSD	172

8.6	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 42 and Band 42 with 2 ULs	172

8.6.1	List of specific combination issues	173

8.6.1.1	 Operating bands for CA	173

8.6.1.2	 Channel bandwidths per operating band for CA	173

8.6.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-42C	173

8.6.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-3A-42C	173

8.6.1.5	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-3A-42C	173

8.5.1.6	MSD	173

8.7	LTE Advanced Carrier Aggregation: CA_1A-42E with 2 ULs	173

8.7.1	List of specific combination issues	174

8.7.1.1	 Operating bands for CA	174

8.7.1.2	 Channel bandwidths per operating band for CA	174

8.7.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42E	174

8.7.1.4	MSD	174

8.8	LTE Advanced Carrier Aggregation: CA_1A-42C-42C with 2 ULs	174

8.8.1	List of specific combination issues	174

8.8.1.1	 Operating bands for CA	174

8.8.1.2	 Channel bandwidths per operating band for CA	175

8.8.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42C-42C	175

8.8.1.4	MSD	175

8.9	LTE Advanced Carrier Aggregation: CA_3A-42E with 2 ULs	175

8.9.1	List of specific combination issues	175

8.9.1.1	 Operating bands for CA	175

8.9.1.2	 Channel bandwidths per operating band for CA	175

8.9.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42E	176

8.9.1.4	MSD	176

8.10	LTE Advanced Carrier Aggregation: CA_3A-42C-42C with 2 ULs	176

8.10.1	List of specific combination issues	176

8.10.1.1	 Operating bands for CA	176

8.10.1.2	 Channel bandwidths per operating band for CA	176

8.10.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42C-42C	176

8.10.1.4	MSD	176

8.11	LTE Advanced Carrier Aggregation: CA_3A-3A-42D with 2 ULs	177

8.11.1	List of specific combination issues	177

8.11.1.1	 Operating bands for CA	177

8.11.1.2	 Channel bandwidths per operating band for CA	177

8.11.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-3A-42D	177

8.11.1.4	MSD	177

8.12	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 and Band 8 with 2 ULs	177

8.12.1	List of specific combination issues	178

8.12.1.1	 Channel bandwidths per operating band for CA	178

8.12.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A-7A-8A	178

8.12.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-3A-7A-7A-8A	179

8.12.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-3A-7A-7A-8A	180

8.12.1.5	MSD	181

8.12.1.6	∆TIB and ∆RIB values	181

8.13	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 7 and Band 26 with 2 ULs	182

8.13.1	List of specific combination issues	182

8.13.1.1	Channel bandwidths per operating band for CA	182

8.13.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-7A-26A	182

8.13.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-7A-26A	184

8.13.1.4	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_1A-3A-7A-7A-26A	186

8.13.1.5	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-3A-7A-7A-26A	188

8.13.1.6	MSD	188

8.13.1.7	∆TIB and ∆RIB values	189

8.14	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 7 and Band 7 with 2ULs	189

8.14.1	List of specific combination issues	189

8.14.1.1	Channel bandwidths per operating band for CA	189

8.14.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C-7C	190

8.14.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3C-7C	192

8.14.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3C-7C	194

8.14.1.5	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C-7C	195

8.14.1.6	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-3C-7C	195

8.14.1.7	 MSD	195

8.14.1.8	 ∆TIB and ∆RIB values	196

Annex A: Change history	197

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for LTE Advanced 2ULs and xDLs Carrier Aggregation with x=3,4,5 under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address 2ULs and xDLs Carrier Aggregation requirements.

This TR covers relevant background information and studies core & performance requirements for the Rel-15 band combinations in Table 1-1, 1-2 and 1-3.

Table 1-1: 3DL/2UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  |  | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A | CA_1A-3A | 1 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-18A | CA_1A-3A, CA_1A-18A, CA_3A-18A | 1 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes | 55 | 0 |
|  |  | 3 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |
|  |  | 18 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_1A-3A-20A | CA_1A-3A CA_1A-20A CA_3A-20A | 1 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes | 60 | 0 |
|  |  | 3 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |
|  |  | 20 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |
| CA_1A-3A-26A | CA_1A-3A, CA_1A-26A, CA_3A-26A | 1 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  | 55 | 0 |
|  |  | 3 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  | 26 |  |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-41A2 | CA_1A-3A | 1 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  | 60 | 0 |
|  |  | 3 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  |  |  |
| CA_1A-7A-26A | CA_1A-7A, CA_1A-26A, CA_7A-26A | 1 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  | 55 | 0 |
|  |  | 7 |  |  |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  | 26 |  |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7C | CA_1A-7A CA_7C | 1 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  | Yes |  | Yes |  |  | Yes | Yes |  | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-28A | CA_1A-7A CA_1A-28A CA_7A-28A | 1 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes | 60 | 1 |
|  |  | 7 |  |  |  |  |  |  | Yes |  | Yes |  |  |  | Yes |  |  |
|  |  | 28 |  |  |  |  |  |  | Yes |  | Yes |  |  |  | Yes |  |  |
| CA_1A-41A-42A | CA_1A-42A | 1 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_1A-42C | CA_42C | 1 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42A-42A | CA_1A-42A | 1 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A | CA_3A-7A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 1 |
|  |  | 7 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-3A-19A | CA_3A-19A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 19 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| CA_3A-3A-21A | CA_3A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 21 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| CA_3A-3A-42A | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-7A-26A | CA_3A-7A, CA_3A-26A, CA_7A-26A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 55 | 0 |
|  |  | 7 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 26 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| CA_3A-7A-28A | CA_3A-28A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-7A-32A | CA_3A-7A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-11A-18A | CA_3A-11A or CA_3A-18A or CA_11A-18A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 45 | 0 |
|  |  | 11 |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| CA_3A-11A-26A | CA_3A-11A or CA_3A-26A or CA_11A-26A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 45 | 0 |
|  |  | 11 |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| CA_3A-20A-32A | CA_3A-20A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-28A-40A | CA_3A-28A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-28A-41A | CA_3A-41A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-41C | CA_3A-41A, CA_41C | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42A | CA_3A-41A, CA_3A-42A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_3A-42A-42A | CA_3A-42A | 3 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-8A | CA_7A-8A | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |
| CA_7A-7A-26A | CA_7A-26A | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 26 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| CA_7A-20A-32A | CA_7A-20A | 7 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_40C-42A | CA_40C | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_41C-48A | 41C | 41 | See CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 48 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_xA-yA-zA | CA_xA-yA or CA_yA-zA or CA_xA-zA | X |  |  |  |  |  |  |  |  |  |  |  |  |  | Up to 60 | 0 |
|  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: B41 and B42 are assumed to be synchronized (i.e. without simultaneous Rx/Tx) NOTE 2: For the corresponding CA configuration, UL transmission in this E-UTRA band is not allowed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 1-2: 4DL/2UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz |  |  | 10 MHz | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A-19A | CA_1A-3A, CA_1A-19A, CA_3A-19A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 19 |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-3A-21A | CA_1A-3A, CA_1A-21A, CA_3A-21A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-3A-42A | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
| CA_1A-3A-7C | CA_1A-3A CA_1A-7A CA_3A-7A CA_7C | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 3 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 1 |
|  |  | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-7A | CA_1A-3A CA_1A-7A CA_3A-7A CA_3C | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 1 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
| CA_1A-3A-7A-26A | CA_1A-3A, CA_1A-7A, CA_1A-26A, CA_3A-7A, CA_3A-26A, CA_7A-26A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 75 | 0 |
|  |  | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 26 |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-7A-28A | CA_1A-7A CA_1A-3A CA_1A-28A CA_3A-7A CA_3A-28A CA_7A-28A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 3 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 28 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_1A-3A-41C2 | CA_1A-3A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-7A-26A | CA_1A-7A, CA_1A-26A, CA_7A-26A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 75 | 0 |
|  |  | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
| CA_1A-7C-28A | CA_1A-7A CA_7C CA_7A-28A CA_1A-28A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 7C | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_1A-41A-42C | CA_1A-42A, CA_42C | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 42 | See CA_42C_BW combination set 1 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-41C-42A | CA_1A-42A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 41 | See CA_41C_BW combination set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_1A-42A-42C | CA_1A-42A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42D | CA_1A-42A | 1 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A-7A | CA_3A-7A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-7C | CA_3C CA_7C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_3A-3A-19A-21A | CA_3A-19A, CA_3A-21A, CA_19A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 19 |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
| CA_3A-3A-42C | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
| CA_3A-7A-7A-26A | CA_3A-7A, CA_3A-26A, CA_7A-26A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 75 | 0 |
|  |  | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  |  | Yes | Yes |  |  |  |  |
| CA_3A-7A-20A-32A | CA_3A_7A CA_3A_20A CA_7A_20A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 7 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 20 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 32 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
| CA_3A-28A-40C | CA_3A-28A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-41A-42A | CA_3A-41A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 70 | 0 |
|  |  | 28 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 42 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_3A-28A-41C | CA_3A-41A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41C-42A | CA_3A-41A, CA_3A-42A, CA_41C | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
| CA_3A-41A-42C | CA_3A-41A, CA_3A-42A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41D | CA_3A-41A, CA_41C | 3 |  |  | Yes |  | Yes |  |  | Yes | Yes |  | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A-42C | CA_3A-42A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42D | CA_3A-42A | 3 |  |  |  | Yes |  |  | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_40C-42C | CA_40C, CA_42C | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-48C | 41C | 41 | See CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |
| CA_41D-48A | 41C | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_wA-xA-yA-zA | CA_wA-xA, CA_wA-yA, CA_wA-zA, CA_xA-yA, CA_xA-zA, CA_yA-zA | W |  |  |  |  |  |  |  |  |  |  | Up to 80 |  |
|  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: B41 and B42 are assumed to be synchronized (i.e. without simultaneous Rx/Tx) NOTE 2: For the corresponding CA configuration, UL transmission in this E-UTRA band is not allowed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 1-3: 5DL/2UL CA configurations part of the WI

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A-19A-21A | CA_1A-3A, CA_1A-19A, CA_1A-21A, CA_3A-19A, CA_3A-21A, CA_19A-21A | 1 |  |  | Yes | Yes |  | Yes |  | Yes | 90 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |
|  |  | 19 |  |  | Yes | Yes |  | Yes |  |  |  |  |
|  |  | 21 |  |  | Yes | Yes |  | Yes |  |  |  |  |
| CA_1A-3A-3A-42C | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  | Yes | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A-7A-26A | CA_1A_3A, CA_1A_7A, CA_1A_26A, CA_3A_7A, CA_3A_26A CA_7A_26A | 1 |  |  | Yes | Yes |  | Yes |  | Yes | 85 | 0 |
|  |  | 3 |  |  | Yes | Yes |  | Yes |  | Yes |  |  |
|  |  | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  | Yes | Yes |  | Yes |  |  |  |  |
| CA_1A-3C-7C | CA_1A-3A CA_1A-7A CA_3A-7A CA_3C CA_7C | 1 |  |  | Yes | Yes | Yes |  |  | Yes | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes |  |  | Yes | 100 | 1 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7C-28A | CA_1A-3A CA_1A-7A CA_3A-7A CA_7C CA_7A-28A CA_1A-28A CA_3A-28A | 1 |  |  | Yes | Yes | Yes |  |  | Yes | 100 | 0 |
|  |  | 3 |  |  |  | Yes | Yes |  |  | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes |  |  | Yes |  |  |
| CA_1A-3A-41D2 | CA_1A-3A | 1 |  |  | Yes | Yes | Yes |  |  | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes |  |  | Yes |  |  |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_1A-41C-42C | CA_1A-42A CA_42C | 1 |  |  | Yes | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 41 | See CA_41C_BW combination set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C_BW combination set 1 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_1A-42C-42C | CA_1A-42A | 1 |  |  | Yes | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_1A-42E | CA_1A-42A | 1 |  |  | Yes | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  | 70 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_3A-41C-42C | CA_42C, CA_3A-41A, CA_3A-42A, CA_41A-42A, CA_41C | 3 |  |  | Yes | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in TS36.101 Table 5.6A.1 |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 Table 5.6A.1 |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-41A-42C | CA_42C | 3 |  |  | Yes | Yes |  | Yes |  | Yes | 90 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 Table 5.6A.1 |  |  |  |  |  |  |  |  |  |
| CA_3A-42E | CA_3A-42A | 3 |  |  | Yes | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_3A-42C-42C | CA_3A-42A | 3 |  |  | Yes | Yes |  | Yes |  | Yes | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-42D | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |
| CA_28A-41C-42C | CA_42C | 28 |  |  | Yes | Yes |  |  |  |  | 90 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in TS36.101 Table 5.6A.1 |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 Table 5.6A.1 |  |  |  |  |  |  |  |  |  |
| CA_41C-48D | 41C | 41 | See CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |
| CA_41D-48C | 41C | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |
| CA_vA-wA-xA-yA-zA | CA_vA-wA or CA_vA-xA or CA_vA-yA or CA_vA-zA or CA_wA-xA or CA_wA-yA or CA_wA-zA or CA_xA-yA or CA_xA-zA or CA_yA-zA | V |  |  |  |  |  |  |  |  | Up to 100 | 0 |
|  |  | W |  |  |  |  |  |  |  |  |  |  |
|  |  | X |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  |  |  |  |  |  |  |
|  |  | Z |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: B41 and B42 are assumed to be synchronized (i.e. without simultaneous Rx/Tx) NOTE 2: For the corresponding CA configuration, UL transmission in this E-UTRA band is not allowed. |  |  |  |  |  |  |  |  |  |  |  |  |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[4]		RP-170946, “New WID on LTE Advanced inter-band CA Rel-15 for xDL/2UL with x=3,4,5”, LG Electronics”

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

Mixed Intra-band and inter-band carrier aggregation: Carrier aggregation of component carriers in same operating band and different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

4	Background

The present document is a technical report for LTE Advanced 2UL and xDL Carrier Aggregation under Rel-15 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs

5	xDLs/2UL Carrier Aggregation: General part

5.1	UE RF specific

5.1.1	UE general architecture

To derive MSD (Maximum Sensitivity Degradation), RAN4 consider xDL/1UL CA UE RF architectures including TDD-FDD CA based on the each CA band combinations in rel-15 for LTE only UE. For the NSA UE CA operation with NR band and LTE band, it is not scope of this xDL/2UL CA WI in rel-15.

Figure 5.1.1-1 UE example RF architecture using cascaded-Diplexer

Figure 5.1.1-1 is an example UE RF architecture using cascaded diplexer for xDL/2UL inter-band CA. RAN4 do not preclude dual Antenna architecture for xDL/2UL CA UE. However, the UE RF requirements could be derived in the single antenna UE RF architecture.

5.1.2	General treatment of ∆TIB and ∆RIB values

Following Table 5.1.2-1 is proposed the additional insertion loss for xDL/2UL CA band combinations. RAN4 should consider reuse the agreed additional insersion loss in xDL/1UL CA UE when the new RF components are not introduce for the xDL/2UL CA band combination. If else, the detail description will be captured in each CA band combination in specific part

Table 5.1.2-1: CA band combinations for xDL/2UL inter-band CA

| CA | DL Band Combination | UL Band Combination | Frequency | Additional ILs [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TIB,c |  | RIB,c |  |
| 3DL/2UL Inter-band CA | CA_1A-3A-3A | CA_1A-3A | 2.1G + 1.8G | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  | CA_1A-3A-26A | CA_1A-3A or CA_1A-26A or CA_3A-26A | 2.1G+1.8G+ 800M | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B26 | TBD | B26 | TBD |
|  | CA_1A-7A-26A | CA_1A-7A or CA_1A-26A | 2.1G+2.6G+800M | B1 | TBD | B1 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B26 | TBD | B26 | TBD |
|  | CA_1A-41A-42A | CA_1A-42A | 2.1G+2.5G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-42A-42A | CA_1A-42A | 2.1+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-3A-7A | CA_3A-7A | 1.8G+1.8G+2.6G | B3 | TBD | B3 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  | CA_3A-3A-19A | CA_3A-19A | 1.8G+800M | B3 | TBD | B3 | TBD |
|  |  |  |  | B19 | TBD | B19 | TBD |
|  | CA_3A-3A-21A | CA_3A-21A | 1.8G+1.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B21 | TBD | B21 | TBD |
|  | CA_3A-3A-42A | CA_3A-42A | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-7A-26A | CA_3A-7A or CA_3A-26A | 1.8G+2.6G+800M | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B26 | TBD | B26 | TBD |
|  | CA_3A-7A-32A | CA_3A-7A | 1.8G+2.6G+1.4G | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B32 | TBD | B32 | TBD |
|  | CA_3A-20A-32A | CA_3A-20A | 1.8G+800M+1.4G | B3 | TBD | B3 | TBD |
|  |  |  |  | B20 | TBD | B20 | TBD |
|  |  |  |  | B32 | TBD | B32 | TBD |
|  | CA_3A-28A-40A | CA_3A-28A | 1.8G+700M+2.3G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B40 | TBD | B40 | TBD |
|  | CA_3A-28A-41A | CA_3A-41A | 1.8G+700M+2.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  | CA_3A-41C | CA_3A-41A | 1.8G+2.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  | CA_3A-41A-42A | CA_3A-41A | 1..8G+2.5G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-42A-42A | CA_3A-42A | 1..8G+3.5G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_7A-7A-8A | CA_7A-8A | 2.6G+2.6G+900M | B7 | TBD | B7 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B8 | TBD | B8 | TBD |
|  | CA_7A-20A-32A | CA_7A-20A | 2.6G+800M+1.4G | B7 | TBD | B7 | TBD |
|  |  |  |  | B20 | TBD | B20 | TBD |
|  |  |  |  | B32 | TBD | B32 | TBD |
| 4DL/2UL inter-band CA | CA_1A-3A-3A-19A | CA_1A-3A or CA_1A-19A or CA_3A-19A | 2.1G+1.8G+800M | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B19 | TBD | B19 | TBD |
|  | CA_1A-3A-3A-21A | CA_1A-3A or CA_1A-21A or CA_3A-21A | 2.1G+1.8G+1.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B21 | TBD | B21 | TBD |
|  | CA_1A-3A-3A-42A | CA_1A-3A or CA_1A-42A or CA_3A-42A | 2.1G+1.8G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-3A-7A-26A | CA_1A-3A or CA_1A-7A or CA_1A-26A or CA_3A-7A or CA_3A-26A | 2.1G+1.8G+2.6G+800M | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B26 | TBD | B26 | TBD |
|  | CA_1A-7A-7A-26A | CA_1A-7A or CA_1A-26A |  | B1 | TBD | B1 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B26 | TBD | B26 | TBD |
|  | CA_1A-41A-42C | CA_1A-42A or CA_42C | 2.1G+2.5G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-41C-42A | CA_1A-42A | 2.1G+2.5G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-42A-42C | CA_1A-42A | 2.1+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-42D | CA_1A-42A | 2.1+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-3A-7A-7A | CA_3A-7A | 1.8G+1.8G+2.6G+2.6G | B3 | TBD | B3 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  | CA_3A-3A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 1.8G+2.6G+900M | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B8 | TBD | B8 | TBD |
|  | CA_3A-3A-19A-21A | CA_3A-19A or CA_3A-21A or CA_19A-21A | 1.8G+800M+1.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B19 | TBD | B19 | TBD |
|  |  |  |  | B21 | TBD | B21 | TBD |
|  | CA_3A-3A-42C | CA_3A-42A | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-7A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 1.8G+2.6G+2.6G+900M | B3 | TBD | B3 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B8 | TBD | B8 | TBD |
|  | CA_3A-7A-7A-26A | CA_3A-7A or CA_3A-26A | 1.8G+2.6G+800M | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B26 | TBD | B26 | TBD |
|  | CA_3A-7A-20A-32A | CA_3A-7A or CA_3A-20A or CA_7A-20A | 1.8G+2.6G+800M+1.4G | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B20 | TBD | B20 | TBD |
|  |  |  |  | B32 | TBD | B32 | TBD |
|  | CA_3A-28A-40C | CA_3A-28A | 1.8G+700M+2.3G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B40 | TBD | B40 | TBD |
|  | CA_3A-28A-41A-42A | CA_3A-41A | 1.8G+700M+2.5G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-28A-41C | CA_3A-41A | 1.8G+700M+2.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  | CA_3A-41C-42A | CA_3A-41A | 1.8G+2.5G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-41A-42C | CA_3A-41A | 1.8G+2.5G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-42A-42C | CA_3A-42A | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-42D | CA_3A-42A | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
| 5DL/2UL inter-band CA | CA_1A-3A-3A-19A-21A | CA_1A-3A or CA_1A-19A or CA_1A-21A or CA_3A-19A or CA_3A-21A or CA_19A-21A | 2.1G+1.8G+800M+1.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B19 | TBD | B19 | TBD |
|  |  |  |  | B21 | TBD | B21 | TBD |
|  | CA_1A-3A-3A-42C | CA_1A-3A or CA_1A-42A or CA_3A-42A_ | 2.1G+1.8G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-41C-42C | CA_1A-42A or, CA_42C | 2.1G+2.5G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-42C-42C | CA_1A-42A | 2.1G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_1A-42E | CA_1A-42A | 2.1G+3.5G | B1 | TBD | B1 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-3A-7A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 1.8G+2.6G+900M | B3 | TBD | B3 | TBD |
|  |  |  |  | B7 | TBD | B7 | TBD |
|  |  |  |  | B8 | TBD | B8 | TBD |
|  | CA_3A-41C-42C | CA_42C | 1.8G+2.5G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-28A-41A-42C | CA_42C | 1.8G+700M+2.5G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-42E | CA_3A-42A | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-42C-42C | CA_3A-42A | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_3A-3A-42D | CA_3A-42A | 1.8G+3.5G | B3 | TBD | B3 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |
|  | CA_28A-41C-42C | CA_42C | 700M+2.5G+3.5G | B28 | TBD | B28 | TBD |
|  |  |  |  | B41 | TBD | B41 | TBD |
|  |  |  |  | B42 | TBD | B42 | TBD |

5.1.3	Fallback CA mode and mandatory support of all paired 2UL CA configurations

RAN4 define the Fallback CA mode by agreed Way-forward (R4-168853) in rel-14.

5.1.4	General TX/RX requirements

5.1.4.1 Transmitter requirements

Dual uplink Transmitter requirements already analysed in TR36.860 for dual uplink inter-band CA. The xDL/2UL CA UE follow the legacy UE transmitter requirements in Table 5.1.4.1-1.

Table 5.1.4.1-1: Common UE RF Tx requirements for dual uplink inter-band CA configuration

| Section / Clause in TS36.101 | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.7.1A | Transmit intermodulation for CA |

5.1.4.2 Receiver requirements

Dual uplink receiver requirements already analysed in TR36.860 for dual uplink inter-band CA according to the self-interference or not.

Additional UE RF receiver requirements for xDL/2UL CA UE are listed in the Table 5.1.4.2-1.

The xDL/2UL CA band combinations without self-interference issues can follow the legacy UE Rx requirements in xDL/1UL CA RF Rx requirements. Since RAN4 already concluded that test coverage of Out-of-band blocking and spurious response for 2DL/2UL are covered by 1UL test (i.e. single carrier LTE and 1UL/2DL CA) [5]. Hence only REFSENS test for xDL/2UL CA UE perform with all downlink component carriers active and both of the uplink carriers active.

Furthermore, RAN4 should define the new maximum sensitivity degradation (MSD) for REFSENS requirements of each CA band combinations when the xDL/2UL CA band combinations has self-interference problems.

Table 5.1.4.2-1: Common UE RF Rx requirements for dual uplink inter-band CA configuration

| Section / Clause in TS36.101 | Description | Comments |
| --- | --- | --- |
| 7.3.1A | Reference sensitivity for CA | Applied to all xDL/2UL CA band combinations |
| 7.6.2.1A | Out-of-band blocking for CA | No need |
| 7.7.1A | Spurious response for CA | No need |

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes the CA band combinations with self-interference problems for CA_3DL/2UL CA.

Table 5.1.5-1: Summary of Self-interference analysis for 3DL/2UL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd band without uplink | intermodulation to 3rd band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-41A-42A | CA_1A-42A | - | 4th IMD | - | Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
|  | CA_41A-42A | - | 4th IMD | - | One critical point is that CA_1A-41A only support 1 uplink with Band 1 due to 41 transmission will be impacted to Band 1 reception in UE. So this uplink CA is not supported. |
| CA_3A-3A-7A | CA_3A-7A | - | - | - | N/A |
| CA_3A-7A-32A | CA_3A-7A | - | 4th IMD | - | 10.5 dB |
| CA_3A-20A-32A | CA_3A-20A | - | - | - | N/A |
| CA_3A-28A-40A | CA_3A-28A | - | - | - | N/A |
| CA_3A-28A-41A | CA_3A-41A | - | 2nd , 3rd IMDs | - | 26.1dB by 2nd IMD 15.8dB by 3rd IMD |
| CA_3A-41C | CA_3A-41A | - | 4th IMD | - | This impact already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101 |
|  | CA_41C | - | - | - | N/A |
| CA_3A-41A-42A | CA_3A-41A | - | 3rd IMD | - | Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
| CA_7A-7A-8A | CA_7A-8A | 3rd harmonic from B8 fall into B7 | - | - | Harmonic issue already covered in 2DL/1UL CA_7A-8A |
| CA_7A-20A-32A | CA_7A-20A | - | - | - | N/A |
| CA_1A-3A-3A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
| CA_1A-3A-26A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-26A | - | - | - | N/A |
|  | CA_3A-26A | - | - | - | N/A |
| CA_1A-7A-26A | CA_1A-7A | - | 5th IMD | - | 3.3 dB |
|  | CA_1A-26A | - | - | - | N/A |
|  | CA_7A-26A | - | - | - | N/A |
| CA_1A-42A-42A | CA_1A-42A | - | - | - | N/A |
| CA_3A-3A-19A | CA_3A-19A | - | 4th IMD |  | Only consider actual spectrum holdings in Japan for B19. The 4th IMD already covered in 2DL/2UL CA_3A-19A in Table 7.3.1A-0f in TS36.101 |
| CA_3A-3A-21A | CA_3A-21A | - | - | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_3A-3A-42A | CA_3A-42A |  | 2nd & 4th IMDs |  | The 2nd & 4th IMDs already covered in 2DL/2UL CA_3A-19A in Table 7.3.1A-0f in TS36.101 |
| CA_3A-7A-26A | CA_3A-7A |  | 2nd & 3rd IMDs |  | - 9.5 dB by 2nd IMD - 17.4 dB by 3rd IMD |
|  | CA_3A-26A |  | 2nd & 3rd IMDs |  | - 28.8 dB by 2nd IMD - 16.5 dB by 3rd IMD |
|  | CA_7A-26A |  | - | - | - N/A |
| CA_3A-42A-42A | CA_3A-42A | B3 2nd harmonics into B42 | 4th& 5th IMDs |  | 2nd harmonics issue already covered in 2DL/1UL CA_3A-42A. No simultaneous Tx/Rx operation in Band 42. |
| CA_1A-3A-18A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-18A | - | - | - | N/A |
|  | CA_3A-18A | - | - | - | N/A |
| CA_1A-3A-20A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-20A | - | - | - | N/A |
|  | CA_3A-20A | - | - | - | N/A |
| CA_1A-3A-41A | CA_1A-3A | - | 5th IMD | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 - 5.0 dB |
| CA_1A-7C | CA_1A-7A | - | - | - | N/A |
|  | CA_7C | - | - | - | N/A |
| CA_1A-7A-28A | CA_1A-7A | - | 5th IMD | - | - 4.5 dB |
|  | CA_1A-28A | - | 2nd IMD | - | - 28.0 dB |
|  | CA_7A-28A | 3rd harmonic | - | - | Harmonic problem already covered in 2DL/1UL CA_1A-28A. |
| CA_3A-7A-28A | CA_3A-28A | - | 3rd IMD | - | - 17.0 dB |
| CA_3A-11A-18A | CA_3A-11A | - | 5th IMD | - | - 5.0 dB |
|  | CA_3A-18A | - | 5th IMD | - | Consider the specific holding spectrum, not necessary to analysis the IMD problem |
|  | CA_11A-18A | - | 5th IMD | - | - 4.0 dB |
| CA_3A-11A-26A | CA_3A-11A | - | 5th IMD | - | - 5.0 dB |
|  | CA_3A-26A | - | 5th IMD | - | - 5.0 dB |
|  | CA_11A-26A | - | 5th IMD | - | - 4.5 dB |
| CA_7A-7A-26A | CA_7A-26A | - | 5th IMD | - | The IMD problem already covered in 2DL/2UL_CA_7A-26A |
| CA_40C-42A | CA_40C | - | - | - | N/A |
| CA_41C-48A | CA_41C | - | - | - | N/A |

Table 5.1.5-2 summarizes the CA_ band combinations with self-interference problems for CA_4DL/2UL CA.

Table 5.1.5-2: Summary of Self-interference analysis for 4DL/2UL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd and 4th band without uplink | intermodulation to 3rd and 4th band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-41A-42C | CA_1A-42A |  | 4th IMD into B41 |  | Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
|  | CA_41A-42A |  | 4th IMD into B1 |  | One critical point is that CA_1A-41A only support 1 uplink with Band 1 due to 41 transmission will be impacted to Band 1 reception in UE. So this uplink CA is not supported. |
|  | CA_42C |  |  |  | N/A |
| CA_1A-41C-42A | CA_1A-42A |  | 4th IMD into B41 |  | Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
|  | CA_41A-42A |  | 4th IMD into B1 |  | One critical point is that CA_1A-41A only support 1 uplink with Band 1 due to 41 transmission will be impacted to Band 1 reception in UE. So this uplink CA is not supported. |
|  | CA_41C |  | - |  | One critical point is that CA_1A-41C only support 1 uplink with Band 1 due to 41 transmission will be impacted to Band 1 reception in UE. So this uplink CA is not supported. |
| CA_3A-3A-7A-7A | CA_3A-7A | - | - | - | N/A |
| CA_3A-7A-7A-8A | CA_3A-7A | - | 3rd IMD into B8 4th IMD into B7 | - | 3rd IMD already covered in 3DL/2UL CA_3A-7A-8A 4th IMD alredy covered in 2DL/2UL CA_3A-7A |
|  | CA_3A-8A | 3rd harmonic from B8 fall into B7 | 2nd IMD into B7 3rd IMD into B7 |  | Harmonic issue already covered in 2DL/1UL CA_7A-8A 2nd & 3rd IMD issues already covered in 3DL/2UL CA_3A-7A-8A |
|  | CA_7A-8A | 2nd harmonic from B8 fall into B3 3rd harmonic from B8 fall into B7 |  |  | 2nd harmonic issus already covered in 2DL/1UL CA_3A-8A 3rd harmonic issus already covered in 2DL/1UL CA_7A-8A |
| CA_3A-7A-20A-32A | CA_3A-7A | - | 2nd IMD into B20 4th IMD into B32 | - | 2nd IMD already covered in 3DL/2UL CA_3A-7A-20A 4th IMD will be covered in 3DL/2UL CA_3A-7A-32A |
|  | CA_3A-20A | - | 2nd & 3rd IMDs into B7 | - | 2nd & 3rd IMDs already covered in 3DL/2UL CA_3A-7A-20A |
|  | CA_7A-20A | - | - | - | N/A |
| CA_3A-28A-40C | CA_3A-28A | - | - | - | N/A |
| CA_3A-28A-41A-42A | CA_3A-41A | 2nd harmonic from B3 fall into B42 | 2nd , 3rd IMDs fall into B28 3rd IMD fall into B42 | - | 2nd harmonic issus already covered in 2DL/1UL CA_3A-42A 2nd & 3rd IMDs will be covered in 3DL/2UL CA_3A-28A-41A Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
| CA_3A-28A-41C | CA_3A-41A |  | 2nd , 3rd IMDs fall into B28 | - | 2nd & 3rd IMDs will be covered in 3DL/2UL CA_3A-28A-41A |
| CA_3A-41C-42A | CA_3A-41A |  | 4th IMD fall into B41 3rd IMD fall into B42 | - | 4th IMD issue already covered in 2DL/2UL CA_3A-41A Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
| CA_3A-41A-42C | CA_3A-41A |  | 3rd IMD fall into B42 |  | Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
| CA_1A-3A-3A-19A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-19A | - | - | - | N/A |
|  | CA_3A-19A | - | - | - | N/A |
| CA_1A-3A-3A-21A | CA_1A-3A | - | 3rd & 5th IMDs into B21 | Yes | - These IMD problems already covered in 3DL_CA_1A-3A-21A 2UL_CA_1A-3A. - Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-21A | - | - | - | N/A |
|  | CA_3A-21A | - | 3rd IMD into B1 | - | - These IMD problems already covered in 3DL_CA_1A-3A-21A 2UL_CA_1A-3A. |
| CA_1A-3A-3A-42A | CA_1A-3A | B3 2nd harmonics into B42 | 4th IMD into B42 | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - The 4th IMD issue already covered in 3DL_CA_1A-3A-42A 2UL_CA_1A-3A. - Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | - | - | - | N/A |
| CA_1A-3A-7A-26A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-7A | - | 5th IMD into B26 | - | - The 5th IMD issue will be covered in 3DL_CA_1A-7A-26A 2UL_CA_1A-7A. |
|  | CA_1A-26A | - | - | - | N/A |
|  | CA_3A-7A | - | 2nd & 3rd IMDs into B26 | - | - The 2nd & 3rd IMDs issue will be covered in 3DL_CA_3A-7A-26A 2UL_CA_3A-7A. |
|  | CA_3A-26A | - | 2nd & 3rd IMDs into B7 |  | - The 2nd & 3rd IMDs issue will be covered in 3DL_CA_3A-7A-26A 2UL_CA_3A-26A. |
|  | CA_7A-26A | - | - | - | N/A |
| CA_1A-7A-7A-26A | CA_1A-7A | - | 5th IMD into B26 | - | - The 5th IMD issue will be covered in 3DL_CA_1A-7A-26A 2UL_CA_1A-7A. |
|  | CA_1A-26A | - | - | - | N/A |
|  | CA_7A-26A | - | 5th IMD into B7 | - | The IMD problem already covered in 2DL/2UL CA_7A-26A |
| CA_1A-42A-42C | CA-1A-42A | - | 4th IMD into B42 | - | - The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_1A-42D | CA_1A-42A | - | 4th IMD into B42 | - | - The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_3A-3A-7A-8A | CA_3A-7A | - | 3rd IMD into B8 | - | - The 3rd IMD issue already covered in 3DL_CA_3A-7A-8A 2UL_CA_3A-7A. |
|  | CA_3A-8A | B8 3rd harmonic into B7 B8 2nd harmonic into B3 | 2nd & 3rd IMDs into B7 4th & 5th IMDs into B3 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-8A and 2DL/1UL CA_7A-8A. - The 2nd & 3rd IMDs issues are already covered in 3DL_CA_3A-7A-8A_2UL_CA_3A-8A - The 4th & 5th IMDs issues are already covered in 2DL/2UL_CA_3A-8A |
|  | CA_7A-8A | B8 2nd harmonics into B3 | - | - | - Harmonics issues already completed in 2DL/1UL CA_3A-8A. |
| CA_3A-3A-19A-21A | CA_3A-19A | - | 4th IMD into B3 | - | - The 4th IMD issue is already covered in 2DL/2UL_CA_3A-19A |
|  | CA_3A-21A | - | 5th IMD into B19 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-21A | - | 5th IMD into B3 | - | - The 5th IMD issues are already covered in 3DL_CA_3A-19A-21A_2UL_CA_19A-21A |
| CA_3A-3A-42C | CA_3A-42A | B3 2nd harmonic into B42 | 2nd & 4th IMDs into B3 4th & 5th IMDs into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - These all IMD problems are already covered in 2DL/2UL_CA_3A-42A |
| CA_3A-7A-7A-26A | CA_3A-7A | - | 2nd & 3rd IMDs into B26 | - | - These 2nd & 3rd IMDs problems will be covered in 3DL_CA_3A-7A-26A_2UL_CA_3A-7A |
|  | CA_3A-26A | - | 2nd & 3rd IMDs into B7 | - | - These 2nd & 3rd IMDs problems will be covered in 3DL_CA_3A-7A-26A_2UL_CA_3A-26A |
|  | CA_7A-26A | - | 5th IMD into B7 | - | The IMD problem already covered in 2DL/2UL CA_7A-26A |
| CA_3A-42A-42C | CA-3A-42A | B3 2nd harmonic into B42 | 4th & 5th IMDs into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_3A-42D | CA_3A-42A | B3 2nd harmonic into B42 | 4th & 5th IMDs into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_1A-3A-7C | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-7A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |
|  | CA_7C | - | - | - | N/A |
| CA_1A-3C-7A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-7A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |
|  | CA_3C | - | - | - | N/A |
| CA_1A-3A-7A-28A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-7A | - | 5th IMD into B28 | - | The IMD problem will be covered in 3DL_CA_1A-7A-28A w/ 2UL_CA_1A-7A |
|  | CA_1A-28A | - | 2nd IMD into B7 5th IMD into B3 | - | 2nd IMD problem will be covered in 3DL_1A-7A-28A w/ 2UL_CA_1A-28A 5th IMD problem already covered in 3DL_1A-3A-28A w/ 2UL_CA_1A-28A |
|  | CA_3A-7A | - | 2nd IMD into B28 | - | 2nd IMD problem already covered in 3DL_3A-7A-28A w/ 2UL_CA_3A-7A |
|  | CA_3A-28A | B28 3rd harmonics into B1 | 3rd IMD into B7 4th IMD into B1 | - | Harmonic problem already covered in 2DL/1UL_CA_1A-28A 3rd IMD problem already covered in 3DL_3A-7A-28A w/ 2UL_CA_3A-28A 4th IMD problem already covered in 3DL_1A-3A-28A w/ 2UL_CA_3A-28A |
|  | CA-7A-28A | B28 3rd harmonics into B1 | 2nd IMD into B3 | - | Harmonic problem already covered in 2DL/1UL_CA_1A-28A 2nd IMD problem already covered in 3DL CA_3A-7A-28A w/ 2UL CA_7A-28A |
| CA_1A-3A-41C | CA_1A-3A | - | 5th IMD into B41 | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 5th IMD problem will be covered in 3DL_CA_1A-3A-41A w/ 2UL_CA_1A-3A |
| CA_1A-7C-28A | CA_1A-7A | - | 5th IMD into B28 | - | 5th IMD problem will be covered in 3DL_CA_1A-7A-28A w/ 2UL_CA_1A-7A |
|  | CA_1A-28A | - | 2nd IMD into B7 | - | 2nd IMD problem will be covered in 3DL_CA_1A-7A-28A w/ 2UL_CA_1A-28A |
|  | CA_7A-28A | B28 3rd harmonic into B1 | - | - | Harmonic problem already covered in 2DL/1UL_CA_1A-28A |
|  | CA_7C | - | - | - | N/A |
| CA_3C-7C | CA_3C | - | - | - | N/A |
|  | CA_7C | - | - | - | N/A |
| CA_3A-41D | CA_3A-41A | - | 4th IMD | - | This impact already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101 |
|  | CA_41C | - | 3rd & 5th IMD into B41 | - | No impact for TDD operation in B41 |
| CA_40C-42C | CA_40C | - | - | - | N/A |
|  | CA_42C | - | - | - | N/A |
| CA_41C-48C | CA_41C | - | - | - | N/A |
| CA_41D-48A | CA_41C | - | 3rd & 5th IMD into B41 | - | No impact for TDD operation in B41 |

Table 5.1.5-3 summarizes the CA_ band combinations with self-interference problems for CA_5DL/2UL CA.

Table 5.1.5-3: Summary of Self-interference analysis for 5DL/2UL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd, 4th and 5th band without uplink | intermodulation to 3rd, 4th and 5th band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-41C-42C | CA_1A-42A | - | 4th IMD into B41 |  | Not necessary to study for the MSD by 4th IMD since B41 and B42 will be operated with synchronized |
|  | CA_41A-42A | - | 4th IMD into B1 | - | One critical point is that CA_1A-41A only support 1 uplink with Band 1 due to 41 transmission will be impacted to Band 1 reception in UE. So this uplink CA is not supported. |
|  | CA_41C |  |  |  | One critical point is that CA_1A-41C only support 1 uplink with Band 1 due to 41 transmission will be impacted to Band 1 reception in UE. So this uplink CA is not supported. |
|  | CA_42C |  |  |  | N/A |
| CA_1A-3A-3A-19A-21A | CA_1A-3A | - | 3rd & 5th IMDs into B21 | Yes | - No MSD due to IMD to be specified according to actural spectrum holdings. - Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-19A | - | - | - | N/A |
|  | CA_1A-21A | - | - | - | N/A |
|  | CA_3A-19A | - | - | - | N/A |
|  | CA_3A-21A | - | 3rd IMD into B1 5th IMD into B19 | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-21A |  | 5th IMD into B3 | - | - The IMD issue was covered in 3DL CA_3A-19A-21A 2UL CA_19A-21A |
| CA_1A-3A-3A-42C | CA_1A-3A | B3 2nd harmonics into B42 | 4th IMD into B42 | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. -The IMD issue was covered 3DL CA_1A-3A-42A with 2UL CA_1A-3A. - Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | B3 2nd harmonics into B42 | 4th & 5th IMDs into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. -The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_1A-42C-42C | CA-1A-42A | - | 4th IMDs into B42 | - | -The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_1A-42E | CA_1A-42A | - | 4th IMDs into B42 | - | -The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_3A-3A-7A-7A-8A | CA_3A-7A | - | 3rd IMD into B8 | - | -The IMD issue was covered 3DL CA_3A-7A-8A with 2UL CA_3A-7A. |
|  | CA_3A-8A | B8 3rd harmonic into B7 | 2nd & 3rd IMDs into B7 | - | - Harmonics issues already completed in 2DL/1UL CA_7A-8A. -The IMD issue were covered 3DL CA_3A-7A-8A with 2UL CA_3A-8A. |
|  | CA_7A-8A | B8 2nd harmonics into B3 | - | - | - Harmonics issues already completed in 2DL/1UL CA_3A-8A. |
| CA_3A-28A-41A-42C | CA_42C | - | - | - | N/A |
| CA_3A-41C-42C | CA_42C | - | - | - | N/A |
| CA_3A-42E | CA-3A-42A | B3 2nd harmonic into B42 | 4th & 5th IMDs into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_3A-42C-42C | CA_3A-42A | B3 2nd harmonic into B42 | 4th & 5th IMDs into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_3A-3A-42D | CA_3A-42A | B3 2nd harmonic into B42 | 4th & 5th IMDs into B42 | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. - The IMD issue were not impact into B42 since simultaneous Tx/Rx is not allowed in B42. |
| CA_28A-41C-42C | CA_42C | - | - | - | N/A |
| CA_1A-3A-7A-7A-26A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-7A | - | 5th IMD into B26 | - | - The 5th IMD issue will be covered in 3DL_CA_1A-7A-26A 2UL_CA_1A-7A. |
|  | CA_1A-26A | - | - | - | N/A |
|  | CA_3A-7A | - | 2nd & 3rd IMDs into B26 | - | - The 2nd & 3rd IMDs issue will be covered in 3DL_CA_3A-7A-26A 2UL_CA_3A-7A. |
|  | CA_3A-26A | - | 2nd & 3rd IMDs into B7 |  | - The 2nd & 3rd IMDs issue will be covered in 3DL_CA_3A-7A-26A 2UL_CA_3A-26A. |
|  | CA_7A-26A | - | - | - | N/A |
| CA_1A-3C-7C | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-7A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |
|  | CA_3C | - | - | - | N/A |
|  | CA_7C | - | - | - | N/A |
| CA_1A-3A-7C-28A | CA_1A-3A | - | - | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 |
|  | CA_1A-7A | - | 5th IMD into B28 | - | The IMD problem will be covered in 3DL_CA_1A-7A-28A w/ 2UL_CA_1A-7A |
|  | CA_1A-28A | - | 2nd IMD into B7 5th IMD into B3 | - | 2nd IMD problem will be covered in 3DL_1A-7A-28A w/ 2UL_CA_1A-28A 5th IMD problem already covered in 3DL_1A-3A-28A w/ 2UL_CA_1A-28A |
|  | CA_3A-7A | - | 2nd IMD into B28 | - | 2nd IMD problem already covered in 3DL_3A-7A-28A w/ 2UL_CA_3A-7A |
|  | CA_3A-28A | B28 3rd harmonics into B1 | 3rd IMD into B7 4th IMD into B1 | - | Harmonic problem already covered in 2DL/1UL_CA_1A-28A 3rd IMD problem already covered in 3DL_3A-7A-28A w/ 2UL_CA_3A-28A 4th IMD problem already covered in 3DL_1A-3A-28A w/ 2UL_CA_3A-28A |
|  | CA_7A-28A | B28 3rd harmonics into B1 | 2nd IMD into B3 | - | Harmonic problem already covered in 2DL/1UL_CA_1A-28A 2nd IMD problem already covered in 3DL CA_3A-7A-28A w/ 2UL CA_7A-28A |
|  | CA_7C | - | - | - | N/A |
| CA_1A-3A-41D | CA_1A-3A | - | 5th IMD into B41 | Yes | Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101 5th IMD problem will be covered in 3DL_CA_1A-3A-41A w/ 2UL_CA_1A-3A |
| CA_41C-48D | CA_41C | - | - | - | N/A |
| CA_41D-48C | CA_41C | - | 3rd & 5th IMD into B41 | - | No impact for TDD operation in B41 |

Based on the MSD test configuration as summarized in Table 5.1.5.1, 5.1.5.2 & 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3DL/2UL inter-band CA with IMD problems.

Table 5.1.5.4 Proposed 3DL/2UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration |  | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) |  | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_3A-7A-32A | CA_3A-7A |  | 3 | 1775 | 5 | 25 | 1870 |  | 5 | N/A | FDD | IMD4 |
|  |  |  | 7 | 2510 | 10 | 50 | 2630 |  | 10 |  |  |  |
|  |  |  | 32 | - | - | - | 1470 |  | 5 | 10.5 |  |  |
| CA_3A-28A-41A | CA_3A-41A |  | 3 | 1720 | 5 | 25 | 1815 |  | 5 | N/A | TDD-FDD | IMD2 |
|  |  |  | 41 | 2510 | 5 | 25 | 2510 |  | 5 |  |  |  |
|  |  |  | 28 | 735 | 5 | 25 | 790 |  | 5 | 26.1 |  |  |
|  | CA_3A-41A |  | 3 | 1720 | 5 | 25 | 1815 |  | 5 | N/A | TDD-FDD | IMD3 |
|  |  |  | 41 | 2660 | 5 | 25 | 2660 |  | 5 |  |  |  |
|  |  |  | 28 | 725 | 5 | 25 | 780 |  | 5 | 15.8 |  |  |
| CA_1A-7A-26A | CA_1A-7A |  | 1 | 1965 | 5 | 25 | 2155 |  | 5 | N/A | FDD | IMD5 |
|  |  |  | 7 | 2510 | 10 | 50 | 2630 |  | 10 |  |  |  |
|  |  |  | 26 | 830 | 5 | 25 | 875 |  | 5 | 3.3 |  |  |
| CA_3A-7A-26A | CA_3A-7A |  | 3 | 1712.5 | 5 | 25 | 1807.5 |  | 5 | N/A | FDD | IMD2 |
|  |  |  | 7 | 2565 | 10 | 50 | 2685 |  | 10 |  |  |  |
|  |  |  | 26 | 816.5 | 5 | 25 | 861.5 |  | 5 | 9.5 |  |  |
|  | CA_3A-7A |  | 3 | 1720 | 5 | 25 | 1815 |  | 5 | N/A | FDD | IMD3 |
|  |  |  | 7 | 2560 | 10 | 50 | 2680 |  | 10 |  |  |  |
|  |  |  | 26 | 835 | 5 | 25 | 880 |  | 5 | 17.4 |  |  |
|  | CA_3A-26A |  | 3 | 1780 | 5 | 25 | 1875 |  | 5 | N/A | FDD | IMD2 |
|  |  |  | 26 | 845 | 5 | 25 | 890 |  | 5 |  |  |  |
|  |  |  | 7 | 2505 | 10 | 50 | 2625 |  | 10 | 28.8 |  |  |
|  | CA_3A-26A |  | 3 | 1750 | 5 | 25 | 1845 |  | 5 | N/A | FDD | IMD3 |
|  |  |  | 26 | 835 | 5 | 25 | 880 |  | 5 |  |  |  |
|  |  |  | 7 | 2545 | 10 | 50 | 2665 |  | 10 | 16.5 |  |  |
| CA_1A-3A-41A |  | CA_1A-3A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 |  | N/A | FDD | IMD5 |  |
|  |  |  | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 |  |  |  |  |  |
|  |  |  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5 |  | 5.0 | TDD |  |  |
| CA_1A-7A-28A |  | CA_1A-7A | 1 | 1935 | 5 | 25 | 2125 | 5 |  | N/A | FDD | IMD5 |  |
|  |  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |  |  |  |
|  |  |  | 28 | 730 | 10 | 50 | 785 | 10 |  | 4.5 |  |  |  |
|  |  | CA_1A-28A | 1 | 1935 | 5 | 25 | 2125 | 5 |  | N/A | FDD | IMD2 |  |
|  |  |  | 28 | 730 | 10 | 50 | 785 | 10 |  |  |  |  |  |
|  |  |  | 7 | 2545 | 10 | 50 | 2665 | 10 |  | 28.0 |  |  |  |
| CA_3A-7A-28A |  | CA_3A-28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 |  | N/A | FDD | IMD3 |  |
|  |  |  | 28 | 743 | 5 | 25 | 798 | 5 |  |  |  |  |  |
|  |  |  | 7 | 2562 | 5 | 25 | 2682 | 5 |  | 17.0 |  |  |  |
| CA_3A-11A-18A |  | CA_3A-11A | 3 | 1725 | 5 | 25 | 1820 | 5 |  | N/A | FDD | IMD5 |  |
|  |  |  | 11 | 1440 | 5 | 25 | 1448 | 5 |  |  |  |  |  |
|  |  |  | 18 | 825 | 5 | 25 | 870 | 5 |  | 5.0 |  |  |  |
|  |  | CA_11A-18A | 11 | 1432 | 5 | 25 | 1481 | 5 |  | N/A | FDD | IMD5 |  |
|  |  |  | 18 | 820 | 5 | 25 | 865 | 5 |  |  |  |  |  |
|  |  |  | 3 | 1753 | 5 | 25 | 1848 | 5 |  | 4.0 |  |  |  |
| CA_3A-11A-26A |  | CA_3A-11A | 3 | 1720 | 5 | 25 | 1815 | 5 |  | N/A | FDD | IMD5 |  |
|  |  |  | 11 | 1438 | 5 | 25 | 1486 | 5 |  |  |  |  |  |
|  |  |  | 26 | 829 | 5 | 25 | 874 | 5 |  | 5.0 |  |  |  |
|  |  | CA_3A-26A | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 |  | N/A | FDD | IMD5 |  |
|  |  |  | 26 | 816.5 | 5 | 25 | 861.5 | 5 |  |  |  |  |  |
|  |  |  | 11 | 1435.5 | 5 | 25 | 1483.5 | 5 |  | 5.0 |  |  |  |
|  |  | CA_11A-26A | 11 | 1440 | 5 | 25 | 1488 | 5 |  | N/A | FDD | IMD5 |  |
|  |  |  | 26 | 824 | 5 | 25 | 869 | 5 |  |  |  |  |  |
|  |  |  | 3 | 1761 | 5 | 25 | 1856 | 5 |  | 4.5 |  |  |  |

5.2	RRM specific

[Editor Note] It will be updated in future

6	3DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

6.1	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 32 with 2 ULs

6.1.1	List of specific combination issues

6.1.1.1	Channel bandwidths per operating band for CA

Table 6.1.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-7A-32A | CA_3A-7A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

6.1.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-32A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.1.2-1

Table 6.1.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits (MHz) | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3420 | 3570 | 5000 | 5140 |
|  | 3420 to 3570 |  | 5000 to 5140 |  |
| 3rd harmonics frequency limits (MHz) | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5130 | 5355 | 7500 | 7710 |
|  | 5130 to 5355 |  | 7500 to 7710 |  |
| 2nd order IMD products and frequency limits (MHz) | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 715 | 860 | 4210 | 4355 |
|  | 715 to 860 |  | 4210 to 4355 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 850 | 1070 | 3215 | 3430 |
|  | 850 to 1070 |  | 3215 to 3430 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 5920 | 6140 | 6710 | 6925 |
|  | 5920 to 6140 |  | 6710 to 6925 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – fy_high + fy_low¦ | ¦fx_high + fy_high – fy_low¦ | ¦fy_low – fx_high + fx_low¦ | ¦fy_high + fx_high – fx_low¦ |
|  | 1640 | 1855 | 2425 | 2645 |
|  | 1640 to 1855 |  | 2425 to 2645 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
|  | 1690 | 1805 | 2480 | 2590 |
|  | 1690 to 1805 |  | 2480 to 2590 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
|  | 2560 | 2855 | 5715 | 6000 |
|  | 2560 to 2855 |  | 5715 to 6000 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 7630 | 7925 | 9210 | 9495 |
|  | 7630 to 7925 |  | 9210 to 9495 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fy_low - 2*fx_high¦ | ¦2*fy_high - 2*fx_low¦ |
|  | 1720 | 1430 | 1430 | 1720 |
|  | 1430 to 1720 |  | 1430 to 1720 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ | ¦2*fy_low + 2*fx_low¦ | ¦2*fy_high + 2*fx_high¦ |
|  | 8420 | 8710 | 8420 | 8710 |
|  | 8420 to 8710 |  | 8420 to 8710 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 8570 | 8215 | 4640 | 4270 |
|  | 8215 to 8570 |  | 4270 to 4640 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 11710 | 12065 | 9340 | 9710 |
|  | 11710 to 12065 |  | 9340 to 9710 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 4290 | 3930 | 355 | 10 |
|  | 3930 to 4290 |  | 10 to 355 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 10920 | 11280 | 10130 | 10495 |
|  | 10920 to 11280 |  | 10130 to 10495 |  |

Based on Table 6.1.1.2-1, the 4th order IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 32; the related MSD study will be covered in clause 6.1.1.3.

6.1.1.3	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7A-32A there are self-interference problems from 2 uplink operation which would interfere the downlink of the Band 32.

Based on the agreed self-desense analysis paper and related proposed contributions in R4-1710486, R4-1710627 and R4-1710736, the merged MSD level was provided from interested companies as below Table 6.1.1.3-1.

Table 6.1.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Qualcomm | Huawei | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B32 | B3 | IMD4 | ¦2*fB7 -2*fB3¦ | 8.7 | 8.63 | 13.5 | 10.5 |
|  | B7 |  |  |  |  |  |  |

6.1.1.4	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_3A-7A-32A are applied.

6.2	LTE Advanced Carrier Aggregation: Band 3 and Band 20 and Band 32 with 2 ULs

6.2.1	List of specific combination issues

6.2.1.1	Channel bandwidths per operating band for CA

Table 6.2.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-20A-32A | CA_3A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

6.2.1.2	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_3A-20A-32A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.2.1.2-1

Table 6.2.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 832 | 862 |
| 2nd harmonics frequency limits (MHz) | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3420 | 3570 | 1664 | 1724 |
|  | 3420 to 3570 |  | 1664 to 1724 |  |
| 3rd harmonics frequency limits (MHz) | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5130 | 5355 | 2496 | 2586 |
|  | 5130 to 5355 |  | 2496 to 2586 |  |
| 2nd order IMD products and frequency limits (MHz) | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 953 | 848 | 2542 | 2647 |
|  | 848 to 953 |  | 2542 to 2647 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 2558 | 2738 | 121 | 14 |
|  | 2558 to 2738 |  | 14 to 121 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 4252 | 4432 | 3374 | 3509 |
|  | 4252 to 4432 |  | 3374 to 3509 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – fy_high + fy_low¦ | ¦fx_high + fy_high – fy_low¦ | ¦fy_low – fx_high + fx_low¦ | ¦fy_high + fx_high – fx_low¦ |
|  | 1680 | 1815 | 757 | 937 |
|  | 1680 to 1815 |  | 757 to 937 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
|  | 1690 | 1805 | 812 | 882 |
|  | 1690 to 1805 |  | 812 to 882 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
|  | 4268 | 4523 | 711 | 876 |
|  | 4268 to 4523 |  | 711 to 876 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 5962 | 6217 | 4206 | 4371 |
|  | 5962 to 6217 |  | 4206 to 4371 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fy_low - 2*fx_high¦ | ¦2*fy_high - 2*fx_low¦ |
|  | 1696 | 1906 | 1906 | 1696 |
|  | 1696 to 1906 |  | 1696 to 1906 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ | ¦2*fy_low + 2*fx_low¦ | ¦2*fy_high + 2*fx_high¦ |
|  | 5084 | 5294 | 5084 | 5294 |
|  | 5084 to 5294 |  | 5084 to 5294 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 1738 | 1543 | 6308 | 5978 |
|  | 1543 to 1738 |  | 5978 to 6308 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 5038 | 5233 | 7672 | 8002 |
|  | 5038 to 5233 |  | 7672 to 8002 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 834 | 1074 | 3691 | 3406 |
|  | 834 to 1074 |  | 3406 to 3691 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 5916 | 6156 | 6794 | 7079 |
|  | 5916 to 6156 |  | 6794 to 7079 |  |

Based on Table 6.2.1.2-1, there is no harmonics and IMDs products fall into own Rx of Band 32.

6.2.1.3	MSD

No MSD issue needs to be addressed.

6.2.1.4	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_3A-20A-32A are applied.

6.3	LTE Advanced Carrier Aggregation: Band 7 and Band 20 and Band 32 with 2 ULs

6.3.1	List of specific combination issues

6.3.1.1	Channel bandwidths per operating band for CA

Table 6.3.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7A-20A-32A | CA_7A-20A | 7 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

6.3.1.2	Co-existence studies for LTE-A UL CA_7A-20A and DL CA_7A-20A-32A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.3.1.2-1

Table 6.3.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 832 | 862 |
| 2nd harmonics frequency limits (MHz) | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 5000 | 5140 | 1664 | 1724 |
|  | 5000 to 5140 |  | 1664 to 1724 |  |
| 3rd harmonics frequency limits (MHz) | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 7500 | 7710 | 2496 | 2586 |
|  | 7500 to 7710 |  | 2496 to 2586 |  |
| 2nd order IMD products and frequency limits (MHz) | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 1738 | 1638 | 3332 | 3432 |
|  | 1638 to 1738 |  | 3332 to 3432 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 4138 | 4308 | 906 | 776 |
|  | 4138 to 4308 |  | 776 to 906 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 5832 | 6002 | 4164 | 4294 |
|  | 5832 to 6002 |  | 4164 to 4294 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – fy_high + fy_low¦ | ¦fx_high + fy_high – fy_low¦ | ¦fy_low – fx_high + fx_low¦ | ¦fy_high + fx_high – fx_low¦ |
|  | 2470 | 2600 | 762 | 932 |
|  | 2470 to 2600 |  | 762 to 932 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
|  | 2480 | 2590 | 812 | 882 |
|  | 2480 to 2590 |  | 812 to 882 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
|  | 6638 | 6878 | 74 | 86 |
|  | 6638 to 6878 |  | 74 to 86 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 8332 | 8572 | 4996 | 5156 |
|  | 8332 to 8572 |  | 4996 to 5156 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fy_low - 2*fx_high¦ | ¦2*fy_high - 2*fx_low¦ |
|  | 3276 | 3476 | 3476 | 3276 |
|  | 3276 to 3476 |  | 3276 to 3476 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ | ¦2*fy_low + 2*fx_low¦ | ¦2*fy_high + 2*fx_high¦ |
|  | 6664 | 6864 | 6664 | 6864 |
|  | 6664 to 6864 |  | 6664 to 6864 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 948 | 758 | 9448 | 9138 |
|  | 758 to 948 |  | 9138 to 9448 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 5828 | 6018 | 10832 | 11142 |
|  | 5828 to 6018 |  | 10832 to 11142 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 2414 | 2644 | 6046 | 5776 |
|  | 2414 to 2644 |  | 5776 to 6046 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 7496 | 7726 | 9164 | 9434 |
|  | 7496 to 7726 |  | 9164 to 9434 |  |

Based on Table 6.3.1.2-1, there is no harmonics and IMDs products fall into own Rx of Band 32.

6.3.1.3	MSD

No MSD issue needs to be addressed.

6.3.1.4	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_7A-20A-32A are applied.

6.4	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 with 2 ULs

6.4.1	List of specific combination issues

6.4.1.1	Channel bandwidths per operating band for CA

Table 6.4.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41C | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | CA_41C | See CA_41C BCS0 in Table 6.Z.1.1-2 |  |  |  |  |  |  |  |

Table 6.4.1.1-2: E-UTRA CA_41C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_41C | 10 | 20 | 40 | 0 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |

6.4.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41C

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.4.1.2-1

Table 6.4.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

Since DL bands included are the same as in 2DL/2UL of 3A-41A, the result obtained for the 2DL/2UL CA can be reused. According to the result, 4th order IMD may fall into own Rx of band 3 and band 41. However the IMD problem is already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101. In addition, 2nd harmonics and 2nd, 3rd order IMD may also fall into Rx frequencies of bands 5-6, 8, 12-14, 17-20, 22, 26-29, 42-44, 46, 48 and 67-68, including bands defined in Rel-14 time frame.

6.4.1.3	MSD

When uplink CA configurations CA_3A-41A is paired with downlink CA configuration CA_3A-41C there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.4.1.4	∆TIB and ∆RIB values

Since the architecture for 2UL/2DL 3A-41A can be reused, the same ∆TIB and ∆RIB can be reused as:

Table 6.4.1.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-41C | 3 | 0.5 |
|  | 41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.4.1.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_3A-41C | 3 | 0 |
|  | 41 | 01 |
|  |  | 0.52 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

6.5	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 with 2 ULs

6.5.1	List of specific combination issues

6.5.1.1	Channel bandwidths per operating band for CA

Table 6.5.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41A-42A | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

Note: Band 41 and Band 42 are assumed to be synchronized.

6.5.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.5.1.2-1

Table 6.5.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

According to the result, 4th order IMD may fall into own Rx of band 3 and band 41. However the IMD problem is already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101. For band 42, the 2nd harmonics from band 3 Tx to Band 42 Rx has already been studied in Table 7.3.1A-0a in TS36.101. Also 3rd order IMD (2*B41-B3) will fall down to band 42 Rx. However, MSD due to the 3rd IMD could be excluded since band 41 and band 42 are assumed to be synchronized.

6.5.1.3	MSD

For band 42, MSD due to the 3rd IMD could be excluded since band 41 and band 42 are assumed to be synchronized. Then 2nd harmonic falldown of band 3 Tx should only be addressed but this MSD was already studied in 2DL/1UL CA_3A-42A which has been captured in 36.101.

Hence, when uplink CA configurations CA_3A-41A is paired with downlink CA configuration CA_3A-41A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.5.1.4	∆TIB and ∆RIB values

∆TIB and ∆RIB can be based on 1UL/3DL of 3A-41A-42A as:

Table 6.5.1.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-41A-42A14 | 3 | 1 |
|  | 41 | 0.35 |
|  |  | 0.86 |
|  | 42 | 0.8 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 14: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. |  |  |

Table 6.5.1.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_3A-41A-42A13 | 3 | 0.5 |
|  | 41 | 05 |
|  |  | 0.56 |
|  | 42 | 0.5 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 13: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. |  |  |

6.6	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 with 2 ULs

6.6.1	List of specific combination issues

6.6.1.1	Channel bandwidths per operating band for CA

Table 6.6.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-28A-41A | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

6.6.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-28A-41A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.6.1.2-1

Table 6.6.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

According to the result, 4th order IMD may fall into own Rx of band 3 and band 41. However the IMD problem is already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101. For band 28, the 2nd order IMD (B41-B3) and 3rd order IMD (2*B3-B41) will fall down to band 28 Rx.

6.6.1.3	MSD

When uplink CA configurations CA_3A-41A is paired with downlink CA configuration CA_3A-28A-41A there are self-interference problems from 2 uplink operation which would interfere the downlink of the Band 28.

Based on the agreed self-desense analysis paper and related proposed contributions, the merged MSD level was provided from interested companies as below Table 6.6.1.3-1.

Table 6.6.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| B3+B28+B41 | B3 | IMD2 | ¦fB3 -fB41¦ | 28.9 | 23.24 | 26.1 |
|  | B41 |  |  |  |  |  |
|  | B3 | IMD3 | ¦2*fB3 -fB41¦ | 16.7 | 14.93 | 15.8 |
|  | B41 |  |  |  |  |  |

6.6.1.4	∆TIB and ∆RIB values

∆TIB and ∆RIB can be based on 1UL/3DL of 3A-28A-41A as:

Table 6.6.1.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-28A-41A | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 41 | 0.35 |
|  |  | 0.86 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.6.1.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_3A-28A-41A | 3 | 0 |
|  | 28 | 0 |
|  | 41 | 05 |
|  |  | 0.56 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

6.7	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 42 with 2 ULs

6.7.1	List of specific combination issues

6.7.1.1	Channel bandwidths per operating band for CA

Table 6.7.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-41A-42A | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

6.7.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.7.1.2-1.

Table 6.7.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 4th IMD falls into the own Rx of Band 1 and Band 41.  As already reported in release 14 version of TR36.714-00-02, the 4th IMD never impacts Band 1 Rx because Tx-Rx separation is always kept in 190MHz.  Therefore, specifying MSD requirement is not necessary.  For 4th IMD impact on Band 41 Rx, it analysis is captured in the next section.

6.7.1.3	MSD

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-41A-42A, RAN4 needs to consider 4th IMD impact on Band 41.  However, when simultaneous Tx/Rx is not considered for this CA configuration (namely, Band 41 and Band 42 are synchronized), 4th IMD never causes desensitization in Band 41 Rx.  Since CA_1A-41A-42A was specified with the assumption that simultaneous Tx/Rx is not occurred among TDD bands 41 and 42, it is reasonable to apply same assumption for this combination.

6.7.1.4	∆TIB and ∆RIB values

Comparing 1UL-3DL CA_1A-41A-42A, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-3DL CA_1A-41A-42A shall apply.

6.8	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 with 2 ULs

6.8.1	List of specific combination issues

6.8.1.1	Channel bandwidths per operating band for CA

Table 6.8.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-3A-7A | CA_3A-7A | 3 | See CA_3A-3A BCS0 in Table 6.8.1.1-2 |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 3 | See CA_3A-3A BCS1 in Table 6.8.1.1-2 |  |  |  |  |  | 50 | 1 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |

Table 6.8.1.1-2: E-UTRA CA_3A-3A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10, 15, 20 |  |  | 30 | 1 |

6.8.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.8.1.2-1

Table 6.8.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the 3rd own Rx band 7. But this impact is already covered in 2UL/2DL CA_3A-7A. So there is no further self-interference issue.

6.8.1.3	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-3A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.8.1.4	∆TIB and ∆RIB values

6.9	LTE Advanced Carrier Aggregation: Band 7 and Band 7 and Band 8 with 2 ULs

6.9.1	List of specific combination issues

6.9.1.1	Channel bandwidths per operating band for CA

Table 6.9.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_7A-7A-8A | CA_7A-8A | 7 | See CA_7A-7A BCS1 in Table 6.9.1.1-2 |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A BCS2 in Table 6.9.1.1-2 |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

Table 6.9.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

6.9.1.2	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_7A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.9.1.2-1

Table 6.9.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 670 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3380 | 3170 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9400 | 9085 | 1160 | 950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5670 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

The 3nd harmonics of Band 8 fall into the own Rx frequency of Band 7. But this impact is already covered in 2DL/1UL CA_7A-8A.

The 5th IMD fall into the own Rx frequency of Band 8. However this impact is already covered in 2DL/2UL CA_7A-8A. Hence there is no further self-interference issue.

6.9.1.3	MSD

When uplink CA configurations CA_7A-8A is paired with downlink CA configuration CA_7A-7A-8A there is no interference components from 2 uplink operation which would interfere the downlink of the third band.

6.9.1.4	∆TIB and ∆RIB values

6.10	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 40 with 2 ULs

6.10.1 Operating bands for CA

Table 6.10.1-1: 3DL/2UL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-28-40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.10.1-2: Supported E-UTRA bandwidths per CA configuration for 3DL/2UL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-28A-40A | CA_3A-28A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

6.10.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-40A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.10.1.2-1

Table 6.10.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

There are no harmonics or IMD products from Band 3 and Band 28 uplinks which would desensitize Band 40 downlink.

6.10.1.3	MSD

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_3A-28A-40A, there is no interference components from 2 uplink operation which would interfere downlink of the Band 40, no further MSD studies for this combination.

6.10.1.4	∆TIB and ∆RIB values

6.11	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 with 2 ULs

6.11.1 	Operating bands for CA

Table 6.11.1-1: Inter-band CA operating bands for CA_1A-3A-3A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-3A-3A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |

### 6.11.2 	Channel bandwidths per operating band for CA

Table 6.11.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-3A-3A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |

### 6.11.3 	Co-existence studies

Based on co-existence studies of CA_1A-3A specified in TR 36.860-12, there is no additional harmonic and intermodulation impact for the additional band 3 receiver.

### 6.11.4 	MSD

As mentioned in 6.11.3, there is no need to specify additional MSD requirement for this UL CA configuration.

6.12	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 19 with 2 ULs

6.12.1 	Operating bands for CA

Table 6.12.1-1: Inter-band CA operating bands for CA_3A-3A-19A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-3A-19A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |

### 6.12.2 	Channel bandwidths per operating band for CA

Table 6.12.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-3A-19A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-3A-19A | CA_3A-19A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  | 55 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |

### 6.12.3 	Co-existence studies

Based on co-existence studies of CA_3A-19A specified in TR 36.860-12, there is no additional harmonic and intermodulation impact for the additional band 3 receiver.

### 6.12.4 	MSD

As mentioned in 6.12.3, there is no need to specify additional MSD requirement for this UL CA configuration.

6.13	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 21 with 2 ULs

6.13.1 	Operating bands for CA

Table 6.13.1-1: Inter-band CA operating bands for CA_3A-3A-21A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-3A-21A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |

### 6.13.2 	Channel bandwidths per operating band for CA

Table 6.13.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-3A-21A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-3A-21A | CA_3A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |

### 6.13.3 	Co-existence studies

Based on co-existence studies of CA_3A-21A specified in TR 36.714-02-02, there is no additional harmonic and intermodulation impact for the additional band 3 receiver.

### 6.13.4 	MSD

As mentioned in 6.13.3, there is no need to specify additional MSD requirement for this UL CA configuration.

6.14	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 42 with 2 ULs

6.14.1 	Operating bands for CA

Table 6.14.1-1: Inter-band CA operating bands for CA_3A-3A-42A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-3A-42A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

### 6.14.2 	Channel bandwidths per operating band for CA

Table 6.14.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-3A-42A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-3A-42A | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.14.3 	Co-existence studies

Based on co-existence studies of CA_3A-42A specified in TR 36.714-02-02, there is no additional harmonic and intermodulation impact for the additional band 3 receiver.

### 6.14.4 	MSD

As mentioned in 6.14.3, there is no need to specify additional MSD requirement for this UL CA configuration.

6.15	LTE Advanced Carrier Aggregation: Band 1 and Band 42 and Band 42 with 2 ULs

6.15.1 	Operating bands for CA

Table 6.15.1-1: Inter-band CA operating bands for CA_1A-42A-42A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-42A-42A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

### 6.15.2 	Channel bandwidths per operating band for CA

Table 6.15.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-42A-42A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-42A-42A | CA_1A-42A | 1 |  |  |  | Yes |  | Yes | Yes | Yes |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |

### 6.15.3 	Co-existence studies

Based on co-existence studies of CA_1A-42A specified in TR 36.860-13, there is no additional harmonic and intermodulation impact for the additional band 1 receiver.

### 6.15.4 	MSD

As mentioned in 6.15.3, there is no need to specify additional MSD requirement for this UL CA configuration.

6.16	LTE Advanced Carrier Aggregation: Band 3 and Band 42 and Band 42 with 2 ULs

6.16.1 	Operating bands for CA

Table 6.16.1-1: Inter-band CA operating bands for CA_3A-42A-42A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-42A-42A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

### 6.16.2 	Channel bandwidths per operating band for CA

Table 6.16.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-42A-42A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-42A-42A | CA_3A-42A | 3 |  |  |  | Yes |  | Yes |  | Yes |  | Yes | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |

### 6.16.3 	Co-existence studies

Based on co-existence studies of CA_3A-42A specified in TR 36.714-02-02, there is no additional harmonic and intermodulation impact for the additional band 3 receiver.

### 6.16.4 	MSD

As mentioned in 6.16.3, there is no need to specify additional MSD requirement for this UL CA configuration.

6.17	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 26 with 2 ULs

6.17.1	List of specific combination issues

6.17.1.1	Channel bandwidths per operating band for CA

Table 6.17.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-7A-26A | CA_1A-7A CA_1A-26A CA_7A-26A | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |

6.17.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-26A

For 2UL / 3DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 6.17.1.2-1

Table 6.17.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

IMD5 of CA_1A-7A may fall into band 26 receiving band. In addition, 3rd harmonics of band 7 may fall into Rx frequency of bands 46 and 47 and 2nd and 3rd order IMD may also fall into Rx freeuencies of bands 32, 45, 50, 51, 71, 75 and 76.

6.17.1.3	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-7A-26A

Table 6.17.1.3-1 lists B7+B26 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.17.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1756 | 3314 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 942 | 4151 | 4326 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

IMD3 and IMD5 of CA_7A-26A may fall into band 26 receiving band and IMD5 of CA_7A-26A may fall into band 7 receiving band. However, these MSD issues were already covered in 2DL/2UL CA_7A-26A. In addition, 3rd harmonics of band 26 may fall into Rx frequency of band 43 and 2nd and 3rd order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38, 41 and 44.

6.17.1.4	MSD

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the 3rd downlink of the band 26.

When uplink CA configurations CA_7A-26A is paired with downlink CA configuration CA_1A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the downlink of the bands 7 and 26. However, these MSD issues were already covered in 2DL/2UL CA_7A-26A.

Based on the agreed self-desense analysis paper and related proposed contributions, the merged MSD level was provided from interested companies as below Table 6.17.1.4-1.

Table 6.17.1.4-1: MSD values provided by companies

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Comment |
| CA_1A-7A-26A | 1 | 1965 | 5 | 25 | 2155 | 5 | N/A | FDD-FDD | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  | 26 | 830 | 5 | 50 | 875 | 5 | 3.5 |  | IMD5 |

6.17.1.5	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_1A-7A-26A are applied.

6.18	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 26 with 2 ULs

6.18.1	List of specific combination issues

6.18.1.1	Channel bandwidths per operating band for CA

Table 6.18.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-7A-26A | CA_3A-7A CA_3A-26A CA_7A-26A | 3 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |

6.18.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-26A

For 2UL / 3DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 6.18.1.2-1

Table 6.18.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

IMD3 of CA_3A-7A may fall into band 26 receiving band. In addition, 2nd harmonics of band 3 may fall into Rx frequency of bands 22, 42, 48 and 49 and 2nd, 3rd order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 42, 46, 47, 67 and 68.

6.18.1.3	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_3A-7A-26A

For 2UL / 3DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 6.18.1.3-1

Table 6.18.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 971 | 2524 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1942 | 1722 | 5048 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3727 | 3432 | 873 | 1128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

IMD2 of CA_3A-26A may fall into band 7 receiving band. In addition, 2nd harmonics of band 3 may fall into Rx frequency of bands 42, 48 and 49 and 3rd harmonics of band 3 may fall into Rx frequency of band 46 and 3rd harmonics of band 26 may fall into Rx frequency of band 41. 2nd, 3rd order IMD may also fall into Rx frequencies of bands 5, 6, 7, 8, 18, 19, 26, 27, 38, 41, 42 and 69.

6.18.1.4	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_3A-7A-26A

Table 6.18.1.4-1 lists B7+B26 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.18.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1756 | 3314 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 942 | 4151 | 4326 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

IMD3 and IMD5 of CA_7A-26A may fall into band 26 receiving band and IMD5 of CA_7A-26A may fall into band 7 receiving band. However, these MSD issues were already covered in 2DL/2UL CA_7A-26A. In addition, 3rd harmonics of band 26 may fall into Rx frequency of band 43 and 2nd and 3rd order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38, 41 and 44.

6.18.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the 3rd own downlink of the band 26.

When uplink CA configurations CA_3A-26A is paired with downlink CA configuration CA_3A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the 3rd own downlink of the band 7.

When uplink CA configurations CA_7A-26A is paired with downlink CA configuration CA_3A-7A-26A there are no self-interference problems from 2 uplink operation which would interfere the 3rd downlink of the band 3.

Based on the agreed self-desense analysis paper and related proposed contributions, the merged MSD level was provided from interested companies as below Table 6.18.1.5-1.

Table 6.18.1.5-1: MSD values provided by companies

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Comment |
| CA_3A-7A-26A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD-FDD | N/A |
|  | 7 | 2560 | 10 | 50 | 2680 | 10 | N/A |  | N/A |
|  | 26 | 835 | 5 | 25 | 880 | 5 | 17.5 |  | IMD3 |
|  | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD-FDD | N/A |
|  | 26 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 10 | 29.0 |  | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

6.18.1.6	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_3A-7A-26A are applied.

6.19	LTE Advanced Carrier Aggregation: Band 7 and Band 7 and Band 26 with 2 ULs

6.19.1	List of specific combination issues

6.19.1.1	Channel bandwidths per operating band for CA

Table 6.19.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_7A-7A-26A | CA_7A-26A | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  | 55 | 0 |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |

6.19.1.2	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_7A-7A-26A

Table 6.19.1.2-1 lists B7+B26 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.19.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1756 | 3314 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 942 | 4151 | 4326 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

IMD3 and IMD5 of CA_7A-26A may fall into band 26 receiving band and IMD5 of CA_7A-26A may fall into band 7 receiving band. However, these MSD issues were already studied in 2DL/2UL CA_7A-26A. In addition, 3rd harmonics of band 26 may fall into Rx frequency of band 43 and 2nd and 3rd order IMD may also fall into Rx frequencies of bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38, 41 and 44.

6.19.1.3	MSD

When uplink CA configurations CA_7A-26A is paired with downlink CA configuration CA_7A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the downlink of the bands 7 and 26. However, the related MSD study results were already covered in 2DL/2UL CA_7A-26A.

6.19.1.4	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_7A-7A-26A are applied.

6.20	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 7 with 2ULs

6.20.1	List of specific combination issues

6.20.1.1	Channel bandwidths per operating band for CA

Table 6.20.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7C | CA_1A-7A CA_7C | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 6.20.1.1-2: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
| CA_7C | 15 | 10, 15 | 40 | 2 |
|  | 20 | 15, 20 |  |  |

6.20.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7C

The 2UL / 3DL own receiver desensitization study by 2UL_CA_1A-7A for 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.20.1.2-1

Table 6.20.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

Based on the results from Table 6.20.1.2-1, there is no harmonics and IMDs products that fall into own Rx band.

6.20.1.3	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-7C

The 2UL / 3DL own receiver desensitization by 2UL_CA_7C study for 2nd and 3rd order harmonics were calculated and presented in Table 6.20.1.3-1

Table 6.20.1.3-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

From the harmonics table, there was no harmonics from Band 7 uplinks which would desensitize the 3rd own Rx frequency of band 1.

Frequency separation of Band 1 downlink and B7 uplink is 330 MHz, therefore there is no impact from Band 7 uplink CA to the band 1 reception.

6.20.1.4		MSD

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-7C there is no interference components from 2 uplink operation which would interfere downlink of the Band 7, no further MSD studies for this combination.

When uplink CA configurations CA_7C is paired with downlink CA configuration CA_1A-7C there is no interference components from 2 uplink operation which would interfere downlink of the Band 1, no further MSD studies for this combination.

6.20.1.5		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_1A-7C are applied.

6.21	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 28 with 2ULs

6.21.1	List of specific combination issues

6.21.1.1	Channel bandwidths per operating band for CA

Table 6.21.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-7A-28A | CA_3A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

6.21.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-7A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.21.1.2-1

Table 6.21.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

For CA_3A-28A we analyse the impact to the 3rd own receiving band by the harmonics and IMD products from the dual uplink transmission. There is self-interference on the 3rd IMD on Band 7, the detail test configuration and MSD level results are provided in section 5.1.5.

6.21.1.3		MSD

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_3A-7A-28A | CA_3A-28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | IMD3 |
|  |  | 28 | 743 | 5 | 25 | 798 | 5 |  |  |  |
|  |  | 7 | 2562 | 5 | 25 | 2682 | 5 | 17.0 |  |  |

6.21.1.4		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_3A-7A-28A are applied.

6.22	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 18 with 2 ULs

6.22.1	List of specific combination issues

6.22.1.1	Channel bandwidths per operating band for CA

Table 6.22.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-18A | CA_1A-3A or CA_1A-18A or CA_3A-18A | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |

6.22.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-18A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.22.1.2-1

Table 6.22.1.2-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 5760 | 5940 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 140 | 270 | 3630 | 3760 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1640 | 2060 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5540 | 5550 | 5740 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3420 | 3980 | 4230 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 280 | 540 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7320 | 7470 | 7720 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 | 7520 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5900 | 5200 | 4860 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2200 | 1500 | 1170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9700 | 8760 | 9100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9500 | 8970 | 9300 |

Based on Table 6.22.1.2-1, the 3rd order IMD products by Band 1 and Band 3 falls into the own Rx frequency of Band 1. This case is already covered in 2DL/2UL CA_1A-3A.

6.22.1.3	Co-existence studies for LTE-A UL CA_1A-18A and DL CA_1A-3A-18A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.22.1.3-1

Table 6.22.1.3-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 5760 | 5940 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1090 | 1165 | 2735 | 2810 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 350 | 260 | 3010 | 3145 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3550 | 3640 | 4655 | 4790 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 465 | 570 | 4930 | 5125 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2180 | 2330 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4365 | 4470 | 6575 | 6770 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5470 | 5620 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7105 | 6850 | 1400 | 1280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4310 | 4100 | 1350 | 1515 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8495 | 8750 | 5180 | 5300 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7390 | 7600 | 6285 | 6450 |

Based on Table 6.22.1.3-1, there are no harmonics and IMDs products fall into own Rx of Band 1, Band 3 and Band 18.

6.22.1.4	Co-existence studies for LTE-A UL CA_3A-18A and DL CA_1A-3A-18A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.22.1.4-1

Table 6.22.1.4-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 880 | 970 | 2525 | 2615 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 155 | 50 | 2590 | 2755 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3340 | 3445 | 4235 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 660 | 780 | 4300 | 4540 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1760 | 1940 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4155 | 4275 | 5945 | 6185 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5050 | 5230 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6325 | 6010 | 1610 | 1475 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3725 | 3470 | 930 | 1125 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7655 | 7970 | 4970 | 5105 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6760 | 7015 | 5865 | 6060 |

Based on Table 6.22.1.4-1, the 4th order IMD products by Band 3 and Band 18 falls into the own Rx frequency of Band 3. This case will be covered in 2DL/2UL CA_3A-18A.

6.22.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-18A, the 3rd order IMD products by Band 1 and Band 3 falls into the own Rx frequency of Band 1. This case is already covered in 2DL/2UL CA_1A-3A.

When uplink CA configurations CA_1A-18A is paired with downlink CA configuration CA_1A-3A-18A, there are no self-interference problems.

When uplink CA configurations CA_3A-18A is paired with downlink CA configuration CA_1A-3A-18A, the 4th order IMD products by Band 3 and Band 18 falls into the own Rx frequency of Band 3. This case will be covered in 2DL/2UL CA_3A-18A.

6.22.1.6	∆TIB and ∆RIB values

For inter-band CA_1-3-18, the same requirements on values for ∆TIB,c and ∆RIB,c of  CA_1-3-19 can be applied. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.22.1.6-1 and in table 6.22.1.6-2:

Table 6.22.1.6-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-18A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |

Table 6.22.1.6-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A_3A-18A | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |

6.23	LTE Advanced Carrier Aggregation: Band 3 and Band 11 and Band 18 with 2 ULs

6.23.1	List of specific combination issues

6.23.1.1	Channel bandwidths per operating band for CA

Table 6.23.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-11A-18A | CA_3A-11A or CA_3A-18A or CA_11A-18A | 3 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |

6.23.1.2	Co-existence studies for LTE-A UL CA_3A-11A and DL CA_3A-11A-18A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.23.1.2-1

Table 6.23.1.2-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 262.1 | 357.1 | 3137.9 | 3232.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1070.8 | 1185.8 | 1972.1 | 2142.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4565.8 | 4680.8 | 4847.9 | 5017.9 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2498.7 | 2633.7 | 3682.1 | 3927.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 524.2 | 714.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5993.7 | 6128.7 | 6557.9 | 6802.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6275.8 | 6465.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5712.1 | 5392.1 | 4081.6 | 3926.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2499.2 | 2234.2 | 923.7 | 713.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8267.9 | 8587.9 | 7421.6 | 7576.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7985.8 | 8250.8 | 7703.7 | 7913.7 |

Based on Table 6.23.1.2-1, the 5th order IMD products by Band 3 and Band 11 falls into the own Rx frequency of Band 18.

6.23.1.3	Co-existence studies for LTE-A UL CA_3A-18A and DL CA_3A-11A-18A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.23.1.3-1

Table 6.23.1.3-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 5130 | 5355 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 880 | 970 | 2525 | 2615 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 155 | 50 | 2590 | 2755 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3340 | 3445 | 4235 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 660 | 780 | 4300 | 4540 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1760 | 1940 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4155 | 4275 | 5945 | 6185 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5050 | 5230 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6325 | 6010 | 1610 | 1475 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3725 | 3470 | 930 | 1125 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7655 | 7970 | 4970 | 5105 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6760 | 7015 | 5865 | 6060 |

Based on Table 6.23.1.3-1, the 4th order IMD products by Band 3 and Band 18 falls into the own Rx frequency of Band 3. This case will be covered in 2DL/2UL CA_3A-18A. Also the 5th order IMD product by Band 3 and Band 18 falls into the own Rx frequency of Band 11. Since Band 18 is only used by KDDI in Japan, the frequency range of Band 3 can be limited to 1710MHz-1750MHz according to Japanese frequency allocation. Then there will be no self-interference problems.

6.23.1.4	Co-existence studies for LTE-A UL CA_11A-18A and DL CA_3A-11A-18A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.23.1.4-1

Table 6.23.1.4-1: Harmonic and IMD analysis

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 1427.9 | 1447.9 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 2855.8 | 2895.8 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 4283.7 | 4343.7 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 597.9 | 632.9 | 2242.9 | 2277.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 182.1 | 232.1 | 2025.8 | 2080.8 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3057.9 | 3107.9 | 3670.8 | 3725.8 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 997.1 | 1062.1 | 3453.7 | 3528.7 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1195.8 | 1265.8 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3872.9 | 3937.9 | 5098.7 | 5173.7 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4485.8 | 4555.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4976.6 | 4881.6 | 1892.1 | 1812.1 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2713.7 | 2623.7 | 365.8 | 450.8 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6526.6 | 6621.6 | 4687.9 | 4767.9 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5913.7 | 6003.7 | 5300.8 | 5385.8 |

Based on Table 6.23.1.4-1, the 5th order IMD products by Band 11 and Band 18 falls into the own Rx frequency of Band 3.

6.23.1.5	MSD

When uplink CA configurations CA_3A_11A is paired with downlink CA configuration CA_3A-11A-18A, the 5th order IMD products by Band 3 and Band 11 falls into the own Rx frequency of Band 18.

When uplink CA configurations CA_3A-18A is paired with downlink CA configuration CA_3A_11A-18A, the 4th order IMD products by Band 3 and Band 18 falls into the own Rx frequency of Band 3. This case will be covered in 2DL/2UL CA_3A-18A. Also the 5th order IMD product by Band 3 and Band 18 falls into the own Rx frequency of Band 11. Also the 5th order IMD product by Band 3 and Band 18 falls into the own Rx frequency of Band 11. Since Band 18 is only used by KDDI in Japan, the frequency range of Band 3 can be limited to 1710MHz-1750MHz according to Japanese frequency allocation. Then there will be no self-interference problems.

When uplink CA configurations CA_11A-18A is paired with downlink CA configuration CA_3A-11A-18A, the 5th order IMD products by Band 11 and Band 18 falls into the own Rx frequency of Band 3.

MSD evaluation is shown in Table 6.23.1.5-1.

Table 6.23.1.5-1: MSD summary for 2UL 3DL CA_3A-11A-18A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_3A-11A-18A | CA_3A-11A | 3 | TBD | 5 | 25 | TBD | 5 | N/A | FDD | N/A |
|  |  | 11 | TBD | 5 | 25 | TBD | 5 | N/A |  | N/A |
|  |  | 18 | TBD | 5 | 25 | TBD | 5 | TBD |  | IMD5 |
|  | CA_11A-18A | 3 | TBD | 5 | 25 | TBD | 5 | TBD | FDD | IMD5 |
|  |  | 11 | TBD | 5 | 25 | TBD | 5 | N/A |  | N/A |
|  |  | 18 | TBD | 5 | 25 | TBD | 5 | N/A |  | N/A |

6.23.1.6	∆TIB and ∆RIB values

For inter-band CA_1-3-18, the same requirements on values for ∆TIB,c and ∆RIB,c of  CA_3-19_21 can be applied. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.23.1.6-1 and in table 6.23.1.6-2:

Table 6.23.1.6-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_3A-11A-18A | 3 | 0.8 |
|  | 11 | 0.3 |
|  | 18 | 0.9 |

Table 6.23.1.6-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_3A-11A-18A | 3 | 0.3 |
|  | 11 | 0 |
|  | 18 | 0.5 |

6.24	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 20 with 2 ULs

6.24.1	List of specific combination issues

6.24.1.1	Channel bandwidths per operating band for CA

Table 6.24.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-20A | CA_1A-3A CA_1A-20A CA_3A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

6.24.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-20A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.24.1.2-1

Table 6.24.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 | 2000 | 1690 | 1805 |

Based on Table 6.24.1.2-1, there is no harmonics and IMDs products fall into own Rx of Band 20.

6.24.1.3	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-3A-20A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.24.1.3-1

Table 6.24.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2496 | 2586 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4566 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 | 2000 | 812 | 882 |

Based on Table 6.24.1.3-1, there is no harmonics and IMDs products fall into own Rx of Band 20.

6.24.1.4	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_1A-3A-20A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.24.1.4-1

Table 6.24.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2496 | 2586 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1148 | 1058 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2978 | 3128 | 316 | 196 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4672 | 4822 | 3584 | 3704 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4898 | 5108 | 516 | 666 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6592 | 6802 | 4416 | 4566 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1528 | 1348 | 7088 | 6818 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5248 | 5428 | 8512 | 8782 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1254 | 1464 | 4276 | 4036 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6546 | 7424 | 7664 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1900 | 2000 | 812 | 882 |

Based on Table 6.24.1.4-1, there is no harmonics and IMDs products fall into own Rx of Band 20.

6.24.1.5	MSD

No MSD issue needs to be addressed.

6.24.1.6	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_1A-3A-20A are applied.

6.25	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 with 2 ULs

6.25.1 	Operating bands for CA

Table 6.25.1-1: Inter-band CA operating bands for CA_3A-41A-42A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-41A-42A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2469 MHz | – | 2690 MHz | 2469 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |

### 6.25.2 	Channel bandwidths per operating band for CA

Table 6.25.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-41A-42A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-41A-42A | CA_3A-42A | 3 |  |  |  | Yes |  | Yes |  | Yes |  | Yes | 60 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |

### 6.25.3 	Co-existence studies

Based on co-existence studies of CA_3A-42A specified in TR 36.714-02-02, there is no additional harmonic and intermodulation impact for the additional band 41 receiver.

### 6.25.4 	MSD

As mentioned in 6.25.3, there is no need to specify additional MSD requirement for this CA configuration.

6.26			LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 28 with 2ULs

6.26.1	List of specific combination issues

6.26.1.1	Channel bandwidths per operating band for CA

Table 6.26.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7A-28A | CA_1A-7A CA_1A-28A CA_7A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |

6.26.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.26.1.2-1

Table 6.26.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

For CA_1A-7A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. From the table above, we see that there is self-interference on the two-tone 5th order IMD on Band 28. The related MSD study will be covered in clause 6.26.1.5

6.26.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-7A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.26.1.3-1

Table 6.26.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

For CA_1A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. From the table above, we see that there is self-interference on the 2nd order IMD on Band 7. The related MSD study will be covered in clause 6.26.1.5

6.26.1.4	Co-existence studies for LTE-A UL CA_7A-28A and DL CA_1A-7A-28A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.26.1.4-1

Table 6.26.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1867 | 1752 | 3203 | 3318 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4252 | 4437 | 1164 | 1004 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5703 | 5888 | 3906 | 4066 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6752 | 7007 | 461 | 256 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8203 | 8458 | 4609 | 4814 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3504 | 3734 | 6406 | 6636 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 242 | 9577 | 9252 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5312 | 5562 | 10703 | 11028 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2756 | 3031 | 6304 | 6004 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7109 | 7384 | 8906 | 9206 |

For CA_7A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. From the table above, we see that there is self-interference on the 3rd order hamonic problem on Band 1. The self-interference problem already covered in 2DL_CA_1A-28A.

6.26.1.5		MSD

When uplink 2CA configurations CA_1A-7A is paired with downlink 3CA_1A-7A-28A there are self-interference issues from concurrently supporting uplink 2CA which could interfere with downlink Band 28. Initial MSD values is provided in Table 6.26.1.5-1 and Table 6.26.1.5-2.

When uplink 2CA configurations CA_1A-28A is paired with downlink 3CA_1A-7A-28A there are self-interference issues from concurrently supporting uplink 2CA which could interfere with downlink Band 7. Initial MSD values is provided in Table 6.26.1.5-1 and Table 6.26.1.5-2.

When uplink 2CA configurations CA_7A-28A is paired with downlink 3CA_1A-7A-28A there are no self-interference issues from concurrently supporting uplink 2CA.

Based on the self-desense analysis conducted in [3]:

Table 6.26.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | LGE MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B7+B28 | B1 | IMD5 | ¦3*fB1 -2*fB7¦ | 1935 | 5 | 25 | 785 | 10 | 0.4 | 4.3 |
|  | B7 |  |  | 2510 | 10 | 50 |  |  |  |  |
|  | B1 | IMD2 | ¦fB1 +fB28¦ | 1935 | 5 | 25 | 2665 | 10 | 1.8 | 27.8 |
|  | B28 |  |  | 730 | 10 | 50 |  |  |  |  |

Table 6.26.1.5-2: MSD values

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |  | Source of IMD |
| CA_1A-7A-28A | CA_1A-7A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | IMD5 |  |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |  |  |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 | 4.5 |  |  |  |
|  | CA_1A-28A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | IMD2 |  |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 |  |  |  |  |
|  |  | 7 | 2545 | 10 | 50 | 2665 | 10 | 28.0 |  |  |  |

6.26.1.6		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 3DL/1UL CA_1A-7A-28A are applied.

6.27	LTE Advanced Carrier Aggregation: Band 1 and Band 42 and Band 42 with 2 ULs

6.27.1 	Operating bands for CA

Table 6.27.1-1: Inter-band CA operating bands for CA_1A-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-42C | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

### 6.27.2 	Channel bandwidths per operating band for CA

Table 6.27.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-42C | CA_42C | 1 |  |  |  | Yes |  | Yes |  | Yes |  | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |

### 6.27.3 	Co-existence studies

Based on co-existence studies of CA_1A-42A specified in TR 36.714-02-02, there is no additional harmonic and intermodulation impact for the additional band 42 receiver.

### 6.27.4 	MSD

As mentioned in 6.27.3, there is no need to specify additional MSD requirement for this CA configuration.

6.28	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 with 2 ULs

6.28.1 	Operating bands for CA

Table 6.28.1-1: Inter-band CA operating bands for CA_3A-41C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-41C | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.28.2 	Channel bandwidths per operating band for CA

Table 6.28.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-41C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-41C | CA_41C | 3 |  |  |  | Yes |  | Yes |  | Yes |  | Yes | 60 | 0 |
|  |  | 42 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |

### 6.28.3 	Co-existence studies

Based on co-existence studies of CA_3A-41A specified in TR 36.714-02-02, there is no harmonic impact while using band 41 as UL. Since only band 41 is used for UL, there is no intermodulation impact for this CA configuration.

### 6.28.4 	MSD

As mentioned in 6.28.3, there is no need to specify MSD requirement for this CA configuration.

6.29	LTE Advanced Carrier Aggregation: Band 3 and Band 42 and Band 42 with 2 ULs

6.29.1 	Operating bands for CA

Table 6.29.1-1: Inter-band CA operating bands for CA_3A-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-42C | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

### 6.29.2 	Channel bandwidths per operating band for CA

Table 6.29.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  | Yes |  | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |  |  |  |

### 6.29.3 	Co-existence studies

Based on co-existence studies of CA_3A-42A specified in TR 36.714-02-02, there is no additional harmonic and intermodulation impact for the additional band 42 receiver.

### 6.29.4 	MSD

As mentioned in 6.29.3, there is no need to specify additional MSD requirement for this CA configuration.

7		4DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

7.1	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 20 and Band 32 with 2 ULs

7.1.1	List of specific combination issues

7.1.1.1	Channel bandwidths per operating band for CA

Table 7.1.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-7A-20A-32A | CA_3A-7A or CA_3A-20A or CA_7A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

7.1.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-20A-32A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.2-1

Table 7.1.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits (MHz) | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3420 | 3570 | 5000 | 5140 |
|  | 3420 to 3570 |  | 5000 to 5140 |  |
| 3rd harmonics frequency limits (MHz) | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5130 | 5355 | 7500 | 7710 |
|  | 5130 to 5355 |  | 7500 to 7710 |  |
| 2nd order IMD products and frequency limits (MHz) | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 715 | 860 | 4210 | 4355 |
|  | 715 to 860 |  | 4210 to 4355 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 850 | 1070 | 3215 | 3430 |
|  | 850 to 1070 |  | 3215 to 3430 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 5920 | 6140 | 6710 | 6925 |
|  | 5920 to 6140 |  | 6710 to 6925 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – fy_high + fy_low¦ | ¦fx_high + fy_high – fy_low¦ | ¦fy_low – fx_high + fx_low¦ | ¦fy_high + fx_high – fx_low¦ |
|  | 1640 | 1855 | 2425 | 2645 |
|  | 1640 to 1855 |  | 2425 to 2645 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
|  | 1690 | 1805 | 2480 | 2590 |
|  | 1690 to 1805 |  | 2480 to 2590 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
|  | 2560 | 2855 | 5715 | 6000 |
|  | 2560 to 2855 |  | 5715 to 6000 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 7630 | 7925 | 9210 | 9495 |
|  | 7630 to 7925 |  | 9210 to 9495 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fy_low - 2*fx_high¦ | ¦2*fy_high - 2*fx_low¦ |
|  | 1720 | 1430 | 1430 | 1720 |
|  | 1430 to 1720 |  | 1430 to 1720 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ | ¦2*fy_low + 2*fx_low¦ | ¦2*fy_high + 2*fx_high¦ |
|  | 8420 | 8710 | 8420 | 8710 |
|  | 8420 to 8710 |  | 8420 to 8710 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 8570 | 8215 | 4640 | 4270 |
|  | 8215 to 8570 |  | 4270 to 4640 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 11710 | 12065 | 9340 | 9710 |
|  | 11710 to 12065 |  | 9340 to 9710 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 4290 | 3930 | 355 | 10 |
|  | 3930 to 4290 |  | 10 to 355 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 10920 | 11280 | 10130 | 10495 |
|  | 10920 to 11280 |  | 10130 to 10495 |  |

Based on Table 7.1.1.2-1, the 2nd order IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 20; this case is already covered in 3DL/2UL CA_3A-7A-20A. In addition, the 4th order IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 32; However, the related MSD study will be covered in 3DL/2UL CA_3A-7A-32A.

7.1.1.3	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_3A-7A-20A-32A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.3-1

Table 7.1.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 832 | 862 |
| 2nd harmonics frequency limits (MHz) | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3420 | 3570 | 1664 | 1724 |
|  | 3420 to 3570 |  | 1664 to 1724 |  |
| 3rd harmonics frequency limits (MHz) | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5130 | 5355 | 2496 | 2586 |
|  | 5130 to 5355 |  | 2496 to 2586 |  |
| 2nd order IMD products and frequency limits (MHz) | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 953 | 848 | 2542 | 2647 |
|  | 848 to 953 |  | 2542 to 2647 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 2558 | 2738 | 121 | 14 |
|  | 2558 to 2738 |  | 14 to 121 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 4252 | 4432 | 3374 | 3509 |
|  | 4252 to 4432 |  | 3374 to 3509 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – fy_high + fy_low¦ | ¦fx_high + fy_high – fy_low¦ | ¦fy_low – fx_high + fx_low¦ | ¦fy_high + fx_high – fx_low¦ |
|  | 1680 | 1815 | 757 | 937 |
|  | 1680 to 1815 |  | 757 to 937 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
|  | 1690 | 1805 | 812 | 882 |
|  | 1690 to 1805 |  | 812 to 882 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
|  | 4268 | 4523 | 711 | 876 |
|  | 4268 to 4523 |  | 711 to 876 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 5962 | 6217 | 4206 | 4371 |
|  | 5962 to 6217 |  | 4206 to 4371 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fy_low - 2*fx_high¦ | ¦2*fy_high - 2*fx_low¦ |
|  | 1696 | 1906 | 1906 | 1696 |
|  | 1696 to 1906 |  | 1696 to 1906 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ | ¦2*fy_low + 2*fx_low¦ | ¦2*fy_high + 2*fx_high¦ |
|  | 5084 | 5294 | 5084 | 5294 |
|  | 5084 to 5294 |  | 5084 to 5294 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 1738 | 1543 | 6308 | 5978 |
|  | 1543 to 1738 |  | 5978 to 6308 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 5038 | 5233 | 7672 | 8002 |
|  | 5038 to 5233 |  | 7672 to 8002 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 834 | 1074 | 3691 | 3406 |
|  | 834 to 1074 |  | 3406 to 3691 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 5916 | 6156 | 6794 | 7079 |
|  | 5916 to 6156 |  | 6794 to 7079 |  |

Based on Table 7.1.1.3-1, there was no harmonics and IMDs products fall into own Rx of Band 32. In addition, the 2nd and the 3rd order IMD products by Band 3 and Band 20 fall into the own Rx frequency of Band 7; However, this MSD issues is already covered in 3DL/2UL CA_3A-7A-20A.

7.1.1.4	Co-existence studies for LTE-A UL CA_7A-20A and DL CA_3A-7A-20A-32A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.1.1.4-1

Table 7.1.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 832 | 862 |
| 2nd harmonics frequency limits (MHz) | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 5000 | 5140 | 1664 | 1724 |
|  | 5000 to 5140 |  | 1664 to 1724 |  |
| 3rd harmonics frequency limits (MHz) | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 7500 | 7710 | 2496 | 2586 |
|  | 7500 to 7710 |  | 2496 to 2586 |  |
| 2nd order IMD products and frequency limits (MHz) | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 1738 | 1638 | 3332 | 3432 |
|  | 1638 to 1738 |  | 3332 to 3432 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 4138 | 4308 | 906 | 776 |
|  | 4138 to 4308 |  | 776 to 906 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 5832 | 6002 | 4164 | 4294 |
|  | 5832 to 6002 |  | 4164 to 4294 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – fy_high + fy_low¦ | ¦fx_high + fy_high – fy_low¦ | ¦fy_low – fx_high + fx_low¦ | ¦fy_high + fx_high – fx_low¦ |
|  | 2470 | 2600 | 762 | 932 |
|  | 2470 to 2600 |  | 762 to 932 |  |
| Two-tone 3rd order IMD products and frequency limits (MHz) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
|  | 2480 | 2590 | 812 | 882 |
|  | 2480 to 2590 |  | 812 to 882 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
|  | 6638 | 6878 | 74 | 86 |
|  | 6638 to 6878 |  | 74 to 86 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 8332 | 8572 | 4996 | 5156 |
|  | 8332 to 8572 |  | 4996 to 5156 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fy_low - 2*fx_high¦ | ¦2*fy_high - 2*fx_low¦ |
|  | 3276 | 3476 | 3476 | 3276 |
|  | 3276 to 3476 |  | 3276 to 3476 |  |
| Two-tone 4th order IMD products and frequency limits (MHz) | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ | ¦2*fy_low + 2*fx_low¦ | ¦2*fy_high + 2*fx_high¦ |
|  | 6664 | 6864 | 6664 | 6864 |
|  | 6664 to 6864 |  | 6664 to 6864 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | 948 | 758 | 9448 | 9138 |
|  | 758 to 948 |  | 9138 to 9448 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 5828 | 6018 | 10832 | 11142 |
|  | 5828 to 6018 |  | 10832 to 11142 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | 2414 | 2644 | 6046 | 5776 |
|  | 2414 to 2644 |  | 5776 to 6046 |  |
| Two-tone 5th order IMD products and frequency limits (MHz) | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 7496 | 7726 | 9164 | 9434 |
|  | 7496 to 7726 |  | 9164 to 9434 |  |

Based on Table 7.1.1.4-1, there was no harmonics and IMDs products fall into own Rx of Band 3 and Band 32.

7.1.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7A-20A-32A, the 4th order IMD products by Band 3 and Band 7 falls into the own Rx frequency of Band 32; this aspect will be covered in 3DL/2UL CA_3A-7A-32A. In addition, the 2nd order IMD products by Band 3 and Band 7 falls into the own Rx frequency of Band 20; however, this case is already covered in 3DL/2UL CA_3A-7A-20A.

When uplink CA configurations CA_3A-20A is paired with downlink CA configuration CA_3A-7A-20A-32A, there is no self-interference problems into the own Rx frequency of Band 32. In addition, the 2nd and the 3rd order IMD products by Band 3 and Band 20 fall into the own Rx frequency of Band 7; however, this case is already covered in 3DL/2UL CA_3A-7A-20A.

When uplink CA configurations CA_7A-20A is paired with downlink CA configuration CA_3A-7A-20A-32A, there is no self-interference problems in 3rd and 4th own Rx frequency bands.

7.1.1.6	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_3A-7A-20A-32A are applied.

7.2	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 42 and Band 42 with 2 ULs

7.2.1	List of specific combination issues

7.2.1.1	Channel bandwidths per operating band for CA

Table 7.2.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-41A-42C | CA_42C CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C BCS1 in TS36.101 |  |  |  |  |  |  |  |

7.2.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.2.1.2-1.

Table 7.2.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 4th IMD falls into the own Rx of Band 1 and Band 41.  As already reported in release 14 version of TR36.714-00-02, the 4th IMD never impacts Band 1 Rx because Tx-Rx separation is always kept in 190MHz.  Therefore, specifying MSD requirement is not necessary.  For 4th IMD impact on Band 41 Rx, it analysis is captured in the MSD section.

7.2.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_1A-41A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.2.1.3-1

Table 7.2.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of bands 1 and 41.

7.2.1.4	MSD

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-41A-42C, RAN4 needs to consider 4th IMD impact on Band 41.  However, when simultaneous Tx/Rx is not considered for this CA configuration (namely, Band 41 and Band 42 are synchronized), 4th IMD never causes desensitization in Band 41 Rx.  Since CA_1A-41A-42C was specified with the assumption that simultaneous Tx/Rx is not occurred among TDD bands 41 and 42, it is reasonable to apply same assumption for this combination.

7.2.1.5	∆TIB and ∆RIB values

Comparing 1UL-4DL CA_1A-41A-42C, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-4DL CA_1A-41A-42C shall apply.

7.3	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 41 and Band 42 with 2 ULs

7.3.1	List of specific combination issues

7.3.1.1	Channel bandwidths per operating band for CA

Table 7.3.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-41C-42A | CA_1A-42A | 1 |  |  | Yes | Yes | Yes |  | Yes | 80 | 0 |
|  |  | 41 | See CA_41C_BCS0 inTS36.101 |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes | Yes |  |  |

7.3.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41C-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.3.1.2-1.

Table 7.3.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 4th IMD falls into the own Rx of Band 1 and Band 41.  As already reported in release 14 version of TR36.714-00-02, the 4th IMD never impacts Band 1 Rx because Tx-Rx separation is always kept in 190MHz.  Therefore, specifying MSD requirement is not necessary.  For 4th IMD impact on Band 41 Rx, it analysis is captured in the next section.

7.3.1.3	MSD

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-41C-42A, RAN4 needs to consider 4th IMD impact on Band 41.  However, when simultaneous Tx/Rx is not considered for this CA configuration (namely, Band 41 and Band 42 are synchronized), 4th IMD never causes desensitization in Band 41 Rx.  Since CA_1A-41C-42A was specified with the assumption that simultaneous Tx/Rx is not occurred among TDD bands 41 and 42, it is reasonable to apply same assumption for this combination.

7.3.1.4	∆TIB and ∆RIB values

Comparing 1UL-4DL CA_1A-41C-42A, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-4DL CA_1A-41C-42A shall apply.

7.4	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 40 and Band 40 with 2 ULs

7.4.1 Operating bands for CA

Table 7.4.1-1: 4DL/2UL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3-28-40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 7.4.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL/2UL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-28A-40C | CA_3A-28A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

7.4.1.2	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_3A-28A-40C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.4.1.2-1

Table 7.4.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

There are no harmonics or IMD products from Band 3 and Band 28 uplinks which would desensitize Band 40 downlink.

7.4.1.3	MSD

When uplink CA configurations CA_3A-28A is paired with downlink CA configuration CA_3A-28A-40C there is no interference components from 2 uplink operation which would interfere downlink of the Band 40, no further MSD studies for this combination.

7.4.1.4	∆TIB and ∆RIB values

7.5	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 and Band 8 with 2 ULs

7.5.1	List of specific combination issues

7.5.1.1	Channel bandwidths per operating band for CA

Table 7.5.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-7A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 | See CA_7A-7A BCS1 in Table 7.5.1.1-2 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 | See CA_7A-7A BCS2 in Table 7.5.1.1-2 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

Table 7.5.1.1-2: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

7.5.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.5.1.2-1

Table 7.5.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 3rd IMD falls into the own Rx band 8. But this impact is already covered in 2UL/3DL CA_3A-7A-8A. So there is no further self-interference issue.

The 4th IMD falls into the own Rx band 7. But this impact is already covered in 2UL/2DL CA_3A-7A. So there is no further self-interference issue.

7.5.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-7A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.5.1.3-1

Table 7.5.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics of Band 8, 4th & 5th IMDs fall into the own Rx frequency of Band 3. And also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. But this impact is already covered in 2UL/2DL CA_3A-8A.

The 3rd harmonics of Band 8 and 2nd &3rd IMDs will interfere own Rx frequency of Band 7. And this impact is already covered in 2UL/3DL CA_3A-7A-8A.So there is no further self-interference issue.

7.5.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-7A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.5.1.4-1

Table 7.5.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 670 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3380 | 3170 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9400 | 9085 | 1160 | 950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5670 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

The 3nd harmonics of Band 8 fall into the own Rx frequency of Band 7. But this impact is already covered in 2DL/1UL CA_7A-8A.

The 5th IMD fall into the own Rx frequency of Band 8. However this impact is already covered in 2DL/2UL CA_7A-8A. Hence there is no further self-interference issue.

7.5.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 7 and Band 8.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-7A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 3, Band 7 and Band 8.

And also when uplink CA configurations CA_7A-8A is paired with downlink CA configuration CA_3A-7A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 8.

However, these impacts are already covered in the fallback combinations as mentioned in the previous section; no further MSD analysis is needed.

7.5.1.6	∆TIB and ∆RIB values

7.6	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 8 with 2 ULs

7.6.1	List of specific combination issues

7.6.1.1	Channel bandwidths per operating band for CA

Table 7.6.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-3A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 3 | See CA_3A-3A BCS0 in Table 7.6.1.1-2 |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 | See CA_3A-3A BCS1 in Table 7.6.1.1-2 |  |  |  |  |  | 60 | 1 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

Table 7.6.1.1-2: E-UTRA CA_3A-3A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10, 15, 20 |  |  | 30 | 1 |

7.6.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.6.1.2-1

Table 7.6.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 3rd IMD falls into the own Rx band 8. But this impact is already covered in 2UL/3DL CA_3A-7A-8A. So there is no further self-interference issue.

The 4th IMD falls into the own Rx band 7. But this impact is already covered in 2UL/2DL CA_3A-7A. So there is no further self-interference issue.

7.6.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.6.1.3-1

Table 7.6.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics of Band 8, 4th & 5th IMDs fall into the own Rx frequency of Band 3. And also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. But this impact is already covered in 2UL/2DL CA_3A-8A.

The 3rd harmonics of Band 8 and 2nd &3rd IMDs will interfere own Rx frequency of Band 7. And this impact is already covered in 2UL/3DL CA_3A-7A-8A.So there is no further self-interference issue.

7.6.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-3A-7A-8A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.6.1.4-1

Table 7.6.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 670 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3380 | 3170 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9400 | 9085 | 1160 | 950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5670 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

The 3nd harmonics of Band 8 fall into the own Rx frequency of Band 7. But this impact is already covered in 2DL/1UL CA_7A-8A.

The 5th IMD fall into the own Rx frequency of Band 8. However this impact is already covered in 2DL/2UL CA_7A-8A. Hence there is no further self-interference issue.

7.6.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-3A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 8 and Band 7.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-3A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 3, Band 7 and Band 8.

And also when uplink CA configurations CA_7A-8A is paired with downlink CA configuration CA_3A-3A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 8.

However, these impacts are already covered in the fallback combinations as mentioned in the previous section; no further MSD analysis is needed.

7.6.1.6	∆TIB and ∆RIB values

7.7	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 with 2 ULs

7.7.1	List of specific combination issues

7.7.1.1	Channel bandwidths per operating band for CA

Table 7.7.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-3A-7A-7A | CA_3A-7A | 3 | See CA_3A-3A BCS0 in Table 7.7.1.1-2 |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A BCS1 in Table 7.7.1.1-3 |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A BCS1 in Table 7.7.1.1-2 |  |  |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A BCS2 in Table 7.7.1.1-3 |  |  |  |  |  |  |  |

Table 7.7.1.1-2: E-UTRA CA_3A-3A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10, 15, 20 |  |  | 30 | 1 |

Table 7.7.1.1-3: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

7.7.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.7.1.2-1

Table 7.7.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the own Rx band 7. But this impact is already covered in 2UL/2DL CA_3A-7A. So there is no further self-interference issue.

7.7.1.3	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-3A-7A-7A there are interference components from 2 uplink operation which would interfere the downlink of the Band 7. However, this impact is already covered in the fallback combination as mentioned in the previous section; no further MSD analysis is needed.

7.7.1.4	∆TIB and ∆RIB values

7.8	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 and Band 42 with 2 ULs

7.8.1	List of specific combination issues

7.8.1.1	Channel bandwidths per operating band for CA

Table 7.8.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41C-42A | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in TS36.101 |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

7.8.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41C-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.8.1.2-1.

Table 7.8.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

According to the result, 4th order IMD may fall into own Rx of band 3 and band 41. However the IMD problem is already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101. For band 42, the 2nd harmonics from band 3 Tx to Band 42 Rx has already been studied in Table 7.3.1A-0a in TS36.101. Also 3rd order IMD (2*B41-B3) will fall down to band 42 Rx. However, MSD due to the 3rd IMD could be excluded since band 41 and band 42 are assumed to be synchronized.

7.8.1.3	MSD

For band 42, MSD due to the 3rd IMD could be excluded since band 41 and band 42 are assumed to be synchronized. Then 2nd harmonic falldown of band 3 Tx should only be addressed but this MSD was already studied in 2DL/1UL CA_3A-42A which has been captured in 36.101.

Hence, when uplink CA configurations CA_3A-41A is paired with downlink CA configuration CA_3A-41C-42A there is no interference components from 2 uplink operation which would interfere the downlink of Band 41 and Band 42.

.

7.8.1.4	∆TIB and ∆RIB values

Comparing 1UL-4DL CA_3A-41C-42A, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-4DL CA_3A-41C-42A shall apply.

7.9	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 and Band 42 with 2 ULs

7.9.1	List of specific combination issues

7.9.1.1	Channel bandwidths per operating band for CA

Table 7.9.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41A-42C | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 |  |  |  |  |  |  |  |

7.9.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-41A-42C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.9.1.2-1.

Table 7.9.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

According to the result, 4th order IMD may fall into own Rx of band 3 and band 41. However the IMD problem is already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101. For band 42, the 2nd harmonics from band 3 Tx to Band 42 Rx has already been studied in Table 7.3.1A-0a in TS36.101. Also 3rd order IMD (2*B41-B3) will fall down to band 42 Rx. However, MSD due to the 3rd IMD could be excluded since band 41 and band 42 are assumed to be synchronized.

7.9.1.3	MSD

For band 42, MSD due to the 3rd IMD could be excluded since band 41 and band 42 are assumed to be synchronized. Then 2nd harmonic falldown of band 3 Tx should only be addressed but this MSD was already studied in 2DL/1UL CA_3A-42A which has been captured in 36.101.

Hence, when uplink CA configurations CA_3A-41A is paired with downlink CA configuration CA_3A-41A-42C there is no interference components from 2 uplink operation which would interfere the downlink of Band 42.

.

7.9.1.4	∆TIB and ∆RIB values

Comparing 1UL-4DL CA_3A-41A-42C, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-4DL CA_3A-41A-42C shall apply.

7.10	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 41 with 2 ULs

7.10.1	List of specific combination issues

7.10.1.1	Channel bandwidths per operating band for CA

Table 7.10.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-28A-41C | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 | See CA_42C Bandwidth combination set 0 in TS36.101 |  |  |  |  |  |  |  |

7.10.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-28A-41C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.10.1.2-1.

Table 7.10.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

According to the result, 4th order IMD may fall into own Rx of band 3 and band 41. However the IMD problem is already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101. For band 28, the 2nd order IMD (B41-B3) and 3rd order IMD (2*B3-B41) will fall down to band 28 Rx.

7.10.1.3	MSD

For band 28, both IMD2 and IMD3 effects should be studied. But it is conducted for CA_3DL_3A-28A-41A_2UL_3A-41A_BCS0 and the results can be reused for this 4DL/2UL CA.

Hence, when uplink CA configurations CA_3A-41A is paired with downlink CA configuration CA_3A-28A-41C there is no new interference components from 2 uplink operation which would interfere the downlink of Band 28 and Band 41..

7.10.1.4	∆TIB and ∆RIB values

Comparing 1UL-4DL CA_3A-28A-41C, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-4DL CA_3A-28A-41C shall apply.

7.11	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 42 with 2 ULs

7.11.1	List of specific combination issues

7.11.1.1	Channel bandwidths per operating band for CA

Table 7.11.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-28A-41A-42A | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

7.11.1.2	Co-existence studies for LTE-A UL CA_3A-41A and DL CA_3A-28A-41A-42A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.11.1.2-1.

Table 7.11.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

According to the result,

- 4th order IMD may fall into own Rx of band 3 and band 41 but the IMD problem is already covered in 2DL/2UL CA_3A-41A in Table 7.3.1A-0f in TS36.101.

- the 2nd harmonics from band 3 Tx to Band 42 Rx has already been studied in Table 7.3.1A-0a in TS36.101.

- 3rd order IMD (2*B41-B3) will fall down to band 42 Rx but this is already covered in 2UL/3DL 3A-41A/3A-41A-42A.

- For band 28, the 2nd order IMD (B41-B3) and 3rd order IMD (2*B3-B41) will fall down to band 28 Rx.

7.11.1.3	MSD

For band 28, both IMD2 and IMD3 effects should be studied. But it is conducted for CA_3DL_3A-28A-41A_2UL_3A-41A_BCS0 and the results can be reused for this 4DL/2UL CA.

Hence, when uplink CA configurations CA_3A-41A is paired with downlink CA configuration CA_3A-28A-41-42A there is no new interference components from 2 uplink operation which would interfere the downlink of Band 28 and Band 41.

.

7.11.1.4	∆TIB and ∆RIB values

Comparing 1UL-4DL CA_3A-28A-41A-42A, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-4DL CA_3A-28A-41A-42A shall apply.

7.12	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 19 with 2 ULs

7.12.1	List of specific combination issues

7.12.1.1	Operating bands for CA

Table 7.12.1.1-1: Inter-band CA operating bands for CA_1A-3A-3A-19A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-3A-3A-19A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |

7.12.1.2	Channel bandwidths per operating band for CA

Table 7.12.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-3A-3A-19A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A-19A | CA_1A-3A, CA_1A-19A, CA_3A-19A | 1 |  |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |
|  |  | 19 |  |  |  | Yes | Yes | Yes |  |  |  |

7.12.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-19A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.12.1.4	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-3A-19A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.12.1.5	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-3A-19A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.12.1.6	MSD

As with 7.12.1.3, 7.12.1.4 and 7.12.1.5, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.13	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 21 with 2 ULs

7.13.1	List of specific combination issues

7.13.1.1	Operating bands for CA

Table 7.13.1.1-1: Inter-band CA operating bands for CA_1A-3A-3A-21A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-3A-3A-21A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |

7.13.1.2	Channel bandwidths per operating band for CA

Table 7.13.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-3A-3A-21A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A-21A | CA_1A-3A, CA_1A-21A, CA_3A-21A | 1 |  |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes | Yes | Yes |  |  |  |

7.13.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-21A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.13.1.4	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-3A-21A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.13.1.5	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-3A-21A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.13.1.6	MSD

As with 7.13.1.3, 7.13.1.4 and 7.13.1.5, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.14	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 42 with 2 ULs

7.14.1	List of specific combination issues

7.14.1.1	Operating bands for CA

Table 7.14.1.1-1: Inter-band CA operating bands for CA_1A-3A-3A-42A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-3A-3A-42A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

7.14.1.2	Channel bandwidths per operating band for CA

Table 7.14.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-3A-3A-42A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A-42A | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes | Yes |  |  |

7.14.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-42A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.14.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-3A-42A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.14.1.5	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-3A-42A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.14.1.6	MSD

As with 7.14.1.3, 7.14.1.4 and 7.14.1.5, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.15	LTE Advanced Carrier Aggregation: CA_1A-42A-42C with 2 ULs

7.15.1	List of specific combination issues

7.15.1.1	Operating bands for CA

Table 7.15.1.1-1: Inter-band CA operating bands for CA_1A-42A-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-42A-42C | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

7.15.1.2	Channel bandwidths per operating band for CA

Table 7.15.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-42A-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-42A-42C | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |

7.15.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42A-42C

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.15.1.4	MSD

As with 7.15.1.3, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.16	LTE Advanced Carrier Aggregation: CA_1A-42D with 2 ULs

7.16.1	List of specific combination issues

7.16.1.1	Operating bands for CA

Table 7.16.1.1-1: Inter-band CA operating bands for CA_1A-42D

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-42D | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

7.16.1.2	Channel bandwidths per operating band for CA

Table 7.16.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-42D

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-42D | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |

7.16.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42D

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.16.1.4	MSD

As with 7.16.1.3, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.17	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 19 and Band 21 with 2 ULs

7.17.1	List of specific combination issues

7.17.1.1	Operating bands for CA

Table 7.17.1.1-1: Inter-band CA operating bands for CA_3A-3A-19A-21A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-3A-19A-21A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |

7.17.1.2	Channel bandwidths per operating band for CA

Table 7.17.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-3A-19A-21A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-3A-19A-21A | CA_3A-19A, CA_3A-21A, CA_19A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |

7.17.1.3	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_3A-3A-19A-21A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.17.1.4	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_3A-3A-19A-21A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.17.1.5	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_3A-3A-19A-21A

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.17.1.6	MSD

As with 7.17.1.3, 7.17.1.4 and 7.17.1.5, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.18	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 42 and Band 42 with 2 ULs

7.18.1	List of specific combination issues

7.18.1.1	Operating bands for CA

Table 7.18.1.1-1: Inter-band CA operating bands for CA_3A-3A-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-3A-42C | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

7.18.1.2	Channel bandwidths per operating band for CA

Table 7.18.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-3A-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-3A-42C | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |

7.18.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-3A-42C

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.18.1.4	MSD

As with 7.18.1.3, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.19	LTE Advanced Carrier Aggregation: CA_3A-42A-42C with 2 ULs

7.19.1	List of specific combination issues

7.19.1.1	Operating bands for CA

Table 7.19.1.1-1: Inter-band CA operating bands for CA_3A-42A-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-42A-42C | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

7.19.1.2	Channel bandwidths per operating band for CA

Table 7.19.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-42A-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-42A-42C | CA_3A-42A | 3 |  |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |

7.19.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42A-42C

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.19.1.4	MSD

As with 7.19.1.3, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.20	LTE Advanced Carrier Aggregation: CA_3A-42D with 2 ULs

7.20.1	List of specific combination issues

7.20.1.1	Operating bands for CA

Table 7.20.1.1-1: Inter-band CA operating bands for CA_3A-42D

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-42D | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

7.20.1.2	Channel bandwidths per operating band for CA

Table 7.20.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-42D

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-42D | CA_3A-42A | 3 |  |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |

7.20.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42D

Co-existence studies of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

7.20.1.4	MSD

As with 7.20.1.3, the MSD of this 4DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

7.21	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 26 with 2 ULs

7.21.1	List of specific combination issues

7.21.1.1	Channel bandwidths per operating band for CA

Table 7.21.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-7A-26A | CA_1A-3A CA_1A-7A CA_1A-26A CA_3A-7A CA_3A-26A CA_7A-26A | 1 |  |  | Yes | Yes | Yes | Yes | 65 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |

7.21.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-26A

For 2UL / 4DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 7.21.1.2-1

Table 7.21.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

IMD5 of CA_1A-7A may fall into band 26 receiving band. However, this impact is already covered in 3DL/2UL CA_1A-7A-26A. Hence there is no further self-interference issue.

7.21.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-26A

For 2UL / 4DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 7.21.1.3-1

Table 7.21.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

IMD3 of CA_3A-7A may fall into band 26 receiving band. However, this impact is already covered in 3DL/2UL CA_3A-7A-26A. Hence there is no further self-interference issue.

7.21.1.4	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_1A-3A-7A-26A

For 2UL / 4DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 7.21.1.4-1

Table 7.21.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 971 | 2524 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1942 | 1722 | 5048 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3727 | 3432 | 873 | 1128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

IMD2 of CA_3A-26A may fall into band 7 receiving band. However, this impact is already covered in 3DL/2UL CA_3A-7A-26A. Hence there is no further self-interference issue.

7.21.1.5	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-3A-7A-26A

Table 7.21.1.5-1 lists B7+B26 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 7.21.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1756 | 3314 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 942 | 4151 | 4326 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

IMD3 and IMD5 of CA_7A-26A may fall into band 26 receiving band and IMD5 of CA_7A-26A may fall into band 7 receiving band. However, this impact is already covered in 2DL/2UL CA_7A-26A. Hence there is no further self-interference issue.

7.21.1.6	MSD

When uplink CA configurations CA_1A-7A, 3A-7A, 3A-26A and CA_7A-26A are paired with downlink CA configuration CA_1A-3A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the downlink of the bands 7 and 26. In addition, uplink CA configuration CA_1A-3A was already captured in Table 7.3.1A-0bA in TS 36.101.

However, these impacts are already covered in the fallback combinations as mentioned in previous section; no futher MSD analysis is needed.

7.21.1.7	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_1A-3A-7A-26A are applied.

7.22	LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 7 and Band 26 with 2 ULs

7.22.1	List of specific combination issues

7.22.1.1	Channel bandwidths per operating band for CA

Table 7.22.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-7A-7A-26A | CA_1A-7A CA_1A-26A CA_7A-26A | 1 |  |  | Yes | Yes | Yes | Yes | 65 | 0 |
|  |  | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |

7.22.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7A-7A-26A

For 2UL / 4DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 7.22.1.2-1

Table 7.22.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

IMD5 of CA_1A-7A may fall into band 26 receiving band. However, this impact is already covered in 3DL/2UL CA_1A-7A-26A. Hence there is no further self-interference issue.

7.22.1.3	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-7A-7A-26A

Table 7.22.1.3-1 lists B7+B26 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 7.22.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1756 | 3314 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 942 | 4151 | 4326 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

IMD3 and IMD5 of CA_7A-26A may fall into band 26 receiving band and IMD5 of CA_7A-26A may fall into band 7 receiving band. However, this impact were already covered in 2DL/2UL CA_7A-26A. Hence there is no further self-interference issue.

7.22.1.4	MSD

When uplink CA configurations CA_1A-7A and CA_7A-26A are paired with downlink CA configuration CA_1A-7A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the downlink of the bands 7 and 26.

However, these impacts are already covered in the fallback combinations as mentioned in previous section; no futher MSD analysis is needed.

7.22.1.5	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_1A-7A-7A-26A are applied.

7.23	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 7 and Band 26 with 2 ULs

7.23.1	List of specific combination issues

7.23.1.1	Channel bandwidths per operating band for CA

Table 7.23.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-7A-7A-26A | CA_3A-7A CA_3A-26A CA_7A-26A | 3 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |

7.23.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-7A-26A

For 2UL / 4DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 7.23.1.2-1

Table 7.23.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

IMD3 of CA_3A-7A may fall into band 26 receiving band. Also IMD4 of CA_3A-7A may fall into band 7 receiving band. However, this impact was already covered in 3DL/2UL CA_3A-7A-26A and 2DL/2UL CA_3A-7A. Hence there is no further self-interference issue.

7.23.1.3	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_3A-7A-7A-26A

For 2UL / 4DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 7.23.1.3-1

Table 7.23.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 971 | 2524 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1942 | 1722 | 5048 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3727 | 3432 | 873 | 1128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

IMD2 and IMD3 of CA_3A-26A may fall into band 7 receiving band. However, this impact is already covered in 3DL/2UL CA_3A-7A-26A. Hence there is no further self-interference issue.

7.23.1.4	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_3A-7A-7A-26A

Table 7.23.1.4-1 lists B7+B26 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 7.23.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1756 | 3314 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 942 | 4151 | 4326 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

IMD3 and IMD5 of CA_7A-26A may fall into band 26 receiving band and IMD5 of CA_7A-26A may fall into band 7 receiving band. However, this impact were already covered in 2DL/2UL CA_7A-26A. Hence there is no further self-interference issue.

7.23.1.5	MSD

When uplink CA configurations CA_3A-7A, CA_3A-26A and CA_7A-26A are paired with downlink CA configuration CA_3A-7A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the downlink of the bands 7 and 26.

However, these impacts are already covered in the fallback combinations as mentioned in previous section; no futher MSD analysis is needed.

7.23.1.6	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_3A-7A-7A-26A are applied.

7.24	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 7 with 2ULs

7.24.1	List of specific combination issues

7.24.1.1	Channel bandwidths per operating band for CA

Table 7.24.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7C | CA_1A-3A CA_1A-7A CA_3A-7A CA_7C | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 1 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 7.24.1.1-2: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
| CA_7C | 15 | 10, 15 | 40 | 2 |
|  | 20 | 15, 20 |  |  |

7.24.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.2-1

Table 7.24.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101

7.24.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.3-1

Table 7.24.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

Based on the results from Table 7.24.1.3-1, there is no harmonics and IMDs products that fall into own Rx band.

7.24.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.24.1.4-1

Table 7.24.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

Based on the results from Table 7.24.1.4-1, there is no harmonics and IMDs products that fall into own Rx band 1.

7.24.1.5	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-3A-7C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 7.24.1.5-1

Table 7.24.1.5-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

From the harmonics analysis table, there was no harmonics from Band 7 uplinks which would desensitize the 3rd own Rx frequency of band 1 or band 3.

Frequency separation of Band 1 downlink and B7 uplink is 330 MHz and Band 3 downlink and B7 uplink is 835MHz, therefore there are no impact from Band 7 uplink CA to the Band 1 and Band 3 reception.

7.24.1.6		MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-7C there is no interference components from 2 uplink operation which would interfere downlink of the Band 7, no further MSD studies for this combination.

When uplink CA configurations CA_1A-7A is paired with downlink CA configuration CA_1A-7C there is no interference components from 2 uplink operation which would interfere downlink of the Band 3, no further MSD studies for this combination.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_1A-7C there is no interference components from 2 uplink operation which would interfere downlink of the Band 1 and Band 7, no further MSD studies for this combination.

When uplink CA configurations CA_7C is paired with downlink CA configuration CA_1A-7C there is no interference components from 2 uplink operation which would interfere downlink of the Band 1 and Band 3, no further MSD studies for this combination.

7.24.1.7		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_1A-3A-7C are applied.

7.25	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 7 with 2ULs

7.25.1	List of specific combination issues

7.25.1.1	Channel bandwidths per operating band for CA

Table 7.25.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3C-7A | CA_1A-3A CA_1A-7A CA_3A-7A CA_3C | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 1 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |

Table 7.25.1.1-2: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

7.25.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.2-1

Table 7.25.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101

7.25.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3C-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.3-1

Table 7.25.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

Based on the results from Table 7.25.1.3-1, there is no harmonics and IMDs products that fall into own Rx band.

7.25.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3C-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.25.1.4-1

Table 7.25.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

Based on the results from Table 7.25.1.4-1, there is no harmonics and IMDs products that fall into own Rx band 1.

7.25.1.5	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C-7A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 7.25.1.5-1

Table 7.25.1.5-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 |

From the harmonics analysis table, there was no harmonics from Band 3 uplinks which would desensitize the 3rd own Rx frequency of Band 1 or Band 7.

The close proximity issues between Band 3 uplink and Band 1 downlink already captured in Table 7.3.1A-0bA in TS36.101.

Frequency separation of Band 7 downlink and B3 uplink is 835 MHz, therefore there is no impact from Band 3 uplink CA to the Band 3 reception.

7.25.1.6		MSD

No MSD issues that need to be addressed.

7.25.1.7		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_1A-3C-7A are applied.

7.26	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 with 2ULs

7.26.1	List of specific combination issues

7.26.1.1	Channel bandwidths per operating band for CA

Table 7.26.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3C-7C | CA_3A-7A CA_3C CA_7C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 7.26.1.1-2: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

Table 7.26.1.1-3: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
| CA_7C | 15 | 10, 15 | 40 | 2 |
|  | 20 | 15, 20 |  |  |

7.26.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3C-7C

Co-existence study already existing in TR 36.715-00-02 Section 6.8

7.26.1.3	Co-existence studies for LTE-A UL CA_3C and DL CA_3C-7C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 7.26.1.3-1

Table 7.26.1.3-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 |

From the harmonics analysis table, there was no harmonics from Band 3 uplinks which would desensitize the 3rd own Rx frequency of band 7.

Frequency separation of Band 7 downlink and B3 uplink is 835 MHz, therefore there is no impact from Band 3 uplink CA to the Band 7 reception.

7.26.1.4	Co-existence studies for LTE-A UL CA_7C and DL CA_3C-7C

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 7.26.1.4-1

Table 7.26.1.4-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

From the harmonics analysis table, there was no harmonics from Band 7 uplinks which would desensitize the 3rd own Rx frequency of band 3.

Frequency separation of Band 3 downlink and B7 uplink is 620 MHz, therefore there are no impact from Band 7 uplink CA to the Band 3 reception.

7.26.1.5		MSD

No MSD issues that need to be addressed.

7.26.1.6		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_3C-7C are applied.

7.27			LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 28 with 2ULs

7.27.1	List of specific combination issues

7.27.1.1	Channel bandwidths per operating band for CA

Table 7.27.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-28A | CA_1A-7A CA_1A-3A CA_1A-28A CA_3A-7A CA_3A-28A CA_7A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |

7.27.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.2-1

Table 7.27.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

For CA_1A-7A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.27.1.2-1, the 5th order IMD products by Band 1 and Band 7 fall into the own Rx frequency of Band 28. However, the IMD problem will be covered in 3DL_CA_1A-7A-28A with 2UL_CA_1A-7A.

7.27.1.3	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-7A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.3-1

Table 7.27.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

For CA_1A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.27.1.4-1, the 2nd order IMD products by Band 1 and Band 28 fall into the own Rx frequency of Band 7. However, the IMD problem will be covered in 3DL_CA_1A-7A-28A with 2UL_CA_1A-28A.

7.27.1.4	Co-existence studies for LTE-A UL CA_1A-3A and CA_1A-3A-7A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.4-1

Table 7.27.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

For CA_1A-3A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.27.1.4-1;

- No issues with downlink Band 28 or Band 7.

- Close proximity issues already captured in TS 36.101 in Table 7.3.1A-0bA

7.27.1.5	Co-existence studies for LTE-A UL CA_3A-7A and CA_1A-3A-7A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.5-1

Table 7.27.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

For CA_3A-7A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.27.1.5-1;

- The 2nd order IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 28. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g.

7.27.1.6	Co-existence studies for LTE-A UL CA_3A-28A and CA_1A-3A-7A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.6-1

Table 7.27.1.6-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

For CA_3A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.27.1.6-1;

- There is self-interference on the 3rd IMD on Band 7, the IMD problem already covered in 3DL CA_3A-7A-28A with 2UL_CA_3A-28A.

7.27.1.7	Co-existence studies for LTE-A UL CA_7A-28A and CA_1A-3A-7A-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.27.1.7-1

Table 7.27.1.7-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1867 | 1752 | 3203 | 3318 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4252 | 4437 | 1164 | 1004 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5703 | 5888 | 3906 | 4066 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6752 | 7007 | 461 | 256 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8203 | 8458 | 4609 | 4814 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3504 | 3734 | 6406 | 6636 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 242 | 9577 | 9252 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5312 | 5562 | 10703 | 11028 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2756 | 3031 | 6304 | 6004 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7109 | 7384 | 8906 | 9206 |

For CA_7A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.27.1.7-1;

- The two-tone 2nd order IMD products by Band 7 and Band 28 fall into the own Rx frequency of Band 3. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g.

- The 3rd order harmonic products by Band 28 fall into the own Rx frequency of Band 1.

7.27.1.8		MSD

When uplink 2CA configurations CA_1A-7A is paired with downlink 4CA_1A-3A-7A-28A there are self-interference issues from concurrently supporting uplink 2CA which could interfere with downlink Band 28. This case is already covered in 3DL/2UL CA_1A-7A-28A, therefore the related MSD study will be covered in 3DL/2UL CA_1A-7A-28A.

When uplink 2CA configurations CA_1A-3A is paired with downlink 4CA_1A-3A-7A-28A there are no self-interference issues from concurrently supporting uplink 2CA. Therefore no further MSD study required.

When uplink 2CA configurations CA_1A-28A is paired with downlink 4CA_1A-3A-7A-28A there are self-interference issues from concurrently supporting uplink 2CA.

- The two-tone 5th order IMD products by Band 1 and Band 28 could interfere with Rx frequency of Band 3. This case is already covered in 3DL/2UL CA_1A-3A-28A in TS 36.101 Table 7.3.1A-0g.

- The 2nd order IMD products by Band 1 and Band 28 fall into the own Rx frequency of Band 7. This case is already covered in 3DL/2UL CA_1A-7A-28A, therefore no further MSD study required.

When uplink 2CA configurations CA_3A-7A is paired with downlink 4CA_1A-3A-7A-28A there are self-interference issues from concurrently supporting uplink 2CA.

- The 2nd order IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 28. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g. No further MSD study required.

- No IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 1, therefore no MSD study required.

When uplink 2CA configurations CA_3A-28A is paired with downlink 4CA_1A-3A-7A-28A there are self-interference issues from concurrently supporting uplink 2CA. There is self-interference on the 3rd IMD on Band 7, this impact already covered in 3DL_CA_3A-7A-28A with 2UL_CA_3A-28A.

When uplink 2CA configurations CA_7A-28A is paired with downlink 4CA_1A-3A-7A-28A there are self-interference issues from concurrently supporting uplink 2CA.

- The two tone 2nd order IMD products by Band 7 and Band 28 fall into the own Rx frequency of Band 3. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g. Therefore no further MSD study required.

- The 3rd order IMD products by Band 7 and Band 28 fall into the own Rx frequency of Band 1. This case is already covered in 3DL/2UL CA_1A-7A-28A, therefore no further MSD study required.

7.27.1.9		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_1A-3A-7A-28A are applied.

7.28			LTE Advanced Carrier Aggregation: Band 1 and Band 7 and Band 7 and Band 28 with 2ULs

7.28.1	List of specific combination issues

7.28.1.1	Channel bandwidths per operating band for CA

Table 7.28.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7C-28A | CA_1A-7A CA_7C CA_7A-28A CA_1A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |

Table 7.28.1.1-2: E-UTRA CA_7C configurations and bandwidth combination sets for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
| CA_7C | 15 | 10, 15 | 40 | 2 |
|  | 20 | 15, 20 |  |  |

7.28.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-7C-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.2-1

Table 7.28.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

For CA_1A-7A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.28.1.2-1, the 5th order IMD products by Band 1 and Band 7 fall into the own Rx frequency of Band 28. However, the IMD problem already covered in 3DL_CA_1A-7A-28A with 2UL_CA_1A-7A.

7.28.1.3	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-7C-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.3-1

Table 7.28.1.3-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

From the harmonics analysis table, there was no harmonics from Band 7 uplinks which would desensitize the 3rd own Rx frequency of Band 1 or Band 28.

7.28.1.4	Co-existence studies for LTE-A UL CA_7A-28A and DL CA_1A-7C-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.4-1

Table 7.28.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1867 | 1752 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4252 | 4437 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5703 | 5888 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6752 | 7007 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8203 | 8458 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3504 | 3734 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 242 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5312 | 5562 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2756 | 3031 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7109 | 7384 | 7166 | 7436 |

For CA_7A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 7.28.1.4-1, the 3rd order harmonic products by Band 28 fall into the own Rx frequency of Band 1. However, the harmonic problem was already covered in 2DL_CA_1A-28A.

7.28.1.5	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-7C-28A

For 2UL / 4DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 7.28.1.5-1

Table 7.28.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

For CA_1A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. From the table above, we see that there is self-interference on the 2nd order IMD on Band 7. However, the IMD problem will be covered in 3DL_1A-7A-28A with 2UL_CA_1A-28A.

7.28.1.6		MSD

When uplink 2CA configurations CA_1A-7A is paired with downlink 4CA_1A-7C-28A there are 5th order IMD self-interference issues from concurrently supporting uplink 2CA which could interfere with downlink Band 28. This case is already covered in 3DL/2UL CA_1A-7A-28A, therefore the related MSD study will be covered in 3DL/2UL CA_1A-7A-28A.

When uplink 2CA configurations CA_7C is paired with downlink 4CA_1A-7C-28A there are no self-interference issues from concurrently supporting uplink 2CA.

When uplink 2CA configurations CA_7A-28A is paired with downlink 4CA_1A-7C-28A there are 3rd harmonic self-interference issues from Band 28uplink which could interfere with downlink Band 1. This case is already covered in 2DL_CA_1A-28A.

When uplink 2CA configurations CA_1A-28A is paired with downlink 4CA_1A-7C-28A there are 2nd order IMD self-interference issues from concurrently supporting uplink 2CA which could interfere with downlink Band 7. This case is already covered in 3DL/2UL CA_1A-7A-28A, therefore no further MSD study required.

7.28.1.7		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 4DL/1UL CA_1A-7C-28A are applied.

8		5DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

8.1	LTE Advanced Carrier Aggregation: Band 1 and Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs

8.1.1	List of specific combination issues

8.1.1.1	Channel bandwidths per operating band for CA

Table 8.1.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-41C-42C | CA_42C CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 41 | See CA_41C BCS0 in TS36.101 |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C BCS1 in TS36.101 |  |  |  |  |  |  |  |

8.1.1.2	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-41C-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.2-1.

Table 8.1.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1420 | 1680 | 5320 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 240 | 560 | 4820 | 5280 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7240 | 7560 | 8720 | 9180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2160 | 2540 | 8220 | 8880 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9160 | 9540 | 12120 | 12780 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3360 | 2840 | 10640 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12480 | 11620 | 4520 | 4080 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15520 | 16380 | 11080 | 11520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6960 | 6240 | 860 | 1440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14040 | 14760 | 12560 | 13140 |

The 4th IMD falls into the own Rx of Band 1 and Band 41.  As already reported in release 14 version of TR36.714-00-02, the 4th IMD never impacts Band 1 Rx because Tx-Rx separation is always kept in 190MHz.  Therefore, specifying MSD requirement is not necessary.  For 4th IMD impact on Band 41 Rx, it analysis is captured in the MSD section.

8.1.1.3	Co-existence studies for LTE-A UL CA_42C and DL CA_1A-41C-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.1.1.3-1

Table 8.1.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of bands 1 and 41.

8.1.1.4	MSD

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-41C-42C, RAN4 needs to consider 4th IMD impact on Band 41.  However, when simultaneous Tx/Rx is not considered for this CA configuration (namely, Band 41 and Band 42 are synchronized), 4th IMD never causes desensitization in Band 41 Rx.  Since CA_1A-41C-42C was specified with the assumption that simultaneous Tx/Rx is not occurred among TDD bands 41 and 42, it is reasonable to apply same assumption for this combination.

8.1.1.5	∆TIB and ∆RIB values

Comparing 1UL-5DL CA_1A-41C-42C, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-5DL CA_1A-41C-42C shall apply.

8.2	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs

8.2.1	List of specific combination issues

8.2.1.1	Channel bandwidths per operating band for CA

Table 8.2.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-41C-42C | CA_42C | 3 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in TS36.101 |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 |  |  |  |  |  |  |  |

8.2.1.2	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-41C-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 8.2.1.2-1

Table 8.2.1.2-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of bands 3 and 41.

8.2.1.3	MSD

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_3A-41C-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3 and Band 41.

8.2.1.4	∆TIB and ∆RIB values

Comparing with 1UL-5DL CA_3A-41C-42C, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-5DL CA_3A-41C-42C shall apply.

8.3	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 41 and Band 42 and Band 42 with 2 ULs

8.3.1	List of specific combination issues

8.3.1.1	Channel bandwidths per operating band for CA

Table 8.3.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_28A-41C-42C | CA_42C | 28 |  |  | Yes | Yes |  |  | 90 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in TS36.101 |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 |  |  |  |  |  |  |  |

8.3.1.2	Co-existence studies for LTE-A UL CA_42C and DL CA_28A-41C-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 8.3.1.2-1

Table 8.3.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of bands 28 and 41.

8.3.1.3	MSD

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_28A-41C-42C there is no interference components from 2 uplink operation which would interfere the downlink of the Band 28 and Band 41. .

8.3.1.4	∆TIB and ∆RIB values

Comparing 1UL-5DL CA_28A-41C-42C, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-5DL CA_28A-41C-42C shall apply.

8.4	LTE Advanced Carrier Aggregation: Band 3 and Band 28 and Band 41 and Band 42 and Band 42 with 2 ULs

8.4.1	List of specific combination issues

8.4.1.1	Channel bandwidths per operating band for CA

Table 8.4.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-28A-41A-42C | CA_42C | 3 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 |  |  |  |  |  |  |  |

8.4.1.2	Co-existence studies for LTE-A UL CA_42C and DL CA_3A-28A-41A-42C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.4.1.2-1

Table 8.4.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 3400 | 3600 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6800 | 7200 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10200 | 10800 |

From the harmonics analysis table, there was no harmonics from Band 42 uplinks which would desensitize the 3rd own Rx frequency of bands 3, 28 and 41.

8.4.1.3	MSD

When uplink CA configurations CA_42C is paired with downlink CA configuration CA_3A-28A-41A-42C, there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3, Band 28 and Band 41.

8.4.1.4	∆TIB and ∆RIB values

Comparing 1UL-5DL CA_3A-28A-41A-42C, no difference is expected from UE architecture point of view for this combination, same values for ∆TIB and ∆RIB as 1UL-5DL CA_3A-28A-41A-42C shall apply.

8.5	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 19 and Band 21 with 2 ULs

8.5.1	List of specific combination issues

8.5.1.1		Operating bands for CA

Table 8.5.1.1-1: Inter-band CA operating bands for CA_1A-3A-3A-19A-21A

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-3A-3A-19A-21A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |

8.5.1.2		Channel bandwidths per operating band for CA

Table 8.5.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-3A-3A-19A-21A

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-28A-41A-42C | CA_42C | 3 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in TS36.101 |  |  |  |  |  |  |  |

8.5.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-19A-21A

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.5.1.4	Co-existence studies for LTE-A UL CA_1A-19A and DL CA_1A-3A-3A-19A-21A

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.5.1.5	Co-existence studies for LTE-A UL CA_1A-21A and DL CA_1A-3A-3A-19A-21A

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.5.1.6	Co-existence studies for LTE-A UL CA_3A-19A and DL CA_1A-3A-3A-19A-21A

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.5.1.7	Co-existence studies for LTE-A UL CA_3A-21A and DL CA_1A-3A-3A-19A-21A

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.5.1.8	Co-existence studies for LTE-A UL CA_19A-21A and DL CA_1A-3A-3A-19A-21A

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.5.1.9	MSD

As with 8.5.1.3 ~ 8.5.1.8, the MSD of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

8.6	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 42 and Band 42 with 2 ULs

8.6.1	List of specific combination issues

8.6.1.1		Operating bands for CA

Table 8.6.1.1-1: Inter-band CA operating bands for CA_1A-3A-3A-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-3A-3A-42C | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

8.6.1.2		Channel bandwidths per operating band for CA

Table 8.6.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-3A-3A-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-3A-42C | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |

8.6.1.3	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-3A-42C

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.6.1.4	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-3A-3A-42C

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.6.1.5	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_1A-3A-3A-42C

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.5.1.6	MSD

As with 8.6.1.3 ~ 8.6.1.5, the MSD of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

8.7	LTE Advanced Carrier Aggregation: CA_1A-42E with 2 ULs

8.7.1	List of specific combination issues

8.7.1.1		Operating bands for CA

Table 8.7.1.1-1: Inter-band CA operating bands for CA_1A-42E

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-42E | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

8.7.1.2		Channel bandwidths per operating band for CA

Table 8.7.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-42E

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-42E | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |

8.7.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42E

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.7.1.4	MSD

As with 8.7.1.3, the MSD of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

8.8	LTE Advanced Carrier Aggregation: CA_1A-42C-42C with 2 ULs

8.8.1	List of specific combination issues

8.8.1.1		Operating bands for CA

Table 8.8.1.1-1: Inter-band CA operating bands for CA_1A-42C-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1A-42C-42C | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

8.8.1.2		Channel bandwidths per operating band for CA

Table 8.8.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_1A-42C-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-42C-42C | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |

8.8.1.3	Co-existence studies for LTE-A UL CA_1A-42A and DL CA_1A-42C-42C

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.8.1.4	MSD

As with 8.8.1.3, the MSD of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

8.9	LTE Advanced Carrier Aggregation: CA_3A-42E with 2 ULs

8.9.1	List of specific combination issues

8.9.1.1		Operating bands for CA

Table 8.9.1.1-1: Inter-band CA operating bands for CA_3A-42E

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-42E | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

8.9.1.2		Channel bandwidths per operating band for CA

Table 8.9.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-42E

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-42E | CA_3A-42A | 3 |  |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |  |

8.9.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42E

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.9.1.4	MSD

As with 8.9.1.3, the MSD of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

8.10	LTE Advanced Carrier Aggregation: CA_3A-42C-42C with 2 ULs

8.10.1	List of specific combination issues

8.10.1.1		Operating bands for CA

Table 8.10.1.1-1: Inter-band CA operating bands for CA_3A-42C-42C

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-42C-42C | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

8.10.1.2		Channel bandwidths per operating band for CA

Table 8.10.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-42C-42C

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-42C-42C | CA_3A-42A | 3 |  |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |  |

8.10.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-42C-42C

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.10.1.4	MSD

As with 8.10.1.3, the MSD of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

8.11	LTE Advanced Carrier Aggregation: CA_3A-3A-42D with 2 ULs

8.11.1	List of specific combination issues

8.11.1.1		Operating bands for CA

Table 8.11.1.1-1: Inter-band CA operating bands for CA_3A-3A-42D

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_3A-3A-42D | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

8.11.1.2		Channel bandwidths per operating band for CA

Table 8.11.1.2-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-3A-42D

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-3A-42D | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS36.101 |  |  |  |  |  |  |  |

8.11.1.3	Co-existence studies for LTE-A UL CA_3A-42A and DL CA_3A-3A-42D

Co-existence studies of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional study is needed.

8.11.1.4	MSD

As with 8.11.1.3, the MSD of this 5DL/2UL CA configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in this TR. Therefore, no additional MSD requirement is needed.

8.12	LTE Advanced Carrier Aggregation: Band 3 and Band 3 and Band 7 and Band 7 and Band 8 with 2 ULs

8.12.1	List of specific combination issues

8.12.1.1		Channel bandwidths per operating band for CA

Table 8.12.1.1-1: E-UTRA CA configuration and bandwidth combination sets for CA_3A-3A-7A-7A-8A

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-3A-7A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 3 | See CA_3A-3A BCS0 in Table 8.12.1.1-2 |  |  |  |  |  | 90 | 0 |
|  |  | 7 | See CA_7A-7A BCS1 in Table 8.12.1.1-3 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 | See CA_3A-3A BCS1 in Table 8.12.1.1-2 |  |  |  |  |  | 70 | 1 |
|  |  | 7 | See CA_7A-7A BCS2 in Table 8.12.1.1-3 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

Table 8.12.1.1-2: E-UTRA CA_3A-3A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10, 15, 20 |  |  | 30 | 1 |

Table 8.12.1.1-3: E-UTRA CA_7A-7A configurations and bandwidth combination sets for intra-band CA

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_7A-7A | - | 5 | 15 |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  | 40 | 3 |

8.12.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-3A-7A-7A-8A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.12.1.2-1

Table 8.12.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 3rd IMD falls into the own Rx band 8. But this impact is already covered in 2UL/3DL CA_3A-7A-8A. So there is no further self-interference issue.

The 4th IMD falls into the own Rx band 7. But this impact is already covered in 2UL/2DL CA_3A-7A. So there is no further self-interference issue.

8.12.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-3A-7A-7A-8A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.12.1.3-1

Table 8.12.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics of Band 8, 4th & 5th IMDs fall into the own Rx frequency of Band 3. And also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. But this impact is already covered in 2UL/2DL CA_3A-8A.

The 3rd harmonics of Band 8 and 2nd &3rd IMDs will interfere own Rx frequency of Band 7. And this impact is already covered in 2UL/3DL CA_3A-7A-8A.So there is no further self-interference issue.

8.12.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-3A-7A-7A-8A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.12.1.4-1

Table 8.12.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 670 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3380 | 3170 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9400 | 9085 | 1160 | 950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5670 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

The 3nd harmonics of Band 8 fall into the own Rx frequency of Band 7. But this impact is already covered in 2DL/1UL CA_7A-8A.

The 5th IMD fall into the own Rx frequency of Band 8. However this impact is already covered in 2DL/2UL CA_7A-8A. Hence there is no further self-interference issue.

8.12.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-3A-7A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 8 and Band 7.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-3A-7A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 3, Band 7 and Band 8.

And also when uplink CA configurations CA_7A-8A is paired with downlink CA configuration CA_3A-3A-7A-7A-8A there are interference components from 2 uplink operation which would interfere the downlink of the Band 8.

However, these impacts are already covered in the fallback combinations as mentioned in the previous section; no further MSD analysis is needed.

8.12.1.6	∆TIB and ∆RIB values

8.13	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 7 and Band 26 with 2 ULs

8.13.1	List of specific combination issues

8.13.1.1	Channel bandwidths per operating band for CA

Table 8.13.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-7A-7A-26A | CA_1A-3A CA_1A-7A CA_1A-26A CA_3A-7A CA_3A-26A CA_7A-26A | 1 |  |  | Yes | Yes | Yes | Yes | 95 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |

8.13.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7A-7A-26A

For 2UL / 5DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 8.13.1.2-1

Table 8.13.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

IMD5 of CA_1A-7A may fall into band 26 receiving band. However, this impact is already covered in 3DL/2UL CA_1A-7A-26A. Hence there is no further self-interference issue.

8.13.1.3	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7A-7A-26A

For 2UL / 5DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 8.13.1.3-1

Table 8.13.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

IMD3 of CA_3A-7A may fall into band 26 receiving band. However, this impact is already covered in 3DL/2UL CA_3A-7A-26A. Hence there is no further self-interference issue.

8.13.1.4	Co-existence studies for LTE-A UL CA_3A-26A and DL CA_1A-3A-7A-7A-26A

For 2UL / 5DL own receiver desensitization study 5th order intermodulation product was calculated and presented in Table 8.13.1.4-1

Table 8.13.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 971 | 2524 | 2634 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1942 | 1722 | 5048 | 5268 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3727 | 3432 | 873 | 1128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

IMD2 of CA_3A-26A may fall into band 7 receiving band. However, this impact is already covered in 3DL/2UL CA_3A-7A-26A. Hence there is no further self-interference issue.

8.13.1.5	Co-existence studies for LTE-A UL CA_7A-26A and DL CA_1A-3A-7A-7A-26A

Table 8.13.1.5-1 lists B7+B26 2UL CA 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 8.13.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1756 | 3314 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 942 | 4151 | 4326 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

IMD3 and IMD5 of CA_7A-26A may fall into band 26 receiving band and IMD5 of CA_7A-26A may fall into band 7 receiving band. . However, this impact is already covered in 2DL/2UL CA_7A-26A. Hence there is no further self-interference issue.

8.13.1.6	MSD

When uplink CA configurations CA_1A-7A, CA_3A-7A, CA_3A-26A and CA_7A-26A are paired with downlink CA configuration CA_1A-3A-7A-7A-26A there are self-interference problems from 2 uplink operation which would interfere the downlink of the bands 7 and 26. In addition, uplink CA configuration CA_1A-3A was already captured in Table 7.3.1A-0bA in TS 36.101.

However, these impacts are already covered in the fallback combinations as mentioned in previous section; no futher MSD analysis is needed.

8.13.1.7	∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 5DL/1UL CA_1A-3A-7A-7A-26A are applied.

8.14	LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 3 and Band 7 and Band 7 with 2ULs

8.14.1	List of specific combination issues

8.14.1.1	Channel bandwidths per operating band for CA

Table 8.14.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3C-7C | CA_1A-3A CA_1A-7A CA_3A-7A CA_3C CA_7C | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-3C-7C | CA_1A-3A CA_1A-7A CA_3A-7A CA_3C CA_7C | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 1 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 8.14.1.1-2: E-UTRA CA_3C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_3C | 5, 10, 15 | 20 | 40 | 0 |
|  | 20 | 5, 10, 15, 20 |  |  |

Table 8.14.1.1-3: E-UTRA CA_7C configurations and bandwidth combination sets defined for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
| CA_7C | 15 | 10, 15 | 40 | 2 |
|  | 20 | 15, 20 |  |  |

8.14.1.2	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3C-7C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.14.1.2-1

Table 8.14.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Close proximity issues already captured in Table 7.3.1A-0bA in TS36.101

8.14.1.3	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3C-7C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.14.1.3-1

Table 8.14.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

Based on the results from Table 8.14.1.3-1, there is no harmonics and IMDs products that fall into own Rx band.

8.14.1.4	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3C-7C

Table 8.14.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

8.14.1.5	Co-existence studies for LTE-A UL CA_3C and DL CA_1A-3C-7C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 8.14.1.5-1

Table 8.14.1.5-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 |

From the harmonics analysis table, there was no harmonics from Band 3 uplinks which would desensitize the 3rd own Rx frequency of band 1 or band 3 or band 7.

The close proximity issues between B3 uplink and B1 reception already captured in Table 7.3.1A-0bA in TS36.101

Frequency separation of Band 7 downlink and B3 uplink is 835 MHz, therefore there is no impact from Band 3 uplink CA to the Band 7 reception.

8.14.1.6	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-3C-7C

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics were calculated and presented in Table 8.14.1.6-1

Table 8.14.1.6-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

From the harmonics analysis table, there was no harmonics from Band 7 uplinks which would desensitize the 3rd own Rx frequency of band 1 or band 3.

Frequency separation of Band 3 downlink and B7 uplink is 620 MHz and B1 downlink and B7 uplink is 330MHz, therefore there are no impact from Band 7 uplink CA to the Band 1 or Band 3 reception.

8.14.1.7		MSD

No MSD issues that need to be addressed.

8.14.1.8		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 5DL/1UL CA_1A-3C-7C are applied.

8.15			LTE Advanced Carrier Aggregation: Band 1 and Band 3 and Band 7 and Band 7 and Band 28 with 2ULs

8.15.1	List of specific combination issues

8.15.1.1	Channel bandwidths per operating band for CA

Table 8.15.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7C-28A | CA_1A-3A CA_1A-7A CA_3A-7A CA_7C CA_7A-28A CA_1A-28A CA_3A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |

Table 8.15.1.1-2: E-UTRA CA_7C configurations and bandwidth combination sets for intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_7C | 10 | 20 | 40 | 1 |
|  | 15 | 15, 20 |  |  |
|  | 20 | 10, 15, 20 |  |  |
| CA_7C | 15 | 10, 15 | 40 | 2 |
|  | 20 | 15, 20 |  |  |

8.15.1.2	Co-existence studies for LTE-A UL CA_1A-7A and DL CA_1A-3A-7C-28A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.15.1.2-1

Table 8.15.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

For CA_1A-7A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission.

Based on the results from Table 8.15.1.2-1, there are no issues with Rx frequency Band 3, the 5th order IMD products by Band 1 and Band 7 fall into the own Rx frequency of Band 28. This case is already covered in 3DL/2UL CA_1A-7A-28A.

8.15.1.3	Co-existence studies for LTE-A UL CA_7C and DL CA_1A-3A-7C-28A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.15.1.3-1

Table 8.15.1.3-1: Harmonic analysis

| UE UL carriers | fx_low | fx_high |
| --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 |

From the harmonics analysis table, there was no harmonics from Band 7 uplinks which would desensitize the 3rd own Rx frequency of Band 1, Band 3 or Band 28.

8.15.1.4	Co-existence studies for LTE-A UL CA_1A-28A and DL CA_1A-3A-7C-28A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.15.1.4-1

Table 8.15.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

For CA_1A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 8.15.1.4-1;

- The two-tone 5th order IMD products by Band 1 and Band 28 fall into the own Rx frequency of Band 3. This case is already covered in 3DL/2UL CA_1A-3A-28A in TS 36.101 Table 7.3.1A-0g.

- The 2nd order IMD products by Band 1 and Band 28 fall into the own Rx frequency of Band 7. This case is already covered in 3DL/2UL CA_1A-7A-28A.

8.15.1.5	Co-existence studies for LTE-A UL CA_7A-28A and DL CA_1A-3A-7C-28A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.15.1.5-1

Table 8.15.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1867 | 1752 | 3203 | 3318 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4252 | 4437 | 1164 | 1004 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5703 | 5888 | 3906 | 4066 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6752 | 7007 | 461 | 256 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8203 | 8458 | 4609 | 4814 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3504 | 3734 | 6406 | 6636 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 242 | 9577 | 9252 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5312 | 5562 | 10703 | 11028 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2756 | 3031 | 6304 | 6004 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7109 | 7384 | 8906 | 9206 |

For CA_7A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 8.15.1.5-1;

- The two tone 2nd order IMD products by Band 7 and Band 28 fall into the own Rx frequency of Band 3. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g.

- The 3rd order harmonic product by Band 28 fall into the own Rx frequency of Band 1. This case is already covered in 2DL_CA_1A-28A.

8.15.1.6	Co-existence studies for LTE-A UL CA_1A-3A and DL CA_1A-3A-7C-28A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.15.1.6-1

Table 8.15.1.6-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

For CA_1A-3A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 8.15.1.6-1;

- No issues with downlink Band 28 or Band 7.

- Close proximity issues already captured in TS 36.101 in Table 7.3.1A-0bA

8.15.1.7	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_1A-3A-7C-28A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.15.1.7-1

Table 8.15.1.7-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

For CA_3A-7A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 8.15.1.7-1;

- The 2nd order IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 28. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g.

- No IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 1.

8.15.1.8	Co-existence studies for LTE-A UL CA_3A-28A and DL CA_1A-3A-7C-28A

For 2UL / 5DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 8.15.1.8-1

Table 8.15.1.8-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

For CA_3A-28A we analyse the impact of the receiving band with the harmonics and IMD products from the dual uplink transmission. Based on the results from Table 8.15.1.8-1;

- There is self-interference on the 3rd IMD on Band 7, it was already covered in 3DL_CA_3A-7A-28A with 2UL_CA_3A-28A.

- There is self-interference on the 4th IMD on Band 1, it was already covered in 3DL_CA_1A-3A-28A with 2UL_CA_3A-28A.

- The 3rd order harmonic product by Band 28 fall into the own Rx frequency of Band 1. This case is already covered in 2DL_CA_1A-28A.

8.15.1.9		MSD

When uplink 2CA configurations CA_1A-7A is paired with downlink 5CA_1A-3A-7C-28A there are self-interference issues from concurrently supporting uplink 2CA which could interfere with downlink Band 28. This case is already covered in 3DL/2UL CA_1A-7A-28A, therefore the related MSD study will be covered in 3DL/2UL CA_1A-7A-28A.

When uplink 2CA configurations CA_7C is paired with downlink 5CA_1A-3A-7C-28A there are no self-interference issues from concurrently supporting uplink 2CA. Therefore no further MSD study required.

When uplink 2CA configurations CA_1A-28A is paired with downlink 5CA_1A-3A-7C-28A there are self-interference issues from concurrently supporting uplink 2CA.

- The two-tone 5th order IMD products by Band 1 and Band 28 could interfere with Rx frequency of Band 3. This case is already covered in 3DL/2UL CA_1A-3A-28A in TS 36.101 Table 7.3.1A-0g. Therefore no further MSD study required.

- The 2nd order IMD products by Band 1 and Band 28 fall into the own Rx frequency of Band 7. This case is already covered in 3DL/2UL CA_1A-7A-28A, therefore no further MSD study required.

When uplink 2CA configurations CA_7A-28A is paired with downlink 5CA_1A-3A-7C-28A there are self-interference issues from concurrently supporting uplink 2CA.

- The two tone 2nd order IMD products by Band 7 and Band 28 fall into the own Rx frequency of Band 3. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g.

- The 3rd order harmonic product by Band 28 fall into the own Rx frequency of Band 1. This case is already covered in 2DL_CA_1A-28A, therefore no further MSD study required.

When uplink 2CA configurations CA_1A-3A is paired with downlink 5CA_1A-3A-7C-28A there are no self-interference issues from concurrently supporting uplink 2CA. Therefore no further MSD study required.

When uplink 2CA configurations CA_3A-7A is paired with downlink 5CA_1A-3A-7C-28A there are self-interference issues from concurrently supporting uplink 2CA.

- The 2nd order IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 28. This case is already covered in 3DL/2UL CA_3A-7A-28A in TS 36.101 Table 7.3.1A-0g., no further MSD study required.

- No IMD products by Band 3 and Band 7 fall into the own Rx frequency of Band 1, therefore no MSD study required.

When uplink 2CA configurations CA_3A-28A is paired with downlink 5CA_1A-3A-7C-28A there are self-interference issues from concurrently supporting uplink 2CA.

- There is self-interference on the 3rd IMD on Band 7, it was already covered in 3DL_CA_3A-7A-28A with 2UL_CA_3A-28A, therefore, no further MSD study required.

- There is self-interference on the 4th IMD on Band 1, it was already covered in 3DL_CA_1A-3A-28A with 2UL_CA_3A-28A, therefore, no further MSD study required.

- The 3rd order harmonic product by Band 28 fall into the own Rx frequency of Band 1. This case is already covered in 2DL_CA_1A-28A, therefore, no further MSD study required

8.15.1.10		∆TIB and ∆RIB values

Same values for ∆TIB and ∆RIB of 5DL/1UL CA_1A-3A-7C-28A are applied.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2017-08 | 3GPP RAN4#84 | R4-1708080 |  |  | TR36.715-00-02 Skeleton for xDL/2UL CA | N/A | 0.0.1 |
| 2017-10 | 3GPP RAN4 #84BIS | R4-1710732 |  |  | Updated TR36.715-00-02 v0.1.0 - Approved TPs in RAN4 #84 R4-1707132, "TP for TR 36.715-00-02: CA_3A-7A-20A-32A with 2 ULs channel bandwidths per operating band," R4-1707133, "TP for TR 36.715-00-02: CA_3A-7A-32A with 2 ULs channel bandwidths per operating band," R4-1707134, "TP for TR 36.715-00-02: CA_3A-20A-32A with 2 ULs channel bandwidths per operating band," R4-1707135, "TP for TR 36.715-00-02: CA_7A-20A-32A with 2 ULs channel bandwidths per operating band," R4-1707428, "TP for TR36.715-00-02: requirements for CA_1A-41A-42A," R4-1707429, "TP for TR36.715-00-02: requirements for CA_1A-41A-42C," R4-1707430, "TP for TR36.715-00-02: requirements for CA_1A-41C-42A," R4-1707431, "TP for TR36.715-00-02: requirements for CA_1A-41C-42C," R4-1707860, "TP for TR 36.715-00-02: Coexistence studies for CA_3A-3A-7A 3DL/2UL CA," R4-1707863, "TP for TR 36.715-00-02: Coexistence studies for CA_7A-7A-8A 3DL/2UL CA," R4-1708061, "TP for TR 36.715-00-02: operating bands, channel bandwidths for CA_4DL_3A-28A-40C_2UL_3A-28A_BCS0," R4-1708062, "TP for TR 36.715-00-02: co-existence studies, UE requirements relaxation for CA_4DL_3A-28A-40C_2UL_3A-28A_BCS0," R4-1708063, "TP for TR 36.715-00-02: operating bands, channel bandwidths for CA_3DL_3A-28A-40A_2UL_3A-28A_BCS0," R4-1708064, "TP for TR 36.715-00-02: co-existence studies, UE requirements relaxation for CA_3DL_3A-28A-40A_2UL_3A-28A_BCS0," R4-1708081, "TP on the general part for xDL/2UL CA," R4-1708088, "Revised WD on xD//2UL inter-band CA in rel-15," R4-1708733, "TP for TR36.715-00-02: Discussions to support CA_3DL_3A-41C_2UL_3A-41A_BCS0," R4-1708734, "TP for TR36.715-00-02: Discussions to support CA_3DL_3A-41A-42A_2UL_3A-41A_BCS0," R4-1708735, "TP for TR36.715-00-02: Discussions to support CA_3DL_3A-28A-41A_2UL_3A-41A_BCS0," R4-1708736, "self-desense analysis for xDL/2UL CA in rel-15," R4-1708741, "TP for TR 36.715-00-02: Co-existence studies for CA_3A-7A-20A-32A with 2 ULs," R4-1708799, "MSD Test configurations for xDL/2UL CA band combinations with self interference issues," | 0.0.1 | 0.1.0 |
| 2017-11 | 3GPP RAN4 #85 | R4-1713034 |  |  | Updated TR36.715-00-02 v0.2.0 - Approved TPs in RAN4 #84BIS R4-1710166, "TP for TR36.715-00-02: Discussions to support CA_4DL_3A-41C-42A_2UL_3A-41A_BCS0," SoftBank Corp. R4-1710167, "TP for TR36.715-00-02: Discussions to support CA_4DL_3A-41A-42C_2UL_3A-41A_BCS0," SoftBank Corp. R4-1710168, "TP for TR36.715-00-02: Discussions to support CA_4DL_3A-28A-41C_2UL_3A-41A_BCS0," SoftBank Corp. R4-1710169, "TP for TR36.715-00-02: Discussions to support CA_4DL_3A-28A-41A-42A_2UL_3A-41A_BCS0," SoftBank Corp. R4-1710741, "Self-desense analysis for new xDL/2UL CA band combinations in rel-15," LG Electronics France R4-1710778, "TP for TR 36.715-00-02: Coexistence studies for CA_3A-7A-7A-8A," CHTTL R4-1710780, "TP for TR 36.715-00-02: Coexistence studies for CA_3A-3A-7A-8A 4DL/2UL CA," CHTTL R4-1710782, "TP for TR 36.715-00-02: Coexistence studies for CA_3A-3A-7A-7A 4DL/2UL CA," CHTTL R4-1711335, "TP for TR 36.715-00-02: Co-existence, MSD and ?TIB/?RIB studies for CA_3A-7A-32A, CA_3A-20A-32A, CA_7A-20A-32A and CA_3A-7A-20A-32A with 2 ULs," TELECOM ITALIA S.p.A. R4-1711523, "TP for TR36.715-00-02: Discussions to support CA_5DL_3A-41C-42C_2UL_42C_BCS0," SoftBank Corp. R4-1711524, "TP for TR36.715-00-02: Discussions to support CA_5DL_28A-41C-42C_2UL_42C_BCS0," SoftBank Corp. R4-1711525, "TP for TR36.715-00-02: Discussions to support CA_5DL_3A-28A-41A-42C_2UL_42C_BCS0," SoftBank Corp. R4-1711545, "TP for TR 36.715-00-02: MSD study for CA_3DL_3A-7A-32A_2UL_3A-7A_BCS0," Huawei, HiSilicon R4-1711546, "TP for MSD for CA_3A-28A-41A," LG Electronics France | 0.1.0 | 0.2.0 |
| 2018-02 | 3GPP RAN4 #86 | R4-1801650 |  |  | Updated TR36.715-00-02 v0.3.0 - Approved TPs in RAN4 #85 R4-1713040, "MSD test results for xDL/2UL CA w/ self-interference problems in rel-15," LG Electronics. R4-1713039, "TP on summry of interference studies for xDL/2UL CA in rel-15," LG Electronics. R4-1712511, "TP for TR 36.715-00-02: Coexistence studies for CA_3A-3A-7A-7A-8A 5DL/2UL CA," CHTTL R4-1712826, "TP for TR 36.715-00-02: CA_3DL_1A-3A-3A_2UL_1A-3A_BCS0," NTT DOCOMO, INC. R4-1712827, "TP for TR 36.715-00-02: CA_3DL_3A-3A-19A_2UL_3A-19A_BCS0," NTT DOCOMO, INC. R4-1712828, "TP for TR 36.715-00-02 CA_3DL_3A-3A-21A_2UL_3A-21A_BCS0," NTT DOCOMO, INC. R4-1712829, "TP for TR 36.715-00-02 CA_3DL_3A-3A-42A_2UL_3A-42A_BCS0," NTT DOCOMO, INC. R4-1712830, "TP for TR 36.715-00-02 CA_3DL_1A-42A-42A_2UL_1A-42A_BCS0," NTT DOCOMO, INC. R4-1712831, "TP for TR 36.715-00-02 CA_3DL_3A-42A-42A_2UL_3A-42A_BCS0," NTT DOCOMO, INC. R4-1712832, "TP for TR 36.715-00-02 CA_4DL_1A-3A-3A-19A_2UL_BCS0," NTT DOCOMO, INC. R4-1712833, "TP for TR 36.715-00-02 CA_4DL_1A-3A-3A-21A_2UL_BCS0," NTT DOCOMO, INC. R4-1712834, "TP for TR 36.715-00-02 CA_4DL_1A-3A-3A-42A_2UL_BCS0," NTT DOCOMO, INC. R4-1712835, "TP for TR 36.715-00-02 CA_4DL_1A-42A-42C_2UL_1A-42A_BCS0," NTT DOCOMO, INC. R4-1712836, "TP for TR 36.715-00-02 CA_4DL_1A-42D_2UL_1A-42A_BCS0," NTT DOCOMO, INC. R4-1712837, "TP for TR 36.715-00-02 CA_4DL_3A-3A-19A-21A_2UL_BCS0," NTT DOCOMO, INC. R4-1712838, "TP for TR 36.715-00-02 CA_4DL_3A-3A-42C_2UL_3A-42A_BCS0," NTT DOCOMO, INC. R4-1712839, "TP for TR 36.715-00-02 CA_4DL_3A-42A-42C_2UL_3A-42A_BCS0," NTT DOCOMO, INC. R4-1712840, "TP for TR 36.715-00-02 CA_4DL_3A-42D_2UL_3A-42A_BCS0," NTT DOCOMO, INC. R4-1712841, "TP for TR 36.715-00-02 CA_5DL_1A-3A-3A-19A-21A_2UL_BCS0," NTT DOCOMO, INC. R4-1712842, "TP for TR 36.715-00-02 CA_5DL_1A-3A-3A-42C_2UL_BCS0," NTT DOCOMO, INC. R4-1712843, "TP for TR 36.715-00-02 CA_5DL_1A-42E_2UL_1A-42A_BCS0," NTT DOCOMO, INC. R4-1712844, "TP for TR 36.715-00-02 CA_5DL_1A-42C-42C_2UL_1A-42A_BCS0," NTT DOCOMO, INC. R4-1712845, "TP for TR 36.715-00-02 CA_5DL_3A-42E_2UL_3A-42A_BCS0," NTT DOCOMO, INC. R4-1712846, "TP for TR 36.715-00-02 CA_5DL_3A-42C-42C_2UL_3A-42A_BCS0," NTT DOCOMO, INC. R4-1712847, "TP for TR 36.715-00-02 CA_5DL_3A-3A-42D_2UL_3A-42A_BCS0," NTT DOCOMO, INC. | 0.2.0 | 0.3.0 |
| 2018-04 | 3GPP RAN4 #86BIS | R4-1803993 |  |  | Updated TR36.715-00-02 v0.4.0 - Approved TPs in RAN4 #86 R4-1803199, "TP for TR 36.715-00-02: Analysis for CA_7A-7A-26A," SK Telecom R4-1803200, "TP for TR 36.715-00-02: Analysis for CA_3A-7A-26A," SK Telecom R4-1803201, "TP for TR 36.715-00-02: Analysis for CA_1A-7A-26A," SK Telecom R4-1803202, "TP for TR 36.715-00-02: Analysis for CA_1A-3A-7A-26A," SK Telecom R4-1803203, "TP for TR 36.715-00-02: Analysis for CA_1A-7A-7A-26A," SK Telecom R4-1803204, "TP for TR 36.715-00-02: Analysis for CA_3A-7A-7A-26A," SK Telecom R4-1803205, "TP for TR 36.715-00-02: Analysis for CA_1A-3A-7A-7A-26A," SK Telecom R4-1802272, "TP for TR 36.715-00-02 CA_3DL_1A-7C," Telstra Corporation Limited R4-1802274, "TP for TR 36.715-00-02 CA_4DL_1A-3A-7C," Telstra Corporation Limited R4-1802275, "TP for TR 36.715-00-02 CA_4DL_1A-3C-7A," Telstra Corporation Limited R4-1802276, "TP for TR 36.715-00-02 CA_4DL_3C-7C," Telstra Corporation Limited R4-1802277, "TP for TR 36.715-00-02 CA_5DL_1A-3C-7C," Telstra Corporation Limited R4-1803178, "TP for TR 36.715-00-02: CA_3DL_3A-7A-28A," Telstra Corporation Limited R4-1802051, "TP for TR 36.715-00-02: CA_3DL_1A_3A-18A_2UL_BCS0," KDDI Corporation R4-1802054, "TP for TR 36.715-00-02: CA_3DL_3A_11A-18A_2UL_BCS0," KDDI Corporation R4-1801652, "TP on summary of interference studies for new xDL/2UL CA band combinations in rel-15," LG Electronics France R4-1801653, "Introduction of additional xDL/2UL CA band combinations in rel-15," LG Electronics France R4-1802134, "TP to TR 36.715-00-02 V0.3.0: CA_3DL_1A-3A-20A_2UL_1A-3A_BCS0 and 2UL_1A-20A_BCS0 and 2UL_3A-20A_BCS0," Nokia | 0.3.0 | 0.4.0 |
| 2018-05 | 3GPP RAN4 #87 | R4-180xxxx |  |  | TR update: TR36.715-00-02 for xDL_2ULs CA_v0.4.0," LG Electronics France - Approved TPs in RAN4 #86BIS R4-1803995, "Self-desense test results for new xDL/2UL CA band combinations in rel-15," LG Electronics France R4-1803996, "Introduction of additional xDL/2UL CA band combinations in rel-15," LG Electronics France R4-1804466, "TP for TR 36.715-00-02 for DC combinations of CA_3DL_1A-42C_2UL_CA_42C_BCS0," KDDI Corporation R4-1804468, "TP for TR 36.715-00-02 for DC combinations of CA_3DL_3A-41A-42A_2UL_3A-42A _BCS0," KDDI Corporation R4-1804469, "TP for TR 36.715-00-02 for DC combinations of CA_3DL_3A-41C_2UL_CA_41C_BCS0," KDDI Corporation R4-1804472, "TP for TR 36.715-00-02 for DC combinations of CA_3DL_3A-42C_2UL_CA_42C_BCS0," KDDI Corporation R4-1805118, "TP for TR 36.715-00-02 CA_3DL_3A-7A-28A," Telstra Corporation Limited R4-1805588, "TP for TR 36.715-00-02 CA_4DL_1A-7C-28A," Telstra Corporation Limited R4-1805589, "TP for TR 36.715-00-02 CA_5DL_1A-3A-7C-28A," Telstra Corporation Limited R4-1805591, "TP for TR 36.715-00-02 CA_4DL_1A-3A-7A-28A," Telstra Corporation Limited R4-1805626, "TP for TR 36.715-00-02 CA_3DL_1A-7A-28A," Telstra Corporation Limited | 0.4.0 | 0.5.0 |