3GPP TSG-RAN4 Meeting #87	R4-1806692

# Busan, South Korea, 21 May - 25 May 2018

Source:		Skyworks Solutions Inc., Ericsson

Title:			Updated A-MPR Measurements of sub-PRB Feature for Cat-M1 Devices

Agenda item:		6.20.2 [LTE_eMTC4-Core]

Document for:	Approval

# Introduction

In RAN4 #86bis, way forward [1] summarizes the agreed assumptions to evaluate A-MPR of Cat-M1 devices using sub-PRB feature:

- For sub-GHz bands (i.e. bands <= 1 GHz): -28 dBc carrier leakage and IQ image rejection ratio,

- For bands above 1 GHz, -25 dBc carrier leakage and IQ image rejection ratio.

In RAN1 #92bis, the final details of 2SC sub-PRB modulation have been agreed to.

In this paper, measurements performed on a power amplifier (PA) are used to update previous A-MPR results [3] based on above simulation assumptions and using 2SC waveform definition.

The updated A-MPR NS values are NS04, NS06 and NS12. MPR results remain unchanged, therefore no MPR is required.

# Discussion

## Measurement Setup

### Measurement Conditions

The input signal used for PA measurements are aligned with [1]  to meet the following RF transceiver impairments:

- IQ image rejection: -25 dBc and – 28 dBc for bands above 1GHz and below 1 GHz respectively,

- LO leakage: -25 dBc and – 28 dBc for bands above 1GHz and below 1 GHz respectively,

- Counter-IM3: not used.

Measurements are performed at carrier frequencies of 782.5 MHz and 1732.5 MHz for low (<1 GHz) and mid-band (>1 GHz) respectively. Measurements focus on meeting SEM mask and band specific requirements associated with each tested NS value for power class 3 (PC3) devices.

The measured device is an LTE Cat0 IoT module which includes the PA, low pass filter and transmit/receive antenna switch. The output of the module is thus representative of the performance at the user equipment (UE) antenna connector. The PA has a fully programmable bias current which facilitates calibration of PA linearity. The PA is calibrated for PC3 using a fully allocated QPSK 1.4 MHz transmission bandwidth (BW) carrier so that ACLR compliance is met with 1 dB back-off.

## Tested Waveform

Simulation in [2] showed that the worst case are the Pi/2 BPSK modulated of sub-carrier length (Lcsc) 2, and QPSK Lcsc 3 waveforms, with the SC being positioned at the lowest possible offset with the lowest PRB of a given cell BW. In [3] measurements using QPSK Lcsc 3 have been presented with FFS to study the impact of Pi/2 BPSK Lcsc 2 waveforms. This contribution aims at completing previous measurements of [3] by testing Pi/2 BPSK Lcsc 2 waveforms with SC located at offset 0. Since 2SC waveforms exhibit the highest power spectral density (PSD), it is believed that these measurements combined with the previous QPSK Lcsc 3 measurements [3] cover the worst case scenario from a spurious emission perspective.

### MPR Results

Figure 1 below shows the measured PSD versus LTE general spectrum emission mask (SEM) for PC3 CAT-M1. Measurements show that no MPR is needed for PC3 CAT-M1 devices for 1.4 MHz channel bandwidth and Lcsc=2. In [3] measurements showed that no MPR was required with Lcsc=3 and Lcsc=6 waveforms. Therefore, no MPR is required for Cat-M1 devices using sub-PRB feature. Also note that on Figure 1, E-UTRA ACLR exhibit 10 dB margin. This margin could be exploited by adequate PA biasing techniques to minimize power consumption, hence sub-PRB feature may improve UE battery life.

Figure 1: LTE general SEM measurements

Figure 1: Measured power spectral density (PSD) for CAT-M1 measured at 0dB MPR, transmission bandwidth configuration of 1.4 MHz, Lcsc=2 with 0 SC offset. Black: measured PA output spectrum, Green: LTE general SEM.

Proposal 1: No MPR is needed for 1.4MHz cell bandwidth Cat-M1 devices for sub-PRB feature.

Observation 1: ACLR margin makes sub-PRB feature a candidate that may reduce CAT-M1 UE power consumption.

### CAT-M1 A-MPR results

Power back-off is evaluated relative to 23 dBm output power for NS_04, NS_06, and NS_12.

NS_06: Figure 2 shows measured spectrum PSD versus NS_06 additional SEM requirement. Measurements restricted to sub-PRB allocation Lcsc 2 indicate there is no A-MPR required for NS_06. This result confirms previous simulated [2] and partial measurement data [3].

Figure 2: NS06 measurement

Figure 2: PC3 CAT-M1 PSD versus NS_06 requirements at 0 dB MPR, Lcsc=2, “0” SC offset. Black: measured PA output spectrum, Green: additional SEM.

NS_12: Figure-3 shows measured spectrum PSD versus NS_12 additional SEM. Measurements indicate 3.5 dB A-MPR is required for NS_12. A-MPR is limited by NS_12 additional spurious emission requirements of -42 dBm/6.25 kHz.

Figure 3: NS12 measurements

Figure 3: Measured power spectral density (PSD) for CAT-M1 versus NS_12 additional SEM measured at 0 dB A-MPR (left) and 3.5 dB A-MPR (right), with UE transmission bandwidth configuration of 1.4 MHz, Lcsc 2 with 0 SC offset. Black: measured PA output spectrum, Green: SEM.

NS_04: Figure-4 shows measured spectrum PSD versus NS_04 additional SEM. Measurements indicate 1.5 dB A-MPR is required for NS_04. A-MPR is limited by NS_12 additional spurious emission requirements of - 25dBm/1 MHz.

Figure 4: NS04 measurements

Figure 4: Measured power spectral density (PSD) for CAT-M1 versus NS_04 additional SEM at 0dB A-MPR (left) and 1.5dB A-MPR (right), with UE transmission bandwidth configuration of 1.4 MHz, Lcsc 2 with 0 SC offset Black: measured PA output spectrum, Green: SEM.

Observation 2: For 1.4MHz cell-bandwidth, measurements show that no A-MPR is required for NS_06. A-MPR of 3.5 and 1.5 dB is required for NS_12 and NS_04 respectively for Lcsc=2, Pi/2 BPSK “0” SC offset waveforms.

# Conclusions

MPR: Measurements with Lcsc=2, Pi/2 BPSK “0” SC offset waveform confirm previous measurement data [3] with QPSK Lcsc=3 and Lcsc=6 : no MPR is required for 1.4MHz cell bandwidth CAT-M1 devices for sub-PRB feature. This observation is  aligned with simulation data [2].

Partial A-MPR measurements are presented for NS_04, NS_06 and NS_12 for CAT-M1.

Proposal 1: No MPR is needed for 1.4MHz cell bandwidth Cat-M1 devices for sub-PRB feature.

Observation 1: For 1.4MHz cell bandwidth, ACLR margin makes sub-PRB feature a candidate that may reduce CAT-M1 UE power consumption.

Observation 2: For 1.4MHz cell-bandwidth, measurements show that no A-MPR is required for NS_06. A-MPR of 3.5 and 1.5 dB is required for NS_12 and NS_04 respectively for Lcsc=2, Pi/2 BPSK “0” SC offset waveforms.

# References

- R4-1805140, WF on UE RF for sub-PRB R15 feature, Ericsson, Skyworks Solutions, Inc., Qualcomm

- R4-1805138, sub-PRB feature impact on MPR_A-MPR of CAT-M1 device, Ericsson

- R4-1805602, MPR_A-MPR measurements of sub-PRB feature for CAT-M1 devices, Skyworks Solutions, Inc.